TSG-RAN Working Group 4 (Radio) meeting #112	R4-2413219

Maastricht, EU, 19th to 23rd August 2024

Source:	Ericsson

Title:	Introduction of OTA spatial emission above the horizon requirement for BS operating in band n104

Agenda item:	8.11.2

Document for:	Approval

# Introduction

At the RAN plenary meeting (RAN#103 in Maastricht) it was decided [1] to start a Work Item (WI) for BS RF requirement evolution (NR_BS_RF_req_evo). Part of the WI objectives is work related to the introduction of a new requirement for AAS BS to protect FSS UL service for band n104. At the last RAN4 meeting (RAN4#111 in Fukuoka) progress related to creation of the BS RF core specification was made and discussion related to conformance testing started. All agreements from last meeting are captured in a way forward contribution [2] for guidance for this meeting and future meeting.

The proposal to develop a new requirement was part of the BS RF Rel-19 package [3]. The background for this new requirement comes from the WRC-23 final act [4] where a new condition was adopted for IMT base stations to protect FSS UL sharing spectrum in a co-channel situation.

At previous meetings we have provided contributions [5, 6] with information with the intention to stimulate the discussion to further progress the work to define BS RF core requirement and corresponding BS RF conformance test requirement and test method.  In this contribution we present new information on topics to be resolved to be able to capture the technical condition from ITU-R as a new requirement in the BS RF core specification TS 38.104. Companion contributions  are also provided: a discussion paper on conformance testing aspects in [7], a draft CR with specification text for TS 38.104 in [8] and text proposal with technical background information to TR 38.xyz in [9].

In this contribution we propose a requirement definition to be captured in TS 38.104.

# Discussion

  - Requirement definition

In ITU-R a new condition for the frequency range 6425 to 7075 MHz was adopted at WRC-23 for protection of FSS UL during co-channel spectrum sharing situations. Table 2.1-1. ITU has indicated that EEIRP limit is defined as the average of the average EIRP radiated above the horizon over horizontal angles between -180o to 180o, within specified vertical elevation ranges.

Table 2.1-1: EEIRP limits as function of elevation angular range

| Elevation angular ranges θLi≤θ<θHi (Degrees) | EEIRP (dBm/MHz) |
| --- | --- |
| 0<q<5 | 27 |
| 5<q<10 | 23 |
| 10<q<15 | 19 |
| 15<q<20 | 18 |
| 20<q<30 | 16 |
| 30<q<60 | 15 |
| 60<q<90 | 15 |

Table 2.1-1 describes the limit, constituted by EEIRPi values creating a profile covering i elevation angular regions.

The EEIRP can be defined as:

EEIRP: average equivalent isotropic radiated power defined for specified azimuth and elevation ranges

The EEIRP limits are defined with respect to the elevation angle, where θ equal to 0 degrees refers to the angle towards the horizon and 90 degrees refers to an angle straight up into the sky. These limits will directly constrain the power generated above the horizon, towards the sky, and will restrict the AAS BS implementation with respect to maximum power (EIRP) towards UEs within the angular coverage range, antenna array geometry and coverage range capability.

For each range of elevation angles EEIRP is defined as upper hemi-sphere two dimensional average EIRP :

EEIRPi=12πsinθHi-sinθLiθLiθHi-ππEIRPθ,φcosθdθdφ

, where EIRPθ,φ  denotes the average 3-D EIRP spectral density pattern, averaged over K EIRP patterns, and θLi≤θ<θHi  is the elevation angular range, i is the index of the vertical bins θ is the elevation angle and φ is the horizontal angle. Angles in the equation, are expressed in radians.

NOTE: Relation to TRP is visible in the case where only one vertical range is considered (where the elevation angle range is defined as θL1 is equal to -π2 and θH1 is equal to π2). In this case EEIRP is equal to TRP.

Figure 2.1-1: Definitions of θL and θH angles relative to the horizontal plane, assumed perpendicular to the antenna array surface (BS placed in the origin).

In TS 28.141-2 clause 4.14 there is a definition of reference coordinate system used for the present requirements. For this requirement, RAN4 may need to define the elevation angle according to conventions used in ITU-R.

.

# Conclusion

Based on the information in this contribution we have observed following:

Observation 1: EEIRP definition tends to become specific to this requirement, as it needs to contain all the information about how this parameter is calculated using averaging over azimuth and elevation, as described in the formula above.

Proposal 1: RAN4 investigate the possibility to adopt a more general definition of the term “EEIRP” that can apply to a larger range of different requirements that may pop-up later. In such case use of term EEIRP must be accompanied by the ranges for azimuth and elevation angles to be considered.

We should not forget that EIRP and EIRP  (and also TRP) are ways to express radiated power and can be used for both wanted and unwanted signals, in different BS RF requirements.

# References

[1]	RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE

[2]	R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE

[3]	R4-2402475, “NR BS RF enhancements in Rel-19”, Ericsson

[4] 	“WRC-23 Final Acts”, ITU-R

[5]	R4-2404757, “On introduction of protection requirement for FSS UL service operating within 6425 to 7125 MHz”, Ericsson

[6]	R4-2409118, “On introduction of OTA spatial emission above the horizon requirement for band n104”, Ericsson

[7]	R-xxxx, “Conformance test aspects for the requirement on OTA spatial emission above the horizon for BS operating in band n104” Ericsson

[8]	R4-xxyyzzz, “Draft CR to TS 38.104: Addition of spatial emission requirement to protect FSS UL within band n104 in subclause 9.9”, Ericsson

[9]	R4-xxyyzzz, “Draft CR to TR 38.xyz: Technical background information for FSS UL protection requirement for band n104”, Ericsson