TSG-RAN Working Group 4 (Radio) meeting #112	R4-2413220

Maastricht, EU, 19th to 23rd August 2024

Source:	Ericsson

Title:	Conformance test aspects for the requirement on OTA spatial emission above the horizon for BS operating in band n104

Agenda item:	8.11.4

Document for:	Approval

# Introduction

In this contribution we present information relevant for BS RF conformance test specification TS 38.141-2. Companion contributions are also provided, a discussion paper on core aspects in [1] draft CR with specification text for TS 38.104  [2] and text proposal with technical background information to TR 38.xyz in [3].

# Discussion

  - Conformance test aspects

    - BS RF test approach

For AAS BS conformance testing the concept including a set of test beams was proposed by ITU-R, where the test beams are supposed to be spread out within the intended coverage range.

Observation 1: When the test requirement and test procedure are defined in RAN4,  usage of the term “horizon” should be avoided. For a BS. the relation to horizon depends on deployment parameters such as mechanical tilt and other site-specific details not part of the BS RF conformance testing specification.

In the BS RF conformance test specification (TS 38.141-2) RAN4 have adopted an approach to use a baseband with limited functionality based on pre-defined test signals (aka. test models). The beamforming weight vectors are proprietary and determined by manufacturer declarations of beamforming direction. Hence, no support for testing the BS during true normal operation in a controlled fading environment is possible with the current approach adopted by RAN4.

During the test the AAS base station is configured with a test baseband functionality that will use the test models and apply beam forming weight vector, as shown in Figure 2.2.1-1. With the beamforming weight vector test beams will be generated according to requirement definitions in TS 38.141-2.

The test is conducted in a Line-of-Sight (LOS) environment without feedback from the measurement receiver (UE). Unlike the UE conformance testing, the BS Down-link and Up-link are tested separately with no dependencies.

# Figure 2.2.1-1: Down-link test object configuration

The base station manufacturer declares the angular range intended for normal operation. This declaration will depend on intended deployment scenario. This would mean that if a specific base station implementation is intended for multiple deployment scenarios, e.g., rural, sub-urban and urban multiple declarations may be needed.

The concept of static test beams allows for conformance testing in conventional antenna test ranges using standard test equipment (network analyser, signal generator, spectrum analyser, etc.).

Currently, the specification does not describe how beam-forming weights associated to test beams are applied to the test signal (test model) during test. Based on declared beam-peak direction, beam-forming weights are derived and applied to the test signal.

Observation 2: With the introduction of OTA spatial emission requirement RAN4 may need to consider how beamforming weights should be applied to the test model to guarantee a relevant test condition.

    - Test concept

The conformance test required to show compliance to the EEIRP limit needs to be developed by RAN4. To show conformance to the EEIRP limits a large number of measured 3-D EIRP patterns for different array excitations will be needed. Since the RAN4 conformance testing is built around pre-defined test signals (test models and fixed refence channels) normal operation in fading environment capturing true layer-1 operation cannot be captured by the conformance test in RAN4 for AAS base stations.

The conformance test for EEIRP limits can be created around the concept of using test beams within a declared coverage angular range (CAR). The CAR is the angular range along vertical and horizontal axis for which the base station is intended to provide coverage within.

In the terminology section (clause 3 of TS 38.141-2), a definition of CAR is needed. For the purpose of EEIRP requirement the CAR can be defined as:

Coverage Angular Range: The coverage angular range is a set of directions describing the intended angular operation range

For each test beam, the 3-D EIRP pattern in the upper hemisphere is measured. Based on measured EIRP patterns the EEIRP profile can be calculated as the output from a multistage averaging process.

Multiple test beams with different directions are required to establish the average EIRP pattern corresponding to intended operation. For this purpose, K different test beams with different beam directions, distributed within the declared CAR for which the base station is intended to operate within, are required.

The intention with the test beams is to excite the array antenna in such a way that the array side-lobe suppression characteristics are captured. For an array antenna with sparse element separation in the vertical domain using sub-arrays, side lobe response and eventual grating lobe response will be captured.

Multiple CARs can be declared for a base station supporting multiple deployment scenarios to map towards different scenarios with different UE distributions. Eventually, for each declared CAR, different beam power profiles are utilized to optimize coverage while meeting the EEIRP profile above the horizon.

For each test beam, the BS manufacturer declares the peak EIRP and direction as q and j angle pair.

For a solid test requirement, the details related to placement of the beam directions and weighting need to be captured in the specification. The location and distribution of beam directions will affect the averaging process significantly. RAN4 needs to define how many beam directions will be needed and how to distribute them.

Observation 3: Since the conformance test concept adopted in RAN4 currently does not capture operation in fading environment, manufacturer declarations will be needed to derive the angular coverage range. This means that the declaration and deployment scenarios are tied together.

The CAR is declared by the base station manufacturer using the angles listed in Table 2.2.2-1.

Table 2.2.2-1: CAR declarations

| Parameter | Description |
| --- | --- |
| qmin | Minimum angle along q axis |
| qmax | Maximum angle along q axis |
| jmin | Minimum angle along j axis |
| jmax | Maximum angle along j axis |

The CAR can be visualized as a rectangular area in the q/j plane, as shown in Figure 2.2.2-1 as a green rectangle.

For a typical wide area base station intended for 3-sector rural deployments values listed in Table 2.2.2-2 is reasonable.

Table 2.2.2-2: Typical values

| Parameter | Values (degrees) |
| --- | --- |
| qmin | 0 |
| qmax | 12 |
| jmin | -60 |
| jmax | 60 |

For other deployment scenarios (e.g., for wide area base station operating in an urban deployment scenario) other values may be declared.

Figure 2.2.2-1: Coverage angular range

The test beams will be located within the declared CAR. The number of test beams K and the location within the q/j plane will impact the measured EEIRP profile. In the background information relevant for the condition ITU-R stipulates that the beam directions should be distributed uniformly within the CAR. To adjust for beams located at the edge of the CAR a beam weight has been introduced.

The average EIRP pattern is calculated from K EIRP patterns as:

EIRPθ,φ=k=1KwkEIRPkθ,φ

, where wk is a beam weight and EIRPkθ,φ is the EIRP pattern per measured test beam.

Beam weight: fraction of the coverage angular range represented by the test beam.

For example, wk=1/K   represents the case where K uniformly and equally spaced test beams are assumed in azimuth and elevation, respectively, and where each beam covers an equal range of angles (e.g., equal area in q/j plane). The weight is required to account for test beam coverage for test beams located on the edge of the CAR, non-uniform distribution or allowing for fewer test beams to be considered as shown in the following examples.

Eventually, the maximum peak EIRP difference between declared beam needs to be limited to a certain value (e.g., 10 dB). Without a cap on the maximum difference, fake beams can be declared during testing that will produce incorrect measured EEIRP levels.

Observation 4: RAN4 need to define test beam set(s) that guarantees acceptable measurement uncertainty.

Proposal 1: RAN4 should define a concept where the test beam directions are related to the declared Coverage Angular Range (CAR) and the number of test beam directions are specified.

    - Conformance test beam set

The beams selected for conformance testing will have direct impact on the robustness of measuring the emission generated above the horizon. To be able to measure the EEIRP, the test object needs to be stimulated to generate beams within the intended coverage angular range. For this purpose, RAN4 need to establish a set of test beams to be used during conformance testing.

In this section we evaluate the EEIRP profiles for different test beam sets for a given array antenna geometry representative of a product in the band and described in Table 2.2.3-1. It should be noticed that these parameters are not selected with the intention to meet the EEIRP requirement profile. As the result will show, the specified EEIRP levels will not meet the ITU-R condition, which would indicate that additional mitigation techniques are needed. The intention with the parameters below is to evaluate the EEIRP characteristics due to different test beam sets.

Table 2.2.3-1: Antenna array assumptions

| Parameter | Value |
| --- | --- |
| Frequency | 6775 MHz |
| Element model | Vertical HPBW: 90° Horizontal HPBW: 90° Sidelobe suppression: 30dB Element directivity: 6.4 dBi |
| Array configuration | Number of rows: 8 Number of columns: 16 Sub-array: 3x1 Sub-array tilt: 6° Horizontal element separation: 0.5λ Vertical element separation: 0.7λ |
| TRP | 27 dBm/MHz |
| Peak EIRP target | 79 dBm/100 MHz |

To evaluate the impact of the number of beams, distribution and beam weights, several beam sets have been defined (Reference, A, B, C, D, E), considering different sizes of the test beam sets, distribution of test beams in the declared CAR, and weights. The CAR is set to -60 to 60 degrees in the horizontal plane and to 0 to 12 degrees (down-tilt) in the vertical plane.

For reference purposes a reference beam set of 325 uniformly distributed beams (25 x 13 equally spaced beams in horizontal and vertical plane, respectively) with uniform beam weight has been defined. This beam set is referred to as “Reference” in figures.

Uniform distribution

We first consider a uniform distribution of beams in the declared CAR. Two cases have been considered, Case A with different beam weights considering beams at the edge of the CAR and Case B, where non weighted average is considered.

Specifically, Beam set A and B are defined by 25 test beams distributed in a uniform lattice with 5 beams along the horizonal axis and 5 beams along the vertical axis, according to Figure 2.2.3-1.

Figure 2.2.3-1: Beam distribution for Beam set cases A and B

For beam set A, the beam pointing direction and corresponding beam weight for all beams are listed in Table 2.2.3-2. The beam directions are weighted to reflect the area that each beam covers within the CAR box.

Table 2.2.3-2: Beam weights for Beam set A

| Test Beam [k] | θ° | φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/32 |
| 3 | 6 | 60 | 1/32 |
| 4 | 6 | -30 | 1/16 |
| 5 | 6 | 30 | 1/16 |
| 6 | 3 | 0 | 1/16 |
| 7 | 9 | 0 | 1/16 |
| 8 | 0 | 0 | 1/32 |
| 9 | 12 | 0 | 1/32 |
| 10 | 0 | -60 | 1/64 |
| 11 | 0 | -30 | 1/32 |
| 12 | 3 | -60 | 1/32 |
| 13 | 3 | -30 | 1/16 |
| 14 | 0 | 30 | 1/32 |
| 15 | 0 | 60 | 1/64 |
| 16 | 3 | 30 | 1/16 |
| 17 | 3 | 60 | 1/32 |
| 18 | 9 | -60 | 1/32 |
| 19 | 9 | -30 | 1/16 |
| 20 | 12 | -60 | 1/64 |
| 21 | 12 | -30 | 1/32 |
| 22 | 9 | 30 | 1/16 |
| 23 | 9 | 60 | 1/32 |
| 24 | 12 | 30 | 1/32 |
| 25 | 12 | 60 | 1/64 |

For Beam set B, uniform beam weighting is assumed. The reason for this is to compare the impact of applying additional beam weighting to the resulted EEIRP profile. The beam directions and beam weight vector are listed in Table 2.2.3-3.

Table 2.2.3-3: Beam weights for Beam set I.B

| Test Beam [k] | θ° | φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/25 |
| 2 | 6 | -60 | 1/25 |
| 3 | 6 | 60 | 1/25 |
| 4 | 6 | -30 | 1/25 |
| 5 | 6 | 30 | 1/25 |
| 6 | 3 | 0 | 1/25 |
| 7 | 9 | 0 | 1/25 |
| 8 | 0 | 0 | 1/25 |
| 9 | 12 | 0 | 1/25 |
| 10 | 0 | -60 | 1/25 |
| 11 | 0 | -30 | 1/25 |
| 12 | 3 | -60 | 1/25 |
| 13 | 3 | -30 | 1/25 |
| 14 | 0 | 30 | 1/25 |
| 15 | 0 | 60 | 1/25 |
| 16 | 3 | 30 | 1/25 |
| 17 | 3 | 60 | 1/25 |
| 18 | 9 | -60 | 1/25 |
| 19 | 9 | -30 | 1/25 |
| 20 | 12 | -60 | 1/25 |
| 21 | 12 | -30 | 1/25 |
| 22 | 9 | 30 | 1/25 |
| 23 | 9 | 60 | 1/25 |
| 24 | 12 | 30 | 1/25 |
| 25 | 12 | 60 | 1/25 |

Reduced number of test beams

The impact on the EEIRP profile for the case where test beam directions are reduced have been evaluated by defining test beam directions according to Figure 2.2.3-2. Now the test beam directions are aligned with the q-axis and j-axis.

Figure 2.2.3-2: Test beam set II, to reduce number of test beam directions

For this beam set, three different variants of beam weight vectors can be evaluated. Therefore, three different beam sets have been defined C, D and E. The beam directions and beam weight vectors are listed in Table 2.2.3-4, Table 2.2.3-5 and Table 2.2.3-6.

Table 2.2.3-4: Beam weights for beam set C

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/32 |
| 3 | 6 | 60 | 1/32 |
| 4 | 6 | -30 | 1/16 |
| 5 | 6 | 30 | 1/16 |
| 6 | 3 | 0 | 1/4 |
| 7 | 9 | 0 | 1/4 |
| 8 | 0 | 0 | 1/8 |
| 9 | 12 | 0 | 1/8 |

Table 2.2.3-5: Beam weights for beam set D (uniform)

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/9 |
| 2 | 6 | -60 | 1/9 |
| 3 | 6 | 60 | 1/9 |
| 4 | 6 | -30 | 1/9 |
| 5 | 6 | 30 | 1/9 |
| 6 | 3 | 0 | 1/9 |
| 7 | 9 | 0 | 1/9 |
| 8 | 0 | 0 | 1/9 |
| 9 | 12 | 0 | 1/9 |

Table 2.2.3-6: Beam weights for beam set E

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/8 |
| 3 | 6 | 60 | 1/8 |
| 4 | 6 | -30 | 1/4 |
| 5 | 6 | 30 | 1/4 |
| 6 | 3 | 0 | 1/16 |
| 7 | 9 | 0 | 1/16 |
| 8 | 0 | 0 | 1/32 |
| 9 | 12 | 0 | 1/32 |

Another alternative considers that the beam weighting is set to adjust for beam directions located on the edge of the CAR and to adjust for vertical angles having only one beam for all horizontal angles. In theory, this beam configuration may allow for an alternative, but incorrect weight values (Case E) as shown in Table 2.2.3-6. With the alternative beam weight , beams with horizontal steering are applied with a significant suppression. Hence the impact of using sub-arrays in the vertical domain will be suppressed, since beam causing grating lobes will be suppressed in the averaging process.

Test beam distribution evaluation

The number of test beams, the distribution within the declared CAR and the weighting, will impact the measured EEIRP level. Therefore, the EEIRP produced by a base station assuming a baseline antenna configuration, already introduced in Table 2.2.3-1, have been evaluated.

In the evaluation 5 different test beam configurations have been evaluated, as listed in Table 2.2.3-7, and compared to the reference beam set and the WRC-23 requirement.

Table 2.2.3-7: Test beam configurations

| Case | Test beam distribution | Beam weight | Detailed description |
| --- | --- | --- | --- |
| A | Uniform | Non-uniform | Table 2.2.3-1 |
| B | Uniform | Uniform | Table 2.2.3-2 |
| C | Non-uniform | Non-uniform | Table 2.2.3-3 |
| D | Non-uniform | Uniform | Table 2.2.3-4 |
| E | Non-uniform | Incorrect Non-uniform | Table 2.2.3-5 |

The EEIRP profiles corresponding to Beam set A, B, C, D and E are plotted in Figure 2.2.3-3.

Figure 2.2.3-3: EEIRP profile for Cases A, B, C, D and E (sub-array size 3x1)

It can be observed that the EEIRP profiles align well with the reference case for beam sets A and B, and quickly converge to it. On the other hand, for the reduced set of beams, beam sets cases C, D and E, deviate from the reference case and from beam sets A and B. As a result, we consider that a test beam-set of 25 beams can represent a good trad-off between accuracy and testing complexity. It can also be noticed that the WRC-23 level is violated for some angular ranges.

It can be noticed that case E significantly deviates from the reference case, which indicates that case E should not be used for conformance testing.

These results, which were already discussed in previous meeting, demonstrate that the test beams should be carefully selected and should be distributed within the whole coverage angular range, not only along the axis. To guarantee repeatability and acceptable measurement uncertainty, RAN4 need to define one common test beam set, taking into account the trade-off between accuracy and testability.

Another aspect to consider in relation to test beam sets is the impact on the EEIRP profile depending on considered array antenna structure, more specifically the size of the sub-array. In what follows we repeat the experiments considering a bigger sub-array size. Specifically, we consider sub-array size of 6x1 and compare it to the previously studied case of sub-array size 3x1.

From the reference beam set, with 325 (25 x 13 equally spaced beams in horizontal and vertical plane, respectively) uniformly distributed beams with uniform beam weights, we already observe a slightly different behaviour as a function of the sub-array size in Figure 2.2.3-4. Specifically, with larger sub-array size the EEIRP exceeds slightly more the requirement, compared to sub-array size 3x1, in certain directions.

Figure 2.2.3-4: EEIRP profile for the reference beam set with 3x1 and 6x1 sub-array size

Similar trend is observed also for other beam sets as described in Figure 2.2.3-5, Figure 2.2.3-6 and Figure 2.2.3-7. It can also be noticed that the same observations done for the sub-array size 3x1 are valid for sub-array size 6x1, in terms of better accuracy and convergence to the reference when considering beam set A and beam set B, with respect to the other cases studied for beam-set C, D and E.

Summary of EEIRP profiles for cases A, B, C, D and E, for sub-array size 6x1 is shown in Figure 2.2.3-8.

Figure 2.2.3-5: EEIRP profile for the beam sets A and B, with 3x1 and 6x1 sub-array size

Figure 2.2.3-6: EEIRP profile for the beam sets C and D, with 3x1 and 6x1 sub-array size

Figure 2.2.3-7: EEIRP profile for the beam set E, with 3x1 and 6x1 sub-array size

Figure 2.2.3-8: EEIRP profile for Cases A, B, C, D and E (sub-array size 6x1)

Observation 5: It can be concluded that the test beams should be carefully selected and should be distributed within the whole coverage angular range, not only along the axis. To guarantee repeatability and acceptable measurement uncertainty, RAN4 need to define one common test beam set.

Observation 6: The impact of the beam weight has significant impact on the measured EEIRP profile.

Observation 7: The trend observed in simulation results, in terms of better accuracy provided by beam sets with sufficient number of sample beams and good distribution, are confirmed also when considering different sub-array sizes.

Proposal 2: RAN4 need to develop a concept where the test beams are distributed within the whole coverage angular, range with a specified beam weight vector.

Observation 8: Comparing the EEIRP result from Beam set A and Beam set B it can be noticed that they produce similar EEIRP profiles, so that case B might be preferred to case A since a simple uniform beamforming weight is used.

Is seems that beam set B provides enough samples for the average to converge and an easy approach to describe how test beams are located within the CAR. From a specification drafting principle it would be good to define beam set B in the conformance test specification including description on how the beam are located within the CAR. For that purpose, equations are required.

Proposal 3: Based on the analysis in this contribution we propose to specify the minimum number if beams used within the CAR to 25 uniformly distributed with wk equal to 1/K and location on test beams in the BS RF conformance specification (referred in this contribution to as beam set B).

    - Description of test method in specification

The information in this section is captured in TS 38.141-2 for each requirement. It gives detailed information about test object configuration and test scope.

      - Initial conditions

In this section information related to initial conditions, test object configuration and test object placement is captured.

The test environment: Normal condition, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: T; see TS 38.141-2, subclause 4.9.1. It is not necessary to test Bottom, Middle and Top frequency. Top frequency is enough since the highest supported frequency sets strictest design constraints.

Base station RF bandwidth positions to be tested for multi-carrier and/or carrier aggregation (CA):

- BRFBW, MRFBW, TRFBW in single band operation; see TS 38.141-2, subclause 4.9.1.

- BRFBW_T’RFBW and B’RFBW_TRFBW in multi-band operation; see TS 38.141-2, subclause 4.9.1.

The test object is located on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is capturing power transmitted towards the sky, it is essential that the BS is mounted on the positioner such that blocking effects due to the positioner is minimized.

NOTE: For regular antenna pattern measurements typically, the focus is on the horizontal angle and then we often see blocking effects in along vertical angle.

For this requirement the same test model as for radiated transmit power should be used.

      - Procedure

The following high level test procedure describes essential steps required to measure the EEIRP profile.

- Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

- Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model described in TS 38.141-2, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1...

- Enable test beam  k for k=1..K.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 5 and 6 for all K test beams.

- Repeat step 4, 5, 6 and 7 for all M and N positioner locations.

- Calculate EEIRPi 〖EEIRP〗_i profile from measured EIRPkθn,φm  patterns as described in section 2.2.5.3.

Observation 9: A test method for EEIRP profile would require EIRP samples to be measured for many test beam directions. Therefore, it is essential to capture a description in the conformance test specification of how EIRP samples are collected for different test beam directions.

Proposal 4: RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission.

    - EIRP pattern measurement

Traditionally, the measurement can be divided into two different stages: Test range calibration and measurement. For an expected EIRP measurement a third stage is introduced.

- Test range calibration

- EIRP pattern measurement

- Post processing

The post-processing stage is required for some test methods, e.g., TRP test methods which are based on spatial EIRP samples. For EEIRP, the post processing stage will be even more complex which would lead to that a new Annex would be required in the conformance test specification for the technical background information.

      - Test range calibration

In the calibration stage the antenna test range transmission loss is characterized with the intention to minimize error sources related to test setup. A common approach used is the substitution method, where a reference antenna with known antenna gain is used to determine the transmission loss in the test range.

In addition, the measurement uncertainty can be improved by calibrating the absolute power measurement uncertainty of the measurement receiver (spectrum analyser with fairly high absolute power measurement uncertainty) using a power meter (with very low measurement uncertainty). A detailed description of suitable test ranges and corresponding calibration procedures can be found in TR 37.941.

Proposal 5: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation.

      - EIRP pattern measurement

For each test beam, the EIRP pattern above the horizon (in the upper hemi-sphere) is measured for horizontal angles within the range -π≤φ≤π and elevation angles within the range 0≤θ≤π2 .

The discrete EIRP pattern EIRPkθn,θm measured per test beam using a specific sampling grid with a specific sampling grid resolution. For the reference test method described in this document a uniform sampling grid is considered. Considering practical measurement aspects, such as test complexity and test time the EIRP pattern per test beam is measured for a set of discrete spatial angles θn,θm, where m=1..M and n=1..N.

The EIRP pattern measurement stage and corresponding details on maximum allowed sampling resolution needs to be documented in an Annex in TS 38.141-2.

Observation 10: For each test beam within the CAR a spatially discrete EIRP pattern is measured in the upper hemi-sphere.

Proposal 6: RAN4 need to describe the EIRP sampling grid in the upper hemi-sphere and define criteria for maximum allowed sampling resolution.

      - Post processing

In technical background information from ITU-R [2] the EEIRP profile is calculated from an integration over continues average EIRP patten. From a testing perspective, the averaging process should be described as an average over discrete spatial samples.

The EEIRP profile can be extracted from measured EIRP spectral density radiation patterns described in section 2.2.5.2 by applying multiple stage of averaging processes.

In the first stage the average EIRP radiation pattern over K test beams are calculated as:

EIRPθn,φm=k=1KwkEIRPkθn,φm

, where EIRPkθn,θm  is the EIRP spectral density radiation pattern measured for the k-th test beam and w_k refers to the weight for the k-th beamforming direction, i.e., the fraction of the steering range represented by the k-th beamforming direction (see detailed description in section 2.3.2).

In the second average stage the EEIRP level for the i-th elevation range is calculated as:

EEIRPi=θHi-θLisinθHi-sinθLiMN∙n=nLinHim=1MEIRPθn,θmcosθn

where N  is the number of sampling points within the θLi and θHibounding range over upper hemisphere in the elevation range, nLiand nHi are the lowest and highest elevation sampling angles within the  (θLi, θHi) bounding range, with  N= nHi-nLi ; M is the number of sampling points in the azimuth range.

The post-processing stage with details description of the averaging calculations should be captured in a dedicated Annex in TS 38.141-2.

Proposal 7: RAN4 need to describe the post-processing calculation in discrete form in the conformance test specification.

    - Measurement uncertainty evaluation

RAN4 needs to evaluate the Measurement Uncertainty (MU) for the reference test method described in this document. The measurement uncertainty for related to a measurement of EEIRP would depend on some new factors never being considered by RAN4 before such as:

- The MU impact due to limited measurement receiver dynamic range.

- The MU impact due to limited number of used test beam directions.

- The MU impact due to selected distribution of test beam directions and usage of the beam weight.

- The MU impact due to the selected EIRP patten spatial sampling resolution.

- The MU impact due to the absolute power measurement uncertainty determined by the calibration process.

- The MU impact due to positioner blocking effects in the vertical domain.

The measurement uncertainty evaluation background will be captured in a new Technical Report (TR).

Due to the complexity and dependence to manufacturer declaration the measurement uncertainty evaluation needs to be performed per vertical angular range over the profile.

Since the requirement on EERIP is a regulatory requirement, the test tolerance defined in RAN4 will not allow for relaxation with respect to the core requirement. Hence, the measurement uncertainty needs to be accounted for in the design. This would mean that a large measurement uncertainty would directly relate to fulfilling a stricter EEIRP level.

Observation 11: RAN4 need to evaluate the measurement uncertainty for EEIRP and capture the outcome in TR 37.941.

# Conclusion

In this contribution we present an overview of the new requirement required to protect FSS UL within band n104 and aspects to consider when the requirement is included in RAN4 BS specifications.

Based on the information in this contribution we have observed following:

Observation 1: When the test requirement and test procedure are defined in RAN4,  usage of the term “horizon” should be avoided. For a BS. the relation to horizon depends on deployment parameters such as mechanical tilt and other site-specific details not part of the BS RF conformance testing specification.

Observation 2: With the introduction of OTA spatial emission requirement RAN4 may need to consider how beamforming weights should be applied to the test model to guarantee a relevant test condition.

Observation 3: Since the conformance test concept adopted in RAN4 currently does not capture operation in fading environment, manufacturer declarations will be needed to derive the angular coverage range. This means that the declaration and deployment scenarios are tied together.

Observation 4: RAN4 need to define test beam set(s) that guarantees acceptable measurement uncertainty.

Observation 5: It can be concluded that the test beams should be carefully selected and should be distributed within the whole coverage angular range, not only along the axis. To guarantee repeatability and acceptable measurement uncertainty, RAN4 need to define one common test beam set.

Observation 6: The impact of the beam weight has significant impact on the measured EEIRP profile.

Observation 7: The trend observed in simulation results, in terms of better accuracy provided by beam sets with sufficient number of sample beams and good distribution, are confirmed also when considering different sub-array sizes.

Observation 8: Comparing the EEIRP result from Beam set A and Beam set B it can be noticed that they produce similar EEIRP profiles, so that Beam set B might be preferred to Beam set A since a simple uniform beamforming weight is used.

Observation 9: A test method for EEIRP profile would require EIRP samples to be measured for many test beam directions. Therefore, it is essential to capture a description in the conformance test specification of how EIRP samples is collected for different test beam directions.

Observation 10: For each test beam within the CAR a spatially discrete EIRP pattern is measured in the upper hemi-sphere.

Observation 11: RAN4 need to evaluate the measurement uncertainty for EEIRP and capture the outcome in TR 37.941.

To further progress the work, we propose following:

Proposal 1: RAN4 should define a concept where the test beam directions are related to the declared Coverage Angular Range (CAR) and the number of test beam directions are specified.

Proposal 2: RAN4 need to develop a concept where the test beams are distributed within the whole coverage angular, range with a specified beam weight vector.

Proposal 3: Based on the analysis in this contribution we propose to specify the minimum number if beams used within the CAR to 25 uniformly distributed with wk equal to 1/K and location on test beams in the BS RF conformance specification (referred in this contribution to as beamset B).

Proposal 4: RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission.

Proposal 5: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation.

Proposal 6: RAN4 need to describe the EIRP sampling grid in the upper hemi-sphere and define criteria for maximum allowed sampling resolution.

Proposal 7: RAN4 need to describe the post-processing calculation in discrete form in the conformance test specification.

# References

[1]	R4-xxxxxx,  “Introduction of OTA spatial emission above the horizon requirement for BS operating in band n104”, Ericsson

[2]	R4-xxyyzzz, “Draft CR to TS 38.104: Addition of spatial emission requirement to protect FSS UL within band n104 in subclause 9.9”, Ericsson

[3]	R4-xxyyzzz, “Draft CR to TR 38.xyz: Technical background information for FSS UL protection requirement for band n104”, Ericsson

# Annex

The is defined with respect to the elevation angle, where θ equal to 0 degrees refers to the angle towards the horizon and 90 degrees refers to an angle straight up into the sky.

The EEIRP for each interval of elevation angles is defined as two dimensional average EIRP :

EEIRPi=12πsinθHi-sinθLiθLiθHi-ππEIRPθ,φcosθdθdφ

, where EIRPθ,φ denotes the average 3-D EIRP spectral density pattern, averaged over K EIRP patterns, and θLi≤θ<θHi  is the elevation angular range, i is the index of elevation range, θ is the elevation angle and φ is the horizontal angle.

The average EIRP pattern is calculated from K EIRP patterns as:

EIRPθ,φ=k=1KwkEIRPkθ,φ

, where wk is a beam weighting and EIRPkθ,φ is the EIRP pattern per measured test beam.

To get the EEIRP discrete form, we assume M points in azimuth and N in elevation, equally spaced, so that:

dφ=2πM

dθ=θHi-θLiN

EEIRPiθn,φm=2πθHi-θLi2πsinθHi-sinθLiMNnLinHim=1MEIRPθn,φmcosθn

EEIRPiθn,φm=θHi-θLisinθHi-sinθLiMNnLinHim=1MEIRPθn,φmcosθn