3GPP TSG-RAN Working Group 4 (Radio) Meeting #112	R4-2413221

# Maastricht, EU, 19th - 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.xyz | CR | <CR#> | rev | - | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.xyz: Technical background information for FSS UL protection requirement for band n104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture technical background information relevant for new the OTA spatial emission requirement for BS operating in band n104, introduced to protect FSS UL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this draft CR information for several subclauses is added to capture essential parts of the technical background relevant for the development of the OTA spatial requirement. Here more detailed information can be found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the TR will not capture essential information. It is essential for BS vendors, operators and administrations to understand the technical background and how the requirement was derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (TR skeleton not available yet) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Clause 2, References:

[1]	Resolution 220, “WRC-23 Final Acts”

Background:

As an outcome of ITU-R sharing studies protection of FSS UL will be required when IMT networks are introduced in the frequency range 6425 to 7125 MHz. A new condition based on Expected EIRP above the horizon has been adopted by ITU-R [1].

The new condition will have impact on the design of the AAS BS with respect to baseband algorithms via the steering capability, radio hardware via the transmitter power capability and antenna hardware via the array geometry. Therefore, it would be appropriate to develop and standardize a BS RF core requirement and corresponding conformance test requirements, including test methods for AAS BS.

The intention with the requirement is to set a limit on the aggregated power at the victim receiver at the satellite from IMT networks on the ground. The requirement will be defined per base station on power radiated above the horizon.

When it is introduced in 3GPP RAN4 NR BS specifications it will be handled as a regulatory BS RF requirement applicable for AAS BS supporting band n104.

The NR BS RF specifications to be impacted of this requirement are:

-	NR; Base Station (BS) radio transmission and reception, TS 38.104

-	NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing, TS 38.141-2

In the ITU-R discussion ideas related to how to measure expected EIRP mask to show conformance to the specified mask emerged. A test concept with a set of test beams was proposed in an effort to mimic normal operation in a RAN4 test environment.

BS RF core aspects:

In ITU-R a new condition for the frequency range 6425 to 7125 MHz was adopted at WRC-23 for protection of FSS UL during co-channel spectrum sharing situations. The new condition is defined as power mask based on a specific elevation profile referred to as expected EIRP (EEIRP) spectral density. The maximum allowed EEIRP for a BS as a function of the vertical angle above the horizon (aka. the elevation angle, defined as the vertical angle from the horizon to a certain angle above the horizon) shall not exceed the limits listed in Table X-1.

Table X-1: EEIRP limits as function of elevation angular range

| Elevation angular ranges θLi≤θ<θHi (Degrees) | EEIRP (dBm/MHz) |
| --- | --- |
| 0<q<5 | 27 |
| 5<q<10 | 23 |
| 10<q<15 | 19 |
| 15<q<20 | 18 |
| 20<q<30 | 16 |
| 30<q<60 | 15 |
| 60<q<90 | 15 |

The requirement will be introduced in a new subclause 9.9 in TS 38.104 in the transmitter section.

Proposed definitions:

EEIRP: average equivalent isotropic radiated power defined for specified azimuth and elevation ranges

Coverage Angular Range: set of directions describing the intended angular operation range

The EEIRP limits are defined with respect to the elevation angle, where θ equal to 0 degrees refers to the angle towards the horizon and 90 degrees refers to an angle straight up into the sky. These limits will directly constrain the power generated above the horizon, towards the sky, and will restrict the AAS BS implementation with respect to maximum power (EIRP) towards UEs within the angular coverage range, antenna array geometry and coverage range capability.

For each range of elevation angles EEIRP is defined as upper hemi-sphere two dimensional average EIRP:

EEIRPi=12πsinθHi-sinθLiθLiθHi-ππEIRPθ,φcosθdθdφ

, where EIRPθ,φ  denotes the average 3-D EIRP spectral density pattern, averaged over K EIRP patterns, and θLi≤θ<θHi  is the elevation angular range, i is the index of the vertical bins, θ is the elevation angle and φ is the horizontal angle. Angles in the equation, are expressed in radians.

NOTE: Relation to TRP is visible in the case where only one vertical range is considered (where the elevation angle range is defined as θL1 is equal to -π2 and θH1 is equal to π2). In this case EEIRP is equal to TRP.

Test beam set evaluation:

In this section we evaluate the EEIRP profiles for different test beam sets for a given array antenna geometry representative of a product in the band, and described in Table XXXX. It should be noticed that theses parameters are not selected with the intention to meet the EEIRP requirement profile. As the result will show, the specified EEIRP levels will not meet the ITU-R condition, which would indicate that additional mitigation techniques are needed. The intention with the parameter below is to evaluate the EEIRP characteristics due to different test beam sets.

Table XXXX: Array antenna assumptions

| Parameter | Value |
| --- | --- |
| Frequency | 6775 MHz |
| Element model | Vertical HPBW: 90° Horizontal HPBW: 90° Sidelobe suppression: 30dB Element directivity: 6.4 dBi |
| Array configuration | Number of rows: 8 Number of columns: 16 Sub-array: 3x1 and 6x1 Sub-array tilt: 6° Horizontal element separation: 0.5λ Vertical element separation: 0.7λ |
| TRP | 27 dBm/MHz |
| Peak EIRP target | 79 dBm/100 MHz |

To evaluate the impact of the number of beams, distribution and beam weights, several beam sets have been defined (Reference, A, B, C, D, E), considering different sizes of the test beam sets, distribution of test beams in the declared CAR (Coverage Angular Range), and weights. The CAR is set to -60 to 60 degrees in the horizontal plane and to 0 to 12 degrees (down-tilt) in the vertical plane.

For reference purposes a reference beam set of 325 uniformly distributed beams (25 x 13 equally spaced beams in horizontal and vertical plane, respectively) with uniform beam weights has been defined. This beam set is referred to as “Reference” in figures.

Uniform distribution

We first consider a uniform distribution of beams in the declared CAR. Two cases have been considered, Case A with different beam weighting at the edge of the CAR and Case B, where non weighted average is considered.

Specifically, Beam set A and B are defined by 25 test beams distributed in a uniform lattice with 5 beams along the horizonal axis and 5 beams along the vertical axis, according to Figure XXX.

Figure XXX: Beam distribution for Beam set cases A and B

For beam set A, the beam pointing direction and corresponding beam weight for all beams are listed in Table XXX. The beam directions are weighted to reflect the area that each beam covers within the CAR box.

Table XXX: Beam weights for Beam set A

| Test Beam [k] | θ° | φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/32 |
| 3 | 6 | 60 | 1/32 |
| 4 | 6 | -30 | 1/16 |
| 5 | 6 | 30 | 1/16 |
| 6 | 3 | 0 | 1/16 |
| 7 | 9 | 0 | 1/16 |
| 8 | 0 | 0 | 1/32 |
| 9 | 12 | 0 | 1/32 |
| 10 | 0 | -60 | 1/64 |
| 11 | 0 | -30 | 1/32 |
| 12 | 3 | -60 | 1/32 |
| 13 | 3 | -30 | 1/16 |
| 14 | 0 | 30 | 1/32 |
| 15 | 0 | 60 | 1/64 |
| 16 | 3 | 30 | 1/16 |
| 17 | 3 | 60 | 1/32 |
| 18 | 9 | -60 | 1/32 |
| 19 | 9 | -30 | 1/16 |
| 20 | 12 | -60 | 1/64 |
| 21 | 12 | -30 | 1/32 |
| 22 | 9 | 30 | 1/16 |
| 23 | 9 | 60 | 1/32 |
| 24 | 12 | 30 | 1/32 |
| 25 | 12 | 60 | 1/64 |

For Beam set B, uniform beam weighting is assumed. The reason for this is to compare the impact of applying additional beam weighting to the resulted EEIRP profile. The beam directions and beam weight vector are listed in Table XXX.

Table XXXX: Beam weights for Beam set I.B

| Test Beam [k] | θ° | φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/25 |
| 2 | 6 | -60 | 1/25 |
| 3 | 6 | 60 | 1/25 |
| 4 | 6 | -30 | 1/25 |
| 5 | 6 | 30 | 1/25 |
| 6 | 3 | 0 | 1/25 |
| 7 | 9 | 0 | 1/25 |
| 8 | 0 | 0 | 1/25 |
| 9 | 12 | 0 | 1/25 |
| 10 | 0 | -60 | 1/25 |
| 11 | 0 | -30 | 1/25 |
| 12 | 3 | -60 | 1/25 |
| 13 | 3 | -30 | 1/25 |
| 14 | 0 | 30 | 1/25 |
| 15 | 0 | 60 | 1/25 |
| 16 | 3 | 30 | 1/25 |
| 17 | 3 | 60 | 1/25 |
| 18 | 9 | -60 | 1/25 |
| 19 | 9 | -30 | 1/25 |
| 20 | 12 | -60 | 1/25 |
| 21 | 12 | -30 | 1/25 |
| 22 | 9 | 30 | 1/25 |
| 23 | 9 | 60 | 1/25 |
| 24 | 12 | 30 | 1/25 |
| 25 | 12 | 60 | 1/25 |

Reduced number of test beams

The impact on the EEIRP profile for the case where test beam directions are reduced have been evaluated by defining test beam directions according to Figure XXX. Now the test beam directions are aligned with the q-axis and j-axis.

Figure XXXX: Test beam set II, to reduce number of test beam directions

For this beam set, three different variants of beam weight vectors can be evaluated. Therefore, three different beam sets have been defined C, D and E. The beam directions and beam weights are listed in Table XXX, Table XXX and Table XXX.

Table XXX: Beam weights for beam set C

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/32 |
| 3 | 6 | 60 | 1/32 |
| 4 | 6 | -30 | 1/16 |
| 5 | 6 | 30 | 1/16 |
| 6 | 3 | 0 | 1/4 |
| 7 | 9 | 0 | 1/4 |
| 8 | 0 | 0 | 1/8 |
| 9 | 12 | 0 | 1/8 |

Table XXX: Beam weights for beam set D (uniform)

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/9 |
| 2 | 6 | -60 | 1/9 |
| 3 | 6 | 60 | 1/9 |
| 4 | 6 | -30 | 1/9 |
| 5 | 6 | 30 | 1/9 |
| 6 | 3 | 0 | 1/9 |
| 7 | 9 | 0 | 1/9 |
| 8 | 0 | 0 | 1/9 |
| 9 | 12 | 0 | 1/9 |

Table XXX: Beam weights for beam set E

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/8 |
| 3 | 6 | 60 | 1/8 |
| 4 | 6 | -30 | 1/4 |
| 5 | 6 | 30 | 1/4 |
| 6 | 3 | 0 | 1/16 |
| 7 | 9 | 0 | 1/16 |
| 8 | 0 | 0 | 1/32 |
| 9 | 12 | 0 | 1/32 |

Another alternative considers that the weighting is set to adjust for beam directions located on the edge of the CAR and to adjust for vertical angles having only one beam for all horizontal angles. In theory, this beam configuration may allow for an alternative but incorrect weight values (Case E) as shown in Table XXX. With the alternative beam weight , beams with horizontal steering are applied with a significant suppression. Hence the impact of using sub-arrays in the vertical domain will be suppressed, since beam causing grating lobes will be suppressed in the averaging process.

Test beam distribution evaluation

The number of test beams, the distribution within the declared CAR and the weighting, will impact the measured EEIRP level. Therefore, the EEIRP produced by a base station assuming a baseline antenna configuration, already introduced in Table XXX, have been evaluated.

In the evaluation 5 different test beam configurations have been evaluated, as listed in Table XXXX, and compared to the reference beam set and the WRC-23 requirement.

Table 2.2.3-7: Test beam configurations

| Case | Test beam distribution | Beam weight | Detailed description |
| --- | --- | --- | --- |
| A | Uniform | Non-uniform | Table 2.2.3-1 |
| B | Uniform | Uniform | Table 2.2.3-2 |
| C | Non-uniform | Non-uniform | Table 2.2.3-3 |
| D | Non-uniform | Uniform | Table 2.2.3-4 |
| E | Non-uniform | Incorrect Non-uniform | Table 2.2.3-5 |

The EEIRP profiles corresponding to Beam set A, B, C, D and E are plotted in Figure XXXX.

Figure XXXX: EEIRP profile for Cases A, B, C, D and E (sub-array size 3x1)

It can be observed that the EEIRP profiles align well with the reference case for beam sets A and B, and quickly converge to it. On the other hand, for the reduced set of beams, beam sets cases C, D and E, deviate from the reference case and from beam sets A and B. As a result, we consider that a test beam-set of 25 beams can represent a good trad-off between accuracy and testing complexity. It can also be noticed that the WRC-23 level is violated for some angular ranges.

It can be noticed that case E significantly deviates from the reference case, which indicates that case E should not be used for conformance testing.

These results, which were already discussed in previous meeting, demonstrate that the test beams should be carefully selected and should be distributed within the whole coverage angular range, not only along the axis. To guarantee repeatability and acceptable measurement uncertainty, RAN4 need to define one common test beam set, taking into account the trade-off between accuracy and testability.

Another aspect to consider in relation to test beam sets is the impact on the EEIRP profile depending on considered array antenna structure, more specifically the size of the sub-array. In what follows we repeat the experiments considering a bigger sub-array size. Specifically, we consider sub-array size of 6x1 and compare it to the previously studied case of sub-array size 3x1.

From the reference beam set, with 325 (25 x 13 equally spaced beams in horizontal and vertical plane, respectively) uniformly distributed beams with uniform beam weights, we already observe a slightly different behaviour as a function of the sub-array size in Figure XXX. Specifically, with larger sub-array size the EEIRP exceeds slightly more the requirement, compared to sub-array size 3x1, in certain directions.

Figure XXX: EEIRP profile for the reference beam set with 3x1 and 6x1 sub-array size

Similar trend is observed also for other beam sets as described in Figure XXX, Figure XXXX and Figure XXXX. It can also be noticed that the same observations done for the sub-array size 3x1 are valid for sub-array size 6x1, in terms of better accuracy and convergence to the reference when considering beam set A and beam set B, with respect to the other cases studied for beam-set C, D and E.

Summary of EEIRP profiles for cases A, B, C, D and E, for sub-array size 6x1 is shown in Figure XXXX.

Figure XXXX: EEIRP profile for the beam sets A and B, with 3x1 and 6x1 sub-array size

Figure XXXX: EEIRP profile for the beam sets C and D, with 3x1 and 6x1 sub-array size

Figure XXXX: EEIRP profile for the beam set E, with 3x1 and 6x1 sub-array size

Figure XXXX: EEIRP profile for Cases A, B, C, D and E (sub-array size 6x1)

BS Conformance test aspects:

The conformance test required to show compliance to the EEIRP limit needs to be developed by RAN4. To show conformance to the EEIRP limits a large number of measured 3-D EIRP patterns for different array excitations will be needed. Since the RAN4 conformance testing is built around pre-defined test signals (test models and fixed refence channels) normal operation in fading environment capturing true layer-1 operation cannot be captured by the conformance test in RAN4 for AAS base stations.

The conformance test for EEIRP limits will be created around a concept of using test beams within a declared CAR. The CAR is the angular range along vertical and horizontal axis for which the base station is intended to provide coverage within.

In the terminology section (clause 3 of TS 38.141-2), a definition of CAR is needed. For the purpose of EEIRP requirement the CAR can be defined as:

Coverage Angular Range: The coverage angular range is a set of directions describing the intended angular operation range.

For each test beam, the 3-D EIRP pattern in the upper hemisphere is measured. Based on measured EIRP patterns the EEIRP profile can be calculated as the output from a multistage averaging process.

Multiple test beam with difference directions is required to establish the average EIRP pattern corresponding to intended operation. For this purpose, K different test beams with different beam directions distributed within the declared CAR for which the base station is intended to operate within is required.

The intention with the test beams is to excite the array antenna in such a way that the array side-lobe suppression characteristics is captured. For an array antenna with sparse element separation in the vertical domain using sub-arrays, side lobe response and eventual grating lobe response will be captured.

Multiple CARs can be declared for a base station supporting multiple deployment scenarios to map towards different scenarios with different UE distributions. Eventually, for each declared CAR different beam power profiles are utilized to optimize coverage while meeting the EEIRP profile above the horizon.

For each test beam, the BS manufacturer declares the peak EIRP and direction as q and j angle pair.

For a solid test requirement, the details related to placement of the beam directions and weighting need to be captured in the specification. The location and distribution of beam directions will affect the averaging process significantly. RAN4 needs to define how many beam directions will be needed and how to distribute them.

The CAR is declared by the base station manufacturer using the angles listed in Table 2.3.2-1.

Table 2.3.2-1: CAR declarations

| Parameter | Description |
| --- | --- |
| qmin | Minimum angle along q axis |
| qmax | Maximum angle along q axis |
| jmin | Minimum angle along j axis |
| jmax | Maximum angle along j axis |

The CAR can be visualized as a rectangular area in the q/j plane as shown in Figure 2.3.2-1 as a green rectangle.

For a typical wide area base station intended for 3-sector rural deployments values listed in Table 2.3.2-2 is reasonable.

Table 2.3.2-2: Typical values

| Parameter | Values (degrees) |
| --- | --- |
| qmin | 0 |
| qmax | 12 |
| jmin | -60 |
| jmax | 60 |

For other deployment scenarios (e.g., for wide area base station operating in an urban deployment scenario) other values may be declared.

Figure 2.3.2-1: Coverage angular range

The test beams will be located within the declared CAR. The number of test beams K and the location within the q/j plane will impact the measured EEIRP profile. In the background information relevant for the condition ITU-R stipulates that the beam directions should be distributed uniformly within the CAR. To adjust for beams located at the edge of the CAR a beam weight has been introduced.

Figure 2.2.3-1: Beam distribution for Beam sets A and B

<<< We need to add equations on how beams are placed based on declarations of CAR >>>

The average EIRP pattern is calculated from K EIRP patterns as:

EIRPθ,φ=k=1KwkEIRPkθ,φ

, where wk is a beam weighting and EIRPkθ,φ is the EIRP pattern per measured test beam.

Beam weight: fraction of the coverage angular range represented by the test beam.

For example, wk=1K  in the case that K uniform equal spaced test beams are assumed in azimuth and elevation, respectively, and where each beam covers an equal range of angles (e.g., equal area in q/j plane). The weight is required to account for test beam coverage for test beams located on the edge of the CAR, non-uniform distribution or allowing for fewer test beams to be considered as show in following examples.

Eventually, the maximum peak EIRP difference between declared beam needs to be limited to a certain value (e.g., 10 dB). Without a cap on the maximum difference, fake beams can be declared during testing that will produce incorrect measured EEIRP levels.

In the conformance specification a reference to the BS RF core specification will be used for the requirement profile. Since, this is a regulatory requirement the test tolerance will be set to zero. Hence, the BS RF requirement profile and conformance test profile will be equivalent.

In technical background information from ITU-R [2] the EEIRP profile is calculated from an integration over continues average EIRP patten. From a testing perspective, the averaging process should be described as an average over discrete spatial samples.

The EEIRP profile can be extracted from measured EIRP spectral density radiation patterns described in section 2.3.5.2 by applying multiple stage of averaging processes.

In the first stage the average EIRP radiation pattern over K test beams are calculated as:

EIRPθn,φm=k=1KwkEIRPkθn,φm

, where EIRPkθn,θm  is the EIRP spectral density radiation pattern measured for the k-th test beam and w_k refers to the weight for the k-th beamforming direction, i.e., the fraction of the steering range represented by the k-th beamforming direction (see detailed description in section 2.3.2).

In the second average stage the EEIRP level for the i-th elevation range is calculated as:

EEIRPi=θHi-θLisinθHi-sinθLiMN∙n=nLinHim=1MEIRPθn,θmcosθn

The post-processing stage with details description of the averaging calculations should be captured in a dedicated Annex in TS 38.141-2.