3GPP RAN TSG-RAN4 Meeting # 112		R4-2411151

Maastricht, NL, August 19 – 23, 2024

Agenda item:	8.1.1.3.4

Source:	Apple

Title:	On SRS IL imbalance issue for 6Rx

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #111, Fukuoka, Japan, May 20-24, 2024), discussions were held on the requirements for 6Rx for handheld and FWA, which are parts of the new  Release 19 WID on RF enhancements for NR FR1/FR2 and EN-DC (NR_ENDC_RF_Ph4) [1]. As captured in the meeting WF [2], one of the 6Rx aspects that  were greatly debated is the SRS antenna switching insertion loss imbalance issue, with the discussion summary highlighted below:

# Topic 4: SRS IL Imbalance Issue ## Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way forward: RAN4 to further discuss if the SRS IL imbalance issue should be solved in Rel-19. Any decision to not solve the SRS IL imbalance issue in Rel-19 will need to be confirmed at RAN Plenary in a Revised WID. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyze the impact of SRS IL imbalance on NW performance degradation. ## Sub-topic 4-2: SRS IL imbalance issue solutions Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behavior in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation

In this discussion paper, we will provide our views on potential solutions that could address the SRS imbalance issue as captured in the  issue 4-2-1 above.

# Discussion

We highlighted in our previous discussion paper [3] that, as the number of Rx antenna ports grows in handheld UEs, it becomes more challenging to accurately perform SRS antenna switching which relies on making UL channel measurements to predict quality of the DL channels. This is because the RF insertion loss differences between the main path (which is used for UL), and the Rx paths are higher and difficult to model. Several companies have illustrated performance loss in case the SRS IL is not compensated or taken into account by the gNB. As reported in [4] for example,  simulations showed that with 70% resource utilization, the DL throughput can degrade as much as 35Mbps/user when the SRS IL imbalance is considered.

In the past meetings, several contributions about potential solutions to address the IL imbalance issue were summitted. The proposed solutions were mostly centered around UE power pre-compensation and UE reporting of the imbalances across the SRS ports to the network.

  - UE Power Pre-compensation

As stated in clause TS38.213 (clause 7.3) , if the SRS resource consists of multiple ports, then its transmitted power is equal across all the SRS antenna ports at all power levels. However, based on TS38.101-1 requirements as highlighted in section 2.2.1 above, RAN4 also specifies ∆TRxSRS for configured maximum output power requirements to allow for some deviation from the maximum configured output power for the UE when transmitting SRS. Therefore, for the transmitted SRS power to be equal on all the SRS antenna ports, it seems to us that the UE could perform, based on its implementation, some amount of power pre-compensation to alleviate the effects of SRS AS IL imbalance.

Observation #1: Based on its implementation, it seems reasonable that the UE could implement some amount of power pre-compensation on its SRS ports to cope with the effects of SRS AS IL imbalance when there is enough power headroom to do so.

Even though power compensation seems like a viable solution to address the SRS IL imbalance issue, there are some UE implementation challenges that need to be considered:

- UE power tolerance

To effectively perform power pre-compensation, the UE the value of DTRxSRS has to be accurately known. This could be challenging given the fact that the specifications on PCMAX tolerances are relatively high. Table 2.2.2-1 below shows the specifications for the PCMAX tolerance, taken from clause 6.2 of the TS38.101-1 specifications. As can be observed from that table, the tolerance can be as high as 7dB for low power levels. This value is higher than the actual DTRxSRS that needs to be compensated for some cases. This make it difficult for the UE to accurately determine the value of DTRxSRS, especially for lower power levels.

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

Table 2.1-1: PCMAX tolerance (from Table 6.2.4-1 of TS38.101-1)

Observation #2: The value of DTRxSRS may not be accurately determined by the UE due to required high values of PCMAX tolerances.

- UE power headroom (PHR)

Pre-compensation can only work if there is enough power headroom to do so. So, pre-compensation is possible only for the cases where the amount of the PA headroom is higher than the IL imbalance that needs to be compensated. This happens when the UE is operating below its maximum power PCMAX.

  - UE SRS IL Imbalance Reporting

Due to the limitations of power pre-compensation as highlighted above, IL imbalance reporting has been explored in the past RAN4 meetings as another potential solution to address the imbalance issue for 4Rx and 8Rx [5]. The main goal of SRS IL reporting is to make the gNB aware of the power imbalance between main SRS port(s) and diversity SRS ports. This awareness of the power offset between DL and UL will be helpful for the gNB to acquire more accurate channel estimation.

  - Potential IL Imbalance Solution based on pre-compensation and reporting

Figure 2.3-1 below is a simplified model of the UE RF front-end when the UE performs SRS antenna switching. The DTRxSRS values for all the SRS branches are shown along with the antenna connectors. In this model, Ant.1 is the main Tx antenna while the diversity antennas used for SRS are Ant.2, Ant.3, Ant.4, and Ant.5. Based on the diagram, DTRxSRS4 which is associated with Ant.5 is the largest imbalance IL value.

Figure 2.3-1: SRS antenna switching IL imbalance modeling

As already mentioned above, if the SRS resource consists of multiple ports, then its transmitted power is equal across all the SRS antenna ports at all power levels, as captured in clause 7.3 of  the TS38.213 specifications.

If the configured maximum power levels at all SRS ports are equal, therefore the lower bound value of that power will be the power at the port with the highest value of  DTRxSRS (in the example captured in the figure above, it would be DTRxSRS4 which is associated to Ant.5). Let’s label that max power “PCMAX_SRSmin”.

If PCMAX_SRSi denotes the maximum configured power for the SRS antenna port i, therefore we have the following two cases:

- Case#1: PCMAX_SRSi ≤ PCMAX_SRSmin:

For this case, the UE has enough power headroom and can perform  pre-compensation on the SRS ports.

- Case #2: PCMAX_SRSi ≥ PCMAX_SRSmin:

For this case, the UE does not have enough headroom to perform pre-compensation. The UE could report to the NW the amount of power back-off (PCMAX_SRSi - PCMAX_SRSmin) for each SRS port so that the NW is aware of imbalance at each antenna port. Table-3-1 shows the values that need to be reported to the NW for this case. The reporting mechanism is triggered when the configured power at each SRS resource reaches PCMAX_SRSmin.

| Antenna Port | SRS Port Power Level (At antenna connectors) | Reported Value to the Network |
| --- | --- | --- |
| 0 | PCMAX_SRS0 | PCMAX_SRS0 -PCMAX_SRSmin |
| 1 | PCMAX_SRS1 | PCMAX_SRS1 -PCMAX_SRSmin |
| 2 | PCMAX_SRS2 | PCMAX_SRS2 -PCMAX_SRSmin |
| 3 | PCMAX_SRS3 | PCMAX_SRS3 -PCMAX_SRSmin |
| 4 | PCMAX_SRS4 | PCMAX_SRS4 -PCMAX_SRSmin |
| 5 | PCMAX_SRS5 | PCMAX_SRS5 -PCMAX_SRSmin |

Table 2.3-1: SRS antenna resource power back-off values reporting

Proposal #1: Solution for IL imbalance issue based on power pre-compensation and reporting:

- Case#1: PCMAX_SRSi ≤ PCMAX_SRSmin:

For this case, the UE has enough power headroom and can perform  pre-compensation on the SRS ports.

- Case #2: PCMAX_SRSi ≥ PCMAX_SRSmin:

For this case, the UE does not have enough headroom to perform pre-compensation. The UE should report to the NW the amount of power back-off (PCMAX_SRSi - PCMAX_SRSmin) for each SRS port as shown in the Table below. The reporting mechanism is triggered when the configured power at each SRS resource reaches PCMAX_SRSmin.

| Antenna Port | SRS Port Power Level (At antenna connectors) | Reported Value to the Network |
| --- | --- | --- |
| 0 | PCMAX_SRS0 | PCMAX_SRS0 -PCMAX_SRSmin |
| 1 | PCMAX_SRS1 | PCMAX_SRS1 -PCMAX_SRSmin |
| 2 | PCMAX_SRS2 | PCMAX_SRS2 -PCMAX_SRSmin |
| 3 | PCMAX_SRS3 | PCMAX_SRS3 -PCMAX_SRSmin |
| 4 | PCMAX_SRS4 | PCMAX_SRS4 -PCMAX_SRSmin |
| 5 | PCMAX_SRS5 | PCMAX_SRS5 -PCMAX_SRSmin |

Proposal #2: The reporting mechanism highlighted above is  also applicable to 2RX/4RX/8RX.

# 3	Conclusions

Observation #1: Based on its implementation, it seems reasonable that the UE could implement some amount of power pre-compensation on its SRS ports to cope with the effects of SRS AS IL imbalance when there is enough power headroom to do so.

Observation #2: The value of DTRxSRS may not be accurately determined by the UE due to required high values of PCMAX tolerances.

Proposal #1: Solution for IL imbalance issue based on power pre-compensation and reporting:

- Case#1: PCMAX_SRSi ≤ PCMAX_SRSmin:

For this case, the UE has enough power headroom and can perform  pre-compensation on the SRS ports.

- Case #2: PCMAX_SRSi ≥ PCMAX_SRSmin:

For this case, the UE does not have enough headroom to perform pre-compensation. The UE should report to the NW the amount of power back-off (PCMAX_SRSi - PCMAX_SRSmin) for each SRS port as shown in the Table below. The reporting mechanism is triggered when the configured power at each SRS resource reaches PCMAX_SRSmin.

| Antenna Port | SRS Port Power Level (At antenna connectors) | Reported Value to the Network |
| --- | --- | --- |
| 0 | PCMAX_SRS0 | PCMAX_SRS0 -PCMAX_SRSmin |
| 1 | PCMAX_SRS1 | PCMAX_SRS1 -PCMAX_SRSmin |
| 2 | PCMAX_SRS2 | PCMAX_SRS2 -PCMAX_SRSmin |
| 3 | PCMAX_SRS3 | PCMAX_SRS3 -PCMAX_SRSmin |
| 4 | PCMAX_SRS4 | PCMAX_SRS4 -PCMAX_SRSmin |
| 5 | PCMAX_SRS5 | PCMAX_SRS5 -PCMAX_SRSmin |

Proposal #2: The reporting mechanism highlighted above is  also applicable to 2RX/4RX/8RX.

# 4	References

R4-2403716 “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, by RAN4 chair (Huawei),  3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

R4-2410590, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, May 20-24, 2024

R4-2407178, “Views on 6Rx for handheld UEs”, by Apple, 3GPP TSG-RAN WG4 Meeting #110Bis, Changsha, China, 15th Apr 2024 - 19th Apr 2024

R1-2308129, “Discussion on RAN4 LS on the UE SRS imbalance issue”, by Ericsson, 3GPP TSG-RAN WG1 Meeting #114, Toulouse, France, August 21st– 25th, 2023

R4-2314695, “WF on support of 8Rx UE RF”, by NTT DOCOMO, INC., 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, August 21st– 25th, 2023