3GPP TSG RAN WG4 #112 meeting 	 	R4-2411648

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.1.1.1.1

Source:	Meta Ireland

Title:	High power UE RF requirements for intra-band CA in TN

Document for:	Approval

- Introduction

In RAN4 #111 meeting, RAN4 agreed the WF [1] on HP UE RF enhancements for UL CA and EN-DC in Rel-19. In the WF, RAN4 decided the reference RF architecture for high power (PC1.5) for intra-band contiguous CA/non-contiguous CA and RAN4 will be further discussed to specify the detail MPR/A-MPR requirements, Pcmax,c and Pcmax for intra-band contiguous CA and intra-band non-contiguous CA operation.

The detail FFS issues are list up based on WF [1] as follow:

PC1.5 UE (HPUE) for intra-band CA in terrestrial network (TN) [1] - A-MPR for n41/n77/n78/n79 Way forward: - A-MPR requirements are not needed for PC1.5 n77/n78/n79 intra-band ULCA - Evaluate A-MPR requirements for PC1.5 n41 intra-band UL CA - PCMAX,C Proposals: - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE) - For dualPA-architecture - PCMAX,C limitation for each component carrier is 26dBm - For TxD (dualTx) - PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3 - PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3 - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721) Way forward: FFS in future meetings - PCMAX Proposals: - Option 1: (Skyworks) - MOP for 2Tx architectures (TxD w/wo UL MIMO) is: 29 dBm - MOP for Dual-PA architectures (one PA/CC, 2LO) is: PCmax=29+10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dBm - Option 2: (Skyworks) MPR for Dual-PA architectures (one PA/CC, 2LO) is modified by: 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dB - Option 3: (Captured in last meeting’s WF) For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10)) - Option 4: For UEs indicating dualPA-Architecture (Architecture #1) the total configured output power PCMAX for the band combination is given by clause 6.2A.4.1.3 for inter-band CA in the current version of 38.101-1, the same applies for TxD (Ericsson) Way forward: FFS in future meetings

In this paper, we suggest how to define the RF requirements for high power UE for intra-band contiguous CA i.e. n41C, n77C and other intra-band non-contiguous CA i.e. CA_n78(2A), CA_n77(2A).

- MPR simulation assumptions of High power intra-band CA UE

  - MPR assumptions for intra-band contiguous CA UE

In this session, we propose the MPR simulation assumptions for PC1.5 CA UE for single carrier.

This is based on the previous RAN4 agreements for high power PC2 CA UE. On top of theses parameters, just modified these assumptions for PC1.5 intra-band contiguous CA UE as follows in Table 1.

Table 1. MPR simulation assumption for intra-band Contiguous CA

MPR simulation assumptions for intra-band contiguous CA - 2CC and 100MHz CBW per CC is assumed. - 2x26dBm PA + 1LO up to 200 MHz aggregated CBW are the baseline for MPR/A-MPR evaluation. - 2x26dBm PA + 2LO also can be considered as vendor implement aspect. - PA calibration for 20MHz QPSK DFT-s-OFDM 100RB0 waveform based on 4dB post PA losses and 1dB MPR. - 26dBm/antenna: 29dBm at 31dB ACLR - ULTR-ACLR is not considered. - Modulation order: QPSK/16QAM/64QAM/256QAM - Carrier leakage: 28 dBc for the allocated CC and non-allocated CC - IQ image: 28 dBc for the allocated CC and non-allocated CC - EVM: Apply per CC in Table 6.4A.2.1.1-1 in TS38.101-1 - SEM: Apply Table 6.5A.2.2.1-1 in TS38.101-1 for both CCs are active - SE: Follow Table 6.5.3.1-2 in TS38.101-1 - Back-off is relative to 29dBm at the antenna. - Equal PSD and Equal back-off power split - Measurements is used where two PA are coupled on the output recreating the 10dB antenna isolation assumption for HH UE. - For FWA UE, RAN4 can consider 20dB antenna isolation - Emission requirements (ACLR/SEM/spurious emissions) are checked by summing the power of the two transmit paths. - Since simulation are not available, at least worst-case corners are evaluated for different modulation order.

In Table 2, we propose the detail MPR simulation assumptions for PC1.5 intra-band non-contiguous CA UE as follows:

Table 2. MPR simulation assumption for intra-band Non-Contiguous CA

MPR simulation assumptions for intra-band Non-Contiguous CA - 2CC and 100MHz CBW per CC is assumed. - 2x26dBm PA + 2LO up to 200 MHz aggregated CBW are the baseline for MPR/A-MPR evaluation. - 2x26dBm PA + 1LO also can be considered as vendor implement aspect. - PA calibration for 20MHz QPSK DFT-s-OFDM 100RB0 waveform based on 4dB post PA losses and 1dB MPR. - 26dBm/antenna: 29dBm at 31dB ACLR - ULTR-ACLR is not considered. - Modulation order: QPSK/16QAM/64QAM/256QAM - SEM: Apply section 6.5A.2.2.2 in TS38.101-1 for both CCs are active. - SE: Apply section 6.5A.3.1 in TS38.101-1 for both CCs are active. - Variable Frequency separation will be considered according to BCS of the intra-band NC CA band. - Back-off is relative to 29dBm at the antenna. - Equal PSD and Equal back-off power split - Measurements is used where two PA are coupled on the output recreating the 10dB antenna isolation assumption. - For FWA UE, RAN4 can consider 20dB antenna isolation - Emission requirements (ACLR/SEM/spurious emissions) are checked by summing the power of the two transmit paths. - Since simulation are not available, at least worst-case corners are evaluated for different modulation order.

Based on the above detail RF simulation parameters, RAN4 can derive the MPR requirements for PC1.5 intra-band contiguous CA UE and PC1.5 intra-band non-contiguous CA UE.

Proposal 1: The proposed MPR simulation assumptions in Table 1 &2, RAN4 can derive the detail MPR requirements for PC1.5 intra-band CA UE.

  - MPR requirements for intra-band contiguous CA with dual Tx

In TS38.101-1 in Rel-18, RAN4 defined the MPR requirements with dual Tx for single carrier as shown in Table 3 (original Table 6.2D.2-2 in TS38.101-1). The same RF architecture with separate PA and separate antenna RF architectures will be assumed for intra-band contiguous CA with dual Tx for CA_n41C, CA_n77C, CA_n78C and CA_n79C.

Table 3: Maximum power reduction (MPR) for power class 1.5 with dual Tx for single carrier

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Based on the agreed reference RF architecture and simulation assumptions, we propose the MPR requirements to support wide intra-band contiguous CA up to 200MHz based on RF simulations as follow two Tables according to contiguous RB allocations and non-contiguous RB allocations.

Table 4: Proposed MPR for intra-band contiguous CA with contiguous RB allocation for PC 1.5 with dual Tx1

| Modulation |  | MPR for bandwidth class B (dB) |  | MPR for bandwidth class C (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 6.0 | ≤ 4.0 | ≤ 8.0 |
|  | QPSK | ≤ 3.5 | ≤ 6.5 | ≤ 4.0 | ≤ 8.0 |
|  | 16QAM | ≤ 3.5 | ≤ 6.5 | ≤ 4.5 | ≤ 8.0 |
|  | 64QAM | ≤ 4.0 | ≤ 6.5 | ≤ 6.5 | ≤ 8.0 |
|  | 256QAM | ≤ 7.0 | ≤ 6.5 | ≤ 8.5 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 6.5 | ≤ 4.5 | ≤ 8.5 |
|  | 16QAM | ≤ 3.5 | ≤ 6.5 | ≤ 4.5 | ≤ 8.5 |
|  | 64QAM | ≤ 5.0 | ≤ 6.5 | ≤ 6.5 | ≤ 8.5 |
|  | 256QAM | ≤ 8.5 | ≤ 8.5 | ≤ 8.5 | ≤ 9.0 |
| NOTE 1: UE indicating TxD supported |  |  |  |  |  |

Table 5: Proposed MPR for intra-band contiguous CA with non-contiguous RB allocation for PC 1.5 with dual Tx4

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 8.5 | ≤ 15.0 | ≤ 5.01 | ≤ 9.5 | ≤ 16.0 |
|  | QPSK | ≤ 5.01 | ≤ 8.5 |  | ≤ 5.51 | ≤ 9.5 |  |
|  | 16QAM | ≤ 5.51 | ≤ 9.0 |  | ≤ 6.01 | ≤ 10.0 |  |
|  | 64QAM | ≤ 7.0 | ≤ 9.0 |  | ≤ 7.5 | ≤ 10.5 |  |
|  | 256QAM | ≤ 8.0 | ≤ 9.5 |  | ≤ 8.5 | ≤ 11.0 |  |
| CP-OFDM | QPSK | ≤ 5.51 | ≤ 9.0 | ≤ 16.5 | ≤ 5.51 | ≤ 9.5 | ≤ 17.0 |
|  | 16QAM | ≤ 6.01 | ≤ 9.0 |  | ≤ 6.01 | ≤ 10.5 |  |
|  | 64QAM | ≤ 7.5 | ≤ 10.0 |  | ≤ 8.0 | ≤ 11.0 |  |
|  | 256QAM | ≤ 9.0 | ≤ 10.5 |  | ≤ 8.5 | ≤ 11.5 |  |
| NOTE 1: the allowed MPR is [5]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

Proposal 2: The proposed MPR requirements in Table 4 and Table 5 for intra-band contiguous CA UE shall be considered to specify the required MPR requirements for PC1.5 intra-band contiguous CA UE with dual Tx in TS38.101-1.

  - SAR solution of PC1.5 intra-band contiguous CA UE

For the SAR regulation requirements, RAN4 can reuse ‘the max uplink dutycycle limitation’ of the single carrier SAR solution for intra-band contiguous CA operation. It means that intra-band contiguous CA UE will be operated by synchronous operation between 2CCs as like simultaneously transmit or reception to comply the SAR limitation.

Proposal 3: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band contiguous CA combinations UE.

  - MPR requirements for intra-band non-contiguous CA UE

In PC2 high power intra-band non-contiguous CA in Rel-17, RAN4 considered 4 candidate RF architectures to support different PA capability combinations and the limitation of supporting channel Bandwidths.

However, RAN4 agreed to consider 2x26dBm PA + 2LO as baseline architecture to support up to 200MHz CBW and the baseline RF architecture to derive MPR/A-MPR requirements for PC1.5 intra-band non-contiguous CA operation with dual PA architecture and 2Tx.

Based on the proposed MPR simulation assumptions and agreed baseline RF architecture, we propose the MPR requirements to support the intra-band non-contiguous CA up to 200MHz based on RF simulations.

Proposed MPR requirements for intra-band Non-Contiguous CA UE in TS38.101-1 MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as: MPR=MAWhere MA is defined as follows MA = [17.0]; 0 ≤ B < 1.44 [16.0]; 1.44 ≤ B < 2.88 [14.5]; 2.88 ≤ B < 5.76 [13.0]; 5.76 ≤ B < 10.8 [11.5]; 10.8 ≤ B < 23.04 [10.0]; 23.04 ≤ B

Proposal 4: The proposed MPR requirements for intra-band non-contiguous CA UE shall be considered to specify the required MPR requirements for PC1.5 intra-band non-contiguous CA UE with dual PA architecture and dual Tx in TS38.101-1.

  - SAR solution of PC1.5 intra-band non-contiguous CA UE

For the SAR regulation requirements, RAN4 can reuse ‘the max. uplink duty-cycle limitation’ of the single carrier SAR solution for intra-band non-contiguous CA operation. The same SAR solution will be applied for all intra-band contiguous CA and intra-band non-contiguous CA UE. It means that intra-band non-contiguous CA UE will be operated by synchronous operation between 2CCs as like simultaneously transmit or reception to comply the SAR limitation.

Proposal 5: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band non-contiguous CA combinations UE.

- Configured transmitted power for PC1.5 intra-band CA UE

  - Pcmax of the configured Tx power for PC1.5 intra-band contiguous CA UE

For PC1.5 intra-band contiguous CA UE, RAN4 can refer the configured transmitted power Pcmax,c for PC2 intra-band contiguous CA UE as baseline and additional points based on the Dual Tx architecture (2x26dBm PA +1LO for 200MHz CBW) of PC1.5 high power CA UE are captured as follows:

- Included ΔPPowerClass,CA = 6 dB for a power class 1.5 CA UE

- Max. ouput power per CC is 26dBm

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA }

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.1-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in clause 6.2A.2 and 6.2A.3, respectively;

-	ΔPPowerClass,CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.1; otherwise ΔPPowerClass,CA = 0 dB;

-	ΔPPowerClass,CA = 6 dB for a power class 1.5 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.1; otherwise ΔPPowerClass,CA = 0 dB;

NOTE: 	UE reports ∆PPowerClass,CA when [∆PPowerClass,CA reporting capability XXX-r18 is present], dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

-	TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

-	∆TRxSRS is the highest value among all serving cells c;

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.

For uplink intra-band contiguous carrier aggregation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in clause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different or same bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.

Proposal 6: RAN4 can define the configured Transmitted power for PC1.5 intra-band contiguous CA as above session 3.1.

  - Pcmax of the configured Tx power for PC1.5 intra-band non-contiguous CA UE

For PC1.5 intra-band non-contiguous CA UE, RAN4 can refer the configured transmitted power Pcmax,c for PC2 intra-band non-contiguous CA UE as baseline and additional points based on the Dual PA architecture (2x26dBm PA + 2LO with 100MHz CBW) of PC1.5 high power CA UE are captured as follows:

- Included ΔPPowerClass,CA = 6 dB for a power class 1.5 CA UE

- PCmax= 29 + 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dBm

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX is defined as follow;

PCmax= 29 + 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dBm

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band non-contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPRc, A-MPRc) + ΔTIB,c + TC + DTRxSRS, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA)}

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.2-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.2A.2 and subclause 6.2A.3 respectively;

-	ΔPPowerClass,CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.2; otherwise ΔPPowerClass,CA = 0 dB;

-	ΔPPowerClass,CA = 6 dB for a power class 1.5 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.1; otherwise ΔPPowerClass,CA = 0 dB;

NOTE:	UE reports ∆PPowerClass,CA when [∆PPowerClass,CA reporting capability XXX-r18 is present], dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

-	TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

-	∆TRxSRS is the highest value among all serving cells c;

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.

For uplink intra-band non-contiguous carrier aggregation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in subclause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different or same bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.

Proposal 7: RAN4 can define the configured Transmitted power for PC1.5 intra-band non-contiguous CA as above session 3.2.

- Conclusions

In this contribution, we propose some RF architectures and SAR solution for intra-band contiguous CA or non-contiguous CA. Also, we propose detail MPR simulation parameters to derive MPR requirements for PC1.5 intra-band contiguous CA/non-contigous CA UE. Based on our detail proposals for the high-power intra-band CA, we proposed as follow

Proposal 1: The proposed MPR simulation assumptions in Table 1 &2, RAN4 can derive the detail MPR requirements for PC1.5 intra-band CA UE.

Proposal 2: The proposed MPR requirements in Table 4 and Table 5 for intra-band contiguous CA UE shall be considered to specify the required MPR requirements for PC1.5 intra-band contiguous CA UE with dual Tx in TS38.101-1.

Proposal 3: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band contiguous CA combinations UE.

Proposal 4: The proposed MPR requirements for intra-band non-contiguous CA UE shall be considered to specify the required MPR requirements for PC1.5 intra-band non-contiguous CA UE with dual PA architecture and dual Tx in TS38.101-1.

Proposal 5: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band non-contiguous CA combinations UE.

Proposal 6: RAN4 can define the configured Transmitted power for PC1.5 intra-band contiguous CA as above session 3.1.

Proposal 7: RAN4 can define the configured Transmitted power for PC1.5 intra-band non-contiguous CA as above session 3.2.

Reference

[1] R4-2410750, “WF on on HPUE UE,” Samsung

[2] RP-241656, “Revised WID on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4,” Huawei

[3] TS38.101-1

[4] TS38.101-3

[5] R4-2407371, “PC1.5 intra-band ULCA MOP for 2Tx and Dual-PA cases,” SKW