3GPP TSG-RAN WG4 Meeting # 112		R4-2411757

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	5.29.1

Source: 	CMCC

Title: 			 (Netw_Energy_NR-Core) Discussion on core maintenance for NES

Document for:	Discussion

- Introduction

In last meeting, RAN4 focused on maintenance issues, the WF has been approved in [1]. However, some issues are still open, which need to be further discussed in this meeting. In this contribution, we focus on these open maintenance issues.

- Discussion

Issue 1-1-1: Power difference conditions

Agreement:

- Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell.

- Only change [9] dB to [12] dB in this meeting.

- Further discuss whether and how to support larger EPRE difference in next meeting:

  - Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)| - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS.

  - Option 2: Further increase [12] dB and the value is FFS

  - Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference

  - Other options are no precluded.

This fundamental side condition issue has been discussed since the first meeting, and then, in RAN4#109 meeting [2], considering that the gap between the component CCs of inter-band CA is typically large, [9]/[12]dB could not cover the pathloss difference plus other power difference between two CCs, for example, nearly 17dB pathloss difference for CA_n28-n79, RAN4 agreed that the spec will capture the wording of ‘EPRE after pre-compensation’, and WF will capture UE behavior that UE carries out pre-compensation for AGC considering [BW difference and carrier frequency difference], which means the pathloss difference between inter-band component carriers is taken into account in EPRE before UE pre-compensation.

Online session (Thursday Nov 16, 2023) Agreement: - The requirements apply provided that [EPRE] difference at UE side is less than [9] dB. - EPRE difference is based on power difference between TRS symbol on SSB-less SCell and SSB symbol on reference cell - Capture in the WF that RAN4 assumes that UE carries out pre-compensation for AGC considering [BW difference and carrier frequency difference]. - Further discuss whether/how to capture the EPRE after pre-compensation in the spec.

In RAN4#110, the issue was opened again, to involve the following Option2, that no need to further clarify EPRE comparison whether it is performed after AGC and no need to capture in spec [3].

First of all, we think we should consist on our agreement that the spec should support the EPRE difference larger [12]dB, unless any technical issue was found. So far, no technical issue was identified. On one hand, for UE supports the feature scellWithoutSSB-interbandCA-r18, it is feasible to carry out the pre-compensation for AGC considering carrier frequency difference, on the other hand, UE does not support this feature is also allowed, since the feature is optional.

Proposal 1:  RAN4 should consist on the previous agreement that the spec should support the EPRE difference larger [12]dB.

Regarding how to support larger EPRE difference, there are three options for further discussion. Our first preference is Option 1, which aligned with our previous agreement and further clarify the component of EPRE pre-compensation by using a approximate pathloss model to cover most of typical cases. But the margin here seems not needed anymore because RAN4 always consider the worst case. Even if the inaccuracy exists in the model, the inaccuracy can be covered by [12]dB.

Proposal 2:  Clarify that EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)|, where f1 and f2 is the center frequency of reference Cell and SSB-less SCell.

For Option 3, currently we use two samples to support [12]dB EPRE difference, if three samples are allowed, there shouldn’t exist any limitation on EPRE difference, since UE behavior is same as blind detection. This method can be used in P-TRS case since network is able to configure 3 or more P-TRS resource set. However, for A-TRS case, network only support to configure up to two A-TRS bursts by fast SCell activation MAC-CE, whether DCI can be used for triggering the A-TRS of deactivated SCell is not clear for us. Hence, whether this method can be used in A-TRS case should be further studied. But anyway, Option 3 will cause longer activation delay when power difference is larger.

Observation 1: The methodology of Option 3 can be used in P-TRS case, to update the requirements with one more P-TRS sample when EPRE difference larger than [12]dB.

Observation 2: Whether the methodology of Option 3 can be used in A-TRS case should be further studied.

Observation 3: The methodology of Option 3 will cause longer activation delay when EPRE difference larger than [12]dB.

Issue 1-1-3: Multiple SSB-less SCells activation

Agreements:

- Also define requirements for following scenario:

  - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous.

  - All to-be-activated SCells have same QCL source cell.

  - P-TRS is used.

    - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR.

    - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay)

  - FFS A-TRS based requirements.

In last meeting, RAN4 agreed to specify the multiple SSB-less SCells activation delay requirement when all to-be-activated SSB-less SCells are in contiguous bands and have same QCL source cell, which is a relative common scenario in practical network.

As analyzed by Apple [4], when multiple to-be activated SSB-less SCells are operated in intra-band contiguous CCs, the signals of SCells are typically sent by same antenna, and go through same propagation condition. We think it is sensible that two/multiple SCells share same timing/frequency sync and AGC information, similar as intra-frequency contiguous CA scenario. Then the requirement for these SSB-less SCells operating in intra-band contiguous CCs should be the minimum value among them. Therefore, the activation delay requirement for multiple SSB-less SCells should be defined as:

If all to-be activated SSB-less SCells are operated in intra-band contiguous CCs:

Tactivation_time = min(Tactivation_time_SCell1, ... , Tactivation_time_SCelln),  n = 1, .. , the number of to-be-activated SSB-less SCells

- Tactivation_time_SCelln = Tfirst_TRS_Scelln + TTRS_Scelln + 5ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17

- Tactivation_time_SCelln =Tfirst_ATRS_Scelln + Tgap_Scelln + TATRS_Scelln + 5ms, if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

  - Tfirst_TRS_Scelln is the time to the end of the first complete periodic CSI-RS burst for the nth SCell activation after slot n + THARQ+3msNR slot length.

  - TTRS_Scelln is the periodicity of the periodic CSI-RS burst for the nth SCell activation.

  - Tfirst_ATRS_Scelln is the time to the end of the first complete CSI-RS burst for the nth SCell activation after slot n + THARQ+3msNR slot length where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

  - TATRS_Scelln is the CSI-RS burst for the nth SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

  - Tgap_Scelln is the gap length between two aperiodic CSI-RS bursts for the nth SSB-less SCell.

Proposal 3:  For all to-be-activated SSB-less SCells are in contiguous bands and have same QCL source cell case, the side condition from single SSB-less SCell case can be reused, the requirement could be defined as:

Tactivation_time = min(Tactivation_time_SCell1, ... , Tactivation_time_SCelln),  n = 1, .. , the number of to-be-activated SSB-less SCells in contiguous bands and have same QCL source cell

- Tactivation_time_SCelln = Tfirst_TRS_Scelln + TTRS_Scelln + 5ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17

- Tactivation_time_SCelln =Tfirst_ATRS_Scelln + Tgap_Scelln + TATRS_Scelln + 5ms, if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

  - Tfirst_TRS_Scelln is the time to the end of the first complete periodic CSI-RS burst for the nth SCell activation after slot n + THARQ+3msNR slot length.

  - TTRS_Scelln is the periodicity of the periodic CSI-RS burst for the nth SCell activation.

  - Tfirst_ATRS_Scelln is the time to the end of the first complete CSI-RS burst for the nth SCell activation after slot n + THARQ+3msNR slot length where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

  - TATRS_Scelln is the CSI-RS burst for the nth SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

  - Tgap_Scelln is the gap length between two aperiodic CSI-RS bursts for the nth SSB-less SCell.

- Conclusion

Based on the discussion above, the following proposals and observations are concluded.

Proposal 1:  RAN4 should consist on the previous agreement that the spec should support the EPRE difference larger [12]dB.

Proposal 2:  Clarify that EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)|, where f1 and f2 is the center frequency of reference Cell and SSB-less SCell.

Observation 1: The methodology of Option 3 can be used in P-TRS case, to update the requirements with one more P-TRS sample when EPRE difference larger than [12]dB.

Observation 2: Whether the methodology of Option 3 can be used in A-TRS case should be further studied.

Observation 3: The methodology of Option 3 will cause longer activation delay when EPRE difference larger than [12]dB.

Proposal 3:  For all to-be-activated SSB-less SCells are in contiguous bands and have same QCL source cell case, the side condition from single SSB-less SCell case can be reused, the requirement could be defined as:

Tactivation_time = min(Tactivation_time_SCell1, ... , Tactivation_time_SCelln),  n = 1, .. , the number of to-be-activated SSB-less SCells in contiguous bands and have same QCL source cell

- Tactivation_time_SCelln = Tfirst_TRS_Scelln + TTRS_Scelln + 5ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17

- Tactivation_time_SCelln =Tfirst_ATRS_Scelln + Tgap_Scelln + TATRS_Scelln + 5ms, if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

  - Tfirst_TRS_Scelln is the time to the end of the first complete periodic CSI-RS burst for the nth SCell activation after slot n + THARQ+3msNR slot length.

  - TTRS_Scelln is the periodicity of the periodic CSI-RS burst for the nth SCell activation.

  - Tfirst_ATRS_Scelln is the time to the end of the first complete CSI-RS burst for the nth SCell activation after slot n + THARQ+3msNR slot length where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

  - TATRS_Scelln is the CSI-RS burst for the nth SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

  - Tgap_Scelln is the gap length between two aperiodic CSI-RS bursts for the nth SSB-less SCell.

- Reference

- R4-2410269, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.

- R4-2321562, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.

- R4-2403526, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.

- R4-2400488, On SSB-less SCell operation for NES, Apple