3GPP TSG-RAN4 Meeting #112	R4-2411792

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda item:	6.1.2.4

Source: 	Qualcomm Incorporated

Title: 	Additional simulation results for 1Rx RedCap UEs without frequency hopping

Document for:	Information

# Introduction

RAN4 revised the simulation assumptions for PRS measurement accuracy for 1Rx RedCap UEs without frequency hopping [2]. This paper includes simulation results for PRS BW of 24 PRBs in fading propagation conditions, which were missing in our previous contribution [1].

- UE Rx-Tx time difference

- PRS-RSRP

- RSTD

# Simulation results for UE Rx-Tx time difference

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -6 dB | SINR = -10 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | 188 | 218 |
|  | 52 |  | 128 | 4 | 1 | 1 |  |  |
|  | 104 |  | 64 | 4 | 1 | 1 |  |  |
|  | 24 | 30 | 128 | 4 | 4 | 1 | 151 | 178 |
|  | 48 |  | 64 | 4 | 1 | 1 |  |  |

Table 1: UE Rx-Tx simulation results for FR1 in TDL-A.

# Simulation results for PRS-RSRP

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -6 dB | SINR = -10 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | 6.4 | 5.6 |
|  | 52 |  | 128 | 4 | 1 | 1 |  |  |
|  | 104 |  | 64 | 4 | 1 | 1 |  |  |
|  | 24 | 30 | 128 | 4 | 4 | 1 | 5.2 | 4.7 |
|  | 48 |  | 64 | 4 | 1 | 1 |  |  |

Table 2: PRS-RSRP simulation results for FR1 in TDL-A.

# Simulation results for RSTD

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINRref = -6 dB, SINRi = -6 dB | SINRref = -6 dB, SINRi = -10 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | 187 | 220 |
|  | 52 |  | 128 | 4 | 1 | 1 |  |  |
|  | 104 |  | 64 | 4 | 1 | 1 |  |  |
|  | 24 | 30 | 128 | 4 | 4 | 1 | 125 | 140 |
|  | 48 |  | 64 | 4 | 1 | 1 |  |  |

Table 3: RSTD simulation results for FR1 in TDL-A.

# References

[1] R4-2311984, Simulation results for 1Rx RedCap UEs without frequency hopping – updated side-condition for fading channels, Qualcomm Inc., RAN4#108

[2] R4-2310075, Updated simulation assumptions for 1Rx RedCap UE PRS measurement without FH, RAN4#107