3GPP TSG-RAN WG4 Meeting #112	R4-2411950

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on NR NTN system parameters for Ku band

Agenda item:	8.9.3

Document for:	Approval

Introduction

In this paper, we provide our views on the support of Ku band for NR NTN. Core parts of the objectives mentioned in the WID are copied below for ease of reading.

Objectives:

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

Discussion

Frequency range

In TS 38.101-5, FR1-NTN frequency range is defined for 410-7125 MHz and FR2-NTN frequency range is defined for 17300 -30000 MHz. Furthermore, in TS 38.101-1/2, FR1 is defined for 410-7125 MHz, FR2-1 for 24250 – 52600 MHz and FR2-2 for 52600 – 71000 MHz. There is no existing frequency range for Ku band in both the NTN and TN specifications. It is proposed to further discuss whether we define a new frequency range covering Ku band or extend the existing frequency range to cover Ku band either from FR1-NTN or FR2-NTN.

The Ku Band is not within any of the existing frequency ranges.

From UE requirement point of view, if the UE RF architecture is similar to FR1-NTN and if its requirement framework (i.e., conducted based) can be reused, we can extend FR1-NTN covering Ku band. Or if the UE RF architecture is similar to FR2-NTN and if its requirement framework (i.e., OTA based) can be reused, we can extend FR2-NTN covering Ku band. If the architecture and requirement framework is unique to Ku band, then a new frequency range should be defined. The frequency range 7-24 GHz was once studied for TN in TS 38.820, which can be used as a baseline for discussing the characteristics of this frequency range.

It is proposed to further discuss whether we define a new frequency range covering Ku band(s) or extend the existing frequency range (either FR1-NTN or FR2-NTN) to cover Ku bands.

Nevertheless, which option taking in proposal 1 the Ku band as it is an NTN band must only impact the NTN frequency ranges captured in TS 38.101-5 and TS 38.108.

Any new or extended frequency range covering Ku band(s) can only be reflected in TS 38.101-5 and TS 38.108.

Operating band

The following four operating bands are tabulated from the WI objective [1] in the format of operating band definition.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| #1a | 13750 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| #1b | 13750 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| #2a | 12750 MHz - 13250 MHz | 10700 MHz - 12750 MHz | FDD |
| #2b | 12700 MHz - 13250 MHz | 10700 MHz - 12700 MHz | FDD |

The operating band n256 and below is reserved for FR1-NTN bands. So far down to n254 is reserved. The next FR1-NTN band number available is n253. The operating band n512 and below is reserved for FR2-NTN bands. The next FR2-NTN band number available is n509. Depending on the conclusion above, we can assign the new band numbers for Ku bands. If we define a new frequency range, we can reserve another range of operating band number.

We wait for the agreement on frequency range issue before agreeing to band numbers.

Channel bandwidth and Subcarrier spacing

Priority 1 in the WI objective is to support 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. As mentioned in the WI objective, for FR1-NTN, 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing can be introduced, though it is not restricted to. For FR2-NTN, 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing can be introduced.

FR1 option in the WID suggests only 30 kHz SCS, thought we could also introduce 15 and 60 kHz. It is to be further discussed we can only focus on 30 kHz SCS for FR1 option.

FR2-NTN option is likely to support 60 kHz and 120 kHz SCS like the existing FR2 bands.

Support of NTN CA is not in the scope of this WID. Single carrier operation should be first completed in this WI. Thus operation of aggregated bandwidth 125 MHz UL and 250 MHz DL should be discussed after this WI is completed.

It is for further discussion whether we nail down SCS options for Ku bands.

Channel raster

NR-ARFCN defined in TS 38.101-5 can be reused to Ku band regardless of FR1 and FR2 options mentioned above. For the frequency range of Ku band, the step size of NR-ARFCN is 15 kHz. We propose to assume SCS based channel raster, i.e., 30 kHz for the FR1 option (if 15 and 60 kHz SCS are excluded) and 60 kHz for the FR2 option.

SCS based channel raster is introduced to Ku bands.

Tx-Rx Separation

For all four band plans #1a, #1b, #2a and #3b above, there is no clear guideline about their channelization schemes and channel pairing of UL and DL like existing FR2-NTN bands. At this moment, it is not possible to define the fixed channelization and fixed duplex separation. Therefore, flexible duplex may be an only option for now.

Flexible Tx-Rx frequency separation can be used for Ku band NTN FDD operation.

Sync raster

Current sync raster design in TS 38.101-5 above 3 GHz can support minimum channel bandwidth at any SCS-based channel raster frequency. The existing sync raster design can be reused for Ku bands.

The existing sync raster design is reused for Ku bands.

Conclusion

The following Observations and Proposals were made:

Observation 1: The Ku Band is not within any of the existing frequency ranges.

Proposal 1: It is proposed to further discuss whether we define a new frequency range covering Ku band(s) or extend the existing frequency range (either FR1-NTN or FR2-NTN) to cover Ku bands.

Proposal 2: Any new or extended frequency range covering Ku band(s) can only be reflected in TS 38.101-5 and TS 38.108.

Proposal 3: We wait for the agreement on frequency range issue before agreeing to band numbers.

Proposal 4: It is for further discussion whether we nail down SCS options for Ku bands.

Proposal 5: SCS based channel raster is introduced to Ku bands.

Proposal 6: Flexible Tx-Rx frequency separation can be used for Ku band NTN FDD operation.

Proposal 7: The existing sync raster design is reused for Ku bands.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.