3GPP TSG-RAN WG4 Meeting #112	R4-2412034

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	On Rel-18 NR MIMO evolution for downlink and uplink RRM core requirements maintenance

Agenda item:	5.27.1

Document for:	Discussion

Introduction

Rel-18 MIMO Evo work item was closed in the last RAN4 meeting including the performance part of the work item. However, some unfinished issues were still listed in the way forward of RAN4#111 meeting in [1]. In this contribution we discuss these issues.

Discussion

Two TAs timing requirements

For two TAs timing requirements, in the last RAN4 meeting RAN4#111, the following agreements were made and issues discussed [1]:

Issue 1-1-2: Applicability of timing requirements for 2 TAs. < Agreement> - Timing requirements for two TAs are applicable for PUCCH/PUSCH/SRS, intra-cell/inter-cell PDCCH ordered RACH CFRA. Issue 1-1-3: For PDCCH order RACH, uplink timing and DL timings association. < Agreement> - For intra-cell, confirm the association in RAN1 LS R4-2407007. - For inter-cell, FFS on: - If the PRACH is triggered towards serving cell PCI, the uplink transmission takes place NTA offset×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the reference cell associated with the coresetPoolIndex having the same TAG as that serving cell, where NTA offset is the first n-TimingAdvanceOffset value, - If the PRACH is triggered towards active additional cell PCI, the uplink transmission takes place NTA offset×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the reference cell associated with the coresetPoolIndex having the same TAG as that active additional PCI, where NTA offset is the second n-TimingAdvanceOffset value.

In the last meeting, we agreed for two TAs on cross-TRP PDCCH order, i.e., where one TRP can trigger through PDCCH order PRACH towards another TRP, assuming only CFRA.

RAN1 LS R4-2407007 specified DL reference timing association for intra-cell deployments. On the other hand, in the last RAN4 meeting we agreed that such DL reference timing association should be specified also for inter-cell deployments, as two TAs has been defined for mDCI mTRP, which can be either intra-cell or inter-cell.

For PRACH transmission for inter-cell multi-DCI based multi-TRP operation with two TAs,

- if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of active DL TCI state(s) of the reference cell associated with the first TAG, where NTA offset is as defined in n-TimingAdvanceOffset.

- if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of active DL TCI state(s) of the reference cell associated with the second TAG, where NTA offset is as defined in n-TimingAdvanceOffset2.

eUTCI DL switching requirements for multi-DCI scenario

For DL TCI state switching requirements, in the last RAN4 meeting RAN4#111, the following agreement was made [1]:

Issue 1-2-1: For mDCI mTRP, OL definition? < Agreement> - OL = 1 if the SSB overlaps or is adjacent to the SSB from the other TRP in FR2 and the SSB is associated to the TRP with the lowest corestPoolIndex, 0, otherwise.

The issue discussed is about overlapping SSBs when performing DL TCI state switching in a mDCI mTRP scenario. Indeed, when the SSBs which are used for fine time/frequency (T/F) tracking for the two activated downlink DL TCI states are overlapping or adjacent, the UE is not expected to be able to do the T/F tracking for each of the first SSBs after processing the MAC-CEs, but the UE would need an additional delay (SSB periodicity, added with the parameter OL=1) to be able to synchronize with both SSBs. This is because the UE may be receiving the two first SSBs from different directions with different panels, and when the UE is not capable of simultaneous reception with two panels, the UE would need to switch the panel to be able to synchronize with both SSBs. When the SSBs are overlapping or adjacent to each other, the UE would simply not have time to switch the panel so fast that it would be able to measure both SSBs on the first occasion (overlapping or adjacent).

On the other hand, in RAN4#109 [2] we made the following agreements on this issue for the case when the SSB periodicity is different between two TRPs:

Issue 3-1-7-a For mDCI mTRP, how to specify DL TCI state switching requirements for eUTCI if UE not supporting two TAs? Agreement: - [Known]: THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc + OL*TSSB) - [Unknown]: THARQ + 3Nslotsubframe,µ+ TL1-RSRP + TOuk*(Tfirst-SSB + TSSB-proc + OL*TSSB) - OL=1 if SSB overlaps or adjacent to SSB from other TRP in FR2 and SSB periodicity is less than that of other TRP, 0 otherwise Issue 3-1-8-a: For mDCI mTRP, how to specify DL TCI state switching requirements for eUTCI if UE supporting two TAs and but not supporting RTD>CP? Agreement: - [Known]: THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc + OL*TSSB) - [Unknown]: THARQ + 3Nslotsubframe,µ+ TL1-RSRP + TOuk*(Tfirst-SSB + TSSB-proc + OL*TSSB) - OL=1 if SSB overlaps or adjacent to SSB from other TRP in FR2 and SSB periodicity is less than that of other TRP, 0 otherwise

In fact, in RAN4#109 we agreed that, for different SSB periodicities between the two TRPs, the additional SSB periodicity delay for a TRP is added if such SSB periodicity is less than that of other TRP. This aspect makes sure that, for example as shown in Figure 1 with SSB1 transmitted by TRP1 and SSB2 transmitted by TRP2, if SSB1 periodicity is smaller than SSB2 periodicity, then the UE first does the T/F tracking for SSB2, waits one SSB1 periodicity, and then does the T/F tracking for SSB1, such that the total TCI switching delay is minimized.

Figure 1. DL TCI state switching delay in multi-DCI mode with overlapping SSBs and different SSB periodicity.

RAN4#109 agreements for MAC CE-based mDCI mTRP DL TCI switching allowed to minimize the total TCI switching delay.

In the agreed CR the previously agreed condition was replaced with the new condition, which led to the case of different SSB periodicity being handled similarly as the same SSB periodicity case. We think there is still benefit to consider the different SSB periodicity case separately, as through the earlier agreement the delay would be shorter in this case. Because of that we think that the last RAN4 meeting RAN4#111 agreement should be amended by clarifying that, only when the two SSB periodicities are the same, then the additional SSB periodicity is added to the TCI state associated to the TRP with lowest coresetPoolIndex, 0.

For mDCI mTRP DL TCI state switching in FR2, clarify the last meeting RAN4#111 agreement including the RAN4#109 agreement as follows:

- OL = 1, if the SSB overlaps or is adjacent to the SSB from the other TRP, and

  - the SSB periodicity is less than that of other TRP, or

  - the SSB periodicity is the same of that of the other TRP and the SSB is associated to the TRP with the lowest coresetPoolIndex.

- Otherwise, OL = 0.

eUTCI UL switching requirements for multi-DCI scenario

For UL TCI state switching requirements, in the last RAN4 meeting RAN4#111, the following issue was discussed [1]:

Issue 1-2-2: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs (RTD<CP and RTD>CP)? < Way forward > - Option 1 (Apple, Samsung) - For joint TCI state, no additional DL RS tracking time for UL TCI state switching. - For separate UL TCI state, If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as: - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms) - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex - For other cases of separate UL TCI state, no additional DL tracking is needed. - Option 2: (Huawei, MediaTek, Ericsson) - No additional DL RS tracking time for UL TCI state switching

Regarding the UL TCI state switching, it is still open how to define the requirements for UEs supporting two TAs. At the same time, it has already been agreed in RAN4#109 [2] for UEs not supporting two TAs that Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently, so with no additional DL RS tracking time for UL TCI state switching. We think that Rel-17 requirements should be applied also for UE supporting two TAs, so we support Option 2, with no additional DL RS tracking time for UL TCI state switching.

Regarding mDCI mTRP UL TCI state switching requirements for UEs supporting two TAs, we support Option 2, i.e., no need to introduce an additional DL RS tracking time.

Conclusion

In this paper, the following Observations and Proposals were made:

Proposal 1: For PRACH transmission for inter-cell multi-DCI based multi-TRP operation with two TAs,

- if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place N"TA offset"×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of active DL TCI state(s) of the reference cell associated with the first TAG, where N"TA offset"  is as defined in n-TimingAdvanceOffset.

- if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place N"TA offset"×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of active DL TCI state(s) of the reference cell associated with the second TAG, where N"TA offset"  is as defined in n-TimingAdvanceOffset2.

Observation 1: RAN4#109 agreements for MAC CE-based mDCI mTRP DL TCI switching allowed to minimize the total TCI switching delay.

Proposal 2: For mDCI mTRP DL TCI state switching in FR2, clarify the last meeting RAN4#111 agreement including the RAN4#109 agreement as follows:

- OL = 1, if the SSB overlaps or is adjacent to the SSB from the other TRP, and

  - the SSB periodicity is less than that of other TRP, or

  - the SSB periodicity is the same of that of the other TRP and the SSB is associated to the TRP with the lowest coresetPoolIndex.

- Otherwise, OL = 0.

Proposal 3: Regarding mDCI mTRP UL TCI state switching requirements for UEs supporting two TAs, we support Option 2, i.e., no need to introduce an additional DL RS tracking time.

Proposal 1: For PRACH transmission for inter-cell multi-DCI based multi-TRP operation with two TAs,

-	if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of active DL TCI state(s) of the reference cell associated with the first TAG, where NTA offset is as defined in n-TimingAdvanceOffset.

-	if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of active DL TCI state(s) of the reference cell associated with the second TAG, where NTA offset is as defined in n-TimingAdvanceOffset2.

Observation 1: RAN4#109 agreements for MAC CE-based mDCI mTRP DL TCI switching allowed to minimize the total TCI switching delay.

Proposal 2: For mDCI mTRP DL TCI state switching in FR2, clarify the last meeting RAN4#111 agreement including the RAN4#109 agreement as follows:

-	OL = 1, if the SSB overlaps or is adjacent to the SSB from the other TRP, and

o	the SSB periodicity is less than that of other TRP, or

o	the SSB periodicity is the same of that of the other TRP and the SSB is associated to the TRP with the lowest coresetPoolIndex.

-	Otherwise, OL = 0.

Proposal 3: Regarding mDCI mTRP UL TCI state switching requirements for UEs supporting two TAs, we support Option 2, i.e., no need to introduce an additional DL RS tracking time.

- References

- R4-2410317, WF on RRM requirements for NR_MIMO_evo_DL_UL, RAN4#111, Fukuoka, Japan, May 20-24, 2024.

- R4-2321563, WF on R18 NR MIMO RRM requirements, Samsung, RAN4#109, Chicago, USA, November 13 – November 17, 2023.