3GPP TSG-RAN WG4 Meeting # 112	R4-2412199

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.27.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on requirements maintenance for Rel-18 MIMO

Document for:	Discussion

# Introduction

The core part and performance part of Rel-18 MIMO was completed with remaining issues captured in WF [1]. In this contribution, we further provide our views on remaining issues.

# Discussion

One of the remaining issues is how to capture the timing requirements for UE supporting two TA considering both intra-Cell and inter-Cell. The relevant agreements are summarized as follows:

Issue 1-1-2: Applicability of timing requirements for 2 TAs. < Agreement> - Timing requirements for two TAs are applicable for PUCCH/PUSCH/SRS, intra-cell/inter-cell PDCCH ordered RACH CFRA. Issue 1-1-3: For PDCCH order RACH, uplink timing and DL timings association. < Agreement> - For intra-cell, confirm the association in RAN1 LS R4-2407007. - For inter-cell, FFS on: - If the PRACH is triggered towards serving cell PCI, the uplink transmission takes place NTA offset×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the reference cell associated with the coresetPoolIndex having the same TAG as that serving cell, where NTA offset is the first n-TimingAdvanceOffset value, - If the PRACH is triggered towards active additional cell PCI, the uplink transmission takes place NTA offset×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the reference cell associated with the coresetPoolIndex having the same TAG as that active additional PCI, where NTA offset is the second n-TimingAdvanceOffset value.

In the following part, we firstly present the relevant RAN1/2 design to have comprehensive understanding of the feature, in terms of following three aspects:

- The mapping between the TA command and TAG

- How to determine the DL reference timing

- How to determine the NTA offset

The mapping between the TA command and TAG

According to following description in RAN2 spec 38.321and 38.331, UE can tell the TAG to which the TA command is applied, which is shown as follows；

TS 38.321

TS 38.331

It could be observed that, UE can tell the TAG to which the TA command is applied by the tag2-flag and TI filed in the TA command. And there is no differentiation between intra-cell and inter-cell.

Observation 1: UE can tell the TAG to which the TA command is applied by the tag2-flag and TI filed in the TA command. And there is no differentiation between intra-cell and inter-cell.

How to determine the NTA offset

Regarding to how to determine the NTA offset, the corresponding requirements could be found in TS 38.213 and TS 38.331 which is shown as follows:

TS 38.331

- For inter-cell

  - n-TimingAdvanceOffset1 applies to transmission toward serving cell PCI

  - n-TimingAdvanceOffset2 applies to transmission toward serving cell with different PCI

For PRACH transmission, the corresponding RAN1 agreements are as follows:

Agreement For PRACH or Msg.A transmission, for inter-cell Multi-DCI Multi-TRP operation with two TAs, first value and first DL reference timing are applied when PRACH is triggered towards serving cell PCI, second value and second DL reference timing are applied when PRACH is triggered towards active additional cell PCI.

UE will use first NTA offset towards serving cell PCI using second NTA offset towards serving cell with different PCI. Based on analysis above, for PRACH and other transmission for inter-Cell, UE determine the NTA offset as follows:

  - n-TimingAdvanceOffset1 applies to transmission toward serving cell PCI

  - n-TimingAdvanceOffset2 applies to transmission toward serving cell with different PCI

Observation 2: For PRACH and other transmission for inter-Cell, UE determines NTA offset when configured with 2 Tas as follows:

  - n-TimingAdvanceOffset applies to transmission toward serving cell PCI

  - n-TimingAdvanceOffset2 applies to transmission toward serving cell with different PCI

How to determine the DL reference timing

To determine the DL reference timing, it is further divided into PRACH transmission and transmission other than PRACH. Per previous RAN4 agreements which is shown as follows, which can apply to both intra-cell and inter-cell.

Issue 1-1-2: Clarification of association of UL TCI states and DL RS < Agreement>: - [One of] downlink reference signal(s) associated with the same coresetPoolIndex as uplink signal

For PRACH transmission, for intra-cell, according to the LS from RAN1, the association is as follows:

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the following for determining the PRACH timing: Alt 4: - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 0: - If “PRACH association indicator” is 0, DL reference timing associated with coresetPoolIndex value 0 is used. - If “PRACH association indicator” is 1, DL reference timing associated with coresetPoolIndex value 1 is used. - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 1: - If “PRACH association indicator” is 0, DL reference timing associated with coresetPoolIndex value 1 is used. - If “PRACH association indicator” is 1, DL reference timing associated with coresetPoolIndex value 0 is used.

For inter-cell, according to RAN1 agreement,

Agreement For PRACH or Msg.A transmission, for inter-cell Multi-DCI Multi-TRP operation with two TAs, first value and first DL reference timing are applied when PRACH is triggered towards serving cell PCI, second value and second DL reference timing are applied when PRACH is triggered towards active additional cell PCI.

Ts 38.212

Based on above RAN1 agreement/spec, for intra-cell, the situation is clear. For inter-cell case, there is no detailed description as intra-cell in the LS from RAN1. The relevant RAN1 agreement states that first DL reference timing when PRACH is triggered towards serving cell PCI and the second DL reference timing applied when PRACH is triggered towards active additional cell PCI. Regarding the definition of “first/second” DL reference timing, we can refer to following requirements in TS 38.212 that the first DL timing is associated with the first TAG and the second DL timing is associated with the second TAG.

Based on the analysis above, we can have a full picture of mDCI with 2 TA for intra-cell and inter-Cell as follows:

For intra-cell:

- The mapping between the TA command and TAG:

  - UE can tell the TAG to which the TA command is applied by the tag2-flag and TI filed in the TA command.

- How to determine the DL reference timing

  - PRACH:

    - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 0:

      - If “PRACH association indicator” is 0, DL reference timing associated with coresetPoolIndex value 0 is used.

      - If “PRACH association indicator” is 1, DL reference timing associated with coresetPoolIndex value 1 is used.

    - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 1:

      - If “PRACH association indicator” is 0, DL reference timing associated with coresetPoolIndex value 1 is used.

      - If “PRACH association indicator” is 1, DL reference timing associated with coresetPoolIndex value 0 is used.

  - Others:

    - [One of] downlink reference signal(s) associated with the same coresetPoolIndex as uplink signal

- How to determine the NTA offset

  - NA

For inter-cell:

- The mapping between the TA command and TAG

  - UE can tell the TAG to which the TA command is applied by the tag2-flag and TI filed in the TA command.

- How to determine the DL reference timing

  - PRACH:

    - If “PRACH association indicator” is 0, DL reference timing associated with first TAG is used.

    - If “PRACH association indicator” is 1, DL reference timing associated with second TAG is used.

  - Others:

    - [One of] downlink reference signal(s) associated with the same coresetPoolIndex as uplink signal

- How to determine the NTA offset

  - n-TimingAdvanceOffset applies to transmission toward serving cell PCI

  - n-TimingAdvanceOffset2 applies to transmission toward serving cell with different PCI

The above observation could be summarized as following Table.

Table I. Timing related issues for UE supporting two TA for intra-cell and inter-cell

|  |  | Intra-cell | Inter-cell |
| --- | --- | --- | --- |
| mapping between the TA command and TAG |  | Determine the TAG to which the TA command is applied by the tag2-flag and TI filed in the TA command. | Determine the TAG to which the TA command is applied by the tag2-flag and TI filed in the TA command. |
| determine the DL reference timing | PRACH | As defined in LS R1-2403752 | If “PRACH association indicator” is 0, DL reference timing associated with first TAG is used. If “PRACH association indicator” is 1, DL reference timing associated with second TAG is used. |
|  | Other | [One of] downlink reference signal(s) associated with the same coresetPoolIndex as uplink signal | [One of] downlink reference signal(s) associated with the same coresetPoolIndex as uplink signal |
| determine the NTA_offset |  | NA | n-TimingAdvanceOffset applies to transmission toward serving cell PCI n-TimingAdvanceOffset2 applies to transmission toward serving cell with different PCI |

Observation 3: Timing related issues for UE supporting two TA for intra-cell and inter-cell is shown in Table I based on RAN1/2 agreements and formal spec.

Then back to the issue about how to precisely reflect the above requirements in RAN4 spec. The suggested test proposals are as follows:

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing for PUCCH/PUSCH/SRS takes place (NTA+NTA offset)×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex having same TAG as the uplink signal, where NTA is commanded by the network independently for each TAG [TS 38.331]. For PRACH transmission for intra-cell multi-DCI based multi-TRP operation with Two TAs, - for PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 0, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1. - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 1, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0. For PRACH transmission for inter-cell multi-DCI based multi-TRP operation with Two TAs, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG, where NTA offset is as defined in n-TimingAdvanceOffset. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG, where NTA offset is as defined in n-TimingAdvanceOffset2.

Proposal 1: Update the timing requirement for multi-TRP operation with two Tas as follows:

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing for PUCCH/PUSCH/SRS takes place (NTA+NTA offset)×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex having same TAG as the uplink signal, where NTA is commanded by the network independently for each TAG [TS 38.331]. For PRACH transmission for intra-cell multi-DCI based multi-TRP operation with Two TAs, - for PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 0, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1. - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 1, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0. For PRACH transmission for inter-cell multi-DCI based multi-TRP operation with Two TAs, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG, where NTA offset is as defined in n-TimingAdvanceOffset. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG, where NTA offset is as defined in n-TimingAdvanceOffset2.

# Conclusions

Observation 1: UE can tell the TAG to which the TA command is applied by the tag2-flag and TI filed in the TA command. And there is no differentiation between intra-cell and inter-cell.

Observation 2: For PRACH and other transmission for inter-Cell, UE determines NTA offset when configured with 2 Tas as follows:

  - n-TimingAdvanceOffset applies to transmission toward serving cell PCI

  - n-TimingAdvanceOffset2 applies to transmission toward serving cell with different PCI

Observation 3: Timing related issues for UE supporting two TA for intra-cell and inter-cell is shown in Table I based on RAN1/2 agreements and formal spec.

Proposal 1: Update the timing requirement for multi-TRP operation with two Tas as follows:

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing for PUCCH/PUSCH/SRS takes place (NTA+NTA offset)×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex having same TAG as the uplink signal, where NTA is commanded by the network independently for each TAG [TS 38.331]. For PRACH transmission for intra-cell multi-DCI based multi-TRP operation with Two TAs, - for PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 0, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1. - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 1, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0. For PRACH transmission for inter-cell multi-DCI based multi-TRP operation with Two TAs, - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG, where NTA offset is as defined in n-TimingAdvanceOffset. - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG, where NTA offset is as defined in n-TimingAdvanceOffset2.

# References

[1] R4-2410317 WF on RRM requirements for NR_MIMO_evo_DL_UL