3GPP TSG RAN WG4 #112bis															     R4-2414919

Hefei, P.R. China, October 14th – 18th, 2024

Agenda Item:		6.25.2

Source:				InterDigital Canada

Title:		Discussion on gap skipping occasions mechanism - NR XR_Ph3

Document for:		Discussion and decision

# Background

During RAN1 Meeting #117, an LS to RAN4 [1] related to the UE assistance information on skipping RRM measurement occasions for NR XR has been approved. Further, in RAN1 Meeting #118 an agreement was made on the NW scheduling indication for a skipping occasion [2].

In the last RAN4 meeting, the workplan has been agreed while the first discussions on this topic took place. As a result, a WF has been agreed in the meeting in [3], while the coffee break offline discussion notes were officially submitted in [4] for reference in future meetings.

# Discussion

In this contribution, we share our analysis on UE assistance information related to measurements occasions for NR XR case and the NW scheduling mechanism. For convenience we are listing below the LS text [2]:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). To RAN4 ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.

In RAN4 meeting #112, the agreed WF in [3] mention that RAN4 will start with Tx/Rx in occasions of L3 measurements:

## Issue 1-1: Workscope < Agreement>: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed.

# NR gap types and their APPLICABILITY FOR Rel-19

During the offline discussions two big gaps categories were considered and then the Rel-18 Concurrent Gaps:

- Type-1 MG: Gap(s) configured via GapConfig without suffix

- Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17

- Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

Basically, Type 1 MG are supported by all UEs and thus it will be easier to address. Along with L3 measurements, we believe that the priority is to address the generic gaps supported by all UEs. The second priority may be Type 2.

Otherwise, most of the NW configured gaps can be considered, but there are different degrees of complexity in the required study for the RRM measurements accuracy impact on each listed case. Moreover, for the rest of categories defined in the WF table we believe that positioning related gaps shouldn’t be considered, due to their possible impact on Public Safety requirements from administrations.

Observation 1: Type 1 MGs are supported by all UEs.

Proposal 1: RAN4 to consider in Rel-19 normative work, in the order of priorities, Type 1 MG configured via GapConfig without suffix and if time allows Type 2 configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r1.

# mechanism for Measurement occasions skipping

In the last RAN1 meeting the agreement to have the NW signal by a scheduling DCI on skipping a gap occasion raises the question of the timeline to be followed.

In our understanding, the process/mechanism can be outlined as in the following steps:

- Let’s assume that NW configures the UE with MGs.

- Upon reception of the UAI, the NW may decide to schedule or not in the following gap occasion, by sending or not a DCI with the MG skipping indication with “X” slots ahead of the gap occasion.

- If the UE receives the MG skipping indication in a scheduling DCI “X” slots before the gap occasion, it will cancel the measurement in the gap and prepares to demodulate PDSCH or prepare for PUSCH transmissions in the skipped MG occasion. If there is no MG skipping indication, the UE proceeds with measurement in the gap occasion.

According to the LS sent by RAN1, a working assumption was made to support Alt. 1-1 (explicit indication by DCI to skip a particular gap/restriction). In our opinion, the time gap of X slots before the gap occasion can be covered by a variety of scheduling DCI format types as suggested during RAN1 discussions on the subject [7]. Since it is important to consider the timing for both DL and UL scheduling upon skipping the gap, we believe that the legacy behavior corresponding to the application delay of the minimum scheduling restrictions mentioned in subclause 5.3.1 in TS 38.214 can be considered as a baseline for the time gap. For convenience, we are outlining the text of the subclause below:

### 5.3.1 Application delay of the minimum scheduling offset restriction When the UE is scheduled with DCI format 0_1, 0_3, 1_1 or 1_3 with a 'Minimum applicable scheduling offset indicator' field in slot n, it shall determine the K0min and K2min values, if configured respectively, to be applied, while the previously applied K0min and/or K2min values are applied until the new values take effect. If the DCI in slot n also indicates an active DL (UL) BWP change for a serving cell, the indicated K0min (K2min) value in the new active DL (UL) BWP, if configured, is applied from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI. Otherwise, change of applied minimum scheduling offset restriction indication carried by DCI in slot n, shall be applied in slot n+X of the scheduling cell. The UE does not expect to be scheduled with DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field indicating another change to K0min or K2min for the same active BWP of the scheduled cell before slot n+X of the scheduling cell. When the DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field indicating a change to the applied K0min or K2min is contained within the first three symbols of slot n, the value of application delay X is determined by, X=maxK0minOld∙2μPDCCH2μPDSCH,Zμ where K0minOld is the currently applied K0min value of the active DL BWP in the scheduled cell and is zero, if minimumSchedulingOffsetK0 is not configured for the active DL BWP in the scheduled cell, Zµ is determined by the subcarrier spacing of the active DL BWP in the scheduling cell in slot n, and given in Table 5.3.1-1, and µPDCCH and µPDSCH are the sub-carrier spacing configurations for PDCCH of the active DL BWP in the scheduling cell and PDSCH of the active DL BWP in the scheduled cell, respectively, in slot n. After indication of a change to the applied K0min or K2min of the scheduled cell in slot n of the scheduling cell, if there is an active DL BWP change in the scheduling cell before slot n+X, the new K0min and/or K2min values are applied from the first slot no earlier than the start of slot n+X based on the sub-carrier spacing configuration of the active DL BWP in the scheduling cell in slot n. When the DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field is received outside the first three symbols of the slot, value of Zµ from Table 5.3.1-1 is incremented by one before determining the application delay X. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining 'Minimum applicable scheduling offset indicator' field is received outside the first three symbols of the slot, the PDCCH candidate that ends later in time is used. Table 5.3.1-1: Definition of Zµ µ; Zµ. 0; 1. 1; 1. 2; 2. 3; 2. 5; 8. 6; 16.

Here are a few DCI formats mentioned in RAN1 discussions for consideration [7]:

| DCI format | Purpose |
| --- | --- |
| 0_1 | Used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE |
| 0_2 | Used for the scheduling of PUSCH in one cell |
| 0_3 | Used for the scheduling of one PUSCH in one cell, or multiple PUSCHs in multiple cells with one PUSCH per cell |
| 1_1 | Used for the scheduling of one or multiple PDSCH in one cell. |
| 1_2 | Used for the scheduling of PDSCH in one cell. |
| 1_3 | Used for the scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell. |

Table 1: DCI formats and their purposes

However, it can be observed that DCI format 0_2 can be covered by format 0_1 and 0_3, while format 1_1 and 1_3 can fulfill 1_2 as well.

Observation 2: For UL PUSCH, DCI format 0_2 can be covered by format 0_1 and 0_3, while format 1_1 and 1_3 can fulfill 1_2 as well for DL PDSCH, and thus 'Minimum applicable scheduling offset indicator’ from subclause 5.3.1 of TS38.214 can be used as a baseline and would cover the legacy behaviour as well.

Proposal 2: RAN4 to assume subclause 5.3.1 of TS38.214 as a baseline for X offset scheduling determination and MG skipping scope expansion.

# Conclusions

This contribution discussed our analysis related to measurements occasions skipping for NR XR case. Based on the discussion, the following observations and proposals are made:

Observation 1: Type 1 MGs are supported by all UEs.

Proposal 1: RAN4 to consider in Rel-19 normative work in order of priorities Type 1 MG configured via GapConfig without suffix and if time allows Type 2 configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r1.

Observation 2: For UL PUSCH, DCI format 0_2 can be covered by format 0_1 and 0_3, while format 1_1 and 1_3 can fulfill 1_2 as well for DL PDSCH, and thus 'Minimum applicable scheduling offset indicator’ from subclause 5.3.1 of TS38.214 can be used as a baseline and would cover the legacy behaviour as well.

Proposal 2: RAN4 to assume subclause 5.3.1 of TS38.214 as a baseline for X offset scheduling determination and MG skipping scope expansion.

# References

- R1-2405736, “LS on UE assistance information”, RAN1 Meeting #117, Fukuoka, Japan, May 2024

- R1-2407512, “LS on gaps/restrictions that are caused by RRM measurements”, RAN1 Meeting #118, Maastricht, NL, August 2024

- R4-2414043 “WF on RRM requirements for XR_Ph3, RAN4 Meeting #112, Maastricht, NL, August 2024

- R4-2414042 “Coffee-break minutes for [112][225] NR_XR_Ph3, RAN4 Meeting #112, Maastricht, NL, August 2024

- TS 38.133, Requirements for support of radio resource management, Rel-18

- TS 38.214, Physical layer procedures for data, v18.3.0, June 2024

- R1-2405666, “Moderator summary - Enabling TX/RX for XR during RRM measurements”, RAN1#118, Aug 2024