3GPP RAN WG 4 Meeting #112bis	R4-2414920

Hefei, China 14- 18 October 2024

Agenda Item:	6.11.4.

Source:	Spark NZ Ltd.

Title:	Text Proposals for TR 38 908 for BS conformance tests for the Validation of EIRP

Document for:	Approva

- Introduction

# The measurement procedure and validation of expected EIRP mask was discussed during RAN4 110 , RAN4 111 and RAN 4 112 meetings but despite a number of meetings and contributions no agreement has been reached on  key issues such as below.

# Numbers of beams for averaging,

# Numbers of samples in an angular bin (to be defined later),

# Weighting factor per beam

# RF channels for conformance testing

# Nor has been an agreement on a detailed and a step wise procedure for measurement and averaging and conformance testing.

# Additionally, WRC resolution requires compliance with the specified EIRP mean values with sufficient accuracy, measured in terms of a high confidence interval (CI) such as 95%. Regretfully compliance with confidence intervals is being mixed with measurement uncertainty but these are two different metrics and should NOT be mixed. Confidence interval arises when the value of a parameter of a random variable is to be estimated from measurements. Its compliance will determine the number of beams needed for the mask measurements and averaging. Measurement uncertainty is like a tolerance on the accuracy of a measurement and even applies to deterministic variables.

# It is to be noted that the measurements are to be performed over a single base station and the expectation is to be computed from a number of measurements – say N and assumed to be an estimate of the expectation of the population

# Text proposals for TR 38 908 are provided.

- Expected EIRP calculation.

# There are two key steps in the WRC Resolution:

1. The expected EIRP, , calculation requires the computation of   Paveθ0,φ0  in equation (1) as the first step of a two-step process. This represents the averaging over N beams, each with unique elevation and azimuth angles, (αn, βn), ∀n={1,2,…,N}, within the steering range of the base station for a given arbitrary reference angles, (θ0, ϕ0), respectively. The reference angles may be assumed as zero. This is given as below:

Paveθ0,φ0=n=1NwnPθ0,φ0 ; αn,βn                                            1

2. The second step- a nested average- involves putting the result of the first step, as shown in equation (2), into another averaging process over the azimuth and elevation angles, denoted as ϕ and θ. Note that in this case, θ is defined as the elevation angle above the horizon, in an abuse of mathematical notation relative to equation (1).

EIRPθL , θH=12πsinθH-sinθLθLθH-ππPaveθ0,φ0 cosθ dφ dθ                          (2)

- Definitions

3.1 Bins and Bin Sizes

The angles θL, θH in eqn (2) are the lower and upper limits of the angles above the horizon in the WRC Resolution. There are seven bins and their ranges are shown below. For example, the values of θL, θH in the first bin are zero and 5º respectively.

Table 1: Maximum allowed Expected level as function of elevation angular range

| Bin Number | Bin Size (Degrees) | Elevation angular ranges θL≤θ<θH (Degrees) | Expected EIRP (dBm/MHz) |
| --- | --- | --- | --- |
| 1 | 5 | 0<q<5 | 27 |
| 2 | 5 | 5<q<10 | 23 |
| 3 | 5 | 10<q<15 | 19 |
| 4 | 5 | 15<q<20 | 18 |
| 5 | 10 | 20<q<30 | 16 |
| 6 | 30 | 30<q<60 | 15 |
| 7 | 30 | 60<q<90 | 15 |

It is to be noted that the bin sizes in the above table are different for bins 6 and 7 relative to bins 1 to 5 respectively.

3.2 Steering Range and Radiation range

There is a need to define two ranges and their values.

Steering range:

- -π/3 ( -60 degrees) ≤Φ ≤ π/3 ( 60 degrees)

- 90º≤θ ≤100º

Radiation range

- -π ( -180 degrees) ≤Φ ≤ π ( 180 degrees)

- 0º≤θ ≤180º

These two ranges are shown in the Figure (1) below:

Fig ( 1) : Radiation and Steering ranges

It is noted that the steering range is a subset of the radiation range. Whilst it is not however stated, is the averaging in step 1 is over the radiation range.

- Proposal 1 : Confirm  steering and radiation range values as per specified above

- Proposal 2 : Step 1 averaging is to be done over the entire radiation range

- Step Wise Procedure

- It is assumed that the angles θ and φ are uniform random variables within the steering range. Choose a tuple (θetilt , φescan) from the steering range to define a beam according to a user location.

- Create a beam P( α1, β1) spanning the radiation range .

- Record values of P( α1, β1)  by using a step size of say 1 degrees  for both the elevation and azimuth angles over the radiation range respectively.  This means that P( α1, β1)  is a  361 x 181 matrix ( 361 rows and 181 columns).This is shown below in Fig(2) :

Fig(2) : Storing measured data in a  matrix

- Proposal 3 : Step size in step 3 must be agreed for both domains. Also, it is required to agree that the step size is equal for both domains. Here it is assumed that the step size is equal and for both domains is 1 degree. Noting that a one-degree step size will only result in 5 points for averaging in a bin of width 5 degrees. The variables θ0,φ0are arbitrary reference angles. The quantity on the left hand side is a function of θ and φ. It should correctly be written as Pave (θ,φ) given θ0,φ0

- Now all the data required to evaluate eqn (2) is available and rest, of the procedure is analysis of the data. Summing the N matrices in step 3 and dividing the sum by N as shown in Fig(3)

Fig(3) : Summing and averaging the N beams

- Proposal 4 : The weighting factor of the beams must be agreed. It is 1/N in Fig (3) as it is assumed that all beams are equally likely.

- Computing the integrals in eqn (2).  The integrand is a function of  θ andφ.   It should be integrated over the limits of   θ and φ  and the result evaluated between the limits  θL and θH There are two alternatives:

Alternative 1: For each bin, compute an average for a given azimuth angle and then average these averages (there are 361 such averages) . Example of bin 1, is shown below:

Fig(4) : computing integrals in eqn (2)- Alternative 1

This is further explained below:

For Bin1, θL =0°, θH = 4° , There are 5 values of Pave(θ)  per azimuth cut. There are 361 azimuth cuts. Sum at all the 361 azimuth cuts:

- θL=0θH=4Pave(θ,φ)φ=-180o + …………+θL=0θH=4Pave(θ,φ)φ=+180o

Each summation contains 5 elements. Scale each sum by Cos (θ)as below, by the center of the angle. Divide the scaled sum by (5 ×361) to get the expected EIRP of the first bin

Scale this again by dividing with  12π(SinθH-SinθL) where  θH  and  θL  bin 1 are 4 and zero degrees respectively.

Alternative 2: The other alternative is averaging over the azimuth and then averaging over the bin as shown below for bin 1:

Fig(5); Computing integrals in step 2, Alternative 2

Each of the cells in a bin are summed over the 361 azimuth cuts. This sum is scaled by Cos (θ)  as shown below. This is similar to Alternative (1). All the five cells in the above are of bin(1) are summed and the sum is divided by (5 ×361)  to  get EIRP of first bin.

Scale this again by dividing the above by   12π(SinθH-SinθL)

Both alternatives will yield the same result.

Proposal 5 : RAN4 should leave it to individual IMs to choose amongst Alternative 1 or 2

# 5	Confidence Intervals

# 5.1 Measurement Uncertainty and Confidence intervals : Are they the same?

# During the RAN4 111 meeting there was discussion that seem to imply that CIs are the same as measurement uncertainty (MU). This is not the case. The MU concept relates to the tolerance levels in making a single measurement, say the EIRP, in a given direction over a large duration of time, where the term “large” implies a duration which is able to sufficiently capture the tolerance levels. Another different example is the value of a simple electrical resistor that has been designed to have a tolerance level of +/- x %. This is the MU one should be thinking about, which is not to be confused with the confidence interval (that in case of EIRP) captures not only the temporal, but the spatial variation levels across different directions of the EIRP, to measure the parameters of its distribution and to show that the measured parameters (with a certain sample size and hence sample statistics) represent the corresponding values of a population. Therefore, to accurately estimate the CIs, say of the EIRP, we have to quantify the tolerance levels in a given direction.

# Observation 1. : MU and CI are different quantities and MU is taken into account when estimating the CI

# Observation 2. : Specific to the case of the expected EIRP mask, an upper limit on the MU value can be defined per elevation bin size (e.g., 0 – 5º for bin 1). This value has to be correlated with the MUs of the constituent components (e.g., RF up-conversion circuits, filters, processing, etc.) involved in emitting the beam in a given direction whose EIRP can be measured. Furthermore, the MU in the stepwise estimation procedure has to be considered, where computation on the 1º matrix elements are suggested (see section 4, point 3 and proposal 3). This implies that the population of the matrix with data is based on accurately estimated data. For example, if the minimum angle which a beam can be sampled at is say 2º, then the matrix size has to be appropriately scaled. This would result in further errors as the size of the bin is only 5º. In other words, in a 5º bin, there would be just 2 samples to average!

# Proposal 6 : MU and CIs are different measures and one should exchange one for the
            other but MU needs to be taken into account when estimating the CI

# 5.2 Computing a Confidence Interval

# Compliance with WRC 23 specified mean values for EIRP requires that the mean values measured from a single base station are sufficiently accurate, for having a high confidence interval (CI) such as 95%, as specified in the Annex to the Resolution of WRC-23. The following text shows theoretically how the CI is derived, and the number of samples needed to achieve a high CI.

# Assume that the expected e.i.r.p values to be complied with are referred to as “Population Means “. The WRC requirement is to have a confidence interval (CI) on the population mean (per elevation angle bin) from a sample of measurements (say from the N beams) on a single base station– an example of 95% is mentioned. CIs are in fact a margin of error on a point estimate of the parameter (in this case the mean of a population) derived from the given samples. # There is no value for standard deviation σ specified of the population for each bin (angular range). Nor is the distribution type of EIRP specified; though it can be assumed that the interferers resulting in interference to the FSS uplink come from very large numbers of base stations, and in this case, the Central Limit Theorem applies, and the resulting interference is Normally distributed, and therefore, the corresponding EIRP requirements are also Normal. # So, the problem boils down to determining the confidence intervals on the mean of a Normal distribution. If the Normality assumption cannot be confirmed, we will need to derive the confidence intervals via the students t-distribution as this is the norm in statistics when the distribution type in unknown. # If the population standard deviation σ is unknown, as is the case here, we will have to estimate it by the sample standard deviation, s. # We also assume that the N beams are unbiased. # We could use the sample standard deviation s as an estimate of σ, but it is better to use the students t distribution when s is used as an estimate for σ.: # Observation 3: The calculation of confidence intervals on the mean requires the knowledge of standard deviations. as this is not specified, we could estimate the standard deviation from that of the given samples. # Observation 4 : The calculation of confidence intervals also requires the knowledge of the underlying distribution type. As this is not specified, we have two choices: (1) Either to assume that the distribution of EIRP is Normal, or (2) a students t distribution. # Observation 5: The calculation of CIs assume the N samples are unbiased. # A 95% confidence interval on the population mean (per-bin) with unknown standard deviation for a normal distribution is then given by: # Lower limit= - Mean- 1.96. s/√N # Upper limit = - Mean + 1.96.s/√N # A 95% confidence interval on the population mean (per bin) with unknown standard deviation for a students t distribution is then given by [ Chapter 9, 2]: # Lower limit= - Mean- 2.04. s/√N # Upper limit = - Mean +2.04. s/√N - In both cases, we can think of s/√Nas an error in the estimated mean. # Observation 6: For both types of distribution choices above, the number of samples ( in this case the beams) N, and the accuracy of samples will significantly influence the upper and lower bounds. # Observation 7: As the distribution of EIRP is not specified, students t distribution may be preferable to estimate the CI. # The students t-distribution is like the normal distribution but does not assume an apriori knowledge of the population mean and standard deviation as the normal distribution does. # The probability density function of the t-distribution is as follows, where Γ represents the gamma function and ν is the degrees of freedom (which is equal to one less than the sample size): # f(t) = Γν+12νπ Γυ2 1+t2υ-υ+12. # For larger sample sizes ( > 20) , both the t and normal distribution are very similar as shown above. - Proposal 7 : if the numbers of samples (N) is large say 30 the t distribution and Normal distribution CIs are similar and any can be chosen. - Proposal 8: Capture all of the above text in sections 2 and 3 in TR for upper 6 GHz- skeleton noted during RAN4 110bis. Specific proposals are given below in accordance with the section names in the blank TR 38 908. # 7 Weighting factors # The measurement procedure requires agreement on the weighting factors for the beams to be used in averaging. We have conducted an analysis of the customer distribution in the azimuths of many sectors in our network. We have observed that the customers are uniformly distributed. Therefore all beams must be uniformly weighted. A non unform weighting may also be negatively viewed by regulators especially if the customer locations are unform and the beam weightings do no match. # Proposal 9: Unform weighting of beams must be used # 8 RF Channels for measurement # The upper 6 GHz band is some 700 MHz wide . assuming the RF channels for measurements are 100 Mhz wide. It is important to do EIRP compliance measurements that represents the performance over all the band. This could be done by doing measurements over multiple channels that are located in various places in the band. However this could lead to excessive numbers of measurements. On the other hand, performing measurements in two channels , one located at each end of the band may be a good compromised between reducing measurement overhead and ensuring the measurements apply all over the band. # Proposal 10 : Two channels should be used for measurement, one at either end of the band. Furthermore measurements should be per polarisation.

# 9	Summary

# This contribution provides a method for the measurement of EIRP so that WRC 23 conditions [1] are complied with. Following proposals are made:

Proposal 1 : Confirm  steering and radiation range values as per specified in section 3- Fig(1).

Proposal 2 : Step 1 averaging is to be done over the entire radiation range.

Proposal 3 : Step size in step 3 must be agreed for both domains. Also, it is required to agree that the step size is equal for both domains. Here it is assumed that the step size is equal and for both domains is 1 degree. The variables θ0,φ0are arbitrary reference angles. The quantity on the left hand side is a function of θ and φ. It should correctly be written as Pave (θ,φ) given θ0,φ0

Proposal 4 : The weighting factor of the beams must be agreed. It is 1/N in Fig(3) as it is assumed that all beams are equally likely.

Proposal 5 : RAN1 should leave it to individual IMs to choose amongst Alternative 1 or 2

Proposal 6 : MU and CIs are different measures and one should exchange one for the other but MU needs to be taken into account when estimating the CI

Proposal 7: If the numbers of samples (N) is large say >20 the students t distribution and Normal distribution CIs are similar, and any can be chosen.

Proposal 8 : Uniform weightings of beams must be used.

Proposal 9: Two channels should be used for measurement, one at either end of the band. Furthermore, the measurements should be per polarisation.

Proposal 10: Capture all of the above text in sections 2 and 3 in TR for upper 6 GHz- skeleton noted during RAN4 110bis. Specific proposals are given below in accordance with the section names in the blank TR 38 908.

# 7.0 Text Proposals

Section 3.1 Definitions and Symbols

Bins and Bin  Sizes

The angles θL, θH in eqn (2) are the lower and upper limits of the angles above the horizon in the WRC Resolution. There are seven bins and their ranges are shown below. For example, the values of θL, θH in the first bin are zero and 5º respectively.

Table 1: Maximum allowed Expected level as function of elevation angular range

| Bin Number | Bin Size (Degrees) | Elevation angular ranges θL≤θ<θH (Degrees) | Expected EIRP (dBm/MHz) |
| --- | --- | --- | --- |
| 1 | 5 | 0<q<5 | 27 |
| 2 | 5 | 5<q<10 | 23 |
| 3 | 5 | 10<q<15 | 19 |
| 4 | 5 | 15<q<20 | 18 |
| 5 | 10 | 20<q<30 | 16 |
| 6 | 30 | 30<q<60 | 15 |
| 7 | 30 | 60<q<90 | 15 |

It is to be noted that the bin sizes in the above table are different for bins 6 and 7 relative to bins 1 to 5 respectively.

Steering Range and Radiation range

There is a need to define two ranges and their values.Type equation here.

Steering range:

- -π/3 ( -60 degrees) ≤Φ ≤ π/3 ( 60 degrees)

- 90º≤θ ≤100º

Radiation range

- -π ( -180 degrees) ≤Φ ≤ π ( 180 degrees)

- 0º≤θ ≤180º

These two ranges are shown in the Figure (1) below:

Fig ( 1) : Radiation and Steering ranges

It is noted that the steering range is a subset of the radiation range.

Section 6.3.1 Testing beams directions

The beams cover the azimuth and elevation steering ranges as described in section 3.1.

Section 6.3.2  Weighting factors for testing beams

As the customers in a sector are uniformly distributed, all beams must be uniformly weighted.  Therefore for say N beams the weighting factor is 1/N for all beams.  A non unform weighting may also be negatively viewed by regulators especially if the customer locations are unform and the beam weightings do not match

Section 6.3.2  Spatial Sampling Grid

The sampling granularity for azimuth shall be no more than 10 degrees and elevation no more than 1 degrees.

Section 6.5.1  Initial RF conditions

The upper 6 GHz band is some 700 MHz wide . assuming the RF channels for measurements are 100 Mhz wide. It is important to do EIRP compliance measurements that represent  the performance over all the band. Measurements in two channels , one located at each end of the band  need to be done :

6425- 6525 MHz

7025- 7125 MHz

This represents a good compromise between reducing measurement overhead and ensuring the measurements apply all over the band.

Section 6.5.2 Test Proceedure

# There are two key steps in the WRC Resolution:

1. The expected EIRP, , calculation requires the computation of   Paveθ0,φ0  in equation (1) as the first step of a two-step process. This represents the averaging over N beams, each with unique elevation and azimuth angles, (αn, βn), ∀n={1,2,…,N}, within the steering range of the base station for a given arbitrary reference angles, (θ0, ϕ0), respectively. The reference angles may be assumed as zero. This is given as below:

Paveθ0,φ0=n=1NwnPθ0,φ0 ; αn,βn                                            1

2. The second step- a nested average- involves putting the result of the first step, as shown in equation (2), into another averaging process over the azimuth and elevation angles, denoted as ϕ and θ. Note that in this case, θ is defined as the elevation angle above the horizon, in an abuse of mathematical notation relative to equation (1).

EIRPθL , θH=12πsinθH-sinθLθLθH-ππPaveθ0,φ0 cosθ dφ dθ                          (2)

# Step Wise Procedure

It is assumed that the angles θ and φ are uniform random variables within the steering range. Choose a tuple (θetilt , φescan) from the steering range to define a beam according to a user location.

Create a beam P( α1, β1) spanning the radiation range .

Record values of P( α1, β1)  by using a step size of say 1 degrees  for both the elevation and azimuth angles over the radiation range respectively.  This means that P( α1, β1)  is a  361 x 181 matrix ( 361 rows and 181 columns). This is shown below in Fig(2) :

Fig(2) : Storing measured data in a  matrix

Now all the data required to evaluate eqn (2) is available and rest, of the procedure is analysis of the data. Summing the N matrices in step 3 and dividing the sum by N as shown in Fig(3)

Fig(3) : Summing and averaging the N beams

Computing the integrals in eqn (2). There are two alternatives:

Alternative 1: For each bin, compute an average for a given azimuth angle and then average these averages (there are 361 such averages) . Example of bin 1, is shown below:

Fig(4) : computing integrals in eqn (2)- Alternative 1

This is further explained below:

For Bin1, θL =0°, θH = 4° , There are 5 values of Pave(θ)  per azimuth cut. There are 361 azimuth cuts. Sum at all the 361 azimuth cuts:

- θL=0θH=4Pave(θ,φ)φ=-180o + …………+θL=0θH=4Pave(θ,φ)φ=+180o

Each summation contains 5 elements. Scale each sum by Cos (θ)as below, by the center of the angle. Divide the scaled sum by (5 ×361) to get the expected EIRP of the first bin

Scale this again by dividing with  12π(SinθH-SinθL) where  θH  and  θL  bin 1 are 4 and zero degrees respectively.

Alternative 2: The other alternative is averaging over the azimuth and then averaging over the bin as shown below for bin 1:

Fig(5); Computing integrals in step 2, Alternative 2

Each of the cells in a bin are summed over the 361 azimuth cuts. This sum is scaled by Cos (θ)  as shown below. This is similar to Alternative (1). All the five cells in the above are of bin(1) are summed and the sum is divided by (5 ×361)  to  get EIRP of first bin.

Scale this again by dividing the above by   12π(SinθH-SinθL)

Both alternatives will yield the same result.

# Section 6.6 Measurement Uncertainty and Confidence Intervals

# Measurement Uncertainty and Confidence intervals : Are they the same?

# During the RAN4 111 meeting there was discussion that seem to imply that CIs are the same as measurement uncertainty (MU). This is not the case. The MU concept relates to the tolerance levels in making a single measurement, say the EIRP, in a given direction over a large duration of time, where the term “large” implies a duration which is able to sufficiently capture the tolerance levels. Another different example is the value of a simple electrical resistor that has been designed to have a tolerance level of +/- x %. This is the MU one should be thinking about, which is not to be confused with the confidence interval (that in case of EIRP) captures not only the temporal, but the spatial variation levels across different directions of the EIRP, to measure the parameters of its distribution and to show that the measured parameters (with a certain sample size and hence sample statistics) represent the corresponding values of a population. Therefore, to accurately estimate the CIs, say of the EIRP, we have to quantify the tolerance levels in a given direction.

# Computing a Confidence Interval

# Compliance with WRC 23 specified mean values for EIRP requires that the mean values measured from a single base station are sufficiently accurate, for having a high confidence interval (CI) such as 95%, as specified in the Annex to the Resolution of WRC-23. The following text shows theoretically how the CI is derived, and the number of samples needed to achieve a high CI.

Assume that the expected e.i.r.p values to be complied with are referred to as “Population Means “. The WRC requirement is to have a confidence interval (CI) on the population mean (per elevation angle bin) from a sample of measurements (say from the N beams) on a single base station– an example of 95% is mentioned. CIs are in fact a margin of error on a point estimate of the parameter (in this case the mean of a population) derived from the given samples. There is no value for standard deviation σ specified of the population for each bin (angular range). Nor is the distribution type of EIRP specified; though it can be assumed that the interferers resulting in interference to the FSS uplink come from very large numbers of base stations, and in this case, the Central Limit Theorem applies, and the resulting interference is Normally distributed, and therefore, the corresponding EIRP requirements are also Normal. So, the problem boils down to determining the confidence intervals on the mean of a Normal distribution. If the Normality assumption cannot be confirmed, we will need to derive the confidence intervals via the students t-distribution as this is the norm in statistics when the distribution type in unknown. If the population standard deviation σ is unknown, as is the case here, we will have to estimate it by the sample standard deviation, s. We also assume that the N beams are unbiased. We could use the sample standard deviation s as an estimate of σ, but it is better to use the students t distribution when s is used as an estimate for σ.: Observation 3: The calculation of confidence intervals on the mean requires the knowledge of standard deviations. as this is not specified, we could estimate the standard deviation from that of the given samples. Observation 4 : The calculation of confidence intervals also requires the knowledge of the underlying distribution type. As this is not specified, we have two choices: (1) Either to assume that the distribution of EIRP is Normal, or (2) a students t distribution. Observation 5: The calculation of CIs assume the N samples are unbiased. A 95% confidence interval on the population mean (per-bin) with unknown standard deviation for a normal distribution is then given by: Lower limit= - Mean- 1.96. s/√N Upper limit = - Mean + 1.96.s/√N A 95% confidence interval on the population mean (per bin) with unknown standard deviation for a students t distribution is then given by [ Chapter 9, 2]: Lower limit= - Mean- 2.04. s/√N Upper limit = - Mean +2.04. s/√N - In both cases, we can think of s/√Nas an error in the estimated mean. Observation 6: For both types of distribution choices above, the number of samples ( in this case the beams) N, and the accuracy of samples will significantly influence the upper and lower bounds. Observation 7: As the distribution of EIRP is not specified, students t distribution may be preferable to estimate the CI. The students t-distribution is like the normal distribution but does not assume an apriori knowledge of the population mean and standard deviation as the normal distribution does. The probability density function of the t-distribution is as follows, where Γ represents the gamma function and ν is the degrees of freedom (which is equal to one less than the sample size): f(t) = Γν+12νπ Γυ2 1+t2υ-υ+12. For larger sample sizes ( > 20) , both the t and normal distribution are very similar as shown above.

# 8. References

[1] WRC 23, Final Acts Resolution 220 (COM 4/7)

[2] J R Crawshaw and J Chambers, Advanced Level Statistics, Nelson Thornes Ltd, 2002

Note that the “horizon” is referred to as 0∘ in a three-dimensional local coordinate system with +90∘ pointing towards the sky and -90∘pointing towards the ground/Earth’s surface.

Care must be exercised when using the term “elevation” angle as this is reflected in both equations (1) and (2) or (3) using different notations. The elevation angle in eqn(1) is not the same as elevation angle in eqn(2).

During RAN4111 meeting there was a suggestion that confidence intervals discussed here are already covered under measurement uncertainty as the concept of a CI is also discussed in TR 37 941 for MU computation. The CI on the EIRP mean value discussed here and a CI for MU are two different issues, and one cannot be exchanged for the other. Measurement uncertainty arises from the randomness of various contributing factors to the MU- (see Table 9.2.3.3-1 in TR 37 941); here the CI arises from the fact that we are trying to determine the parameters of the distribution of a population from a limited sample. Our aim is to ensure that the parameters values derived from the sample have a high degree of confidence in making a claim that they represent the corresponding parameter values of the population.

Note that the “horizon” is referred to as 0∘ in a three-dimensional local coordinate system with +90∘ pointing towards the sky and -90∘pointing towards the ground/Earth’s surface.

Care must be exercised when using the term “elevation” angle as this is reflected in both equations (1) and (2) or (3) using different notations. The elevation angle in eqn(1) is not the same as elevation angle in eqn(2).

During RAN4111 meeting there was a suggestion that confidence intervals discussed here are already covered under measurement uncertainty as the concept of a CI is also discussed in TR 37 941 for MU computation. The CI on the EIRP mean value discussed here and a CI for MU are two different issues, and one cannot be exchanged for the other. Measurement uncertainty arises from the randomness of various contributing factors to the MU- (see Table 9.2.3.3-1 in TR 37 941); here the CI arises from the fact that we are trying to determine the parameters of the distribution of a population from a limited sample. Our aim is to ensure that the parameters values derived from the sample have a high degree of confidence in making a claim that they represent the corresponding parameter values of the population.