3GPP TSG RAN WG4 Meeting#112-bis                                R4-2414944

Hefei, CN, Oct 14 - Oct 18, 2024

Title:	RRM Core requirements on measurements related enhancements for LTM

Source:	China Telecom

Agenda item:			6.24.2.1

Document for:	Discussion

- Introduction

In RAN#105 meeting, the objective on measurements related enhancements in [1] has been updated as follow. According to the WF in last RAN4 meeting, we will continue to discuss the RRM Core requirements on measurements related enhancements for LTM.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

- Discussion

2.1. Event triggered L1 measurement reporting

RRM scope of Event triggered L1 measurement reporting

- Candidate solutions: - Option 1: RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value. RAN4 can wait for more RAN2 input. (CATT) - Option 2: (Apple) - Requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. Details are FFS. - Existing framework in L3 event triggered reporting RRM requirements can be reused to cover L1 event triggered reporting. - Option 3: define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay. (CMCC, Apple, Samsung, HW, ZTE, Nokia, E///, vivo, CTC) - Option 3a: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3. - Option 3b: RAN4 can start the discussion based on SSB firstly. Further RAN1/RAN2 progress are needed. (Samsung) - Option 3c: RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. (Nokia, E///, vivo, CMCC) - Option 4: RAN4 should discuss the accuracy requirement for event triggered L1 measurement reporting. (ZTE) - Option 5: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements, for example (Nokia) - Known TCI state condition for early TCI state activation and cell switch - Applicability of cell switch delay requirements - early DL sync (Tfirst-RS = 0 conditions) - early ASN.1 decoding and validity check - LTM L1 measurement requirements - Option 6: For the topic of ‘Measurements related enhancements for the purpose of supporting LTM,’ RAN4 to wait for further progress to be made in other working groups under this WI . (QC, vivo) - Recommended WF - Continue discussion.

RAN2 has introduced the event triggered L1 measurement reporting is introduced in RAN2, it is necessary to define related measurement reporting requirements.

We have defined the measurement reporting requirements for event triggered reporting of SSB based L3 measurements and CSI-RS based L3 measurements. And it is copied as follow,

Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). Reported CSI-RSRP, CSI-RSRQ, and CSI- SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.3, 10.1.3.3, 10.1.7.2, 10.1.8.2, 10.1.12.2 and 10.1.13.2.

So we think RAN4 should to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. In view of on-going discussion on CSI-RS based L1 measurement, we can focus on SSB based L1 measurement firstly, but do not exclude the CSI-RS based L1 measurement.

In addition, we think the existing framework in L3 event triggered reporting RRM requirements can be as a starting point. And the accuracy requirement for event triggered L1 measurement reporting should also be defined.

Proposal 1: RAN4 should define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. In view of on-going discussion on CSI-RS based L1 measurement, we can focus on SSB based L1 measurement firstly, but do not exclude the CSI-RS based L1 measurement.

Proposal 2: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point

Proposal 3: The accuracy requirement for event triggered L1 measurement reporting should also be defined

filtering assumption in event evaluation

- Candidate solutions: - Option 1: Define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Wait for more RAN1/2 progress to discuss the detailed requirements. (MTK) - Recommended WF - Continue discussion.

For event triggered SSB based L1 measurement and CSI-RS based L1 measurement, the related requirements on filtering is shown as follows

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected The event triggered measurement reporting delay, measured without L3 filtering shall be less than the CSI-RS based measurement defined in clause 9.10.2.5. When L3 filtering is used an additional delay can be expected.

However, RAN2 only assume the filtering at present, it is necessary to wait the conclusion of RAN1. As we think, it is possible that the filtering is up to UE implementation. So we think it is better to wait the conclusion of RAN1.

Proposal 4: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements.

event triggered reporting delay

- Candidate solutions: - Option 1: The event triggered L1 measurement reporting delay, measured without L3 filtering, is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. (CMCC) - Option 2: For the delay of event triggered L1 measurement reporting requirements based on SSB, current LTM L1-RSRP measurement delay can be reused. (Samsung) - Option 3: Starting point for the SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap. CSI-RS based delay is pending on Rel-19 work. (Nokia) - Option 4: RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (E///) - Recommended WF - Continue discussion.

We think it is related the last issue, when filtering is used an additional delay can be expected. In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline.

Proposal 5: Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected.

In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline.

2.2. CSI-RS based L1 measurement

Definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

- Candidate solutions: - Option 1: following CSI-RS based L3 measurement, (Apple) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. (MTK) - Option 3: For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement. (vivo) - Recommended WF - Continue discussion.

We prefer to follow the definition on CSI-RS based L3 measurement. So option 1 is OK for us.

Proposal 6: Following CSI-RS based L3 measurement

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

  - It is applied for SCS = 60KHz

- the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

RTD assumption

- Candidate solutions: - Option 1: RTD<CP is taken as baseline. For the case of RTD>CP between serving cell and neighbour cell on the same carrier, UE capability should be introduced. (Apple) - Option 2: Further study whether to support RTD>CP case for CSI-RS based L1-RSRP measurement. (MTK) - Recommended WF - Continue discussion.

If restrict the RTD between serving cell and neighbour cell to be within CP, some cases with higher SCS are also restricted. So we think it better that support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 7: Support RTD>CP case for CSI-RS based L1-RSRP measurement.

whether to cover both known and unknown cell for CSI-RS L1 measurement

- Candidate solutions: - Option 1: only consider known cell in CSI-RS based L1-RSRP measurement. (Apple, MTK) - Recommended WF - Continue discussion.

As the objective saying, the purpose of measurements related enhancements is supporting LTM. And we have achieved the conclusion that when the target cell and the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, the LTM cell switch delay is defined in R18. So we think it is ok that only consider known cell in CSI-RS based L1-RSRP measurement.

Proposal 8: Only consider known cell in CSI-RS based L1-RSRP measurement.

The relationship on SSB based L3 measurement and configured CSI-RS L1 measurement.

For either FR1 or FR2, UE needs to perform SSB based L3 measurement on the neighbour cells firstly to obtain the coarse timing and SSB index information. So SSB based L3 measurement can be as the pre-requisite condition of configured CSI-RS L1 measurement.

But For FR2, UE needs to perform RX beam sweeping to receive the RS. So if the repetition is ON, UE can receive the configured CSI-RS resources and perform CSI-RS measurement with Rx beam sweeping. If the repetition is OFF, UE needs to perform measurement on the associated SSB firstly, but we do not capture the significant benefit of second situation.

Proposal 9: SSB based L3 measurement can be as the pre-requisite condition of configured CSI-RS L1 measurement for either FR1 or FR2.

Proposal 10: If the repetition is ON, UE can receive the configured CSI-RS resources and perform CSI-RS measurement with Rx beam sweeping directly. If the repetition is OFF, UE needs to perform measurement on the associated SSB firstly, but we do not capture the significant benefit of second situation.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: RAN4 should define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. In view of on-going discussion on CSI-RS based L1 measurement, we can focus on SSB based L1 measurement firstly, but do not exclude the CSI-RS based L1 measurement.

Proposal 2: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point.

Proposal 3: The accuracy requirement for event triggered L1 measurement reporting should also be defined.

Proposal 4: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements.

Proposal 5: Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected.

In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline.

Proposal 6: Following CSI-RS based L3 measurement

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

  - It is applied for SCS = 60KHz

- the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

Proposal 7: Support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 8: Only consider known cell in CSI-RS based L1-RSRP measurement.

Proposal 9: SSB based L3 measurement can be as the pre-requisite condition of configured CSI-RS L1 measurement for either FR1 or FR2.

Proposal 10: If the repetition is ON, UE can receive the configured CSI-RS resources and perform CSI-RS measurement with Rx beam sweeping directly. If the repetition is OFF, UE needs to perform measurement on the associated SSB firstly, but we do not capture the significant benefit of second situation.

- References

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.