3GPP TSG RAN WG4 Meeting#112-bis                                R4-2414945

Hefei, CN, Oct 14 - Oct 18, 2024

Title:	RRM Core requirements on conditional intra-CU LTM

Source:	China Telecom

Agenda item:			6.24.2.2

Document for:	Discussion

- Introduction

In RAN#105 meeting, the objective on conditional intra-CU LTM in [1] has been updated as follow. From this meeting, we will start to discuss the RRM Core requirements on conditional intra-CU LTM.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

- Discussion

RRM scope of conditional intra-CU LTM

In RAN#105 meeting, the supporting of conditional intra-CU LTM RAN has been introduced.

So we need to specify RRM requirements related to the conditional intra-CU LTM as necessary.

For R18 LTM, we consider the RRM scope of LTM as follows

- TCI state activation for LTM candidate cell - PDCCH ordered Random Access for LTM - LTM PCell/ PSCell Cell Switch - Link Recovery Procedures - TRP specific Link Recovery Procedures - L1-SINR measurements for Reporting - Intra-frequency L1-RSRP measurements for neighbor cell - NR inter-frequency L1 measurement

Because this is the first meeting on conditional intra-CU LTM in RAN4, we are open to this issue. And we think conditional intra-CU LTM delay, TCI state activation for LTM candidate cell and the synchronization with the target cell should be discussed in the scope of conditional intra-CU LTM.

Proposal 1: Conditional intra-CU LTM delay, TCI state activation for LTM candidate cell and the synchronization with the target cell should be discussed in the scope of conditional intra-CU LTM.

Conditional Intra-CU LTM delay

Firstly, we should define the starting point and ending point of conditional intra-CU LTM delay. As a reference, the definitions the starting point and ending point of LTM delay and CHO delay are copied as follows

CHO: When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. LTM: LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

So, we think the starting point of conditional intra-CU LTM delay is When the UE receives a RRC message implying conditional intra-CU LTM. If we want to align with the definition of LTM delay, we can treat it as the starting point of conditional intra-CU LTM delay is the end of the last TTI containing the RRC command for conditional intra-CU LTM.

Similarly, the ending point of conditional intra-CU LTM delay is the time the UE transmits the first UL message on the target cell.

Proposal 2: Conditional intra-CU LTM delay is the delay from the end of the last TTI containing the RRC command for conditional intra-CU LTM until the time the UE transmits the first UL message on the target cell.

Base on the definition of the conditional intra-CU LTM delay as shown above. The components of Conditional intra-CU LTM delay can contain these as follows

DIntra-CLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TIntra-CLTM_execution + Tinterrupt.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- TTCI-state activate is the time for active the TCI state of candidate cell(s).

- Tmeasure is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

- TIntra-CLTM_execution is the UE execution preparation time for conditional intra-CU LTM, and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Proposal 3:

DIntra-CLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TIntra-CLTM_execution + Tinterrupt.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- TTCI-state activate is the time for active the TCI state of candidate cell(s).

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TIntra-CLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Due to this is the first meeting on conditional intra-CU LTM in all RAN WGs. We should revisit this issue when other WGs achieve further conclusions.

Proposal 4：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

DL-sync, UL-sync and Early TCI state activate

For early TCI state activate and DL-sync, we think R18 LTM early TCI state activate and DL-sync can be as baseline.

For UL-sync, during the conditional intra-CU LTM, UE does not necessitate a signaling exchange with source cell. So, we think PDCCH ordered RACH will not be suitable for conditional intra-CU LTM. In this case, we can consider some situations as follows,

- Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0.

- RAR-based TA acquisition.

- UE based TA measurement.

Proposal 5:

- For early TCI state activate and DL-sync, we think R18 LTM early TCI state activate and DL-sync can be as baseline.

- For UL-sync, we can consider some situations as follows,

  - Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0.

  - RAR-based TA acquisition.

  - UE based TA measurement.

Whether to cover both RACH-based and RACH-less conditional intra-CU LTM

As the previous issue saying, we think UE can acquire TA without RAR-based TA acquisition. So, RACH-less conditional intra-CU LTM is important. We should cover both RACH-based and RACH-less conditional intra-CU LTM.

Proposal 6: Cover both RACH-based and RACH-less conditional intra-CU LTM.

whether to cover both known and unknown cell for conditional intra-CU LTM

The definition of known cell for R18 LTM is copied as follow,

The target cell in the LTM cell switch command is known if the following conditions are met: - During the last 5 seconds before the reception of the cell switch command: - The UE has sent a valid L1 or L3 measurement report for the target cell, and - One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell, - One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell, Otherwise, the cell is unknown.

However, since there is not cell switch command for conditional intra-CU LTM, we think the condition can be revised as,

During the last 5 seconds before the reception of the RRC command that contain conditional intra-CU LTM configuration:

……., the details are TBD.

We think the conditional intra-CU LTM configuration is based on previous L3 measurement result. In other words, if UE is configured conditional intra-CU LTM, it is better that UE has sent a valid L3 measurement report x seconds early. And the value of x should not be too large.

So we think all the cell(s) configured as candidate cell(s) is/are known in this case.

Proposal 7: The known cell condition for conditional intra-CU LTM can be revised as, during the last 5 seconds before the reception of the RRC command that contain conditional intra-CU LTM configuration…….

Proposal 8: All the cell(s) configured as candidate cell(s) conditional intra-CU LTM is/are known.

- Summary

In this paper, we provide our views on conditional intra-CU LTM. From this discussion we have derived the following proposals:

Proposal 1: Conditional intra-CU LTM delay, TCI state activation for LTM candidate cell and the synchronization with the target cell should be discussed in the scope of conditional intra-CU LTM.

Proposal 2: Conditional intra-CU LTM delay is the delay from the end of the last TTI containing the RRC command for conditional intra-CU LTM until the time the UE transmits the first UL message on the target cell.

Proposal 3:

DIntra-CLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TIntra-CLTM_execution + Tinterrupt.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- TTCI-state activate is the time for active the TCI state of candidate cell(s).

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TIntra-CLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Proposal 4：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

Proposal 5:

- For early TCI state activate and DL-sync, we think R18 LTM early TCI state activate and DL-sync can be as baseline.

- For UL-sync, we can consider some situations as follows,

  - Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0.

  - RAR-based TA acquisition.

  - UE based TA measurement.

Proposal 6: Cover both RACH-based and RACH-less conditional intra-CU LTM.

Proposal 7: The known cell condition for conditional intra-CU LTM can be revised as, during the last 5 seconds before the reception of the RRC command that contain conditional intra-CU LTM configuration…….

Proposal 8: All the cell(s) configured as candidate cell(s) for conditional intra-CU LTM is/are known.

- References

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.