3GPP TSG-RAN WG4 Meeting # 112bis	             R4-2414947

# Hefei, China, 14th - 18th, Oct 2024

Source: 	Samsung, TELUS, Bell mobility

Title: 	Views on UE RF requirements for fragmented carriers

Agenda Item:	6.6.3

Document for:	Discussion

- Introduction

In RAN#103, a SID was approved [1] with following objectives on the study of how to process the fragmented carriers with one Rx Chain. Initial discussion was organized in RAN4#112 and the agreements are captured in [2].

We present our initial consideration in this paper.

- 2. Discussion/Conclusion

There are some recommended WF or FFS points during last meeting final round discussion but unfortunately nothing on UE performance evaluation was agreed in the end.

To facilitate moderator’s summary work and to be efficient, we list the items we are supportive as following, using the wording captured in the Annex of the WF, which was polished by companies already. More details can be found in our paper of last meeting [3].

Assumptions on test configurations for evaluation

  - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos

  - Other configurations are not precluded if identified as having performance impact

Requirements for evaluation

- ΔRIBNC, ACS, IBB, NBB requirements on both PCC and SCC, all need to be evaluated

- Out-of-band blocking and spurious response do not need requirement adjustment

- Adjusted requirements for fragmented carriers shall be separated from the existing FDD/TDD intra-band carrier non-contiguous aggregation ΔRIBNC, ACS, IBB, NBB

- Discussion on new requirement(s) if identified is not precluded

For details related to the gap between fragmented carriers, or other configurations (apart from the configurations in current spec for requirements verification), we provide the following ones for RAN4 consideration.

      - Some examples of gaps between the fragmented carriers are:

        - n2/n25: 10 MHz, 15 MHz

        - n7: 20 MHz and 30 MHz

        - n66: 15 MHz

        - n26: 5 MHz and 10 MHz

- 3. Reference

[1] RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MTK, RAN#104

[2] R4-2414311, WF on fragmented DL carriers study, MediaTek Inc., RAN4 #112

[3] R4-2411311, Views on UE RF requirements for fragmented carriers, Samsung, RAN4#112