3GPP TSG-RAN WG4 Meeting # 112bis	R4-2414951

# Hefei, China, 14th - 18th, Oct 2024

Source: 	Samsung

Title: 	Discussion on the referenced UE architecture table for non-collocated

Agenda Item:	6.5.2.3

Document for:	Discussion

- Introduction

In RAN#103, a new WID on Non-collocated deployment was approved [1] which focus on specifying UE RF/RRM requirements for Type-4a/b UE capability and clarifying the UE behaviour for Type-4a/b supported UE. Discussion was organized in RAN4#112 and the agreements were captured in [2] and the Chairman notes.

In this paper, we share the views on how the referenced UE architecture table could be possibly modified to better reflect the common understanding (if any), and invite companies to discuss whether the change is necessary.

- Discussion and conclusion

As described in the WID [1], “UE types are defined in R4-2217734”, note that here our understanding is “UE types” is intended for “UE capability types”. The following table is excerpted from R4-2217734 with some new change in red.

During the discussion of Rel-19 SI fragmented carriers, it was found out that the architecture of Type-3a/3b (so called “partially shared Rx Chain” architecture) had been selected in LTE period for intra-band DL NC CA requirements specification. While from what we know, the popular UE implementation is the “fully separated Rx Chain” architecture with existing requirements being complied with but MIMO layer/Diversity sacrificed. In addition, Type-1 architecture (so called “fully shared Rx Chain” architecture) was selected in last meeting as the baseline architecture in Rel-19 fragmented carriers to study the possibility of receiving two NC CCs in a single Rx RF Chain, in order to free up more RF resources for other bands/CCs. Note our understanding of “Rx Chain” is from antenna to the end of ADC.

Based on above understanding, we further modify the table in R4-2217734 as following (modification in red). Please note that the RF architecture assumption (i.e., number of antenna, mixer, BB etc) has not been changed, just the “comment” column which is informative has been modified to better reflect the common understanding (if there is).

If companies think there is no ambiguity on non-collocated discussion, it is also ok for us to change nothing.

Proposal 1: Companies to discuss whether the change in “comment” column is needed, to better align the understanding.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) Can support up to 4 layer/4Rx per CC if there is no in-gap interferer or the level of in-gap interferer is tolerable. May fallback to 2layer/2Rx per CC to fully avoid the in-gap interference issue. [Note 1: In-gap interference issue is a general issue for collocated deployment, not specific for this non-collocated WI.] [Note 2: In-gap interference issue is likely to be covered by Rel-19 SI fragmented carriers, approach(es) can be leveraged whenever feasible. ] |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but nNeed 2LO frequencies Can support up to 2layer/2Rx per CC |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 eLNAs + 1BB/Rx => common AGC on eLNA => 25dB only for some range Can support up to 4layer for NR CC and up to 2layer for LTE CC |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 eLNAs + 1BB/Rx => common AGC on eLNA => 25dB only for some range Can support up to 4layer per CC |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNAs => is it compatible with smartphone? (for which frequency range), FWA only Can support up to 4layer/4Rx for NR CC and up to 2layer/2Rx for LTE CC |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNAs => is it compatible with smartphone? (for which frequency range), FWA only Can support up to 4layer/4Rx per CC |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

- 3. Reference

[1] RP-240838, R19 WID_Intra-band non-collocated EN-DC and NR-CA, RAN#103

[2] R4-2414453, WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#112