3GPP TSG-RAN WG4 Meeting #112bis	R4-2414982
Hefei, Anhui, China, 14th – 18th October, 2024

Title:	R19 AMPR for n28 40MHz

Source:	OPPO

Agenda item:	5.20.2

Document for:	Approval

- Introduction

This paper discusses the NS_18 AMPR for n28 40MHz for both PC3 and PC2 per WF [1].

- Discussion

  - AMRP for PC3 with 1Tx

2.1.1 AMRP region definition

The configurations are:

- power class PC3

- One PC3 low band PA

- 40MHz CBW with 30khz scs (figure 1)

- CP-OFDM / DFT-S-OFDM

- Modulation pi/2 bpsk, qpsk, 16qam, 64qam, 256qam

Figure 1 40MHz CBW location and NS_18 emission region

The AMPR results can be found in annex A.

From the simulation results, it was found the current AMPR region definition may not well fit the AMPR status. The AMPR region definition in this paper is defined as in figure 2. From figure 3 it can be seen that the slope of AMPR region boundaries between A7, A10 and A8 is 1.

Figure 2 AMPR region definition

Figure 3 Shown of AMPR region definition (30khz scs, DFT QPSK AMPR results)

The region definition is in table 1.

Table 1 AMPR region definition

| CBW, MHz | Tx BW, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart *12*SCS (MHz) | LCRB *12*SCS (MHz) |  |
| 40MHz | 703-743 | > 12*scs* LCRB + 5.04 | > max (0, -12*scs*RBstart + 12*scs*N -3.6) | A7 |
|  |  | > 0 | > 12*scs* RBstart + 6.48 | A8 |
|  |  | <= 11.52 | <= 6.48 | A9 |
|  |  | <= 12*scs* Y + 5.04 >= 12*scs* Y - 6.48 | > 6.48 | A10 |

2.1.2 AMRP requirements

Based on above AMPR region definition, the AMPR requirements are summarized in table 2 for DFT-S-OFDM and CP-OFDM with implementation margin considered.

Table 2 AMPR for 1T PC3

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A7 | A8 | A9 | A10 | A7 | A8 | A9 | A10 |
| Pi/2 BPSK | 2 | 9 | 4 | 4 | N/A | N/A | N/A | N/A |
| QPSK | 3 | 9 | 4 | 4 | 5.5 | 10 | 6 | 6 |
| 16QAM | 3 | 9 | 5 | 5 | 5.5 | 10 | 6 | 6 |
| 64QAM | 4 | 10 | 6 | 6 | 5.5 | 10.5 | 6.5 | 6.5 |
| 256QAM | 4 | 10 | 6 | 6 | 5.5 | 10.5 | 8.5 | 8.5 |

Proposal 1:         Consider Table 1 as the AMPR region definition and Table 2 as the PC3 AMPR requirements.

  - AMRP for PC2

2.2.1 1T PC2 results

Similar as 1T PC3, the AMPR results for 1T PC2 can be found in annex B.

Below table 3 summarized the AMPR results for 1T PC2 based on the region definition in Table 1.

Table 3 AMPR for 1T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A7 | A8 | A9 | A10 | A7 | A8 | A9 | A10 |
| Pi/2 BPSK | 4 | 11 | 6 | 6 |  |  |  |  |
| QPSK | 4 | 11 | 6 | 6 | 6.5 | 13 | 7.5 | 7.5 |
| 16QAM | 5 | 12 | 8 | 8 | 6.5 | 13 | 8 | 8 |
| 64QAM | 6 | 12 | 8.5 | 8.5 | 6.5 | 13 | 8.5 | 8.5 |
| 256QAM | 6 | 12 | 8.5 | 8.5 | 6.5 | 13 | 9 | 9 |

2.2.2 2T PC2 results

Similar as 2T PC2, the AMPR results for 2T PC2 can be found in annex C.

Below table 4 summarized the AMPR results for 2T PC2 based on the region definition in Table 1.

Table 4 AMPR for 2T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A7 | A8 | A9 | A10 | A7 | A8 | A9 | A10 |
| Pi/2 BPSK | 3 | 12 | 6 | 6 |  |  |  |  |
| QPSK | 4 | 12 | 6 | 6 | 6 | 13 | 8 | 8 |
| 16QAM | 4 | 12 | 6 | 6 | 6 | 13 | 8 | 8 |
| 64QAM | 5 | 13 | 7.5 | 7.5 | 6 | 13 | 9 | 9 |
| 256QAM | 5.5 | 13 | 8.5 | 8.5 | 6 | 13 | 10 | 10 |

2.2.3 AMRP requirements

From table 3 and table 4, it can be seen the difference between 1T and 2T are around 0-2dB. If one AMPR table for 1Tx and 2Tx, then AMPR table 5 can be used.

Table 5 AMPR for both 1T and 2T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A7 | A8 | A9 | A10 | A7 | A8 | A9 | A10 |
| Pi/2 BPSK | 4 | 12 | 6 | 6 | N/A | N/A | N/A | N/A |
| QPSK | 4 | 12 | 6 | 6 | 6.5 | 13 | 8 | 8 |
| 16QAM | 5 | 12 | 8 | 8 | 6.5 | 13 | 8 | 8 |
| 64QAM | 6 | 13 | 8.5 | 8.5 | 6.5 | 13 | 9 | 9 |
| 256QAM | 6 | 13 | 8.5 | 8.5 | 6.5 | 13 | 10 | 10 |

Proposal 2:         If one AMPR table for PC2 with 1Tx and 2Tx, then AMPR table 5 can be used.

- Conclusion

This paper shared the AMPR simulation results for NS_18 with 40MHz CBW in n28 and got below observations and proposals.

Proposal 1:         Consider Table 1 as the AMPR region definition and Table 2 as the PC3 AMPR requirements.

Table 1 AMPR region definition

| CBW, MHz | Tx BW, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart *12*SCS (MHz) | LCRB *12*SCS (MHz) |  |
| 40MHz | 703-743 | > 12*scs* LCRB + 5.04 | > max (0, -12*scs*RBstart + 12*scs*N -3.6) | A7 |
|  |  | > 0 | > 12*scs* RBstart + 6.48 | A8 |
|  |  | <= 11.52 | <= 6.48 | A9 |
|  |  | <= 12*scs* Y + 5.04 >= 12*scs* Y - 6.48 | > 6.48 | A10 |

Table 2 AMPR for 1T PC3

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A7 | A8 | A9 | A10 | A7 | A8 | A9 | A10 |
| Pi/2 BPSK | 2 | 9 | 4 | 4 | N/A | N/A | N/A | N/A |
| QPSK | 3 | 9 | 4 | 4 | 5.5 | 10 | 6 | 6 |
| 16QAM | 3 | 9 | 5 | 5 | 5.5 | 10 | 6 | 6 |
| 64QAM | 4 | 10 | 6 | 6 | 5.5 | 10.5 | 6.5 | 6.5 |
| 256QAM | 4 | 10 | 6 | 6 | 5.5 | 10.5 | 8.5 | 8.5 |

Proposal 2:         If one AMPR table for PC2 with 1Tx and 2Tx, then AMPR table 5 can be used.

Table 5 AMPR for both 1T and 2T PC2

|  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A7 | A8 | A9 | A10 | A7 | A8 | A9 | A10 |
| Pi/2 BPSK | 4 | 12 | 6 | 6 | N/A | N/A | N/A | N/A |
| QPSK | 4 | 12 | 6 | 6 | 6.5 | 13 | 8 | 8 |
| 16QAM | 5 | 12 | 8 | 8 | 6.5 | 13 | 8 | 8 |
| 64QAM | 6 | 13 | 8.5 | 8.5 | 6.5 | 13 | 9 | 9 |
| 256QAM | 6 | 13 | 8.5 | 8.5 | 6.5 | 13 | 10 | 10 |

- Reference

[1] R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, #112 2024-08

Annex A: 1T PC3

Annex A.1 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B: 1T PC2

Annex B.1 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex C: 2T PC2

Annex C.1 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex C.2 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]