3GPP TSG-RAN WG4 Meeting #112bis	R4-2414984
Hefei, Anhui, China, 14th – 18th October, 2024

Title:	R19 MPR for PC1.5 contiguous CA and NCCA

Source:	OPPO

Agenda item:	6.1.1.1.2

Document for:	Approval

- Introduction

This paper discusses the Rel-19 PC1.5 contiguous CA and NCCA MPR based on WF [1] and also requirement clarifications in [2][3].

- PC1.5 contiguous CA

2.1 Non-Contig RB allocation MRP

2.1.1 Initial MRP results

With the understanding of EVM / IBE requirements are NOT verified when both CCs are configured with PRBs, below tables give the initial simulation results. It includes CP-OFDM and DFT-S-OFDM with Class B and Class C and PRB configured in both CCs.

And it should be noticed that in the simulation TxD/1LO is used to achieve PC1.5, and the IMD impacts are accounted in the simulation, and some additional values need to be added to cope with it in the final MPR results.

The configurations are:

- CA class B or C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 15khz or 30khz

- LO number is 1LO+TxD

- Modulations are same in both CCs

Table 1 CP-OFDM non-contig RB allocation: CA Class B and Class C

| CP-OFDM: CA Class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam | 64qam | 256qam |
| 20 | 20 | 1 | 73 | 1 | 32 | 15khz | inner | 2.3 | 2.6 | 2.2 | 2.2 | 2.2 |
| 20 | 20 | 1 | 72 | 1 | 33 | 15khz | inner | 2.3 | 2.6 | 2.2 | 2.2 | 2.2 |
| 20 | 20 | 1 | 56 | 1 | 1 | 15khz | inner | 2.8 | 2.8 | 2.8 | 2.8 | 2.4 |
| 20 | 20 | 1 | 55 | 1 | 1 | 15khz | inner | 2.4 | 2.8 | 2.4 | 2.4 | 2.4 |
| 50 | 50 | 1 | 183 | 1 | 86 | 15khz | inner | 2.3 | 2.6 | 2.2 | 2.2 | 2.2 |
| 50 | 50 | 1 | 139 | 1 | 1 | 15khz | inner | 2.4 | 2.8 | 2.4 | 2.4 | 2.4 |
| 50 | 50 | 1 | 138 | 1 | 1 | 15khz | inner | 1.3 | 1.5 | 1.3 | 1.5 | 1.2 |
| Inner Max MPR |  |  |  |  |  |  |  | 2.8 | 2.8 | 2.8 | 2.8 | 2.4 |
| 20 | 20 | 1 | 72 | 1 | 50 | 15khz | outer1 | 4.5 | 4.5 | 4.5 | 5.1 | 5.1 |
| 20 | 20 | 1 | 54 | 1 | 50 | 15khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.8 |
| 20 | 20 | 1 | 42 | 1 | 63 | 15khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.8 |
| 20 | 20 | 1 | 1 | 1 | 1 | 15khz | outer1 | 5.5 | 5.5 | 4.9 | 5.5 | 5.5 |
| 50 | 50 | 1 | 137 | 1 | 60 | 15khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.8 |
| 50 | 50 | 1 | 137 | 1 | 20 | 15khz | outer1 | 4.5 | 4.5 | 4.5 | 5.1 | 5.1 |
| 50 | 50 | 1 | 109 | 1 | 160 | 15khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.8 |
| 50 | 50 | 1 | 108 | 1 | 161 | 15khz | outer1 | 5.1 | 5.1 | 4.5 | 5.8 | 5.1 |
| 50 | 50 | 1 | 1 | 1 | 1 | 15khz | outer1 | 5.5 | 5.5 | 5.5 | 6.1 | 6.1 |
| Outer1 Max MPR |  |  |  |  |  |  |  | 5.5 | 5.5 | 5.5 | 6.1 | 6.1 |
| 20 | 20 | 1 | 40 | 1 | 65 | 15khz | outer2 | 7.4 | 7.3 | 6.5 | 7.3 | 6.5 |
| 20 | 20 | 1 | 1 | 1 | 105 | 15khz | outer2 | 5.3 | 5.3 | 5.3 | 5.9 | 5.9 |
| 20 | 20 | 1 | 1 | 1 | 80 | 15khz | outer2 | 5.3 | 5.3 | 5.3 | 5.9 | 5.3 |
| 20 | 20 | 1 | 41 | 1 | 64 | 15khz | outer2 | 7.4 | 7.3 | 6.5 | 7.3 | 6.5 |
| 50 | 50 | 1 | 50 | 1 | 162 | 15khz | outer2 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 50 | 50 | 1 | 1 | 1 | 269 | 15khz | outer2 | 5.3 | 5.3 | 5.3 | 5.3 | 5.3 |
| 50 | 50 | 1 | 1 | 1 | 80 | 15khz | outer2 | 5.3 | 5.3 | 4.7 | 5.3 | 5.3 |
| Outer2 Max MPR |  |  |  |  |  |  |  | 7.4 | 7.3 | 6.5 | 7.3 | 6.5 |
| CP-OFDM: CA Class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam | 64qam | 256qam |
| 60 | 100 | 1 | 110 | 1 | 51 | 30khz | inner | 1.3 | 1.0 | 1.1 | 1.3 | 1.3 |
| 60 | 100 | 1 | 109 | 1 | 52 | 30khz | inner | 2.3 | 2.2 | 1.6 | 2.2 | 1.6 |
| 60 | 100 | 1 | 84 | 1 | 1 | 30khz | inner | 1.3 | 1.2 | 1.2 | 1.5 | 1.2 |
| 60 | 100 | 1 | 83 | 1 | 1 | 30khz | inner | 1.3 | 1.2 | 1.2 | 1.3 | 1.2 |
| 100 | 100 | 1 | 184 | 1 | 51 | 30khz | inner | 1.1 | 1.1 | 1.1 | 1.1 | 1.0 |
| 100 | 100 | 1 | 183 | 1 | 52 | 30khz | inner | 1.6 | 1.6 | 1.3 | 1.6 | 1.6 |
| 100 | 100 | 1 | 140 | 1 | 1 | 30khz | inner | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| 100 | 100 | 1 | 139 | 1 | 1 | 30khz | inner | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| Inner Max MPR |  |  |  |  |  |  |  | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| 60 | 100 | 1 | 120 | 1 | 118 | 30khz | outer1 | 5.1 | 5.1 | 4.5 | 5.1 | 5.1 |
| 60 | 100 | 1 | 82 | 1 | 118 | 30khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 60 | 100 | 1 | 43 | 1 | 118 | 30khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 60 | 100 | 1 | 42 | 1 | 118 | 30khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 60 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 5.3 | 5.3 | 5.3 | 6.0 | 5.3 |
| 100 | 100 | 1 | 182 | 1 | 182 | 30khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 100 | 100 | 1 | 138 | 1 | 182 | 30khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 100 | 100 | 1 | 110 | 1 | 150 | 30khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 100 | 100 | 1 | 109 | 1 | 163 | 30khz | outer1 | 5.1 | 5.1 | 5.1 | 5.8 | 5.1 |
| 100 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 5.5 | 5.5 | 5.5 | 6.1 | 6.1 |
| Outer1 Max MPR |  |  |  |  |  |  |  | 5.5 | 5.5 | 5.5 | 6.1 | 6.1 |
| 60 | 100 | 1 | 41 | 1 | 120 | 30khz | outer2 | 6.6 | 6.5 | 5.8 | 6.5 | 5.7 |
| 60 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 5.2 | 5.1 | 5.1 | 5.8 | 5.1 |
| 60 | 100 | 1 | 1 | 1 | 180 | 30khz | outer2 | 5.2 | 5.1 | 5.1 | 5.8 | 5.1 |
| 100 | 100 | 1 | 108 | 1 | 180 | 30khz | outer2 | 5.1 | 5.1 | 4.5 | 5.8 | 5.1 |
| 100 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 5.3 | 5.3 | 5.3 | 5.9 | 5.3 |
| 100 | 100 | 1 | 1 | 1 | 100 | 30khz | outer2 | 5.3 | 5.3 | 5.3 | 5.3 | 5.3 |
| Outer2 Max MPR |  |  |  |  |  |  |  | 6.6 | 6.5 | 5.8 | 6.5 | 5.7 |

Table 2 DFT-S-OFDM non-contig RB allocation: CA Class B and Class C

| DFT-S-OFDM: CA Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam |  | 64qam |  | 256qam |
| 20 | 20 | 1 | 73 | 1 | 32 | 15khz | inner | 2.0 | 2.4 | 2.4 | 2.4 |  | 2.5 |  |
| 20 | 20 | 1 | 72 | 1 | 33 | 15khz | inner | 2.0 | 2.4 | 2.4 | 2.4 |  | 2.5 |  |
| 20 | 20 | 1 | 56 | 1 | 1 | 15khz | inner | 2.1 | 2.6 | 2.7 | 2.6 |  | 2.7 |  |
| 20 | 20 | 1 | 55 | 1 | 1 | 15khz | inner | 2.5 | 2.6 | 2.7 | 2.6 |  | 2.7 |  |
| 50 | 50 | 1 | 183 | 1 | 86 | 15khz | inner | 2.0 | 2.4 | 2.4 | 2.4 |  | 2.5 |  |
| 50 | 50 | 1 | 139 | 1 | 1 | 15khz | inner | 2.5 | 2.6 | 2.7 | 2.6 |  | 2.7 |  |
| 50 | 50 | 1 | 138 | 1 | 1 | 15khz | inner | 1.4 | 1.5 | 1.6 | 1.6 |  | 1.7 |  |
| Inner Max MPR |  |  |  |  |  |  |  | 2.5 | 2.6 | 2.7 | 2.6 |  | 2.7 |  |
| 20 | 20 | 1 | 72 | 1 | 50 | 15khz | outer1 | 2.8 | 3.3 | 3.9 | 4.4 |  | 4.5 |  |
| 20 | 20 | 1 | 54 | 1 | 50 | 15khz | outer1 | 3.3 | 4.4 | 4.4 | 5.0 |  | 5.1 |  |
| 20 | 20 | 1 | 42 | 1 | 63 | 15khz | outer1 | 3.3 | 4.4 | 4.4 | 5.0 |  | 5.1 |  |
| 20 | 20 | 1 | 1 | 1 | 1 | 15khz | outer1 | 3.7 | 4.2 | 4.8 | 4.8 |  | 4.9 |  |
| 50 | 50 | 1 | 137 | 1 | 60 | 15khz | outer1 | 3.3 | 4.4 | 4.4 | 4.4 |  | 5.1 |  |
| 50 | 50 | 1 | 137 | 1 | 20 | 15khz | outer1 | 2.8 | 3.3 | 3.9 | 3.8 |  | 4.5 |  |
| 50 | 50 | 1 | 109 | 1 | 160 | 15khz | outer1 | 3.3 | 4.4 | 4.4 | 5.0 |  | 5.1 |  |
| 50 | 50 | 1 | 108 | 1 | 161 | 15khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 50 | 50 | 1 | 1 | 1 | 1 | 15khz | outer1 | 3.7 | 4.7 | 4.8 | 4.8 |  | 5.5 |  |
| Outer1 Max MPR |  |  |  |  |  |  |  | 3.7 | 4.7 | 4.8 | 5.0 |  | 5.5 |  |
| 20 | 20 | 1 | 40 | 1 | 65 | 15khz | outer2 | 7.4 | 7.3 | 7.3 | 7.3 |  | 7.4 |  |
| 20 | 20 | 1 | 1 | 1 | 105 | 15khz | outer2 | 3.5 | 4.6 | 4.6 | 4.6 |  | 5.3 |  |
| 20 | 20 | 1 | 1 | 1 | 80 | 15khz | outer2 | 3.5 | 4.0 | 4.6 | 4.6 |  | 5.3 |  |
| 20 | 20 | 1 | 41 | 1 | 64 | 15khz | outer2 | 6.5 | 7.3 | 6.5 | 7.3 |  | 7.4 |  |
| 50 | 50 | 1 | 50 | 1 | 162 | 15khz | outer2 | 3.3 | 4.4 | 4.4 | 5.0 |  | 5.1 |  |
| 50 | 50 | 1 | 1 | 1 | 269 | 15khz | outer2 | 3.5 | 4.0 | 4.6 | 4.6 |  | 4.7 |  |
| 50 | 50 | 1 | 1 | 1 | 80 | 15khz | outer2 | 3.5 | 4.0 | 4.6 | 4.6 |  | 4.7 |  |
| Outer2 Max MPR |  |  |  |  |  |  |  | 7.4 | 7.3 | 7.3 | 7.3 |  | 7.4 |  |
| DFT-S-OFDM: CA Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam |  | 64qam |  | 256qam |
| 60 | 100 | 1 | 110 | 1 | 51 | 30khz | inner | 1.3 | 1.3 | 1.4 | 1.4 |  | 1.5 |  |
| 60 | 100 | 1 | 109 | 1 | 52 | 30khz | inner | 2.0 | 2.0 | 2.1 | 2.0 |  | 2.2 |  |
| 60 | 100 | 1 | 84 | 1 | 1 | 30khz | inner | 1.4 | 1.2 | 1.3 | 1.3 |  | 1.2 |  |
| 60 | 100 | 1 | 83 | 1 | 1 | 30khz | inner | 1.4 | 1.5 | 1.3 | 1.6 |  | 1.4 |  |
| 100 | 100 | 1 | 184 | 1 | 51 | 30khz | inner | 1.3 | 1.0 | 1.1 | 1.1 |  | 1.5 |  |
| 100 | 100 | 1 | 183 | 1 | 52 | 30khz | inner | 2.0 | 2.0 | 2.1 | 2.1 |  | 2.2 |  |
| 100 | 100 | 1 | 140 | 1 | 1 | 30khz | inner | 2.1 | 2.6 | 2.7 | 2.6 |  | 2.7 |  |
| 100 | 100 | 1 | 139 | 1 | 1 | 30khz | inner | 2.1 | 2.2 | 2.3 | 2.6 |  | 2.7 |  |
| Inner Max MPR |  |  |  |  |  |  |  | 2.1 | 2.6 | 2.7 | 2.6 |  | 2.7 |  |
| 60 | 100 | 1 | 120 | 1 | 118 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 60 | 100 | 1 | 82 | 1 | 118 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 60 | 100 | 1 | 43 | 1 | 118 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 60 | 100 | 1 | 42 | 1 | 118 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 60 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 3.5 | 4.0 | 4.7 | 4.6 |  | 5.3 |  |
| 100 | 100 | 1 | 182 | 1 | 182 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 100 | 100 | 1 | 138 | 1 | 182 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 100 | 100 | 1 | 110 | 1 | 150 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 100 | 100 | 1 | 109 | 1 | 163 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 5.0 |  | 5.1 |  |
| 100 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 3.7 | 4.2 | 4.8 | 4.8 |  | 5.5 |  |
| Outer1 Max MPR |  |  |  |  |  |  |  | 3.7 | 4.2 | 4.8 | 5.0 |  | 5.5 |  |
| 60 | 100 | 1 | 41 | 1 | 120 | 30khz | outer2 | 6.5 | 6.5 | 5.7 | 6.4 |  | 6.5 |  |
| 60 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 60 | 100 | 1 | 1 | 1 | 180 | 30khz | outer2 | 3.3 | 3.8 | 4.4 | 4.4 |  | 5.1 |  |
| 100 | 100 | 1 | 108 | 1 | 180 | 30khz | outer2 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 100 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 3.5 | 4.0 | 4.6 | 4.6 |  | 4.7 |  |
| 100 | 100 | 1 | 1 | 1 | 100 | 30khz | outer2 | 3.5 | 4.0 | 4.6 | 4.6 |  | 4.7 |  |
| Outer2 Max MPR |  |  |  |  |  |  |  | 6.5 | 6.5 | 5.7 | 6.4 |  | 6.5 |  |

2.2.2 Summary of MRP results for non-contiguous RB allocations

Above MPR results can be summarized as below table 3 with some necessary margins added.

Table 3 MPR for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 6.5 | 10 | 5 | 6.5 | 9 |
|  | QPSK | 5 | 7.5 | 10 | 5 | 7 | 9 |
|  | 16QAM | 5.5 | 7.5 | 10 | 5.5 | 7.5 | 9 |
|  | 64QAM | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
|  | 256QAM | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
| CP-OFDM | QPSK | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
|  | 16QAM | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
|  | 64QAM | 5.5 | 9 | 10 | 5.5 | 9 | 9 |
|  | 256QAM | 5.5 | 9 | 10 | 5.5 | 9 | 9 |

Proposal 1:         Consider MPR results for non-contiguous RB allocations in two CCs in table 3.

  - Contiguous RB allocation MRP

2.2.1 Initial MRP results with RB in both CCs

The MPR results for contiguous RB allocation for CP-OFDM/DFT-S-OFDM are given in below tables. And the RBs are configured in both CCs which means the EVM/IBE are not verified in the simulation. The IMD impacts have been accounted in the simulation.

The configurations are:

- CA class B or C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 15khz or 30khz

- LO number is 1LO+TxD

- Modulations are same in both CCs

Table 4 CP-OFDM contig RB allocation in two CCs: CA Class B and Class C

| CP-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 54 | 1 | 15 | inner | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 |
| 20 | 20 | 53 | 53 | 53 | 1 | 15 | inner | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 |
| 20 | 20 | 52 | 52 | 53 | 1 | 15 | inner | 1.6 | 2.3 | 2.3 | 2.3 | 1.6 |
| 50 | 50 | 136 | 134 | 136 | 1 | 15 | inner | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 |
| 50 | 50 | 135 | 135 | 135 | 1 | 15 | inner | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 |
| 50 | 50 | 134 | 136 | 135 | 1 | 15 | inner | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 |
| Inner Max MPR |  |  |  |  |  |  |  | 1.6 | 2.3 | 2.3 | 2.3 | 1.6 |
| 20 | 20 | 106 | 1 | 106 | 1 | 15 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 20 | 20 | 81 | 25 | 80 | 1 | 15 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 20 | 20 | 80 | 26 | 80 | 1 | 15 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 20 | 20 | 79 | 27 | 80 | 1 | 15 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.1 |
| 50 | 50 | 270 | 1 | 270 | 1 | 15 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 50 | 50 | 203 | 67 | 202 | 1 | 15 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 50 | 50 | 202 | 68 | 202 | 1 | 15 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 50 | 50 | 201 | 69 | 202 | 1 | 15 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| Outer Max MPR |  |  |  |  |  |  |  | 3.0 | 3.0 | 3.0 | 3.0 | 3.1 |
| CP-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 54 | 108 | 162 | 1 | 30 | inner | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 53 | 109 | 162 | 1 | 30 | inner | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 100 | 100 | 137 | 136 | 137 | 1 | 30 | inner | 0.9 | 0.0 | 0.9 | 0.9 | 0.9 |
| 100 | 100 | 136 | 137 | 136 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 100 | 100 | 135 | 138 | 135 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 1 | 273 | 1 | 30 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 60 | 100 | 125 | 37 | 209 | 1 | 30 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 60 | 100 | 125 | 37 | 208 | 1 | 30 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 60 | 100 | 125 | 37 | 207 | 1 | 30 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 100 | 100 | 273 | 1 | 273 | 1 | 30 | outer | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |
| 100 | 100 | 205 | 68 | 204 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 100 | 100 | 204 | 69 | 204 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 100 | 100 | 203 | 70 | 204 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| Outer Max MPR |  |  |  |  |  |  |  | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 |

Table 5 DFT-S-OFDM contig RB allocation in two CCs: CA Class B and Class C

| DFT-S-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 54 | 1 | 15 | inner | 1.0 | 1.1 | 1.5 | 1.5 | 1.5 |
| 20 | 20 | 54 | 52 | 50 | 1 | 15 | inner | 0.0 | 1.1 | 1.2 | 1.5 | 1.2 |
| 20 | 20 | 50 | 56 | 50 | 1 | 15 | inner | 0.0 | 0.8 | 1.2 | 1.2 | 1.2 |
| 50 | 50 | 144 | 126 | 128 | 1 | 15 | inner | 0.0 | 0.0 | 0.9 | 1.0 | 1.0 |
| 50 | 50 | 144 | 126 | 125 | 1 | 15 | inner | 0.0 | 0.8 | 0.9 | 0.9 | 1.0 |
| 50 | 50 | 135 | 135 | 135 | 1 | 15 | inner | 0.0 | 0.0 | 0.9 | 1.0 | 1.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 1.0 | 1.1 | 1.5 | 1.5 | 1.5 |
| 20 | 20 | 100 | 6 | 100 | 1 | 15 | outer | 1.0 | 2.0 | 2.5 | 2.5 | 2.5 |
| 20 | 20 | 90 | 16 | 72 | 1 | 15 | outer | 1.3 | 2.0 | 2.5 | 2.5 | 2.5 |
| 20 | 20 | 80 | 26 | 80 | 1 | 15 | outer | 1.3 | 2.0 | 2.1 | 2.5 | 2.5 |
| 20 | 20 | 80 | 26 | 72 | 1 | 15 | outer | 1.0 | 2.0 | 2.1 | 2.5 | 2.1 |
| 50 | 50 | 270 | 0 | 270 | 1 | 15 | outer | 0.0 | 1.4 | 2.1 | 2.5 | 2.5 |
| 50 | 50 | 216 | 54 | 192 | 1 | 15 | outer | 0.0 | 1.4 | 2.1 | 2.1 | 2.1 |
| 50 | 50 | 216 | 54 | 180 | 1 | 15 | outer | 0.0 | 1.4 | 2.1 | 2.1 | 2.1 |
| 50 | 50 | 200 | 70 | 200 | 1 | 15 | outer | 0.0 | 1.1 | 1.5 | 2.1 | 2.1 |
| Outer Max MPR |  |  |  |  |  |  |  | 1.3 | 2.0 | 2.5 | 2.5 | 2.5 |
| DFT-S-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 54 | 108 | 162 | 1 | 30 | inner | 0.0 | 0.8 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 50 | 112 | 162 | 1 | 30 | inner | 0.0 | 0.8 | 1.2 | 1.2 | 1.2 |
| 100 | 100 | 144 | 129 | 135 | 1 | 30 | inner | 0.0 | 0.8 | 1.2 | 1.2 | 1.2 |
| 100 | 100 | 135 | 138 | 135 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 100 | 100 | 135 | 138 | 128 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.8 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 1 | 270 | 1 | 30 | outer | 1.0 | 2.0 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 135 | 27 | 200 | 1 | 30 | outer | 1.0 | 1.4 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 128 | 34 | 200 | 1 | 30 | outer | 1.0 | 1.4 | 2.1 | 2.5 | 2.5 |
| 60 | 100 | 128 | 34 | 192 | 1 | 30 | outer | 1.0 | 1.4 | 2.1 | 2.5 | 2.5 |
| 100 | 100 | 270 | 3 | 270 | 1 | 30 | outer | 0.0 | 1.4 | 2.1 | 2.5 | 2.5 |
| 100 | 100 | 216 | 57 | 200 | 1 | 30 | outer | 0.0 | 1.4 | 2.1 | 2.1 | 2.1 |
| 100 | 100 | 216 | 57 | 192 | 1 | 30 | outer | 0.0 | 1.4 | 2.1 | 2.1 | 2.1 |
| 100 | 100 | 200 | 73 | 192 | 1 | 30 | outer | 0.0 | 1.1 | 1.5 | 2.1 | 1.5 |
| Outer Max MPR |  |  |  |  |  |  |  | 1.0 | 2.0 | 2.5 | 2.5 | 2.5 |

2.2.1 Initial MRP results with RB in one CC

To further check the EVM /IBE impacts in contiguous CA, below Table 6 and 7 give the simulation results for CP-OFDM and DFT-S-OFDM respectively when only CC1 is configured with PRBs. The assumptions are similar as two CC cases.

Table 5 CP-OFDM contig RB allocation in one CC: CA Class B and Class C

| CP-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 20 | 20 | 52 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 20 | 20 | 50 | 56 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.7 |
| 50 | 50 | 136 | 134 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 50 | 50 | 135 | 135 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 50 | 50 | 144 | 126 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.4 | 0.6 | 1.8 | 2.7 |
| 20 | 20 | 106 | 1 | 0 | 1 | 15 | outer | 3.1 | 3.1 | 3.1 | 3.6 | 3.6 |
| 20 | 20 | 100 | 6 | 0 | 1 | 15 | outer | 2.6 | 3.1 | 3.1 | 3.6 | 3.6 |
| 20 | 20 | 90 | 16 | 0 | 1 | 15 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 20 | 20 | 81 | 25 | 0 | 1 | 15 | outer | 2.2 | 2.2 | 2.2 | 2.6 | 3.1 |
| 20 | 20 | 80 | 26 | 0 | 1 | 15 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 3.1 |
| 50 | 50 | 270 | 1 | 0 | 1 | 15 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 50 | 50 | 216 | 54 | 0 | 1 | 15 | outer | 1.8 | 1.8 | 2.2 | 2.2 | 3.1 |
| 50 | 50 | 203 | 67 | 0 | 1 | 15 | outer | 1.8 | 1.8 | 1.8 | 2.2 | 3.1 |
| Outer Max MPR |  |  |  |  |  |  |  | 3.1 | 3.1 | 3.1 | 3.6 | 3.6 |
| CP-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 55 | 107 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 60 | 100 | 60 | 102 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 60 | 100 | 50 | 112 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 100 | 100 | 136 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 100 | 100 | 144 | 129 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 100 | 100 | 135 | 138 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 60 | 100 | 162 | 1 | 0 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 60 | 100 | 125 | 37 | 0 | 1 | 30 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 3.1 |
| 60 | 100 | 135 | 27 | 0 | 1 | 30 | outer | 1.8 | 1.8 | 2.2 | 1.8 | 3.1 |
| 100 | 100 | 273 | 1 | 0 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 100 | 100 | 270 | 3 | 0 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 100 | 100 | 205 | 68 | 0 | 1 | 30 | outer | 0.9 | 0.9 | 0.9 | 1.2 | 2.6 |
| 100 | 100 | 216 | 57 | 0 | 1 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.8 | 2.6 |
| Outer Max MPR |  |  |  |  |  |  |  | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |

Table 6 DFT-S-OFDM contig RB allocation in one CC: CA Class B and Class C

| DFT-S-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 20 | 20 | 52 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 20 | 20 | 50 | 56 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 50 | 50 | 136 | 134 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 50 | 50 | 135 | 135 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 50 | 50 | 144 | 126 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 20 | 20 | 106 | 1 | 0 | 1 | 15 | outer | 0.0 | 0.7 | 1.6 | 2.0 | 2.4 |
| 20 | 20 | 100 | 6 | 0 | 1 | 15 | outer | 0.0 | 0.7 | 1.6 | 1.6 | 2.0 |
| 20 | 20 | 90 | 16 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 1.6 | 2.0 |
| 20 | 20 | 81 | 25 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 20 | 20 | 80 | 26 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 50 | 50 | 270 | 1 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 50 | 50 | 216 | 54 | 0 | 1 | 15 | outer | 0.0 | 0.1 | 0.5 | 0.6 | 2.0 |
| 50 | 50 | 203 | 67 | 0 | 1 | 15 | outer | 0.0 | 0.0 | 0.3 | 0.6 | 2.0 |
| Outer Max MPR |  |  |  |  |  |  |  | 0.0 | 0.7 | 1.6 | 2.0 | 2.4 |
| DFT-S-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 55 | 107 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 1.6 |
| 60 | 100 | 60 | 102 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 1.6 |
| 60 | 100 | 50 | 112 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 100 | 100 | 136 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 100 | 100 | 144 | 129 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 100 | 100 | 135 | 138 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.0 | 0.0 | 0.3 | 2 |
| 60 | 100 | 162 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.8 | 0.9 | 2.0 |
| 60 | 100 | 135 | 27 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.5 | 0.6 | 2.0 |
| 60 | 100 | 125 | 37 | 0 | 1 | 30 | outer | 0.0 | 0.0 | 0.5 | 0.6 | 2.0 |
| 100 | 100 | 273 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.9 | 0.9 | 2.0 |
| 100 | 100 | 270 | 3 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.9 | 0.9 | 2.0 |
| 100 | 100 | 205 | 68 | 0 | 1 | 30 | outer | 0.0 | 0.0 | 0.0 | 0.3 | 1.6 |
| 100 | 100 | 216 | 57 | 0 | 1 | 30 | outer | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| Outer Max MPR |  |  |  |  |  |  |  | 0.0 | 0.1 | 0.9 | 0.9 | 2.0 |

2.2.2 Summary of MRP results for Contiguous RB allocation in one CC and two CCs

Table 7 has summarized the MPR results for contiguous RB allocations in two CCs case, and Table 8 has summarized the MPR results for contiguous RB allocations in one CC case with necessary additional implementation margins being added.

Table 7 MPR for contiguous RB allocation in two CCs

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 4 | 2.5 | 3.5 |
|  | QPSK | 3.5 | 4.5 | 3.5 | 4.5 |
|  | 16QAM | 4 | 5 | 4 | 5 |
|  | 64QAM | 4 | 5 | 4 | 5 |
|  | 256QAM | 4 | 5 | 4 | 5 |
| CP-OFDM | QPSK | 5 | 5.5 | 4 | 5.5 |
|  | 16QAM | 5 | 5.5 | 4 | 5.5 |
|  | 64QAM | 5 | 5.5 | 4 | 5.5 |
|  | 256QAM | 5 | 5.5 | 4 | 5.5 |

Table 8 MPR for contiguous RB allocation in one CC

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 2.5 | 2.5 | 2.5 |
|  | QPSK | 2.5 | 3.5 | 2.5 | 3 |
|  | 16QAM | 2.5 | 4 | 2.5 | 3.5 |
|  | 64QAM | 3 | 4.5 | 3 | 4 |
|  | 256QAM | 4.5 | 5 | 4.5 | 4.5 |
| CP-OFDM | QPSK | 3 | 6 | 3 | 5.5 |
|  | 16QAM | 3 | 6 | 3 | 5.5 |
|  | 64QAM | 4.5 | 6.5 | 4.5 | 5.5 |
|  | 256QAM | 5.5 | 6.5 | 5.5 | 6.5 |

The combined one CC and two CCs MPR results are given in Table 9.

Table 9 MPR for contiguous RB allocation in one CC and two CCs

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 4 | 2.5 | 3.5 |
|  | QPSK | 3.5 | 4.5 | 3.5 | 4.5 |
|  | 16QAM | 4 | 5 | 4 | 5 |
|  | 64QAM | 4 | 5 | 4 | 5 |
|  | 256QAM | 4.5 | 5 | 4.5 | 5 |
| CP-OFDM | QPSK | 5 | 6 | 4 | 5.5 |
|  | 16QAM | 5 | 6 | 4 | 5.5 |
|  | 64QAM | 5 | 6.5 | 4.5 | 5.5 |
|  | 256QAM | 5.5 | 6.5 | 5.5 | 6.5 |

Proposal 2:         Consider MPR results for contiguous RB allocations in table 9.

- PC1.5 NCCA

3.1.1 Initial MRP results

The configurations are:

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- 20MHz+40MHz NCCA combinations with 20MHz gap in between

- scs is 15khz

- LO number include 2LO, and 1LO+TxD

- Modulations 64QAM in both CCs

- Equal PSD

Both forward and reverse IMD impacts and necessary margins have been accounted in the simulation. Based on simulation results in [2] it was observed that the MPR results are similar for QPSK and 64QAM, therefore here only 64QAM is further analysed.

The MPR results are in table 10 and 11 for DFT-S-OFDM and CP-OFDM respectively to meet -30dBm/MHz SEM requirements.

Table 10 DFT-S-OFDM NCCA MPR results

| DFT-S-OFDM (64qam) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | B | 2LO | 1LO + TxD |
| 106 | 1 | 216 | 1 | 57.96 | 6.4 | 9.5 |
| 106 | 1 | 150 | 66 | 46.08 | 5.9 | 9.5 |
| 100 | 1 | 100 | 116 | 36 | 5.4 | 9.5 |
| 72 | 1 | 50 | 116 | 21.96 | 5.4 | 9.5 |
| 61 | 1 | 61 | 157 | 21.96 | 5.9 | 9.6 |
| 60 | 1 | 60 | 156 | 21.6 | 5.9 | 9.5 |
| 46 | 1 | 46 | 170 | 16.56 | 6.4 | 10.4 |
| 32 | 1 | 60 | 156 | 16.56 | 6.3 | 10.4 |
| 32 | 1 | 32 | 184 | 11.52 | 6.4 | 13.6 |
| 28 | 1 | 28 | 188 | 10.08 | 6.4 | 13.6 |
| 24 | 1 | 32 | 184 | 10.08 | 6.9 | 13.6 |
| 18 | 1 | 18 | 198 | 6.48 | 6.9 | 15.0 |
| 15 | 1 | 15 | 201 | 5.4 | 6.9 | 16.6 |
| 14 | 1 | 16 | 200 | 5.4 | 6.8 | 16.6 |
| 14 | 1 | 14 | 202 | 5.04 | 7.4 | 16.6 |
| 12 | 1 | 18 | 198 | 5.4 | 7.4 | 16.6 |
| 12 | 1 | 16 | 200 | 5.04 | 7.5 | 16.6 |
| 12 | 1 | 12 | 204 | 4.32 | 7.5 | 16.6 |
| 9 | 1 | 9 | 207 | 3.24 | 7.4 | 16.6 |
| 8 | 1 | 8 | 208 | 2.88 | 7.5 | 18.6 |
| 6 | 1 | 12 | 204 | 3.24 | 7.5 | 18.6 |
| 6 | 1 | 6 | 210 | 2.16 | 8.1 | 18.6 |
| 4 | 1 | 8 | 208 | 2.16 | 8.1 | 18.6 |
| 4 | 1 | 4 | 212 | 1.44 | 8.1 | 18.6 |
| 3 | 1 | 3 | 213 | 1.08 | 8.9 | 18.7 |
| 2 | 1 | 4 | 212 | 1.08 | 8.9 | 18.7 |
| 2 | 1 | 2 | 214 | 0.72 | 8.8 | 18.6 |
| 1 | 1 | 2 | 214 | 0.54 | 8.2 | 18.8 |
| 1 | 1 | 1 | 215 | 0.36 | 9.0 | 18.8 |

Table 11 CP-OFDM NCCA MPR results

| CP-OFDM (64qam) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | B | 2LO | 1LO + TxD |
| 106 | 1 | 216 | 1 | 57.96 | 7.6 | 9.6 |
| 106 | 1 | 150 | 66 | 46.08 | 7.0 | 9.5 |
| 100 | 1 | 100 | 116 | 36 | 6.5 | 9.5 |
| 72 | 1 | 50 | 116 | 21.96 | 6.5 | 10.4 |
| 61 | 1 | 61 | 157 | 21.96 | 7.0 | 9.6 |
| 60 | 1 | 60 | 156 | 21.6 | 7.0 | 10.4 |
| 46 | 1 | 46 | 170 | 16.56 | 7.0 | 10.4 |
| 32 | 1 | 60 | 156 | 16.56 | 7.6 | 10.4 |
| 32 | 1 | 32 | 184 | 11.52 | 8.2 | 12.4 |
| 28 | 1 | 28 | 188 | 10.08 | 7.6 | 13.6 |
| 24 | 1 | 32 | 184 | 10.08 | 8.2 | 13.6 |
| 18 | 1 | 18 | 198 | 6.48 | 8.1 | 14.9 |
| 15 | 1 | 15 | 201 | 5.4 | 8.1 | 14.9 |
| 14 | 1 | 16 | 200 | 5.4 | 8.2 | 16.7 |
| 14 | 1 | 14 | 202 | 5.04 | 8.2 | 16.6 |
| 12 | 1 | 18 | 198 | 5.4 | 8.8 | 16.6 |
| 12 | 1 | 16 | 200 | 5.04 | 8.9 | 16.6 |
| 12 | 1 | 12 | 204 | 4.32 | 8.8 | 16.6 |
| 9 | 1 | 9 | 207 | 3.24 | 8.9 | 16.6 |
| 8 | 1 | 8 | 208 | 2.88 | 8.8 | 16.6 |
| 6 | 1 | 12 | 204 | 3.24 | 8.8 | 16.6 |
| 6 | 1 | 6 | 210 | 2.16 | 9.5 | 18.6 |
| 4 | 1 | 8 | 208 | 2.16 | 9.6 | 18.6 |
| 4 | 1 | 4 | 212 | 1.44 | 9.5 | 18.5 |
| 3 | 1 | 3 | 213 | 1.08 | 10.3 | 18.5 |
| 2 | 1 | 4 | 212 | 1.08 | 9.6 | 21.2 |
| 2 | 1 | 2 | 214 | 0.72 | 9.6 | 18.6 |
| 1 | 1 | 2 | 214 | 0.54 | 9.7 | 18.6 |
| 1 | 1 | 1 | 215 | 0.36 | 9.7 | 21.3 |

2.2.2 Summary of MRP results for NC CA

From above results, it can be seen the results are similar for DFT-S-OFDM and CP-OFDM. Figure 1 use the max value from DFT and CP OFDM.

Figure 1 NC CA MPR results

The proposed MPR values for PC1.5 NC CA are as below:

- If dualPA-Architecture is not indicated (1LO+TxD)

MPR =   20;		0 < B < 2.16

18;		2.16 =< B < 10.08

15;		10.08 =< B < 16.56

13;		16.56 =< B < 21.96

10;		21.96 =< B

- If dualPA-Architecture is indicated (2LO)

MPR =     11;		0 < B < 3.24

10;		3.24 =< B < 5.4

9;		5.4 =< B < 11.52

8			11.52 =< B

Proposal 3:         Consider MPR results for NCCA as above to meet -30dBm/MHz SEM requirements.

- Conclusion

This paper analyzed the MPR for PC1.5 intra-band contiguous CA, got below proposals.

Proposal 1:         Consider MPR results for non-contiguous RB allocations in two CCs in table 3.

Table 3 MPR for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 6.5 | 10 | 5 | 6.5 | 9 |
|  | QPSK | 5 | 7.5 | 10 | 5 | 7 | 9 |
|  | 16QAM | 5.5 | 7.5 | 10 | 5.5 | 7.5 | 9 |
|  | 64QAM | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
|  | 256QAM | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
| CP-OFDM | QPSK | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
|  | 16QAM | 5.5 | 8 | 10 | 5.5 | 8 | 9 |
|  | 64QAM | 5.5 | 9 | 10 | 5.5 | 9 | 9 |
|  | 256QAM | 5.5 | 9 | 10 | 5.5 | 9 | 9 |

Proposal 2:         Consider MPR results for contiguous RB allocations in table 9.

Table 9 MPR for contiguous RB allocation in one CC and two CCs

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 4 | 2.5 | 3.5 |
|  | QPSK | 3.5 | 4.5 | 3.5 | 4.5 |
|  | 16QAM | 4 | 5 | 4 | 5 |
|  | 64QAM | 4 | 5 | 4 | 5 |
|  | 256QAM | 4.5 | 5 | 4.5 | 5 |
| CP-OFDM | QPSK | 5 | 6 | 4 | 5.5 |
|  | 16QAM | 5 | 6 | 4 | 5.5 |
|  | 64QAM | 5 | 6.5 | 4.5 | 5.5 |
|  | 256QAM | 5.5 | 6.5 | 5.5 | 6.5 |

Proposal 3:         Consider MPR results for NCCA as below to meet -30dBm/MHz SEM requirements.

- If dualPA-Architecture is indicated (2LO)

MPR =     11;		0 < B < 3.24

10;		3.24 =< B < 5.4

9;		5.4 =< B < 11.52

8		11.52 =< B

- Reference

[1] R4-2414277 WF on HPUE for CA in TN, Samsung, #112 2024-08

[2] R4-2412349 R19 MPR for PC1.5 contiguous CA, OPPO, #112 2024-08

[3] R4-2412350 R19 MPR for PC1.5 NC CA, OPPO, RAN4#112 2020-08