3GPP TSG-RAN WG4 Meeting # 112bis                                                           R4-2415016

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.10.2.3

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with UL-MIMO

Document for:	Discussion

# 1. Introduction

RAN#105 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on ATG with UL-MIMO. This paper will focus on the remaining issues.

# 2. Discussion

2.1. ULFPTx mode for UL-MIMO

Uplink full power transmission is a method for UE transmission enhancement. Be limited by power control, UE could not reach full power transmission in some cases, and it will influence the uplink coverage and transmission rate. To ensure PA could reach the max output power for the power class, different ULFPTx Modes are designed for UEs with different capabilities. Considering the characteristics of ATG UE with high altitude and wide coverage, it is better to reserve the possibility of improving ATG UE UL coverage and transmission rate. We think ULFPTx mode should be supported for ATG UL-MIMO.

Proposal 1: Support ULFPTx mode for ATG UL-MIMO.

2.2. Single antenna port and coherent requirement for UL-MIMO

Issue 4-1-2: whether preclude the single antenna port related requirement Agreement: - FFS on whether to preclude the single antenna port related requirement. Issue 4-2-2: whether to apply coherent UL MIMO requirement for ATG UE Agreement: - FFS on whether to apply coherent UL MIMO requirement for ATG UE

In a similar situation regarding whether to support ULFPTx mode for UL-MIMO, we have not yet reached a consensus on supporting a single antenna port and coherent UL-MIMO. Some companies believe that the benefits of implementing these capabilities are minimal, and they are not widely used in legacy User Equipment (UE). However, in our view, these concerns should not be used as reasons to exclude these capabilities, as they can make the ATG use scenarios more flexible. Moreover, no additional special requirements need to be specified for ATG UE, and the existing requirements for TN UE can be reused.

Proposal 2: Apply the single antenna port related requirement and coherent UL MIMO requirement for ATG UE, and the legacy requirements for TN UE could be reused.

2.3. Tx requirements

Last meeting we have reached the following WF:

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18 Agreement: - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated [at each TAB connector]. - If the legacy requirement for UL MIMO is defined per layer, FFS - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer. - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector.

Based on the WF, the Tx requirements could be defined as the sum of all antenna connectors or at each antenna connector. In our opinion, the option could be chosen and the details are shown as following tables.

1. Transmitter power for UL MIMO

Since UL MIMO supports the ULFPTx, the UE maximum output power could be defined as the sum of all antenna connectors.

Table 1. Transmitter power for UL MIMO

| UE maximum output power for UL MIMO | Defined as the sum of all antenna connectors Single antenna-port transmission without indicating Tx diversity with precoding matrix W=1, apply for at least one antenna connector Single antenna-port transmission indicating Tx diversity with precoding matrix W=1, apply as the sum of all antenna connectors Single antenna-port transmission with dual Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 with precoding matrix W=1, defined for at least one antenna connector |
| --- | --- |
| UE maximum output power reduction for UL MIMO | Defined as the sum of all antenna connectors |
| Configured transmitted power for UL MIMO | For UE supporting UL MIMO, the transmitted power is configured per each UE. |

2. Output power dynamics for UL MIMO

For minimum output power and power control, they should be defined as the sum of all antenna connectors. OFF power and ON/OFF time mask should be defined at each antenna connector.

Table 2. Output power dynamics for UL MIMO

| Minimum output power for UL MIMO | Defined as the sum of all antenna connectors for single antenna-port transmission indicating Tx diversity with precoding matrix W=1 |
| --- | --- |
| Transmit OFF power for UL MIMO | Defined at each antenna connector |
| Transmit ON/OFF time mask for UL MIMO | Defined at each antenna connector |
| Power control for UL MIMO | Defined as the sum of all antenna connectors |

3. Output power dynamics for UL MIMO

Transmit signal quality should be defined per layer or as the sum of emissions from both antennas to account for the UL MIMO scheme.

Table 3. Output power dynamics for UL MIMO

| Frequency error for UL MIMO | per layer |
| --- | --- |
| Error Vector Magnitude | per layer |
| Carrier leakage | per layer |
| In-band emissions | Apply at each transmit antenna connector |
| EVM equalizer spectrum flatness for UL MIMO | per layer |

4. Output RF spectrum emissions for UL MIMO

Output RF spectrum emissions requirements should be defined as the sum of all antenna connectors.

Table 4. Output RF spectrum emissions for UL MIMO

| Occupied bandwidth for UL MIMO | Defined as the sum of all antenna connectors |
| --- | --- |
| Out of band emission for UL MIMO | Defined as the sum of all antenna connectors |
| Spurious emission for UL MIMO | Defined as the sum of all antenna connectors |
| Transmit intermodulation for UL MIMO | Defined as the sum of all antenna connectors |

# 3. Conclusions

In this contribution, UE RF requirements for ATG with UL-MIMO are discussed with the following proposals:

Proposal 1: Support ULFPTx mode for ATG UL-MIMO.

Proposal 2: Apply the single antenna port related requirement and coherent UL MIMO requirement for ATG UE, and the legacy requirements for TN UE could be reused.

Proposal 3:

If the legacy requirement for UL MIMO is defined at each antenna connector,

-	For ATG UE with antenna array, the requirement should be updated [at each TAB connector].

If the legacy requirement for UL MIMO is defined per layer

-	For ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer.

Use the above Tables to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18.

# 4. References

[1] RP-242381, Revised WID on Enhancements for Air-to-ground network for NR