3GPP TSG-RAN WG4 Meeting #112bis	R4-2415035
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.1.1.2.1

Source: 	OPPO

Title: 	Discussion on MPR reduction for single carrier

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, the power domain enhancements for single carrier were discussed and some agreements were approved in WF [1].

Issue 1-1-1: Approaches for scenario 1 - WF - Further evaluate the reduced MPR values with relaxation of ACLR/SEM to certain levels or waived - Other ways to reduce MPR are not precluded Issue 1-2-1: Clarification of BS CBW - Proposals - Proposal 1: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios. (Huawei) - WF - FFS in next meeting Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - Proposals - Approach 1: UE CBW based - Option 1: The extended UE CBW based method as captured in the WF in last meeting. - Approach 2: BS CBW based - Option 2: BS CBW based inner/outer method. (Skyworks) - BS NRB instead of UE NRB. - Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart. - The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR. - For margin or stricter emissions, the BS can signal a reduced NRB and RBshift. - Option 2a: the feature ‘narrower UE channel BW within wider BS bandwidth’ is specified for a UE configured with a UE-specific CHBW within a wider BS bandwidth, the said UE compliant with selected OOBE requirements applicable for the wider BS bandwidth (the cell bandwidth). (Ericsson, Nokia) - WF - Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach

#### Issue 1-2-3: whether extended UE CBW edge could exceed the BS CBW edge - Proposals - Proposal 1: The extended UE CBW should not exceed the BS CBW. (Samsung, Sony, vivo, MTK, LGE) - Proposal 2: One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge. (ZTE) - WF - proposal 1 is agreeable for singe CC case, FFS multi-CC/BS transmitted BW case #### Issue 1-2-4: Whether and where to use IBE in the larger BS CBW - WF - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW #### Issue 1-2-5: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. (Samsung, Sony, ZTE, vivo, LGE) - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth. (Nokia, Ericsson) - WF - FFS in next meeting #### Issue 1-2-6: Boundary to apply SE - WF - [SE applicability depends on the extended CBW] #### Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE - Proposals - Option 1: The integral region and the boundary of OOBE should be based on UE CBW. (Samsung, vivo, ZTE, China Telecom) - Option 2: The integral region and boundary of OOBE is based on extended UE CBW. (Sony) - WF - FFS in next meeting #### Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW - Proposals - Proposal 1: (Apple) - If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel. - If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third. - Proposal 2: (Qualcomm) - If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW. - If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW. - Proposal 3: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point. (vivo) - Proposal 4: Consider extended UE CBW approach to improve MPR starting from 1.5*CBW (OPPO) - Proposal 5: (Huawei) - For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication. - Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency. - For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK - WF - FFS with consideration of above proposals in next meeting #### Issue 1-2-9: Whether and how to consider the Asymmetrical extended CBW approach - WF - Asymmetrical extended CBW approach can be considered, FFS the details #### Issue 1-2-11: Signaling aspects - Proposals - Proposal 1: no changes of signalling are specified for the feature ‘narrower UE channel BW within wider BS bandwidth’ except possibly specification of a capability bit to indicate support of the feature. (Ericsson) - Proposal 2: Need to consider capability for extended UE CBW procedure. (LGE) - Proposal 3: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability. (Sony) - Proposal 4: The frequency intervals for the UE CBW and the BS CBW should be judged by the NW and the indication of MPR reduction could be directly sent to the UE when the corresponding conditions are met. (vivo) - Proposal 5: Establish a start point and continue to details for BS indication with the deepen in MPR reduction mechanism. (CTC) - WF - To discuss the signalling aspects after sufficient evaluation of power boosting and/or MPR reduction in terms of relaxed requirements.

This contribution further discusses power domain enhancement for single carrier.

- Discussion

2.1 The clarification of BS channel bandwidth and BS RF bandwidth

In RAN4 #110bis meeting, RAN4 has agreed to prioritize scenario 1-1 and scenario 2 for initial study of power domain enhancements for single carrier in terms of relaxed requirements and scenario 1-2 will be studied after scenario 1-1 and scenario 2:

- Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator)

- Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators)

- Scenario 2: Narrower UE channel BW within wider BS bandwidth

In RAN4 #111 meeting, for scenario 2 it was agreed that:

- Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g., each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects

- Other mechanisms are not precluded

Scenario 2 was described as narrower UE channel BW within wider BS bandwidth, and in the previous approved WF, some other descriptions have emerged, i.e., BS CBW for one operator holding spectrum, inside a larger BS CBW. Hence, what’s the meaning of BS channel bandwidths was discussed in last meeting and need further discuss issue 1-2-1 in this meeting:

Here excerpt the definitions of BS RF Bandwidth and BS channel bandwidth from TS 38.104 and TS 37.104:

In TS 38.104:

In TS 37.104:

And in clause 5.3.1 of Spec TS 38.104, it describes that

The BS channel bandwidth supports a single NR RF carrier in the uplink or downlink at the Base Station.

Based on above definitions and description, we can get that

- BS channel bandwidth means BS RF bandwidth only covers a single carrier

- BS RF Bandwidth can

  - Case 1: Only cover single carrier, in this case, the BS RF bandwidth equals to BS channel bandwidth.

  - Case 2: cover multiple contiguous carriers, these carriers can belong to single RAT or multi-RATs.

  - Case 3: cover multiple sub-blocks, the carriers in these sub-blocks can belong to single RAT or multi-RATs.

That is, one BS RF bandwidth can include single or multiple BS channel bandwidths, but BS channel bandwidth only supports a single RF carrier, as below figure shown:

FC,block 1,low

Sub-block Bandwidth, BWChannel,block[MHz]

Fedge,block 1, low

Fedge,block n, low

FC,block n,high

FC,block 1,high

Sub block n

Sub block 1

Base Station RF Bandwidth

Transmission Bandwidth Configuration of the highest carrier in a sub-block [RB]

Transmission Bandwidth Configuration of the lowest carrier in a sub-block  [RB]

Transmission Bandwidth Configuration of the highest carrier in a sub-block [RB]

Fedge,block 1,high

Upper Sub-block Edge

Lower Sub-block Edge

Lower Sub-block Edge

Foffset_high

Fedge,block n,high

Sub-block Bandwidth, BWChannel,block [MHz]

Resource block

Foffset_low

FC,block n,low

Upper Sub-block Edge

Foffset_high

Resource block

Foffset_low

Transmission Bandwidth Configuration of the lowest carrier in a sub-block  [RB]

...

Figure 2.1-1 Illustration of Base Station RF Bandwidth

Therefore, about Issue 1-2-1, the clarification should be:

Proposal 1: clarify that

- BS channel bandwidth can only cover single carrier.

- BS RF bandwidth can cover a single carrier, multi-carriers and multi-RATs scenarios.

- BS RF bandwidth equals to BS channel bandwidth only when it covers a single carrier.

2.2 Scenario 2: Narrower UE channel BW within wider BS bandwidth

Based on the clarification of BS channel bandwidth and BS RF bandwidth, scenario 2 can be divided to two sub-scenarios:

- Scenarios 2-1: For single carrier operation, narrower UE channel BW within a wider BS channel bandwidth.

- Scenarios 2-2: For multiple carrier operation, UE channel BW within a wider BS RF bandwidth, the UE channel BW can be narrower or equal to that BS channel bandwidth which it is located in.

Here, we can first discuss scenario 2-1 based on symmetrical extended CBW. The purpose of this scenario is to convert outer RB allocation to inner RB allocation by shifting the emission requirements. In last meeting, two approaches were discussed:

- Approach 1: extended UE CBW based

- Approach 2: BS CBW based

Actually, for the Boundary to apply ACLR/SEM and OOBE, there is no essential difference between extended UE CBW based and BS CBW based. In my understanding, the key issue is how to extend the IBE for usage between the edge of UE CBW and extended UE CBW or BS CBW.

Since IBE is defined based on transmission bandwidth configuration of UE including the related parameters for general requirements, carrier leakage and IQ image, i.e., NRB and LCRB. Currently, NRB is defined based on the regular defined channel bandwidth, i.e., 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 and 100MHz.

So the issue how to decide the NRB for extended IBE based on extended UE CBW approach need consider, when the extended UE CBW is not the regular defined channel bandwidth. For example, if the extended UE CBW is 37.5MHz (extended 1.5 times based on UE CBW, i.e., 25MHz), the gap between the edge of UE CBW and extended UE CBW is 6.25MHz, it can put down the integer RB numbers.

BS CBW is defined the same with UE CBW, the extended IBE can adopt the NRB of the BS CBW directly, there is no additional NRB issue for extended IBE based on BS CBW approach.

Considering the extended IBE, using BS CBW based to convert the outer RB allocation to inner RB allocation and as the boundary to apply emission requirements is more reasonable, if the size of extended UE CBW is not the regular defined channel bandwidth. But the UE emission requirements based on BS CBW will conflict with current definition of UE emission requirements.

Observation 1: It needs to define additional configuration of NRB for extended IBE based on extended UE CBW approach, if the extended UE CBW is not the regular defined channel bandwidth.

To keep the spectrum efficiency and the flexibility of network scheduling, it is better to shift the emission and apply extended IBE based on extended UE CBW, like this, when the BS CBW is larger than the extended UE CBW of one UE, the left RBs in BS CBW could assign to another UE. To avoid define additional configuration of NRB for extended IBE, the extended UE CBW should be the regular defined channel bandwidth for extended UE CBW based approach.

Proposal 2: Adopt extended UE CBW based approach to convert the outer RB allocation to inner RB allocation.

Proposal 3: The extended IBE and the boundary to apply ACLR/SEM and OOBE should be defined based on extended UE CBW.

Proposal 4: The extended UE CBW should belong to the regular defined channel bandwidth, i.e., 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 and 100MHz.

In last meeting, we provided the related simulation results for 20MHz channel bandwidth CP-OFDM QPSK modulation in [2] research the relationship of the MPR reduction and the extended UE CBW. And we got that:

- When extended UE CBW is 2 times of the UE CBW, all outer RB allocation converts to inner RB allocation and the MPR can be improved up to 2.5dB.

- When extended UE CBW is 1.5 times of the UE CBW, the MPR can still be improved up to 2dB although not all outer RB allocation are converted to inner RB allocation.

- When extended UE CBW is 1.25 times of the UE CBW, only small part of regions for outer RB allocation can get improvement.

To keep extended UE CBW be the regular defined channel bandwidth, RAN 4 can first define 2 times of the UE CBW as the largest extended UE CBW.

And in last meeting, it was agreed that

- Extended UE CBW should not exceed the BS CBW for single carrier operation.

Therefore, for 2 times extended method, it’s obvious that the extended CBW for UE CBW of 5, 10, 15, 20, 25, 30, 35, 40, 45, 50MHz are still the regular defined channel bandwidth.

For the extended UE CBW larger than the largest BS CBW of a band, the largest BS CBW of this band can be as the extended UE CBW, and how many MPR improvements can be defined based on the ratio of the extended UE CBW and UE CBW. For example, the 2 times extended CBW of 60MHz UE CBW is 120MHz that is larger than the largest BS CBW 100MHz, so 100MHz extended CBW can be adopted and the extended ratio is 1.7.

Proposal 5: RAN4 could define multiple extended ratios based on extended UE CBW approach:

- If the extended UE CBW is less than or equal to the largest BS CBW of a band, 2 * UE CBW as the default extended UE CBW in which all outer RB allocation are converted into inner RB allocation, the MPR for inner RB allocation could be used.

- If the extended UE CBW is larger than the largest BS CBW of a band, using the largest BS CBW of this band as the extended UE CBW, than define the MPR improvement level based on the ratio of extended UE CBW and UE CBW, i.e,

  - X dB MPR improvement level for the extended ratio belongs to [1.5, 2]

  - Y dB MPR improvement level for the extended ratio belongs to [1, 1.5]

There is no need to consider the asymmetrical extended CBW approach, it should use the narrowest gap between the edge of UE CBW to the BS CBW as the shifting value for extended UE CBW and then consider it as the symmetrical extended CBW.

Proposal 6: Don’t need consider the asymmetrical extended CBW approach, use the narrowest gap between the edge of UE CBW to the BS CBW as the shifting value for extended UE CBW and then consider it as the symmetrical extended CBW.

For the largest UE CBW for a band, especially for the 100MHz UE CBW, it can’t be considered in the scenario that Narrower UE channel BW within wider BS bandwidth for single carrier operation, since there is no any gap between the edge of UE CBW and BS CBW.

Proposal 7: The largest UE CBW for a band, especially for the 100MHz UE CBW can’t be considered in the scenario that Narrower UE channel BW within wider BS bandwidth for single carrier operation.

For UE channel BW within a wider BS RF bandwidth for multiple carriers operation, the UE channel BW must be narrower or equal to that BS channel bandwidth which it is located in, although the BS RF bandwidth covers multiple BS CBW.

When the UE channel BW is equal to the BS channel bandwidth, i.e., the largest UE CBW for a band, it can be considered in scenario 1-1 that no adjacent in-band/out-of-band co-existence issue (single operator).

When the UE channel BW is less than the BS channel bandwidth for multiple carriers operation, it can use the same method with single carrier operation as the starting point, whether further consider ACLR/SEM relaxation can further study.

Proposal 8: For multiple carriers operation of scenario 2:

- When the UE channel BW is equal to the largest BS channel bandwidth, i.e., the largest UE CBW for a band, it can be considered in scenario 1-1 that no adjacent in-band/out-of-band co-existence issue (single operator).

- When the UE channel BW is less than the largest BS channel bandwidth for multiple carriers operation, it can use the same method with single carrier operation as the starting point, whether further consider the relaxation like scenario 1-1 can further study.

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band.

Proposal 1: clarify that

- BS channel bandwidth can only cover single carrier.

- BS RF bandwidth can cover a single carrier, multi-carriers and multi-RATs scenarios.

- BS RF bandwidth equals to BS channel bandwidth only when it covers a single carrier.

Observation 1: It needs to define additional configuration of NRB for extended IBE based on extended UE CBW approach, if the extended UE CBW is not the regular defined channel bandwidth.

Proposal 2: Adopt extended UE CBW based approach to convert the outer RB allocation to inner RB allocation.

Proposal 3: The extended IBE and the boundary to apply ACLR/SEM and OOBE should  be define based on extended UE CBW.

Proposal 4: The extended UE CBW should belong to the regular defined channel bandwidth, i.e., 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 and 100MHz.

Proposal 5: RAN4 could define multiple extended ratios based on extended UE CBW approach:

- If the extended UE CBW is less than or equal to the largest BS CBW of a band, 2 * UE CBW as the default extended UE CBW in which all outer RB allocation are converted into inner RB allocation, the MPR for inner RB allocation could be used.

- If the extended UE CBW is larger than the largest BS CBW of a band, using the largest BS CBW of this band as the extended UE CBW, than define the MPR improvement level based on the ratio of extended UE CBW and UE CBW. I.e,

  - X dB MPR improvement level for the extended ratio belongs to [1.5, 2]

  - Y dB MPR improvement level for the extended ratio belongs to [1, 1.5]

Proposal 6: Don’t need consider the asymmetrical extended CBW approach, use the narrowest gap between the edge of UE CBW to the BS CBW as the shifting value for extended UE CBW and then consider it as the symmetrical extended CBW.

Proposal 7: The largest UE CBW for a band, especially for the 100MHz UE CBW can’t be considered in the scenario that Narrower UE channel BW within wider BS bandwidth for single carrier operation.

Proposal 8: For multiple carriers operation of scenario 2:

- When the UE channel BW is equal to the largest BS channel bandwidth, i.e., the largest UE CBW for a band, it can be considered in scenario 1-1 that no adjacent in-band/out-of-band co-existence issue (single operator).

- When the UE channel BW is less than the largest BS channel bandwidth for multiple carriers operation, it can use the same method with single carrier operation as the starting point, whether further consider the relaxation like scenario 1-1 can further study.

- Reference

[1] R4-2414441 WF on power domain enhancement, Huawei, RAN4 #112

[2] R4-2412351 R19 MPR reduction for single carrier, OPPO, RAN4 #112