3GPP TSG-RAN WG4 Meeting #112bis	R4-2415036
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.1.1.2.2

Source: 	OPPO

Title: 	Discussion on the applicable MPR for FR1 intra-band UL CA with single activated cell

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, the power domain enhancements for FR1 intra-band CA were discussed and some agreements were approved in WF [1].

Sub-topic 2-1: Intra-band contiguous UL CA Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung) - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE) - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated; - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm). - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4 - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4 - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4 - WF - FFS in next meeting Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung) - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei) - WF - FFS in next meeting

#### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks) - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo) - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson) - WF - FFS in next meeting #### Issue 2-1-4: Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell - Proposals - Proposal 1: Add the following description into clause 6.2A.2.1: - “For intra-band contiguous carrier aggregation with single CC with activated cell, MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.” - WF - FFS the details of draft CR - Sub-topic 2-2: Intra-band non-contiguous CA #### Issue 2-2-1: General considerations - Agreement - The work scope for FR1 NC CA should be limited to only study MPR applicability. #### Issue 2-2-2: Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. (Qualcomm, ZTE, Huawei) - Proposal 2: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC. (Skyworks) - Proposal 3: for non-contiguous UL CA configurations with sub-blocks consisting of one cell and supported by dual PA architecture, the non-CA (single CC) MPR applies for one cell active among the configured uplink serving cells, the other cell deactivated. (Ericsson) - Proposal 4: for non-contiguous UL CA configurations with sub-blocks consisting of one cell not supported by a dual PA architecture, applicability of the non-CA (single CC) MPR for one active cell among the configured uplink serving cells, the other cell deactivated, is subject to UE capability, e.g. indication of [mpr-singleCC-activated-FR1] for the band combination. (Ericsson) - WF - FFS in next meeting

This contribution further discusses power domain enhancement for FR1 intra-band CA based on the open issues in last meeting.

- Discussion

2.1 Intra-band contiguous CA

According to the discussion in last meeting, the applicable MPR for intra-band contiguous CA with single activated cell can follow below description:

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

And the key issue is which configured Tx power requirements should be used for intra-band contiguous CA with single activated cell, especially for PC2 intra-band contiguous CA when TxD capability is indicated.

From current Spec, we can see:

- For single carrier, configured Tx power requirements are defined per CC.

- For intra-band contiguous UL CA, its total configured maximum output power PCMAX was defined based on its configured Tx power requirements per CC.

- For UE supporting Tx diversity, the transmitted power is configured per each UE.

Actually, the configured Tx power requirements for single carrier, intra-band contiguous UL CA and Tx diversity are the same when only with single activated cell.

Since in the clause 6.2A.4.1.1 for configured transmitted power for Intra-band contiguous CA, it is indicated that the MPR for Intra-band contiguous CA are specified in clause 6.2A.2. The intra-band contiguous CA with single activated cell could still follow the definition of configured transmitted power for Intra-band contiguous CA in clause 6.2A.4.1.1,  the only thing we need to do is indicating the reference table number clearly for intra-band contiguous CA with single activated cell in clause 6.2A.2.

Don’t need follow different definitions of configured transmitted power based on different PC classes and different UE capability.

Proposal 1: The following applicable MPR for intra-band contiguous CA with single activated cell need capture into clause 6.2A.2 clearly:

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal 2: Intra-band contiguous CA with single activated cell still follow the definition of configured transmitted power for Intra-band contiguous CA in clause 6.2A.4.1.1

Although intra-band contiguous CA with single activated cell only transmit UL signals in one CC at the same time, it still belongs to the intra-band contiguous UL CA from the network perspective. The spurious emissions/ACLR/SEM need keep for aggregated/configured CBW.

Proposal 3: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell.

2.2 intra-band non-contiguous CA

In last meeting, it agreed that

- The work scope for FR1 NC CA should be limited to only study MPR applicability.

Therefore, for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

Proposal 4: The applicable MPR for intra-band non-contiguous UL CA with only one UL CC activated doesn’t need further enhancement

- Conclusion

This paper discussed the applicable MPR for intra-band UL CA with single activated cell and proposed:

Proposal 1: The following applicable MPR for intra-band contiguous CA with single activated cell need capture into clause 6.2A.2 clearly:

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal 2: Intra-band contiguous CA with single activated cell still follow the definition of configured transmitted power for Intra-band contiguous CA in clause 6.2A.4.1.1

Proposal 3: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell.

Proposal 4: The applicable MPR for intra-band non-contiguous UL CA with only one UL CC activated doesn’t need further enhancement

- Reference

[1] R4-2414441 WF on power domain enhancement, Huawei, RAN4 #112