3GPP TSG-RAN WG4 Meeting #112bis	R4-2415037
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.6.2

Source: 	OPPO

Title: 	Discussion on reducing the number of UE Rx chains

Document for:	Discussion

- Introduction

In RAN4 #112meeting, how to reduce the number of UE Rx chains for fragmented carriers were discussed and some agreements were approved in WF [1]:

Agreements: Issue 2-1-1: Applicability and clarification on the scope - The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain - The two non-contiguous CCs is still under the current DL non-contiguous CA framework - Each fragment is treated as an individual CC - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz. - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands. - Provide the details related to gap between fragmented DL blocks] Issue 3-1-4: Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements - Proposal 1: put off the signalling related discussions until the UE performance aspects are clear. Open issues: Issue 2-1-2: Reference architecture - The existing requirements for DL NCCA was specified based on partially-shared Rx architecture according to TR36.823. - FFS on following options for reducing number of Rx chains for DL non-contiguous CCs: - Option 1: Only the “fully shared Rx chain architecture” needs to be studied - Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz - Other options are not precluded - Companies are encouraged to provide baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study - FFS on whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier. Issue 2-1-4: Study on power spectral density difference between carriers of co-located adjacent channel operators - FFS on following options in next meeting - Option 1: Prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure) - For evaluation purpose and for simplicity, assume the two gNBs of two co-location operators are both transmitting at maximal allowed power level - equal PSD between two co-location operators could be assumed from gNB transmisstion point of view for evaluation purpose, as the starting point - The PSD difference from UE reception point of view could vary as per the different antenna type, antenna pointing direction, etc between operators - Option 2: For inter-operator deployment scenario, both shared RRU case and separate RRU case are considered - Other options are not precluded Issue 2-1-5: Prerequisite conditions - This is discussed in issue 3-1-2 Issue 2-1-6: Others - FFS following proposals in next meeting: - Proposal 1 - The impact of UE switching between common RF chain and separate RF chains, including potential interruption, should be studied. If there is RRM impact, RAN4 should consider it. - Proposal 2 - The new CA configuration supported only by adjusted RF requirements could be reported to the NW together with other default supported CA configuration. - Proposal 3 - gNB trigger UE to measure gap could be considered.

This contribution further discussed the scope and architecture for fragmented carriers.

- Discussion

In the SID [2], it’s described that the reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) is for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario.

In last meeting, it was further clarified in the WF that

- The two non-contiguous CCs is still under the current DL non-contiguous CA framework

  - Each fragment is treated as an individual CC

- The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

- [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

  - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

    - Provide the details related to gap between fragmented DL blocks]

The fragments here are just used to describe the spectrums in one band are not contiguous for one operator and each fragment is treated as an individual CC. And the purpose of SID is trying to make these fragmented carriers can be fully utilized in the form of intra-band non-contiguous CA by using single RX chain..

Therefore, single DL band with frequency span ≤ 100 MHz containing two non-contiguous CCs and the fragments can be described as the similar description of frequency separation class in FR2 intra-band non-contiguous CA as below:

Proposal 1: Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz.

In last meeting, it agreed that two non-contiguous CCs is still under the current DL non-contiguous CA framework and the impact of UE switching between common RF chain and separate RF chains may need consider, therefore, RAN4 should first consider the fragmented carriers for the existing intra-band non-contiguous DL CA, where the frequency separation between these two non-contiguous CCs is less than or equal to 100MHz, then discuss whether extend to other bands in which the intra-band non-contiguous DL CA aren’t defined.

Proposal 2: RAN4 should first consider the fragmented carriers for those bands, in which the intra-band non-contiguous DL CA have been defined, where the frequency separation between these two non-contiguous CCs is less than or equal to 100MHz, then discuss whether extend to other bands.

Since there is no intra-band non-contiguous DL CA in band n39 in current Spec, it can be removed tentatively.

Proposal 3: Remove band n39 from the given example bands of n2/n25, n3, n7, n66, n41, n39.

In current Spec, the corresponding Rx requirements for intra-band non-contiguous CA are defined for intra-band non-contiguous carrier aggregation of two or more sub-blocks with channel bandwidth larger than or equal to 5 MHz, and the 3MHz channel bandwidth for CA are discussing in other R-19 WID, to keep the progress of this topic, RAN4 doesn’t consider 3MHz for fragmented carriers.

Proposal 4: RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers.

About Wgap between fragmented DL blocks, in current Spec, the maximum Wgap for FDD bands supporting intra-band non-contiguous DL CA is the largest separation in the DL frequency range considering the aggregated channel bandwidth. And for some bands, the smaller Wgap are defined to reduce the impact of Tx transmission to the Rx receiver, as below table 2-1 listed:

Table 2-1 The Wgap and DL frequency range for FDD bands supporting intra-band non-contiguous DL CA

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode | DL freqency range |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD | 60 |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD | 60 |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD | 75 |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD | 25 |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD | 70 |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD | 17 |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD | 65 |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |  |
| CA_n25(2A) CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 |  |  |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD | 35 |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD | 90 |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 | FDD | 35 |
|  |  | 25MHz + 5MHz11 | Wgap = 5.0 | 20 (RBstart = 19) | 25.1 |  |  |

From above table, we can see the size of Wgap is related with aggregated channel bandwidth and DL frequency range. And the current configurations for intra-band non-contiguous DL CA represent the real request of the operators. To avoid deterioration in the receiver performance too much due to share the IF and BB filter, we can first choose the Wgap that ΔRIBNC=0 of intra-band non-contiguous DL CA for fragmented carriers

Proposal 5: About the maximum Wgap between fragmented DL blocks:

- For the example FDD bands, the maximum Wgap can use the below Wgap values that ΔRIBNC=0 of intra-band non-contiguous DL CA and corresponding aggregated channel bandwidth as the Starting point:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | 30 | 25 | 0 |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | 45 | 25 | 0 |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | 30 | 50 | 0 |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | 30 | 25 | 0 |
| CA_n66(2A) | 15/15 | 5MHz + 5MHz | 80 | 25 | 0 |

- For the example TDD band n41, using 80MHz Wgap as the starting point as below table:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] |
| --- | --- | --- | --- |
| CA_n41(2A) | 15/15 | 10MHz + 10MHz | 80 |

In current Spec, the minimum Wgap for intra-band non-contiguous DL CA need meet:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

The fragment carriers is still treated as intra-band non-contiguous DL CA, it also need meet the request of minimum Wgap.

Proposal 6: The minimum Wgap should meet:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

Reference the partially-shared Rx architecture for the existing requirements for DL NCCA according to TR36.823. RAN4 should use “fully shared Rx chain architecture” as the starting point for fragmented carriers, to further increase the flexibility of fragmented carriers for one operator through the intra-band non-contiguous DL CA frame by reducing the Rx chain.

Proposal 7: Using “fully shared Rx chain architecture”as the the starting point to reduce number of Rx chains for DL non-contiguous CCs, other architecture can further study.

- Conclusion

This paper analyzed the application of fragmented carriers and proposed:

Proposal 1: Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz.

Proposal 2: RAN4 should first consider the fragmented carriers for those bands, in which the intra-band non-contiguous DL CA have been defined, where the frequency separation between these two non-contiguous CCs is less than or equal to 100MHz, then discuss whether extend to other bands.

Proposal 3: Remove band n39 from the given example bands of n2/n25, n3, n7, n66, n41, n39.

Proposal 4: RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers.

Proposal 5: About the maximum Wgap between fragmented DL blocks:

- For the example FDD bands, the maximum Wgap can use the below Wgap values that ΔRIBNC=0 of intra-band non-contiguous DL CA and corresponding aggregated channel bandwidth as the Starting point:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | 30 | 25 | 0 |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | 45 | 25 | 0 |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | 30 | 50 | 0 |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | 30 | 25 | 0 |
| CA_n66(2A) | 15/15 | 5MHz + 5MHz | 80 | 25 | 0 |

- For the example TDD band n41, using 80MHz Wgap as the starting point as below table:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] |
| --- | --- | --- | --- |
| CA_n41(2A) | 15/15 | 10MHz + 10MHz | 80 |

Proposal 6: The minimum Wgap should meet:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

Proposal 7: Using “fully shared Rx chain architecture”as the the starting point to reduce number of Rx chains for DL non-contiguous CCs, other architecture can further study.

- Reference

[1] R4-2414457, WF on fragmented DL carriers study, MediaTek Inc., RAN4#112 2024-08

[2]RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc., RAN#104 2024-06