3GPP TSG-RAN WG4 Meeting #112bis	R4-2415038
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.6.3

Source: 	OPPO

Title: 	Discussion on impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

In RAN4 #112meeting, the impacts on UE RF requirements and DL performance were discussed in WF [1]:

Open issues: Issue 3-1-1: Assumptions on test configurations for evaluation - See if RAN4 can agree on the following test configurations: - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos - UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB - Reuse the Rel-15 assumption of Rx chain image rejection, which 25dB - At this stage RAN4 does not take DL MIMO into account - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc. - Other configurations are not precluded if identified as having performance impact Issue 3-1-2: Prerequisite conditions - FFS on following proposals in next meeting: - Proposal 1: Maintain the current ACS and IBB requirements to ensure UE selectivity - Proposal 2: The fourth sub-bullet in the SID should be simplified to “Determine a reasonable upper level of the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs” - Proposal 3: RAN4 to confirm it is feasible to receive two non-contiguous CCs in a shared RF chains with the assumption that power spectral density imbalance between any of the CC and also the signal in the gap are within 6dB - FFS on assuming higher PSD for the signal in the gap of 2CC Issue 3-1-3: Requirements for evaluation Companies’ views are diverged but not controversial. Further discuss following bullets see if they can be agreeable as a package - Both TDD and FDD intra-band DL contiguous CA should be included - ΔRIBNC, ACS, IBB, NBB requirements [on both PCC and SCC] as well as the minimum guard band sizes for single Rx RF chain, all need to be evaluated - Out-of-band blocking and spurious response do not need requirement adjustment - Adjusted requirements for fragmented carriers shall be separated from the existing FDD/TDD intra-band carrier non-contiguous aggregation ΔRIBNC, ACS, IBB, NBB - Discussion on if a rejection level of the image caused by the in-gap interferer is needed - Discussion on new requirement(s) if identified is not precluded Issue 3-1-5: Others - FFS following proposal in next meeting: - RAN4 shall discuss expected UE fallback behaviour when an in-gap interferer precludes the UE to operate in fragmented carrier mode

- Discussion

In the companion contribution, we proposed using the Wgap that ΔRIBNC=0 of intra-band non-contiguous DL CA for fragmented carriers to try to reduce the impact of Tx transmission. In these cases of ΔRIBNC=0, the UL PCC allocation is Full RB allocation.

Proposal 1: RAN4 can agree on the following test configurations:

  - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC=0, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos

    - UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

    - Reuse the Rel-15 assumption of Rx chain image rejection, which 25dB

    - At this stage RAN4 does not take DL MIMO into account

    - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc.

  - Other configurations are not precluded if identified as having performance impact

The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain. The main work is to research the impact of interferer in-gap for the inter-operator co-located scenario due to no rejection of IF and/or BB filters.

And for the ACS of intra-band non-contiguous carrier aggregation, The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active. That is, the largest interferer power in-gap and out-of-gap is -25dBm. For fragmented carriers, the largest interferer power in-gap shouldn’t larger than -25dBm.

Proposal 2: The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain.

Due to no rejection of IF and/or BB filters, the UE selectivity in-gap may not be consistent with current levels. Therefore, RAN4 will should study a single Rx RF chain from two aspects: first is how much the UE selectivity will worsen in-gap; Second is how much the minimum interferer can be achieved considering no rejection of IF and/or BB filters and the impact of Tx transmission.

In current Spec, ACS are tested under two cases for non-CA and intra-band CA: case 1 is for smaller interferer, case 2 is for larger interferer. No matter for case 1 or case 2, the power difference between wanted signal and interferer is ACS-SNR-IM.

Based on above analysis, the purpose of fourth sub-bullet in the SID that determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators is to decide a reasonable ACS in-gap. In the parameters of ACS, the wanted signal and interferer are specified through total power in the bandwidth, and the bandwidth of interferer is 5MHz for all wanted signal channel bandwidths. So it doesn’t make sense to determine the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs.

Proposal 3: RAN4 should determine a reasonable upper level of the power difference of the in-gap signal compared to the two non-contiguous CCs.

Proposal 4: The parameters for test case 1 and case 2 in-gap need re-evaluate based on new power difference of the in-gap signal compared to the two non-contiguous CCs.

Proposal 5: ΔRIBNC, IBB, NBB requirements [on both PCC and SCC] as well as the minimum guard band sizes for single Rx RF chain, all need to be re-evaluated.

Since the frequency offset of out-of-band blocking are specified based on the lowest frequency and highest of the downlink operating band， there is no out-of-band blocking and spurious response in-gap of two non-contiguous CCs in one band.

Proposal 6: Out-of-band blocking and spurious response do not need requirement adjustment.

The purpose of reducing number of Rx chain for one operator is to make them fully utilizing their fragmented spectrum holdings in a single CA combination. That is, it is to support more intra-band non-contiguous CCs in a single band. Therefore, the UE fallback mode behaviour should be considered according to the aggregated numbers of CCs. For example:

- If the UE supporting highest order of intra-band non-contiguous CA is CA_n41(2A), the UE can fallback to multiple Rx chains in the same order CA or non-CA when an in-gap interferer precludes the UE to operate in fragmented carrier mode;

- If the UE supports the highest order of intra-band non-contiguous CA is DL CA_n41(3A) that 2 CCs within the frequency span of a single Rx chain is received by a single Rx chain and another CC outside of the frequency span is received by anther Rx chain, the UE could fallback to next fallback level CA when an in-gap interferer precludes the UE to operate in fragmented carrier mode:

Proposal 7: The UE can fallback to multiple Rx chains in the same order CA or next fallback level CA including non-CA when an in-gap interferer precludes the UE to operate in fragmented carrier mode.

- Conclusion

This paper discussed the RF requirements for fragmented carriers and proposed:

Proposal 1: RAN4 can agree on the following test configurations:

  - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC=0, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos

    - UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

    - Reuse the Rel-15 assumption of Rx chain image rejection, which 25dB

    - At this stage RAN4 does not take DL MIMO into account

    - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc.

  - Other configurations are not precluded if identified as having performance impact

Proposal 2: The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain.

Proposal 3: RAN4 should determine a reasonable upper level of the power difference of the in-gap signal compared to the two non-contiguous CCs.

Proposal 4: The parameters for test case 1 and case 2 in-gap need re-evaluate based on new power difference of the in-gap signal compared to the two non-contiguous CCs.

Proposal 5: ΔRIBNC, IBB, NBB requirements [on both PCC and SCC] as well as the minimum guard band sizes for single Rx RF chain, all need to be re-evaluated.

Proposal 6: Out-of-band blocking and spurious response do not need requirement adjustment.

Proposal 7: The UE can fallback to multiple Rx chains in the same order CA or next fallback level CA including non-CA when an in-gap interferer precludes the UE to operate in fragmented carrier mode.

- Reference

[1] R4-2414457 WF on fragmented DL carriers study, MediaTek Inc., RAN4#112 2024-08