3GPP TSG-RAN WG4 Meeting #112bis	R4-2415039
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.8.2.2.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Agreements: Issue 3-1-1: General considerations - Agreement - The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements. Issue 3-1-2: Consideration of band(s) - Agreement: - Use 2GHz for the co-existence study - WI can be completed when the band specific requirements for one pair of {n256,256}and {n255, 255} are completed in this WI - The contributions on the other existing Rel-18 FR1-NTN bands for specifying common requirements are allowed. - Specify the band specific requirements for other potential NTN bands in the separate spectrum WI Issue 3-1-3: Consideration of power classes - Agreement - All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed Issue 3-1-4: Consideration of regulation(s) - Agreement - Applicable regulations should be taken into account for requirements discussions. - If any identified corrections to be made for PC3, those should be considered in another work item, it is not in the current scope of this WI. Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - For PC2 IoT NTN, at least 1Tx shall be considered - For other PCs (both IoT NTN and NR NTN), FFS Issue 3-2-1: NR-NTN HPUE feasibility for different power classes Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes - Agreement of Issue 3-2-1 to Issue 3-2-3: - For PC2, it is feasible to support both handheld and non-handheld devices. - FFS other power classes for applicable device type. Issue 3-3-1: Maximum output power - Agreement - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling Issue 3-3-5: SEM - Agreement: - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs Issue 3-3-6: ACLR - Agreement: - The ACLR requirements will depend on co-existence study outcomes.

This contribution further discussed the UE Tx RF requirements for  NR NTN HPUE.

- Discussion

In the R-19, the purpose of NTN HPUE is to introduce PC1, PC1.5 and PC2 for NTN bands and

- PC2 will apply to both handheld and non-handheld UE

- PC1 and PC1.5 are prioritized for non-handheld UE

In last meeting, the architecture of NTN HPUE was discussed, and it agreed For PC2 (NR NTN), 1 Tx and 2 Tx for PC2 NR NTN UE are to be considered for requirements and the architecture for other power class need further study.

PC1.5 for TN was introduced only for TDD currently and the typical architecture is 2Tx and 4Tx also was introduced. For TN FDD, currently there is still no PC1.5 definition due the high power 26dBm PA. Fortunately, PC2 can be implemented by 1Tx for TN FDD band, and RAN4 also agree consider 1Tx for NTN PC2 UE. Therefore, we can use 2Tx for PC1.5 NTN UE as the starting point. Considering the implementation complexity of UE, 3Tx has been introduced for CA, and it is discussing for single carrier for TN PC3 and PC2 in related MIMO WID. 3Tx can apply to NTN UE to reduce the implementation complexity compared to the 4Tx.

Proposal 1: 2Tx and 3 Tx for PC1.5 NR NTN UE can be considered for requirements as the starting point.

About PC1, it applied to non-handheld UE only, the main limitation is still because of the higher power 26dBm PA. to reduce the implementation complexity and make PC1 can be widely used in TN and NTN UE, 3Tx can study as the starting point.

Proposal 2: 3 Tx for PC1 NR NTN UE can be studied for requirements as the starting point.

For the SAR issue for handheld UE, currently, 3GPP has introduced the uplink duty cycle and higher power class than default power class. From NTN UE perspective, especially for current usage of NTN UE, the most important and feasible use case is message and phone call. In such case, a large duty cycle is easy to maintain while not much influencing the service quality. Furthermore, with the spirit of introducing the higher power class, if contradictorily, larger P-MPR is used to fulfil the SAR requirement, this will make the HPUE feature unreasonable. In such case, we believe the duty cycle solution is a much better way which can ensure the SAR regulation as well as the service quality and the coverage. However, as also discussed in other places, the duty cycle method needs to fulfil the regulation and this needs more investigation.

Proposal 3: Further investigate the duty cycle method for regulation of NTN hand-held UE.

- Conclusion

In this paper we give further discussion on the TX requirement for HPUE NTN. The proposals are as following:

Proposal 1: 2Tx and 3 Tx for PC1.5 NR NTN UE can be considered for requirements as the starting point.

Proposal 2: 3 Tx for PC1 NR NTN UE can be studied for requirements as the starting point.

Proposal 3: Further investigate the duty cycle method for regulation of NTN hand-held UE.

- Reference

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112 2024-08