3GPP TSG-RAN WG4 Meeting #112bis	R4-2415040
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.8.2.2.2

Source: 	OPPO

Title: 	Discussion on Rx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Rx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Agreements: Issue 4-1: General considerations - Agreement - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class. Issue 4-2: RSD for NR-NTN - Agreement - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE. - Further discuss the following aspects: - Use similar way in TN spec by introducing a new table of RSD compared to the PC3 - Whether default Tx-Rx frequency separation is used for test point

This contribution further discussed the UE Rx RF requirements for NR NTN HPUE.

- Discussion

The REFSENS requirements for both of FDD bands and TDD bands, regardless of the influence of its own Tx, there is no difference for different power classes. In R-15 for FDD bands for PC3, the impact of the Tx transmission to its own Rx has been considered in the REFSENS. The interferer from the Tx transmission will increase when the transmission power increases for some FDD bands, due to the limitation of isolation between the Tx and its own Rx. Therefore, in TN for FDD bands, the RSD was introduced for higher power PC2 compared to PC3. FR1 NR NTN bands are FDD bands, when introducing HPUE for the example band is for n255, n256, the similar influence should be considered as TN FDD HPUE.

Proposal 1: Use similar way in TN spec by introducing new tables of RSD for PC1, PC1.5 and PC2 compared to the PC3, separately.

Compared NTN band n256 to TN band n1, the gap between UL frequency and DL frequency of n256 is 60MHz that is larger than the 30MHz gap of n1.

|  | UL frequency | DL frequency |  |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |

For n1, 0 dB RSD for PC2 is defined for different channel bandwidth. And in last meeting, it agreed 1 Tx and 2 Tx for PC2 NR NTN UE are to be considered for requirements, that is similar with the architecture of TN PC2 UE. Band n256 can reuse the RSD of n1.

For band n255, the gap between UL frequency and DL frequency of n256 is 65.5 MHz, there is no influence of Tx transmission to the REFSENS of PC3, compared band n1, band n255 can also reuse the RSD of n1.

Proposal 2: 0dB RSD can be specified for PC2 NR NTN UE in n256 and n255.

Proposal 3: For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE.

- Conclusion

In this paper we give further discussion on the RX requirement for HPUE NTN. The proposals are as following:

Proposal 1: Use similar way in TN spec by introducing new tables of RSD for PC1, PC1.5 and PC2 compared to the PC3, separately.

Proposal 2: 0dB RSD can be specified for PC2 NR NTN UE in n256 and n255, FFS for PC1, PC1.5.

Proposal 3: For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE.

- Reference

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112 2024-08