3GPP TSG-RAN WG4 Meeting #112bis	R4-2415041
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.26.2.1

Source: 	OPPO

Title: 	Discussion on RedCap UE RF requirements with NR NTN operating in FR1-NTN bands

Document for:	Discussion

- Introduction

In previous RAN4 meeting, the RedCap UE RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1, 2, 3]:

Agreements: - Consider both 1Rx and 2Rx for requirements for RedCap and eRedCap UE. - Specify both eRedCap variants, i.e. with and without bandwidth reduction. - Target power class is PC3. - Adopt the same bandwidth as well as SCS as TN RedCap and TN eRedCap UE. - Add RedCap and eRedCap NTN requirements under the same new suffix into TS 38.101-5. Consider re-use of TN specification structure, i.e. which clauses are under suffix I in TS 38.101-1, as starting point. - All the NR-NTN FR1-NTN bands will be considered. - Re-use non-RedCap NTN UE refsens for 2Rx FD-FDD NTN (e)RedCap UE. - 0.5 dB tightening for n254 and n256, FFS for n255, for 2Rx HD-FDD refsens - Re-use TN (e)RedCap UE 1 Rx refsens allowance for NTN (e)RedCap UE as follows Operating band; Channel bandwidth (MHz); ΔR1R (dB). FDD band; 5; 2.5. FDD band; 10, 15, 20; 3.0. - 2.5 dB relaxation for 1Rx HD-FDD refsens compared to 2Rx HD-FDD - Full UL RB allocation for HD-FDD Refsens, both for 1Rx and 2Rx.

This contribution further discussed the UE RF requirements for RedCap NTN.

- Discussion

In previous meeting, it agreed that the requirement for eRedCap with bandwidth reduction should be specified. But the detail requirement didn’t discuss.

Based on the agreement of TN eRedCap UE with bandwidth reduction, we proposed for NTN eRedCap with bandwidth reduction:

Proposal 1: The maximum transmission bandwidth for NTN eRedCap with bandwidth reduction should be 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

Proposal 2: REFSENS of NTN eRedCap with bandwidth reduction for wider channel BW in FR1-NTN bands, it should be defined as below:

- The 5MHz REFSENS requirements are applicable for all the channel bandwidths including 5MHz, 10MHz, 15MHz, 20MHz and 30MHz channel bandwidth.

- 25 contiguous RB placed in middle of channel BW both in UL and DL for both HD-FDD and FD-FDD

  - Where both UL and DL allocations are following the 5MHz NTN RedCap REFSENS configurations and located at the center of the channel BW.

  - UL RB size follow the 5MHz UL configuration table

- Reuse the same scaling factor of NTN (e)RedCap without bandwidth reduction for REFSENS for NTN eRedCap with bandwidth reduction with 2Rx HD-FDD, 1Rx HD-FDD and FD-FDD.

- Conclusion

This paper the UE RF requirements for NTN eRedCap with bandwidth reduction and proposed:

Proposal 1: The maximum transmission bandwidth for NTN eRedCap with bandwidth reduction should be 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

Proposal 2: REFSENS of NTN eRedCap with bandwidth reduction for wider channel BW in FR1-NTN bands, it should be defined as below:

- The 5MHz REFSENS requirements are applicable for all the channel bandwidths including 5MHz, 10MHz, 15MHz, 20MHz and 30MHz channel bandwidth.

- 25 contiguous RB placed in middle of channel BW both in UL and DL for both HD-FDD and FD-FDD

  - Where both UL and DL allocations are following the 5MHz NTN RedCap REFSENS configurations and located at the center of the channel BW.

  - UL RB size follow the 5MHz UL configuration table

- Reuse the same scaling factor of NTN (e)RedCap without bandwidth reduction for REFSENS for NTN eRedCap with bandwidth reduction with 2Rx HD-FDD, 1Rx HD-FDD and 1Rx FD-FDD.

- Reference

[1] R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm Inc., RAN4#110bis 2024-04

[2] R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm Inc., RAN4#111 2024-05

[3] R4-2413527, Way Forward for [112][311] NR_NTN_Ph3_UE_RF, Qualcomm Inc., RAN4#112 2024-08