3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2415264

# Hefei, China, 14th – 18th October, 2024

Source: 	China Telecom

Title: 	Views on SAR Solution

Agenda Item:	6.1.1.1.2	Inter-band UL NR-CA/EN-DC with 2 bands and 2Tx and/or 3Tx

Document for:	Discussion

Introduction

In meeting RAN4#112[1], almost all companies agreed that duty cycle solution can be used for PC1.5 inter-band UL CA/EN-DC with 2Tx or 3Tx but the methodology and scenarios need further discussion. Agreement is still far away and some details should be clarified.

So in this contribution we reiterate our views and further clarify it.

Discussion

- 2.1 PC1.5 HPUE for inter-band EN-DC

For inter band EN-DC, the legacy duty-cycle solution has two modes: FDD+TDD and TDD+TDD, and in Rel-19 in this WI we extend the scenarios to include PC1.5 and FDD+FDD, so total SAR solution need to be updated.

So if the PC1.5 FDD+TDD and TDD+TDD EN-DC configuration is adopted, the duty-cycle solution should also be introduced and the mechanism and UE capability can be reused. But we should further study the default duty-cycle threshold and trigger condition.

We can start with taking half of PC2 default threshold and 0.5*PC2capability(e.g. maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16/ maxUplinkDutyCycle-FDD-TDD-EN-DC1) as new default duty-cycle threshold and trigger condition of SAR solution for PC1.5 of FDD+TDD/TDD+TDD. So for PC1.5, 15% and 0.5*PC2capability should be the threshold.

Proposal1: If the PC1.5 FDD+TDD and TDD+TDD EN-DC configuration is adopted in this WI, the corresponding duty-cycle solution should be introduced and 15% and 0.5*PC2capability should be the threshold.

For EN-DC with LTE FDD band, the legacy methodology has two E-UTRA FDD threshold 40%/70%. If the E-UTRA duty-cycle is in 0~40% NR uses maxUplinkDutyCycle-FDD-TDD-EN-DC1 and in 40%~70% using maxUplinkDutyCycle-FDD-TDD-EN-DC2.

So the design of FDD+FDD EN-DC duty-cycle solution can follow this scheme that keeping the E-UTRA FDD threshold 40%/70% and introducing new NR threshold. On the other hand, if FDD+FDD EN-DC duty-cycle solution need specified in Rel-19, the UE capability for NR UL threshold should be considered.

Proposal2: If specify FDD+FDD EN-DC duty-cycle solution in Rel-19, the UE capability for NR UL threshold should be considered.

- 2.2 PC1.5 HPUE for intra/inter-band CA

The follow content is the resubmission of R4-2412432 in last meeting, and our view about this part is unchanged.

For PC1.5 HPUE for intra/inter-band CA, since only the MOP requirements are changed, the general SAR solution framework above can be kept using. The maximum UplinkDutyCycle threshold should have two values: maxUplinkDutyCycle-interBandCA-PC2 and 0.5* maxUplinkDutyCycle-interBandCA-PC2, referring to the PC2 UE for CA. When the total average percentage of uplink symbols is between maxUplinkDutyCycle-interBandCA-PC2 and 0.5* maxUplinkDutyCycle-interBandCA-PC2, ΔPPowerClass,CA =3dBm; When the total average percentage of uplink symbols is lager than maxUplinkDutyCycle-interBandCA-PC2, ΔPPowerClass,CA =6dBm.

Proposal3: For PC1.5 HPUE for intra/inter-band CA, the general SAR solution framework and the threshold of average percentage of uplink symbols should both refer to PC2 UE for CA and PC1.5 UE for single CC.

The average percentage of uplink symbols should also be defined as 50%×( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y) since the power class is per band rather than per Tx. The MOP restriction of UE for two band with 2Tx is same as UE for two band with 3Tx. And the default value of maxDutyNR,x/y should be 25% according to the baseline of SAR requirements.

Proposal4: For PC1.5 HPUE for intra/inter-band CA, if power class of one or both of the bands within the band combination is power class 1.5, the default value of maxDutyNR,x/y should be 25%.

Conclusion

Proposal1: If the PC1.5 FDD+TDD and TDD+TDD EN-DC configuration is adopted in this WI, the corresponding duty-cycle solution should be introduced and 15% and 0.5*PC2capability should be the threshold.

Proposal2: If specify FDD+FDD EN-DC duty-cycle solution in Rel-19, the UE capability for NR UL threshold should be considered.

Proposal3: For PC1.5 HPUE for intra/inter-band CA, the general SAR solution framework and the threshold of average percentage of uplink symbols should both refer to PC2 UE for CA and PC1.5 UE for single CC.

Proposal4: For PC1.5 HPUE for intra/inter-band CA, if power class of one or both of the bands within the band combination is power class 1.5, the default value of maxDutyNR,x/y should be 25%.

References

- R4-2414277, WF on HPUE for CA in TN, RAN4#112