3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2415266

# Hefei, China, 14th – 18th October, 2024

Source: 	China Telecom

Title: 	Further study on SRS IL imbalance

Agenda Item:	6.1.1.3.4	SRS IL imbalance

Document for:	Discussion

Introduction

In last RAN4 meeting, we made some progress that SRS IL imbalance issue is indeed vital but we need some compromised solutions with minimal impact to the specification.

Companies submitted a bulk of SRS IL imbalance analysis papers and the discussion was extremely heated. So in this contribution, we reaffirm our perspective and provide some compromised schemes.

Discussion

- 2.1 Whether to solve SRS IL imbalance issue in Rel-19

The WF[1] was captured as below:

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751.

About this section, our point is unchanged that SRS IL imbalance is non-negligible and also included in WID, so we have to find a solution in Rel-19.

Proposal1: SRS IL imbalance is non-negligible and continue to pursue a solution in Rel-19.

- 2.2 Initial Considerations for SRS IL imbalance issue

The WF[1] was captured as below:

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

The follow content is the resubmission of R4-2412434 in last meeting, and our view about this part is unchanged.

With the introduce of 6Rx UE the power imbalance between SRS ports are more non-negligible and the value of ∆TRxSRS will be higher. For 8Rx handheld UE, the IL relaxation in TS 38.101-1 could reach 10.3dB at some scenarios and it is too large in performance evaluation.

Due to the diversity of UEs implement the compensation mechanism at UE side is unpredictable much less the limited power hea droom. On the one hand, the 3GPP specification only address the upper boundary of SRS IL issue. On the other hand, the relaxation in PCMAX_L,f formula are ∆TC,c, ΔTIB,c, and ∆TRxSRS and each of them contributes to the MOP so the IL imbalance is difficult to acquire for UE, which most likely reflect directly to the NW.

The actually power needed for SRS transmission can be assume as:

PSRS,total = PSRS,b,f,c(I,qs,l) + PIL

Where PSRS,b,f,c(I,qs,l) can be calculated according to TS 38.213.

If PSRS,total ≤ PCMAX_H,f,c, which means the power headroom is enough for UE to compensate the SRS IL (i.e. if PSRS,total ≤ PCMAX,f,c, UE can simply increase PSRS,total to compensate IL and do not have to change PCMAX,f,c; if PSRS,total ≥ PCMAX,f,c, UE also have chance to compensate IL through increasing PCMAX,f,c since the upper bound will not be exceeded.);

If PSRS,total ≥ PCMAX_H,f,c, UE is impossible to compensate the whole SRS IL, but can still compensate partial of it by extending output power to PCMAX_H,f,c.

No matter what strategy UE takes to compensate the imbalance, a situation will arise that UE cannot compensate all the SRS IL due to limit power headroom, so NW will misunderstand the actual SRS power and cannot perform compensation at it is side. Therefore, an SRS IL imbalance reporting mechanism should exist to handle the mismatch between UE and NW when UE cannot compensate the total IL.

Obsveration1: SRS IL imbalance reporting mechanism is necessarily to be introduced.

- 2.3 solutions

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

The following proposals are the resubmission from our paper in last meeting[2].

Proposal2: UE should indicate to NW whether it support insertion loss compensation ability.

Proposal3: From complexity point, the static reporting is more feasible. But if we want more accurate report, dynamic method should be considered.

Proposal4: Both static and dynamic SRS IL report need to consider reporting granularity and other information.

Besides, the power headroom of SRS is also important for BS to determine the SRS transmit power for each SRS port no matter the UE compensates the insertion loss or not. But according to RAN2 spec, non-CA/DC scenarios (i.e. single entry MAC-CE PHR) has no Type3 PHR reporting and Type1 PHR is no longer accurate for convey SRS power headroom due to the IL imbalance.

So accurate SRS power headroom reporting should be introduced for non-CA/DC scenarios since only has a little impact to specification.

Proposal5: Introduce accurate SRS power headroom reporting for non-CA/DC scenarios.

And we also need to further study the granularity of new SRS power headroom report, per cell or per port or per SRS resource.

Proposal6: Further study the granularity of new SRS power headroom report.

To minimize the impact to specification, only introducing UE compensation and new SRS power headroom reporting can be a compromised solution.

Proposal7: Only introducing UE compensation and new SRS power headroom reporting can be a compromised solution.

Conclusion

Obsveration1: SRS IL imbalance reporting mechanism is necessarily to be introduced.

Proposal1: SRS IL imbalance is non-negligible and continue to pursue a solution in Rel-19.

Proposal2: UE should indicate to NW whether it support insertion loss compensation ability.

Proposal3: From complexity point, the static reporting is more feasible. But if we want more accurate report, dynamic method should be considered.

Proposal4: Both static and dynamic SRS IL report need to consider reporting granularity and other information.

Proposal5: Introduce accurate SRS power headroom reporting for non-CA/DC scenarios.

Proposal7: Only introducing UE compensation and new SRS power headroom reporting can be a compromised solution.

References

- R4-2414285, WF on for 6Rx UE requirements, RAN4#112

- R4-2412434, On SRS IL imbalance, China Telecom, RAN4#112