# 3GPP TSG-RAN WG4 Meeting #112-bis	                                 R4-2415272

# Hefei, China, 14 – 18 Oct., 2024

Agenda item:	6.20.1

Source: 	CableLabs, Charter Communications

Title: 	Consideration of band/channel bandwidth support for SBFD

Document for:	Discussion

# 1. Introduction

For the Rel-19 WI NR_duplex_evo, the bands and minimum channel bandwidth where SBFD could apply are being discussed [1]-[2]. It is agreed that “the supported channel bandwidth shall be no less than [X] MHz,” and [X] is for further study [1]. It was also agreed that “the supported channel bandwidth can be impacted by the subband/guard band size discussion” [2]. In this contribution, a method that evaluates the minimum channel bandwidth with respect to the inter-subband guard band is provided.

- Discussion

The single-carrier channel bandwidth ranges from 5 to 100 MHz, and the corresponding max transmission bandwidth is defined in TS 38.101-1 Table 5.3.2-1: Maximum transmission bandwidth configuration NRB: FR1. It was agreed in RAN4 that only DUD and DU patterns are allowed for the sub-band configuration. There is only one guard band between the DL and UL subbands for the DU configuration, and there are two guard bands for the DUD configurations. The total bandwidth of the guard band(s) reduces the transmission bandwidth, and such a reduction percentage should be considered as a reference when deciding the minimum channel bandwidth for SBFD operating bands.

Proposal 1: The total bandwidth of the guard band(s) reduces the transmission bandwidth, and such a reduction percentage should be considered as a reference when deciding the minimum channel bandwidth for SBFD operating bands.

The bandwidth of the inter-subband guard band, [B], is still under study [2]. There were proposals for five RBs with 30 kHz SCS [1] and [3]. Tentatively using five RBs as an example, the transmission bandwidth reduction for DU configuration is listed in Table 1, and for DUD configuration is listed in Table 2. The transmission bandwidth reduction of R and R=[2~5]% is suggested to determine the minimum channel bandwidth of an SBFD operating band.

Proposal 2: The transmission bandwidth reduction, R, is suggested between 2% and 5%, and the value of R could be further discussed.

Proposal 3: Based on proposals 1 and 2 and an assumed 1.8 MHz inter-subband guard bandwidth (five RBs), to maintain the spectrum efficiency, SBFD should operate in bands with the minimum channel bandwidth of 50 MHz for the DU configuration and 100 MHz for the DUD configuration.

Table 1. Transmission bandwidth reduction due to SBFD inter-subband guard band for the DU configuration (one guard band).

| DU configuration with one inter-subband guard band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth (MHz) | Max transmission bandwidth |  | Inter-subband guard band |  | Transmission bandwidth reduction due to SBFD inter-subband guard band |
|  | Number of RBs | Bandwidth (MHz) | Number of RBs per guard | Total guard band bandwidth (MHz) |  |
| 5 | 11 | 3.96 | 5 | 1.8 | 45.5% |
| 10 | 24 | 8.64 | 5 | 1.8 | 20.8% |
| 15 | 38 | 13.68 | 5 | 1.8 | 13.2% |
| 20 | 51 | 18.36 | 5 | 1.8 | 9.8% |
| 25 | 65 | 23.4 | 5 | 1.8 | 7.7% |
| 30 | 78 | 28.08 | 5 | 1.8 | 6.4% |
| 35 | 92 | 33.12 | 5 | 1.8 | 5.4% |
| 40 | 106 | 38.16 | 5 | 1.8 | 4.7% |
| 45 | 119 | 42.84 | 5 | 1.8 | 4.2% |
| 50 | 133 | 47.88 | 5 | 1.8 | 3.8% |
| 60 | 162 | 58.32 | 5 | 1.8 | 3.1% |
| 70 | 189 | 68.04 | 5 | 1.8 | 2.6% |
| 80 | 217 | 78.12 | 5 | 1.8 | 2.3% |
| 90 | 245 | 88.2 | 5 | 1.8 | 2.0% |
| 100 | 273 | 98.28 | 5 | 1.8 | 1.8% |

Table 2. Transmission bandwidth reduction due to SBFD inter-subband guard band for the DUD configuration (two guard bands).

| DUD configuration with two inter-subband guard band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth (MHz) | Max transmission bandwidth |  | Inter-subband guard band |  | Transmission bandwidth reduction due to SBFD inter-subband guard band |
|  | Number of RBs | Bandwidth (MHz) | Number of RBs per guard | Total guard band bandwidth (MHz) |  |
| 5 | 11 | 3.96 | 5 | 3.6 | 90.9% |
| 10 | 24 | 8.64 | 5 | 3.6 | 41.7% |
| 15 | 38 | 13.68 | 5 | 3.6 | 26.3% |
| 20 | 51 | 18.36 | 5 | 3.6 | 19.6% |
| 25 | 65 | 23.4 | 5 | 3.6 | 15.4% |
| 30 | 78 | 28.08 | 5 | 3.6 | 12.8% |
| 35 | 92 | 33.12 | 5 | 3.6 | 10.9% |
| 40 | 106 | 38.16 | 5 | 3.6 | 9.4% |
| 45 | 119 | 42.84 | 5 | 3.6 | 8.4% |
| 50 | 133 | 47.88 | 5 | 3.6 | 7.5% |
| 60 | 162 | 58.32 | 5 | 3.6 | 6.2% |
| 70 | 189 | 68.04 | 5 | 3.6 | 5.3% |
| 80 | 217 | 78.12 | 5 | 3.6 | 4.6% |
| 90 | 245 | 88.2 | 5 | 3.6 | 4.1% |
| 100 | 273 | 98.28 | 5 | 3.6 | 3.7% |

- Conclusions

In this contribution, a method that evaluates the minimum channel bandwidth with respect to the inter-subband guard band is provided.

Proposal 1: The total bandwidth of the guard band(s) reduces the transmission bandwidth, and such a reduction percentage should be considered as a reference when deciding the minimum channel bandwidth for SBFD operating bands.

Based on a temporal assumption of SBFD inter-subband guard bandwidth of 1.8 MHz (FFS), the transmission bandwidth reduction for both DU and DUD configurations is provided in Tables 1 and 2, respectively. To maintain spectrum efficiency, a transmission bandwidth reduction of R is proposed between 2% and 5% which could be further discussed. The corresponding minimum channel bandwidth is 50 MHz for the DU configuration and 100 MHz for the DUD configuration for an SBFD operating band.

Proposal 2: The transmission bandwidth reduction, R, is suggested between 2% and 5%, and the value of R could be further discussed.

Proposal 3: Based on proposals 1 and 2 and an assumed 1.8 MHz inter-subband guard bandwidth (five RBs), to maintain the spectrum efficiency, SBFD should operate in bands with the minimum channel bandwidth of 50 MHz for the DU configuration and 100 MHz for the DUD configuration.

# 4. Reference

- R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General,” Samsung, RAN4 #112, August 2024, Maastricht, Netherland.

- R4-2409889, “Way Forward for [110bis][313] NR_duplex_evo,” RAN4 #111, May 2024, Fukuoka, Japan.

- R4-2413238, “On SBFD general aspects and system parameters,” Nokia, RAN4 #112, August 2024, Maastricht, Netherlands.