# 3GPP TSG-RAN WG4 Meeting #112bis	                 R4-2415308

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.13.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR2 OTA test method on STxMP

Document for:	Approval

# Introduction

In RAN4#112, the WF on NR FR2 OTA in [1] was approved. In this paper, we provide our further considerations on FR2 OTA test method on transmission with multi-panel (STxMP) testing.

# 2. Discussion

## EIRP PUMAX,f,c,k testing

With the assumption that TE has the capability of distinguishing EIRP per TCI, the key testability issue for peak EIRP PUMAX,f,c,k testing is how to select the proper AoA pair and UE orientation

In [1], the below agreements were made on AoA separation and UE orientation for EIRP PUMAX,f,c,k testing.

Issue 1-2-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreement - Take below proposal as the starting point: - For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enabled (i.e., from Beam Peak Search for MOP in 6.2 of TS 38.101-2, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and AoA pair selecting from set of {30deg, 60deg, 90deg, 120deg, 150deg}. - FFS whether a test direction other than the beam peak direction of single CC operation without STxMP enabled can be used for first AoA - Due to lack of absolute probe location definition of the multi-AoA UE RF test system, defined in Clause 5.2.2 of TR38.871, FFS on whether or how such test system can guarantee the same relative angular orientations between specific UE test directions and probes.

As agreed in [1], the potential testability issue is whether the test system can guarantee the same relative angular orientations between specific UE test directions and probes due to different implementation solutions for multi-Rx testing in [2]. One potential solution to solve the ambiguity issue of relative angular orientations between specific UE test directions and probes can be considered as follows.

- Step 1): Since the beam peak direction of single CC operation without STxMP is unique with reference UE orientation defined in [4], the first AoA, i.e., beam peak direction without STxMP should be searched assuming the same UE orientation declared for the second AoA by UE.

- Step 2): The second AoA declared by UE assumes AoA pair is measured based on one specific example test system in [5] under UE orientation used in Step 1)

- Step 3): TE transforms the declared AoA pair and UE orientation to guarantee the same relative angular orientations if the test system is different from the example test system in Step 2).

Proposal 1: RAN4 considers the below solution to solve the ambiguity of relative angular orientations between specific UE test directions and probes can be considered as follows.

- Step 1): Since the beam peak direction of single CC operation without STxMP is unique with reference UE orientation defined in [4], the first AoA, i.e., beam peak direction without STxMP should be searched assuming the same UE orientation declared for the second AoA by UE.

- Step 2): The second AoA declared by UE assumes AoA pair is measured based on one specific example test system in [5] under UE orientation used in Step 1)

- Step 3): TE transforms the declared AoA pair and UE orientation to guarantee the same relative angular orientations if the test system is different from the example test system in Step 2).

Since the AoA separation and UE orientation for EIRP PUMAX,f,c,k testing will be reused for both aggregated EIRP PUMAX,f,c and aggregated TRP PTMAX,f,c testing, it is suggested not to consider the test direction other than the beam peak direction of single CC operation without STxMP for first AoA in EIRP PUMAX,f,c,k testing for simplification.

Proposal 2: It is suggested not to consider the test direction other than the beam peak direction of single CC operation without STxMP for first AoA in EIRP PUMAX,f,c,k testing.

## 2.2 Aggregated EIRP PUMAX,f,c testing

In [1], the additional margin for skipping rule for EIRPmax is agreed as follows.

Issue 1-3-2: Additional margin for skipping rule for EIRPmax testing - Agreement - Further discuss additional margin considering the range of [0.5dB, 2dB]. - Encourage companies to provide technical Analysis and/or simulation results.

For the additional margin for skipping rule, two aspects need further analysis [5]:

- The error due to the beam peak search grid for the first AoA: For this part, the MU of measurement grid for beam peak search defined for the beam peak direction of single CC operation without STxMP can be covered, and therefore no need to be considered for additional margin for this part in skipping rule.

- The error due to the potential change in the antenna pattern between Multi-Tx and single-Tx: Since the same codebook of beamforming is most likely to be used for Multi-Tx and single-Tx. It is reasonable to consider a maximum of 0.5dB additional margin for the skipping rule for EIRPmax testing

Proposal 3: A maximum of 0.5dB additional margin can be considered in the skipping rule for EIRPmax testing.

## 2.3 Aggregated TRP PTMAX,f,c testing

For Aggregated TRP PTMAX,f,c, testing, the issue is similar to aggregated EIRP PUMAX,f,c testing. Similar agreements were made in [1].

Issue 1-4-2: Additional margin for skipping rule for TRPmax testing - Agreement - Further discuss additional margin considering the range of [0.5dB, 2dB]. - Encourage companies to provide technical Analysis and/or simulation results.

Similarly, a maximum of 0.5dB additional margin in the skipping rule for TRPmax testing can be considered based on the analysis in section 2.2.

Proposal 4: A maximum of 0.5dB additional margin can be considered in the skipping rule for TRPmax testing.

# 3.	Conclusion

In this paper, we provide our considerations on test methodology for STxMP and have the following observations and proposals:

Proposal 1: RAN4 considers the below solution to solve the ambiguity of relative angular orientations between specific UE test directions and probes can be considered as follows.

- Step 1): Since the beam peak direction of single CC operation without STxMP is unique with reference UE orientation defined in [4], the first AoA, i.e., beam peak direction without STxMP should be searched assuming the same UE orientation declared for the second AoA by UE.

- Step 2): The second AoA declared by UE assumes AoA pair is measured based on one specific example test system in [5] under UE orientation used in Step 1)

- Step 3): TE transforms the declared AoA pair and UE orientation to guarantee the same relative angular orientations if the test system is different from the example test system in Step 2).

Proposal 2: It is suggested not to consider the test direction other than the beam peak direction of single CC operation without STxMP for first AoA in EIRP PUMAX,f,c,k testing.

Proposal 3: A maximum of 0.5dB additional margin can be considered in the skipping rule for EIRPmax testing.

Proposal 4: A maximum of 0.5dB additional margin can be considered in the skipping rule for TRPmax testing.

# 4.	References

- R4-2413534, WF on Rel-19 NR_FR2_OTA, Qualcomm

- TR 38.871, Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- 3GPP TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment

- R4-2411698, On FR2 OTA testing of STxMP, Samsung