3GPP TSG-RAN WG4 Meeting #112bis													R4-2415369

Hefei, China, Oct 14 – 18, 2024

Title:	Power domain enhancements for single carrier

Agenda item:			6.1.1.2.1

Source:	Nokia

Document for:	Approval

# Introduction

The following MPR reduction work has been approved as part of the RAN4 Rel-19 [1]. One motivation for this work is generally relaxing requirements for UEs that have such frequency allocation that makes meeting the requirements less important e.g. inner allocation, UE channel bandwidth within BS bandwidth. Based on RAN4 discussions another motivation is lower complexity UEs such as RedCap UEs that may not be able to meet all the current requirements. Such UEs would need to be operated so that they always have allocation allowing relaxed requirements e.g. the UE channel bandwidth is within BS channel bandwidth.

In RAN4#111 meeting, the WF[1] has been agreed and the details about power domain enhancement for single carrier are as follows:

1.1 Consideration on relaxation of requirements for identified scenarios Agreement in main session: - No relaxation of level of spurious emission requirements - FFS on whether to change the spurious emission boundary ## 1.2. Mechanisms for enabling MPR reduction and/or power boosting Way forward: - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW - Other mechanisms are not precluded

# Discussion

In RAN4#112 meeting [2], several issues have been highlighted for the power domain enhancement for single carrier :

#### Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - WF #### Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach #### Issue 1-2-3: whether extended UE CBW edge could exceed the BS CBW edge - WF - proposal 1 is agreeable for singe CC case, FFS multi-CC/BS transmitted BW case #### Issue 1-2-4: Whether and where to use IBE in the larger BS CBW - WF - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW #### Issue 1-2-5: Boundary to apply ACLR and SEM

To ensure consistent performance and compliance, the extended UE channel bandwidth (UE CBW) should adhere to the same conditions and rules originally applied to the UE CBW, including parameters like ACLR, IBE, and SEM. This approach maintains the integrity of the system and avoids any potential performance degradation or non-compliance issues. To maintain efficient resource allocation and avoid interference, the extended UE channel bandwidth (UE CBW) should always be contained within the base station channel bandwidth (BS CBW). This ensures that all UE resource blocks (RBs) remain within the allocation of the BS CBW, preventing any potential conflicts or overlaps with other users in the cell.

Proposal 1: UE CBW extension should not exceed the BS CBW, such that we want to keep all the UE RBs allocation inside BS CBW

To preserve the integrity and compatibility of the existing system, it is crucial to maintain the current ACLR (Adjacent Channel Leakage Ratio) and SEM (Spectral Emission Mask) requirements without relaxation at the edge the BS CBW

Observation 1: No relaxation of parameters should be applied outside BS CBW.

Proposal 2: Maintain the existing ACLR value/limit without any adjustment when extending the UE bandwidth to match the BS CBW.

Proposal 3: The ACLR and SEM should be applied to edge of BS CBW

Observation 2: An additional guard band configuration or a configured maximum transmission bandwidth can be defined to limit UE possible RB allocation within initial UE CBW regardless of its extension method or reference CBW.

Furthermore, the In-Band Emission (IBE) should be applied only within the extended UE CBW that should match BS CBW, between the edge of BS CBW and the edge of UE RB allocation as illustrated in the Figure 1.

Figure 1: OOB style in Rel 18 versus OOB candidate for Rel 19

It is important to highlight, when the UE CBW is extended to match the BS CBW, the additional guard bands on the left (GBL) and right (GBR) sides of the UE CBW can be non-symmetrical, as showed in figure 2.

Figure 2: Extended UE CBW matches BS CBW with additional guard band GBL and GBR

By configuring the additional guard bands according to BS CBW, MPR can be reduced while minimizing the interference to adjacent channels and maintaining the legacy ACLR/SEM limits.

To guarantee tolerable level of interference within BS CBW, UE should satisfy IBE requirement within whole BS CBW (which is UE extended BW size). Otherwise, UEs in CBW would have constructive interference from all UEs especially in FR1 and thus UL performance significant degradation.

Proposal 4: The IBE should be applied within BS CBW according to UE RB allocation.

In some cases, NW may tolerate larger UE in-band emissions if it is well-confined in small bandwidth. For such cases, it would be possible to further enhance MPR and/or have a additional power boost by providing UE an additional band (e.g., 1 RB) adjacent to UE RB allocation where IBE is omitted or not considered as shown Figure 3 (e.g, IBE mask shift by X RB(s) on both sides of RB allocation).

Figure 3: Further MPR enhancement with additional adjacent band X

Proposal 5: RAN4 to further study MPR enhancement and power boost technique based on the definition and applicability of IBE within CBW.

# Conclusion

Proposal 1: UE CBW extension should not exceed the BS CBW, such that we want to keep all the UE RBs allocation inside BS CBW

Observation 1: No relaxation of parameters should be applied outside BS CBW.

Proposal 2: Maintain the existing ACLR value/limit without any adjustment when extending the UE bandwidth to match the BS CBW.

Proposal 3: The ACLR and SEM should be applied to edge of BS CBW

Observation 2: An additional guard band configuration or a configured maximum transmission bandwidth can be defined to limit UE possible RB allocation within initial UE CBW regardless of its extension method or reference CBW.

Proposal 4: The IBE should be applied within BS CBW according to UE RB allocation.

Proposal 5: RAN4 to further study MPR enhancement and power boost technique based on the definition and applicability of IBE within CBW.

# 4	Reference

[1] RP-241656 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, 3GPP TSG RAN Meeting #104

[2] R4-2406584 “WF on power domain enhancements” TSG-RAN WG4 Meeting #110bis, April, 2024

[3] R4-2414441 “WF on power domain enhancements” TSG-RAN WG4 Meeting #112bis, August, 2024