3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415452

Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			On CW cancellation capability

Agenda item:		6.21.3.4

Document for:		Approval

# Introduction

In this paper, we present our thoughts on CW cancellation for A-IoT RAN4 study.

# Discussion

In WF [1], the CW cancellation is FFS as below and our view for CW cancellation capability is discussed in rest of paper.

### Issue 2-6: CW for D1T1

- Agreement in RAN4#112:

  - FFS on methods and feasible values for CW cancellation

## Link budget analysis

In WF [1], the SNR degradation is to be investigated as below and we think the link budget should be discussed and an understanding of the link budget in relation to the allowed SINR degradation is needed.

Agreement in RAN4#112:

- Companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different.

  - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation

  - CW transmission impact on the received SINR, e.g. degradation

The latest link budget parameters from RAN1 are listed below in Table 1 and used as starting point for discussion. The highlighted parameters are modified to simplify the discussion.

Table 1: Link budget parameters

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A2/B D2T2-A2 | D1T1-A2/B D2T2-A2 |
| [0A1] | CW case | N/A | 1-1/1-2 |
| [0B] | Device 1/2a/2b | Device 1/2a | Device 1/2a |
| [0C] | Center frequency (MHz) | 900MHz | 900MHz |
| [0D] | Topology/Pathloss model | - Free space pathloss | - Free space pathloss |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: 2 | 1 |
| [1E] | Total Tx Power (dBm) | For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) | For device 1/2a: - [1E]-D2R-Alt2: (For scenarios ‘A1’ and ‘A2’) - The Device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. |
| [1E1] | CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ 33 dBm if CW transmit in DL spectrum 23 dBm if CW transmit in UL spectrum Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | - BS Tx ant gain Note: only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | For scenarios ‘A1’ and ‘A2’ - Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note: only applicable for device 1/2a |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1E5] | CW received power (dBm) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), | 180kHz(M), |
| [1G] | Tx antenna gain (dBi) | - For BS for indoor, 6 dBi(M) | - For A-IoT device, 0dBi |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | - OOK: 6 dB - PSK: 0 dB - FSK: Y dB It is applicable for device 1 and 2a |
| [1J] | Ambient IoT on-object antenna penalty | Not applicable | 0.9dB |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | 0 for device 1 10 for device 2a |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | - For BS, X dB, X =3 | N/A |
| [1M] | EIRP (dBm) | Calculated | Calculated |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a] [receiver bandwidth?] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver - 20dB, | For BS as reader - 10dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated | Calculated |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt2 | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | Refer to [2k2] |
| [2K1] | Remaining CW interference (dB) | N/A | Refer to [2k2] |
| [2K2] | Receiver sensitivity loss(dB) | N/A | X dB degradation |
| [2L] | Receiver Sensitivity (dBm) | - Calculated | Calculated |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | N/A (free space propagation assumption) | N/A (free space propagation assumption) |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | N/A | N/A |

With the link budget parameter and assuming the free space pathloss, the received power at the device for R2D link can be modelled as below:

Pdevice=PTx+Greader+Lfeeder+Lpolarization+Lobject_penalty+10log10(λ4∙π∙r)2+Gdevice

Also, the received power for reader for D2R link can be described as below:

Preader=Pdevice+Gdevice+Lbackscatter+Lpolarization+Lobject_penalty+10log10(λ4∙π∙r)2 +Lfeeder  +Greader

Table 2: parameters list for received power calculation

| PTx | 33 dBm, A-IoT BS for DL signal and CW transmitting in DL spectrum 23 dBm, CW transmitting in UL spectrum |
| --- | --- |
| Greader | 6 dBi |
| Lfeeder | -3 dB |
| Lpolarization | -3 dB |
| Lobject_penalty | -0.9 dB |
| Gdevice | 0 dBi |
| Lbackscatter | -6dB for OOK |
| r | Distance from reader to A-IoT device |
| λ | Wave-length for 900MHz |

The device REFSENS and A-IoT receiver REFSENS decide the min received power to decode the signal and REFSNES is calculated using the formula below:

REFSENS=kTB + SNR + 10log10(RX BW) + (NF+ IM) – Diversity gain

where

-	kT: Thermal noise power spectrum density which is -174dBm/Hz

-	NF:  10 dB for A-IoT BS and 20 dB for device 1

-	IM: 2.5 dB is assumed

-	Diversity gain: 0 dB

The target SNR with 1% BLER is simulated with LLS from Figure 1 and Figure 2 [3] and listed in Table 3.

Table 3: target SNR from LLS

|  | SNR | comments |
| --- | --- | --- |
| D2R | 7.5 dB | OOK1, single tone, no FEC for 1% BLER |
| R2D | 18 dB | OOK1 for 1% BLER |

Table 4: Calculated REFSENS for Device 1/2a and A-IoT BS

|  | REFSENS | Comment |
| --- | --- | --- |
| Device 1 | -71.3 dBm | NF = 24 dB |
| Device 2a | -81 dBm | NF = 20 |
| A-IoT BS | -107 dBm | NF = 10 dB |

Figure 1: R2D BLER vs. SNR for 1 PRB across different values of M under the assumption of perfect synchronization

Figure 2: BER performance of OOK schemes under AWGN channel

It is stated in WID [4] that as below:

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- X is to be decided in WGs.

- Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

For device type 1, the power consumption is around 1 µW and this corresponds to -30 dBm. With the conjugate matching at the antenna at device, the radiated power at antenna and power delivered IC load will be equal. Considering the power conversion efficiency of 50%, there is 25% percent of the power to power the IC and min received power at the antenna should be -24 dBm (-30 dBm + 6) and this is for device 1 where the RF energy is harvested from either A-IoT BS DL signal power or CW transmission power. Figure 3 and Figure 4 illustrate the max range the A-IoT BS can reach the device 1. The calculated A-IoT BS REFSENS is also marked together with Energy Harvest (EH) sensitivity. It can be observed that for device 1 harvesting the RF power from A-IoT BS, the forward link is bottleneck and the needed A-IoT BS min received power is -84 dBm at the distance where the min received power to power on device is -24 dBm in R2D.

The A-IoT BS REFSENS could be relaxed with 23 dB or 33 dB for device 1 harvesting the RF energy from A-IoT BS depending on the CW transmission power.

RAN4 also agree to use the hardcoded device 1 and 2a sensitivity for coexisting simulation and Figure 5 and Figure 6 illustrate the link budget for the hardcoded sensitivity device. It can be observed from Figure 5 and Figure 6 that A-IoT BS REFSENS can be relaxed 9 dB for device 1 with 23 dB CW transmission power where the reading range is limited by R2D sensitivity. When CW transmission power is 33 dBm, R2D is not limiting factor and D2R REFSENS decides the maximum range.

R2D will be limiting factor for link budget for device 1 with 23 dBm CW signal and A-IoT BS REFSENS can be relaxed with 9 dB.

D2R will be limiting factor for link budget for device 1/2a with 33 dBm CW signal for hardcoded device 1/2a sensitivity.

Figure 3: device 1 range considering the energy harvesting with CW transmission power of 33 dBm

Figure 4: device 1 range considering the energy harvesting with CW transmission power of 23 dBm

Figure 5: device 1/2a range with hardcoded sensitivity and CW transmission power of 23 dBm

Figure 6: device 1/2a range with hardcoded sensitivity and CW transmission power of 33 dBm

## CW cancelation

For D1T1-A2, with RAN4 agreed CW RF parameters, the CW transmission can be different hardware or shared hardware with the A-IoT BS or UE as reader. Therefore, for CW cancellation for D1T1-A2/D2T2-A2, both shared hardware and separate hardware options should be discussed. As the backscattering signal needs the CW signal to operate, it can be observed from link budget analysis that the minimum received power for A-IoT BS/UE as reader should be defined with the maximum distance between reader and device  in the presence of the CW signal.

Both shared and separated hardware for CW transmission and A-IoT transmission should be studied.

The minimum received power for A-IoT BS/UE as reader is defined with the maximum distance from link budget analysis and in the presence of CW transmission.

From link budget analysis the min received power could be -74 dBm limited by EH sensitivity, the signal with such low level received power should be decoded in the presence of the CW transmission which is either 33 dBm or 23 dBm. The CW transmission can leak to the receiver chain via either reflection from environment or the imperfection of the isolation between transmitter and receiver for the shared hardware architecture. As the receiver has limited dynamic range and linearity, this CW leakage needs to be attenuated/isolated to avoid the desensitization of the receiver.  The CW signal frequency is used to modulate the backscatter signal and the frequency separation of the CW signal and modulated signal is around 100kHz for example as illustrated in Figure 7 according to the current D2R waveform design. The RF filter Q value should be 9000 (900e6/100e3) and is difficult to implement. This means using an RF filter before the LNA to attenuate the CW signal is not possible. As such, a high level CW leakage signal will be present together with a low wanted backscattered signal before LNA.

Using the RF filter to attenuate the CW signal is difficult to design.

Figure 7: D2R waveform in frequency domain

To analysis the impact of the receiver desensitization, the backscattered signal is modelled as below [7]:

Sb (t)=Abcos⁡(ωb∙t+φb)∙cosωc∙t+φc

with ωc representing the CW frequency and ωb is the message frequency.

And the leakage CW signal is modelled as below:

SI (t)=AI∙cosωc∙t+φI

Then the receiver nonlinearity is modelled with a power series below assuming the second-order nonlinear product can be filtered:

yot=a1∙xt+a3∙xt3

Where a1 is the small signal gain of the receiver and a3<0 represents the nonlinear behaviour of the system.

The input signal before LNA is then:

xt=Sb t+SI (t)

Assuming φb=φc=φI=0, at output of the LNA, only the terms associated with fundamental frequency coswbtcos⁡(ωct) are of interest and they are below:

yoIt=a1Ab+916a3Ab3+94a3AbAI2cosωb∙t∙cosωc∙t

As the CW signal is much larger than backscattered signal, the term 916∙a3∙Ab3 can be dropped and further simplified with:

yoIt=a1Ab+94a3AbAI2cosωb∙t∙cosωc∙t

The voltage of the fundamental signal at output of LNA can be reformulated as:

vt=a1∙Ab∙1+94∙a3a1∙AI2

Since a3<0, we can write:

vt=a1∙Ab∙1-94∙a3a1∙AI2

It can be observed from the above that the wanted signal level is degraded with scaling factor which directly reduces the SNR, therefore the desensitization factor is:

SNRD=20log101-94a3a1∙AI2

Further the IIP3 of LNA can be expressed with:

AIIP3_LNA2=|a1||a3|∙43

Therefore, the SNR degradation can be formulated below:

SNRD=20log101-3∙AI2AIIP3LNA2=20log101-3∙Pcw_leakgePIIP3_LNA

Considering the shared hardware architecture and monostatic antenna configuration, with the return loss of 15 dB, the CW leakage power will be 18 dBm and if the SNR degradation is allowed with 3 dB, the IIP3 of LNA should be 28 dBm. Such high IIP3 is demanding for design. For other combination value for SNR degradation and CW leakage power, it is illustrated in Figure 8 and the IIP3 value is listed in Table 5. Referring to TR 38.858, the reported IIP3 for WA BS is from the range of 10 dBm to -27.6 dBm, this in turn demands some CW cancellation capability before LNA.

SNR degradation occurs with limited receiver linearity and amount of degradation is depending both the CW leakage power input to LNA and linearity of the LNA.

CW cancellation capability is needed if the IIP3 is to be relaxed and limited SNR degradation is allowed.

Table 5:  IIP3 of LNA with different SNR degradation and CW leakage power

|  |  | SNR degradation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | -1 | -3 | -10 |
| CW leakage power (dBm) | 18 | 32 | 28 | 24 |
|  | 8 | 22 | 18 | 14 |
|  | 0 | 14 | 10 | 6 |
|  | -10 | 4 | 0 | -4 |

Figure 8: IIP3 of LNA with different SNR degradation and CW leakage power

For CW cancellation, there are passive or active cancellation schemes existing for RFID reader front-end. In both schemes, a copy of the CW leakage signal is created with same amplitude but opposite phase. The cancellation scheme is illustrated in Figure 9. For the passive cancellation scheme, the cancellation path needs to be calibrated and for active cancellation scheme, the cancellation path can be tuned to adapt to the leakage CW signal. However, the cancellation is limited by either the hardware imperfection or the detecting signal amplitude/phase accuracy. In [6], it is mentioned that for 40 dB CW cancellation, the amplitude error should be 0.09 dB and phase error to be 0.5 degree. This is quite demanding in design considering the current RRM measurement accuracy for RSRP is +/-2 dB in normal condition. In this sense, the cancellation capability of around 10 dB seems achievable. The above is for shared hardware architecture so the leakage signal can be detected or measured/calibrated within the same hardware.

However, when the CW and A-IoT signal are transmitted from different hardware, there is no way to create of local copy of leakage CW signal in A-IoT BS/UE as reader. Therefore, the passive CW cancelation scheme is not possible. Alternatively, the active CW cancellation scheme should be used in this case. This applies to the case where the CW node is co-located with A-IoT BS for D1T1-A2 or D2T2-A2.

The CW cancellation capability is limited by the achievable cancellation signal amplitude error and phase error.

CW cancellation around 10 dB before LNA is achievable for shared hardware.

Figure 9: CW cancellation scheme

Overall, from the above discussion, it can be observed that CW leakage will result in receiver sensitivity degradation. Such SNR degradation relates to the LNA linearity design and CW leakage power level. To relax the LNA linearity requirement, either CW cancellation scheme should be implemented, or the system allows more SNR degradation or both. The SNR degradation should be investigated in the context of the link budget which is in RAN1 scope. CW cancellation capability is limited by the implementation and around 10 dB could be achievable. In our opination, the SNR degradation range from 1 dB to 10 dB should studied as it shows usually R2D could be limiting factors for link budget and therefore, the D2R sensitivity is decided by R2D range and therefore some degradation would be allowed.

Study the SNR degradation range from 1 dB to 10 dB caused by imperfection of CW cancellation, e.g 1 dB, 3 dB and 10 dB.

- TP start here:

### 6.7.5	  Feasibility study

#### 6.7.5.1	CW cancellation

The min received power could be -74 dBm or lower, the signal with such low level received power should be decoded in the presence of the CW transmission which is either 33 dBm or 23 dBm. The CW transmission can leak to the receiver chain via either reflection from environment or the imperfection of the isolation between transmitter and receiver when CW is transmitted by A-IoT BS or UE as reader. As the receiver has limited dynamic range and linearity, this CW leakage needs to be attenuated/isolated to mitigate the desensitization of the receiver.  The CW signal frequency is used to modulate the backscatter signal and the frequency separation of the CW signal and modulated signal is around 100kHz for example as illustrated in Figure 6.7.5.1-1 according to the current D2R waveform design. Using an RF filter before the LNA to attenuate the CW signal is not possible. As such, a high level CW leakage signal will be present together with a low wanted backscattered signal before LNA.

Figure 6.7.5.1-1: D2R waveform in frequency domain

To analysis the impact of the receiver desensitization, the backscattered signal is modelled as below[2]:

Sb (t)=Abcos⁡(ωb∙t+φb)∙cosωc∙t+φc

with ωc representing the CW frequency and ωb is the message frequency.

And the leakage CW signal is modelled as below:

SI (t)=AI∙cosωc∙t+φI

Then the receiver nonlinearity is modelled with a power series below assuming the second-order nonlinear product can be filtered:

yot=a1∙xt+a3∙xt3

Where a1 is the small signal gain of the receiver and a3<0 represents the nonlinear behaviour of the system.

The input signal before LNA is then:

xt=Sb t+SI (t)

Assuming the φb=φc=φI=0, at output of the LNA, only the terms associated with fundamental frequency coswbtcos⁡(ωct) are of interest and they are below:

yoIt=a1Ab+916a3Ab3+94a3AbAI2cosωb∙t∙cosωc∙t

As the CW signal is much larger than backscattered signal, the term 916∙a3∙Ab3 can be dropped and further simplified with

yoIt=a1Ab+94a3AbAI2cosωb∙t∙cosωc∙t

The voltage of the fundamental signal at output of LNA can be reformulated as:

vt=a1∙Ab∙1+94∙a3a1∙AI2

Since a3<0, we can write:

vt=a1∙Ab∙1-94∙a3a1∙AI2

It can be observed from the above that the wanted signal level is degraded with scaling factor which directly reduces the SNR, therefore the desensitization factor is:

SNRD=20log101-94a3a1∙AI2

Further the IIP3 of LNA can be expressed with:

AIIP3_LNA2=|a1||a3|∙43

Therefore, the SNR degradation can be formulated below:

SNRD=20log101-3∙AI2AIIP3LNA2=20log101-3∙PcwleakgePIIP3LNA

Assuming CW signal is transmitted by A-IoT BS or UE as reader and monostatic antenna configuration, with the return loss of 15 dB, the CW leakage power will be 18 dBm. If the SNR degradation is allowed with 3 dB, the IIP3 of LNA should be 28 dBm. Such high IIP3 is demanding for design. For other combination value for SNR degradation and CW leakage power, it is illustrated in Figure 6.7.5.1-2 and the IIP3 value is listed in Table 6.7.5.1-1. Referring to TR 38.858, the reported IIP3 for WA BS is from the range of 10 dBm to -27.6 dBm, this in turn demands some CW cancellation capability before LNA.

Table 6.7.5.1-1:  IIP3 of LNA with different SNR degradation and CW leakage power

|  |  | SNR degradation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | -1 | -3 | -10 |
| CW leakage power (dBm) | 18 | 32 | 28 | 24 |
|  | 8 | 22 | 18 | 14 |
|  | 0 | 14 | 10 | 6 |
|  | -10 | 4 | 0 | -4 |

Figure 6.7.5.1-2: IIP3 of LNA with different SNR degradation and CW leakage power

For CW cancellation, there are passive or active cancellation schemes existing for RFID reader front-end. In both schemes, a copy of the CW leakage signal is created with same amplitude but opposite phase. The cancellation scheme is illustrated in Figure 6.7.5.1-3. For the passive cancellation scheme, the cancellation path needs to be calibrated and for active cancellation scheme, the cancellation path can be tuned to adapt to the leakage CW signal. However, the cancellation is limited by either the hardware imperfection or the detecting signal amplitude/phase accuracy. In [6], it is mentioned that for 40 dB CW cancellation, the amplitude error should be 0.09 dB and phase error to be 0.5 degree. This is quite demanding in design considering the current RRM measurement accuracy for RSRP is +/-2 dB in normal condition.

When the CW and A-IoT signal are transmitted from different hardware, the active CW cancellation scheme should be used in this case. This applies to the case where the CW node is co-located with A-IoT BS for D1T1-A2 or D2T2-A2.

Figure 6.7.5.1-3: CW cancellation scheme

# Conclusions

In this contribution, we present our overview on the A-IoT CW cancellation issues with below observations and proposal:

Observation 1 The A-IoT BS REFSENS could be relaxed with 23 dB or 33 dB for device 1 harvesting the RF energy from A-IoT BS depending on the CW transmission power.

Observation 2 R2D will be limiting factor for link budget for device 1 with 23 dBm CW signal and A-IoT BS REFSENS can be relaxed with 9 dB.

Observation 3 D2R will be limiting factor for link budget for device 1/2a with 33 dBm CW signal for hardcoded device 1/2a sensitivity.

Observation 4 Both shared and separated hardware for CW transmission and A-IoT transmission should be studied.

Observation 5 The minimum received power for A-IoT BS/UE as reader is defined with the maximum distance from link budget analysis and in the presence of CW transmission.

Observation 6 Using the RF filter to attenuate the CW signal is difficult to design.

Observation 7 SNR degradation occurs with limited receiver linearity and amount of degradation is depending both the CW leakage power input to LNA and linearity of the LNA.

Observation 8 CW cancellation capability is needed if the IIP3 is to be relaxed and limited SNR degradation is allowed.

Observation 9 The CW cancellation capability is limited by the achievable cancellation signal amplitude error and phase error

Observation 10 CW cancellation around 10 dB before LNA is achievable for shared hardware.

Proposal-1: Study the SNR degradation range from 1 dB to 10 dB caused by imperfection of CW cancellation, e.g 1 dB, 3 dB and 10 dB.

# References

- R4-2414305, WF on impacts of A-IoT on RF requirements, Huawei

- R1-2405824, Evaluation assumptions and results for Ambient IoT, Ericsson

- R1-2405826, General aspects of physical layer design for Ambient IoT, Ericsson

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

- TR 38.858

- Jun Chen, Tao Yan, Zhong Huang, “Carrier and Noise Cancellation Effects in a Self-Jammer Canceller for UHF RFID Readers” 2018 International Conference on Microwave and Millimeter Wave Technology (ICMMT)

- Ali Ghahremani, etc “A UHF-RFID Transceiver With a Blocker-Canceller Feedback and 30 dBm Output Power” IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS—I: REGULAR PAPERS, VOL. 60, NO. 11, NOVEMBER 2013