# 3GPP TSG RAN WG4 Meeting#112-bis	R4-2415647

# Hefei, CN, Oct 14 – Oct 18, 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on the demodulation performance requirements for 6Rx devices

Agenda item:	6.1.1.3.2

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues pertaining to performance requirements for 6Rx devices as well as agreed on a tentative set of simulation assumptions for performance evaluations [1]. In this paper, we share our views and simulation results for MIMO layer evaluation for the 6Rx devices.

2. 6-layer Support

6-layer Support

Way Forward: RAN4 to further discuss the following options.

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices.

-	Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point.

-	Feasibility criteria

-	Consider 4-layer vs 6-layer throughput performance.

-	Simulation assumptions

-	Only consider link-level simulation for performance evaluations.

-	Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM.

-	Option 4: 6-layer support is not feasible for handheld UEs.

During the last meeting, RAN4 discussed the feasibility of introducing 6-layer performance requirements for 6Rx devices. To this end, several simulation assumptions were agreed for initial performance evaluations. The related way forward is copied below [1]. In this section, we present our simulation results for 6Rx handheld device showing the feasibility of 6-layer support.

2.1 Simulation Assumptions

We consider the assumptions provided in Table 1 for our simulations. We consider a 4L configuration with 6 receive antennas and 8 transmit antenna ports as a reference scenario. Instead of considering 16 transmit antenna ports as in the simulation assumptions [1], we consider 32 transmit antenna ports as the field data indicates 32 transmit antenna ports configuration is widely configured for TDD scenarios. Also, we used fixed PMI (Rel-15/Type 1 and Rel-18/e-Type 1) and SVD based precoding (to evaluate beamforming gain) for our simulations.

Table 1: Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 100MHz |
| SCS |  | 30 |
| Antenna configuration |  | [32Tx, 6Rx] [8Tx, 6Rx] |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank [4,5,6] |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 1 and Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Fixed PMI (Rel-15 and Rel-18) and SVD based precoding |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | [Rel-18/Rel-15 1+1(4L/6L), Rel-15 2+0(6L)] [Rel-15 2+2(4L), Rel-15 2+2(6L)] |
|  | Precoding | Fixed PMI (Rel-15 and Rel-18) and SVD based precoding |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 |

2.2 Simulation Results

2.2.1 Impact of Demodulation Reference Symbols

We note that if more than four MIMO layers shall be supported, six or more DMRS antenna ports need to be configured. This can be achieved by multiple DMRS configurations, such as Rel-15 and Rel-18 eType 1 DMRS configurations.

- Rel-15 Type1 DMRS with two frontloaded DMRS symbols: Type1 DMRS supports only four orthogonal DMRS ports with one frontloaded DMRS symbol.

- Rel-18 eType1 DMRS with one frontloaded and one additional DMRS symbols: eType1 DMRS allows to configure up to eight orthogonal DMRS ports with one frontloaded DMRS symbol [2]. The larger number of DMRS ports per symbol comes at the expense of a reduced frequency density of DMRS resource elements. Using only a single FL DMRS is sensitive to higher Doppler spread. Therefore, we consider a configuration with one frontloaded and one additional DMRS symbols.

The simulations apply MCS link adaptation via ACK/NACK feedback targeting 10% first-tx BLER and fixed rank  {4,5,6} for 6Rx antennas. The number of Tx antenna ports is assumed eight and an advanced R-ML receiver is considered for these results. It is assumed that precoder for data transmission is fixed in our simulations. Table 2 shows relative throughput for five and six layers with respect to a reference scenario for each SNR value with MCS Table 1. The receiver correlation matrix is option 1 from Table 1. Also, we assume zero Tx antenna correlation. The reference scenario applies four layers and 6Rx antennas with 8Tx antenna ports and configured with Rel-18 1+1 DMRS configuration. A positive relative throughput means that five and/or six layers provide a gain over four layers for 6Rx antennas with the same number of transmit antenna ports.

Table 2: Throughput performance relative to 6Rx/4L Rel-15 1+1 DMRS with MCS Table 1

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+0 | 26 | 9.4 |
|  |  | 30 | 25.2 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
|  | Rel-18 1+1 | 26 | 7.2 |
|  |  | 30 | 24.5 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
| 6RX/6L | Rel-15 2+0 | 28 | 11.5 |
|  |  | 30 | 26.8 |
|  |  | 34 | 48.9 |
|  |  | 38 | 50.4 |
|  | Rel-18 1+1 | 28 | 8.8 |
|  |  | 30 | 23.1 |
|  |  | 34 | 49.6 |
|  |  | 38 | 50.4 |

From Table 2, it can be observed that Rel-18 1+1 DMRS configurations offers comparable performance with that of Rel-15 2+0 configuration. Having one additional DMRS symbol, it is expected that Rel-18 1+1 DMRS combination will outperform that of Rel-15 2+0 configuration at higher Doppler scenarios. Furthermore, comparing between 5L and 6L performances with Rel-18 1+1 DRMS configurations, it is noted that 6L starts to outperform 5L at around 30dB, showing the feasibility of 6L performance.

Observation 1: Rel-18 1+1 DMRS configuration provides comparable performance with that of Rel-15 2+0 configuration.

Observation 2: It is expected that Rel-18 1+1 DMRS will perform better at higher Doppler compared to that of Rel-15 2+0 configuration due to additional DMRS symbol.

Observation 3: With Rel-18 1+1 DMRS configuration and MCS Table 1, 6L starts to outperform 5L at around 30dB and 5L starts to outperform 4L at around 24dB.

Table 3: Throughput performance relative to 6Rx/4L Rel-15 1+1 DMRS with MCS Table 2

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+0 | 30 | 3.9 |
|  |  | 34 | 10.8 |
|  |  | 38 | 23.9 |
|  |  | 42 | 24.7 |
|  | Rel-18 1+1 | 30 | 1.2 |
|  |  | 34 | 14.8 |
|  |  | 38 | 24.7 |
|  |  | 42 | 24.7 |
| 6RX/6L | Rel-15 2+0 | 30 | 3.5 |
|  |  | 36 | 6.0 |
|  |  | 38 | 13.1 |
|  |  | 42 | 21.3 |
|  | Rel-18 1+1 | 30 | 2 |
|  |  | 34 | 2 |
|  |  | 38 | 21.3 |
|  |  | 42 | 32.8 |

With the same simulation assumptions for the results presented in Table 2, now we analyze the performance with MCS Table 2. As expected, it is observed that with MCS table 2, a higher absolute peak throughput is achieved. Besides, it is seen from Table 3 that Rel-18 1+1 DRMS is already outperforming that of Rel-15 2+0 configuration starting 32dB and the performance gap is further widening at higher SNR.

Observation 4: Rel-18 1+1 DMRS configuration shows noticeable performance gain compared to that of Rel-15 2+0 configuration starting from 32dB SNR.

Observation 5: With Rel-18 1+1 DMRS configuration and MCS Table 2, 6L starts to outperform 5L at around 38dB and 5L starts to outperform 4L at around 22dB.

Table 4: Throughput performance relative to 6Rx/4L Rel-18 1+1 DMRS with MCS Table 1 across correlation matrices from options 1 and 2

| RX/Layer | Rel-18 DMRS Configuration/Correlation Matrix | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 1+1 (Option 1) | 28 | 20 |
|  |  | 30 | 24.5 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
|  | 1+1 (Option 2) | 28 | 17.7 |
|  |  | 30 | 24.4 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
| 6RX/6L | 1+1 (Option 1) | 28 | 8.8 |
|  |  | 30 | 23.1 |
|  |  | 34 | 49.6 |
|  |  | 38 | 50.4 |
|  | 1+1 (Option 2) | 28 | 6.1 |
|  |  | 30 | 20.2 |
|  |  | 34 | 48.9 |
|  |  | 38 | 50.4 |

Now we evaluate performance with correlation matrix in option 2 from Table 1 and compare with performance observed with correlation matrix from option 1. We note that correlation matrix in option 2 shows higher correlation on average. Therefore, it is of interest to see how much performance degradation is observed due to higher correlation across receive antennas. Compared to performance presented in Table 2, we see from Table 4 that 5L performance is similar across options 1 and 2 and degradation for 6L with correlation matrix from option 2 is minimal, i.e., less than 3%.

Observation 6: Compared to performance with correlation matrix from option 1, negligible performance loss (<3%) is observed when compared to performance with correlation matrix from option 2.

We now compare performance results with Rel-15 2+2 DMRS configurations. For these results, we assume the reference scenario is 4L/Rel-15 2+2 DMRS configuration. The objective is to compare the 5L and 6L performance against this reference scenario. We note that the DMRS overhead is quite heavy with this configuration, which can be observed with lower peak throughput, e.g., compared with 2+0 configuration. We consider the correlation matrix in option 1 for these simulations with fixed precoding and MCS Table 1.

Table 5: Throughput performance relative to 6Rx/4L Rel-15 2+2 DMRS with MCS Table 1

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+2 | 24 | 5.6 |
|  |  | 26 | 18.8 |
|  |  | 28 | 26.2 |
|  |  | 30 | 25.2 |
| 6RX/6L | Rel-15 2+2 | 26 | 3.9 |
|  |  | 28 | 22.8 |
|  |  | 30 | 36.9 |
|  |  | 32 | 49.5 |

Table 6: Throughput performance relative to 6Rx/4L Rel-15 2+2 DMRS with MCS Table 2

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+2 | 30 | 11.2 |
|  |  | 34 | 23.8 |
|  |  | 36 | 24.9 |
|  |  | 38 | 25.7 |
| 6RX/6L | Rel-15 2+2 | 32 | 0.4 |
|  |  | 34 | 8.7 |
|  |  | 36 | 20.9 |
|  |  | 38 | 31.4 |

From Table 5, we observe that 6L starts to outperform both 4L and 5L around 26dB and 28dB, respectively, with Rel-15 2+2 configuration and MCS Table 1. With MCS Table 2, the relative performance is captured in Table 6.  From these results, we observe that 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively for Rel-15 2+2 configuration.

Observation 7: For Rel-15 2+2 DMRS configuration with MCS Table 1, 6L starts to outperform both 4L and 5L around 26dB and 28dB, respectively.

Observation 8: For Rel-15 2+2 DMRS configuration with MCS Table 2, 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively.

Observation 9: With Rel-15 2+2 DMRS configuration, significant throughput loss is observed due to increased DMRS overhead.

2.2.2 Impact of Number of Transmit Antenna Ports

In this section, we provide our results with larger number of transmit antenna ports. In particular, we compare earlier performance with 8 transmit antenna ports that is discussed in the previous section with that of 32Tx antenna ports. To this end, it is assumed that SRS antenna switching is available for TDD scenarios and SVD based beamforming at the transmitter. We assume an update periodicity of 40 slots (i.e., 20ms for TDD scenario), which results in a reasonable trade-off between SRS overhead and tracking the time-selective channel in a realistic deployment.

Table 7: Throughput performance relative to 8Tx/4L Rel-18 1+1 DMRS with MCS Table 1 and 32 transmit antenna ports

| RX/Layer | Rel-18 DMRS Configuration/Correlation Matrix | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 1+1 (Option 1) | 24 | 21.2 |
|  |  | 26 | 25.2 |
|  |  | 30 | 25.2 |
|  |  | 34 | 25.2 |
|  | 1+1 (Option 2) | 28 | 25.2 |
|  |  | 30 | 25.2 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
| 6RX/6L | 1+1 (Option 1) | 28 | 40 |
|  |  | 30 | 47.4 |
|  |  | 34 | 50.4 |
|  |  | 38 | 50.4 |
|  | 1+1 (Option 2) | 28 | 7.9 |
|  |  | 30 | 23.5 |
|  |  | 34 | 59.6 |
|  |  | 38 | 50.4 |
| 6RX/4L | 1+1 (Option 1) | 20 | 19.4 |
|  |  | 24 | 0.7 |
|  |  | 26 | 0 |
|  |  | 28 | 0 |
|  | 1+1 (Option 2) | 20 | 22 |
|  |  | 24 | 13.2 |
|  |  | 26 | 0.7 |
|  |  | 28 | 0 |

From Table 7, we observe that 32 Tx antenna ports offer noticeable performance gain with respect to the reference 8Tx scenario even before reaching peak throughput. In particular, the SNR gain for 5L and 6L is observed to be 3dB and 4dB with 32 Tx antenna ports for both correlation matrices, respectively. We also see a 4dB improvement in 5L to 6L switchover point with 32Tx antenna ports when compared to 8Tx configuration for both correlation matrices. Even a 4L configuration sees a 20% throughput gain with 32 Tx antenna ports at SNR=20dB. Furthermore, with 32 Tx antenna ports, 6L configuration starts to outperform that of 4L at around 22 dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices.

Observation 10: For the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 32 transmit antenna ports offers roughly 3dB and 4dB SNR gain for 5L and 6L when compared to 8Tx configuration for both correlation matrices, respectively.

Observation 11: For the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 5L to 6L switchover point with 32Tx antenna ports is improved by 4dB for both correlation matrices.

Observation 12: For the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 6L configuration starts to outperform that of 4L at around 22 dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices.

2.3 Summary of Simulation Results

With the aid of agreed correlation matrices, in this section we have provided simulation results with different DMRS overhead (both Rel-15 and Rel-18) and increased number of transmit antenna ports. It is observed that Rel-18 1 1+1 DMRS configuration provides comparable performance compared to Rel-15 2+0 configurations. We also observed 6Rx/6L configuration outperforms that of 6Rx/4L at around SNR = 27 dB and 6Rx/5L at around SNR = 30dB for MCS Table 1. When MCS table 2 is considered, the switch over points to 6L from 4L and 5L shifts to 33dB and 39dB, respectively. Furthermore, considering the correlation matrix in option 2 with higher correlation coefficients, we see negligible (<3%) performance loss, suggesting 6L performance is quite robust to correlation values. Furthermore, performance results with Rel-15 2+2 DMRS configurations suggest significant throughput loss due to increased DMRS overhead.

Additionally, we have examined performance with 32 transmit antenna ports and compared with the baseline configuration of 8 transmit antenna ports. It has been noticed that a configuration of 32 transmit antennas can offer significant gain, up to 3dB and 4dB for 5L and 6L, respectively. This is assuming all other simulation assumptions remain unchanged. Furthermore, we see a 4dB improvement in 5L to 6L switch over points with 32Tx antenna ports.

Based on the observations from the simulation results with different DMRS types and configurations, as well as number of transmit antenna ports and practical correlation assumptions, we think that it is feasible to define 6-layer performance requirements for 6Rx devices.

Proposal 1: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

- FFS whether to use same requirements for handheld and FWA devices.

3. Conclusions

Observation 1: Rel-18 1+1 DMRS configuration provides comparable performance with that of Rel-15 2+0 configuration.

Observation 2: It is expected that Rel-18 1+1 DMRS will perform better at higher Doppler compared to that of Rel-15 2+0 configuration due to additional DMRS symbol.

Observation 3: With Rel-18 1+1 DMRS configuration and MCS Table 1, 6L starts to outperform 5L at around 30dB and 5L starts to outperform 4L at around 24dB.

Observation 4: Rel-18 1+1 DMRS configuration shows noticeable performance gain compared to that of Rel-15 2+0 configuration starting from 32dB SNR.

Observation 5: With Rel-18 1+1 DMRS configuration and MCS Table 2, 6L starts to outperform 5L at around 38dB and 5L starts to outperform 4L at around 22dB.

Observation 6: Compared to performance with correlation matrix from option 1, negligible performance loss (<3%) is observed when compared to performance with correlation matrix from option 2.

Observation 7: For Rel-15 2+2 DMRS configuration with MCS Table 1, 6L starts to outperform both 4L and 5L around 26dB and 28dB, respectively.

Observation 8: For Rel-15 2+2 DMRS configuration with MCS Table 2, 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively.

Observation 9: With Rel-15 2+2 DMRS configuration, significant throughput loss is observed due to increased DMRS overhead.

Observation 10: For the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 32 transmit antenna ports offers roughly 3dB and 4dB SNR gain for 5L and 6L when compared to 8Tx configuration, respectively.

Observation 11: For the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 5L to 6L switchover point with 32Tx antenna ports is improved by 4dB.

Observation 12: For the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 6L configuration starts to outperform that of 4L at around 22 dB and 5L configuration starts to outperform that of 4L below 20dB.

Proposal 1: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

- FFS whether to use same requirements for handheld and FWA devices.

4. References

[1] R4-2414285, “WF on requirements for 6Rx,” 3GPP TSG RAN WG Meeting #112, Maastricht, NL, Aug 19 – 23, 2024.

[2] TS 38.211, ‘Physical channels and modulation (Release 18)’, v18.2.0, March 2024