3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415728

Hefei, China, Oct. 14th – 18th, 2024

Source: 	Murata Manufacturing Co. Ltd

Title: 	MSD for DC_(n)8AA

Agenda item:	5.2.2	UE RF requirements for EN-DC and NE-DC of 2 DL with 2 UL (DC_R19_1BLTE_1BNR_2DL2UL)	[DC_R19_xBLTE_yBNR-Core]

Document for:	Approval

- Introduction

MSD test cases for DC_(n)8AA are discussed in this contribution.

# 2. Discussion

## 2.1 Background Information

In RAN4#112, The following has been approved as a way forward for DC_(n)8AA test point [1].

Table 1: Test point for DC_(n)8AA

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (dB) |  |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | FFS | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | FFS |  |
|  | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | FFS |  |
|  | n8 | N/A | 20 | N/A | 940 | FFS |  |
| NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

In this contribution, we evaluate the MSD for 1UL DC_(n)8AA using the test point in Table 1.

## 2.2 Results

### 2.2.1 Architecture assumption

Referring to [2][3], single PA and single antenna UE architecture has been assumed for low-band intra-band DC requirements. Accordingly, the MSD analysis for PC3 DC_(n)8AA is based on UE reference architecture as shown in Figure 1.

Figure 1: Reference architecture for MSD analysis

### 2.2.2 Power Amplifier Calibration

PC3 PA is calibrated at MPR1 for -30 dBc ACLR using a fully allocated 20 MHz DFT-s-OFDM QPSK waveform.

### 2.2.3 MSD Evaluation

We evaluated MSD values for PC3 DC_(n)8AA using the following assumptions.

- Tx-Rx duplexer isolation: 55dB

- Tx antenna duplex rejection in Rx range: 40dB

- Primary antenna to diversity antenna isolation: 10dB

- MRC combining and -1 dB Signal to-Noise Ratio (SNR)

Figure 2: DC_(n)8AA MSD analysis

Proposal 1: Specify MSD for PC3 DC_(n)8AA as follows.

Table 2: MSD for DC_(n)8AA

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (dB) |  |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 21.5 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | 0 |  |
|  | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | 0 |  |
|  | n8 | N/A | 20 | N/A | 940 | 2.3 |  |
| NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

# Conclusion

Proposal 1: Specify MSD for PC3 DC_(n)8AA as follows.

Table 2: MSD for DC_(n)8AA

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (dB) |  |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 21.5 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | 0 |  |
|  | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | 0 |  |
|  | n8 | N/A | 20 | N/A | 940 | 2.3 |  |
| NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

# References

- R4-2414349, “WF on DC_(n)8AA”, Skyworks, Murata, China Unicom, CHTTL, RAN4#112, Maastricht, Netherlands

- R4-2005123, “TP for TR37.716-11-11: UE MSD requirements for DC_(n)5AA”, RAN4#94-2-Bis, Electronic Meeting

- R4-1714344, “WF on DC_71-n71”, Intel, T-Mobile, Skyworks, Nokia, Ericsson, RAN4#85, Reno, USA