3GPP TSG-RAN4 Meeting #112bis	R4-2415754

# Hefei, China, Oct 11 – 18, 2024

Agenda item:			6.1.1.2.2

Source:	vivo

Title:	Discussion on MPR applicability for FR1 intra-band UL CA

Document for:	Discussion

- Introduction

In RAN#112 meeting, the WF[1] has been agreed, and the remaining issue is as follows:

### Sub-topic 2-1: Intra-band contiguous UL CA #### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung) - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE) - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated; - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm). - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4 - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4 - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4 - WF - FFS in next meeting #### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung) - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei) - WF - FFS in next meeting #### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks) - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo) - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson) - WF - FFS in next meeting #### Issue 2-1-4: Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell - Proposals - Proposal 1: Add the following description into clause 6.2A.2.1: - “For intra-band contiguous carrier aggregation with single CC with activated cell, MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.” - WF - FFS the details of draft CR - Sub-topic 2-2: Intra-band non-contiguous CA #### Issue 2-2-1: General considerations - Agreement - The work scope for FR1 NC CA should be limited to only study MPR applicability. #### Issue 2-2-2: Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. (Qualcomm, ZTE, Huawei) - Proposal 2: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC. (Skyworks) - Proposal 3: for non-contiguous UL CA configurations with sub-blocks consisting of one cell and supported by dual PA architecture, the non-CA (single CC) MPR applies for one cell active among the configured uplink serving cells, the other cell deactivated. (Ericsson) - Proposal 4: for non-contiguous UL CA configurations with sub-blocks consisting of one cell not supported by a dual PA architecture, applicability of the non-CA (single CC) MPR for one active cell among the configured uplink serving cells, the other cell deactivated, is subject to UE capability, e.g. indication of [mpr-singleCC-activated-FR1] for the band combination. (Ericsson) - WF - FFS in next meeting

- Discussion

## 2.1. Sub-topic 2-1: Intra-band contiguous UL CA

As it has been agreed that for PC3 and PC2 contiguous UL CA the corresponding single CC’s MPR would be used when only 1 CC is activated. If each CC has a separate LO then the MPR would fall back to the single carrier level. As to the BW basis of the out-of-band emission requirements such as SEM\ACLE\SE, it is recommended that the evaluation bandwidth of ACLR\SEM\SE should be based on the BW of the activated CC rather than the aggregated BW.

From one side, the transmission mode is almost the same as single CC case. From the other side, the MPR reduction from CA to single carrier means the transmission power is increased. On this promise if the evaluation bandwidth of EVM and ACLR still keeps as wide as aggregated BW including multiple CCs activated in CA, these requirements will be relaxed compared to single-carrier. Then the transmit power could be much higher and the emission is more likely to cause interference with the adjacent UEs.

Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, when dualPA-Architecture IE is indicated, the single CC MPR could apply. And the integral region and boundary of SE/ACLR/SEM should be based on the activated CBW.

However, in the last meeting according to the testing data and analysis of [2], when there is only one CC activated but the LO does not switch to the middle of the CC accordingly, the image would fall into the frequency range of the deactivated CC. And if the activated CBW is too narrow the image would be higher than the SEM mask of the activated single carrier, therefore a bigger MPR is needed to guarantee the output performance.

Based on the issue above, there are a couple of different schemes in the non-dualPA architecture:

Option 1. For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, the MPR and out-of-band emission requirements of the single carrier would be applied. At the same time, specifying a new MPR value which is larger than original single carrier in the case the that bandwidth of the activated CC is too narrow.

Option 2. For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, the MPR and out-of-band emission requirements of the single carrier would be applied with the premise that the LO could switch with the CC activating.

Option 3. For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, the MPR of the single carrier would be applied, and the out-of-band emission requirements should refer to the CA CBW.

For option 1, it is similar to the “exception case” with narrow RB allocation bandwidth in non-contiguous CA configuration, setting a higher MPR to leave some margin for the IMD product exceeding the spurious limit. But specifying a new MPR is not in the WID scope so option 1 should not be prioritized. As to option 2, it would limit the UE implementation and transmission interruption problems may occur. So option3 is preferred.

Proposal 2: For PC3/PC2 intra-band contiguous CA with single CC with activated cell, when dualPA-Architecture IE is not indicated, the MPR of the single carrier could apply. And the integral region and boundary of SE/ACLR/SEM should be based on the aggregated CBW.

## 2.2. Sub-topic 2-2: Intra-band UL non-contiguous CA

For NC CA with 1CC activated, if each CC has a separate LO then the MPR could fall back to the single carrier level without any problem, and no spec changes are needed expect for adding 1CC activated as a supplementary scenario after configuring 1CC. However, if there is only one LO which could not switch with the activating state changed, and the RB numbers of the two CCs are allocated asymmetrical then the image of the activated CC would fall into the gap and exceed the SEM sometimes.

In the current spec, the MPR of 1CC configured with B < 9/15 MHz is specified to be higher than the single carrier as the IMD product of the configured narrow CC would fail the spurious limit requirement. If considering the image issue together the MPR value and the width of B should be re-evaluated to provide enough room for the worst transmission cases. But re-evaluation should be triggered in other items as it is out of scope.

LO-switching is an implementation matter and should not be directly covered in specifications. At the same time, MPR is the maximum power reduction, if the LO can be switched then the output power of the activated narrow CC can naturally become lower or even as low as the level of single carrier’s MPR. We should retain a larger MPR for the UE without capability of LO-switching. Therefore, there is no need to force all intra-band NC CA with single CC activated to follow single carrier MPR.

Proposal 3: For PC3 and PC2 intra-band non-contiguous CA with single CC with activated cell, when dualPA-Architecture IE is indicated, the MPR of single carrier applies and no spec changes are needed except for adding 1CC activation as a supplementary scenario.

For UE indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively Else If AND( FIM3,low_block,low > SEM-13,low , FIM3,high_block,high < SEM-13,high ) MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively. Else MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively.

Proposal 4: For PC3 and PC2 intra-band non-contiguous CA with single CC with activated cell, when dualPA-Architecture IE is not indicated,

- Activating 1CC could be added as a supplementary scenario after configuring 1CC.

For UE without indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used; For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used; For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

- No need to specify all intra-band NC CA with single CC activated to follow single carrier’s MPR.

- If 5.5dB and 6.5dB could not guarantee the output performance of narrow RB allocation cases then it should be re-evaluated in other items so that the worst scenario could be covered.

- Conclusion

Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, when dualPA-Architecture IE is indicated, the single CC MPR could apply. And the integral region and boundary of SE/ACLR/SEM should be based on the activated CBW.

Proposal 2: For PC3/PC2 intra-band contiguous CA with single CC with activated cell, when dualPA-Architecture IE is not indicated, the MPR of the single carrier could apply. And the integral region and boundary of SE/ACLR/SEM should be based on the aggregated CBW.

Proposal 3: For PC3 and PC2 intra-band non-contiguous CA with single CC with activated cell, when dualPA-Architecture IE is indicated, the MPR of single carrier applies and no spec changes are needed except for adding 1CC activation as a supplementary scenario.

For UE indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively Else If AND( FIM3,low_block,low > SEM-13,low , FIM3,high_block,high < SEM-13,high ) MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively. Else MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively.

Proposal 4: For PC3 and PC2 intra-band non-contiguous CA with single CC with activated cell, when dualPA-Architecture IE is not indicated,

- Activating 1CC could be added as a supplementary scenario after configuring 1CC, i.e.,

For UE without indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used; For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used; For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

- No need to specify all intra-band NC CA with single CC activated to follow single carrier’s MPR.

- If 5.5dB and 6.5dB could not guarantee the output performance of narrow RB allocation cases then it should be re-evaluated in other items so that the worst scenario could be covered.

- References

- R4-2414441, WF on power domain enhancements, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #112.

- R4-2411049, On improved MPR for intra-band ULCA when only one CC has RBs allocated, Skyworks Solutions, Inc. 3GPP TSG-RAN WG4 Meeting #112.