3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415790

Hefei, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Feasibility study and analysis for Iot NTN HPUE Tx Requirements

Agenda Item:	6.8.2.3.1

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. A paper [4] was submitted including an initial feasibility study and analysis for both NR-based and Iot-based NTN HPUE Tx requirements.

In this paper, further feasibility analysis based on some measurements for NB-Iot based Iot NTN were collected, and a number of proposals were made based on last meeting’s progress.

Verification and feasibility Analysis

General

Background

An analysis has been provided in [4], in which the following observations and proposals have been provided:

NB-Iot Based

Observation 4: It is easier for NB-Iot to implement higher power with its half-duplex and very narrow bandwidth.

Proposal 2: For NB-Iot based Iot-NTN, including handheld and non-handheld:

    - Power Class 2 is feasible;

    - Power Class 1 is likely to be feasible.

eMTC Based

Observation 5: eMTC based Iot-NTN feasibility is largely comparable to NR-NTN.

Proposal 3: For eMTC based Iot-NTN:

    - Power Class 2 is likely to be feasible for handheld and non-handheld;

    - Power Class 1 feasibility is FFS;

In the WF [3], the following WF is provided

Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes

Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes

- Agreement of Issue 3-2-1 to Issue 3-2-3:

  - For PC2, it is feasible to support both handheld and non-handheld devices.

  - FFS other power classes for applicable device type.

So the main target is for whether PC1 is feasible or not for different Iot NTN cases.

Constraints analysis

Among the restrictions of the power classes that can be achieved, Power amplifier and duplexer are two main constrains for Iot NTN UE to reach higher power. What is different from NR NTN is, only eMTC based full duplex Iot-NTN need duplexer, while half duplex eMTC based Iot-NTN and NB-Iot based Iot NTN do not need a duplexer.

In the case that duplexer is not needed, the IL budget that reserved for front end can be reduced from 5dB to 3dB, this alone would bring a 2dB UE output power raise in the UE antenna port.

Observation 1a: Power amplifier is the main constraint for all the half duplex cases, including NB-Iot based Iot-NTN and half duplex eMTC based Iot-NTN;

Observation 1b: Power amplifier and duplexer are two main constraints for full duplex case, which is eMTC based Iot-NTN.

Table 1. Whether a constrain exists for different Iot NTN

|  | PA | Duplexer |
| --- | --- | --- |
| NB-Iot Based | Yes | No |
| half duplex eMTC based | Yes | No |
| full duplex eMTC based | Yes | Yes |

In order to have more insight on the state-of-the-art technology, we made testing on two different commercial PAs. One is specially designed for satellite communication, and another is a GSM PA. It should be noted that those are all based on handheld UE type, which is designed for mobile phones.

Proposal 1: Confirm that only full duplex eMTC based Iot-NTN need duplexer in Iot NTN.

Handheld and non-handheld for Iot NTN

Although no official definition, handheld and non-handheld is a concept that usually connected with phones, which is usually for voice and data service used by human. By comparison, the NB-Iot and eMTC, as their names indicated, were designed for other purposes such as Iot devices. So in LTE, there were no discussion on whether NB-Iot and eMTC device are handheld or non-handheld.

Observation 2a: In LTE, NB-Iot and eMTC are not differentiate between handheld and non-handheld.

However, as these were extended to Iot-NTN, things may have slight changes, and the scenarios and deployments may have differences from legacy Iot systems. E.g. some of them might be used in handheld devices.

In addition, in the HPUE WI, they have already been discussed together with NR NTN as a package, and apply the categorization of handheld and non-handheld in the WID description such as following:

So we can assume here that similar categorization can also be done for Iot NTN devices.

Observation 2b: Iot NTN may face different scenarios compared to legacy Iot in LTE.

Proposal 2: Iot NTN also can be differentiated between handheld and non-handheld devices.

1Tx vs 2Tx

In last RAN4 meeting, there has been discussion on whether 2Tx should be considered for PC2 Iot NTN. And in the WF, “at least 1Tx” is used for Iot to leave some room for future discussion.

During the specification of NB-Iot and eMTC, 1Tx is one of the most basic assumptions for UE architecture, and all the physical layer and RF specs are based on 1Tx.

Observation 3a: 1Tx is one of the most basic assumptions for NB-Iot and eMTC.

Although 2Tx is a way to achieve higher Tx power without introducing higher power components such as PA and duplexer, this way actually has some intrinsic drawbacks such as more complexities, more cost and IC area, and less predictable performance. So, this may not be a good extension, particularly for the Iot which usually more sensitive to cost and complexity.

Even Iot NTN may have some difference, this sensitivity is still likely to be kept.

Observation 3b: Further extend to 2Tx may bring undue complexities and cost for Iot.

In addition, as can be seen from later testing, it would be easier for Iot device to achieve higher power. Thus the need for 2Tx extension is further degraded.

Observation 3c: Iot can achieve higher power even with 1Tx, which makes the 2Tx even less necessary.

Based on the previous observations, the following proposal is provided:

Proposal 3: Only 1Tx is considered for Iot NTN, and 2Tx implementation not considered.

PA-1 (Satellite) Testing & Analysis

PA introduction

In order to have an understanding of the state-of-the-art PA performance for satellite communication for handheld UE, we select one commercial PAM designed for satellite communication. It should be noted that this is a different PA compared to what was verified for NR-NTN in [7], although some design targets are similar.

Here are some basic information of the PA and the testing scenarios

- PAM designed for Satellite voice, SMS, and data services application with optimized PAE and harmonic suppression.

- Package Size: Approximate 3.6 x 3.6 x 0.7 mm

- Multi-band PA cover:1980M~2010MHz,1608M~1624MHz and 1668M~1675MHz

- Verification using typical PA configurations (VCC etc.)

Testing configurations and results

The testing configuration are referenced from Iot NTN testing spec 36.521-4

- NB-Iot based NR signal is verified for n256 Freq = 1995MHz

- PA output power Prated = 29/33/37 dBm

- Configuration 36.521-4, Table 6.2B.2.4.1-1, Configuration ID: 1,3,6,10; + Annex A.2.3.

Table 2. Configuration table (Reference from 36.521-4)

It is noted that MPR is not considered yet. The GSM_ACLR and UTRA_ACLR is tested. The requirements can reference the following table:

Table 3. ACLR definition (Reference from 36.521-4)

Based on the previous configurations, the following measurement results were obtained.

Table 4. Measurement results for PA1

From the results, it can be seen that the 12-tone configuration #10 generally have the worst results under each PA output power. So the later analysis could be based on this configuration.

Analysis

The measured Pout is the output power of the PA, and did not consider the insertion loss of front end yet. In the production, we would assume a 3dB IL needed to leave more room for the cases that no duplexer is needed.

Based on this value, the Pout of PA of 33/37dBm corresponds to an expected 30/34dBm output in the UE antenna port, which is well beyond what PC1 is assumed should be (31dBm). Even at the 37dBm output level, the GSM_ACLR and UTRA_ACLR still have quite large value 28.5/45.3. If a 2dB MPR is also considered in this 12-tone configuration which is allowed by the current spec, even more margins can be expected.

Though ACLR is still depending on co-existence study, we can still use the current PC3 value as reference.

Observation 4: Based on the measurement of a PA designed for handheld UE in satellite communication, PC1 1Tx have quite sufficient margin for NB-Iot based Iot-NTN.

PA-2 (GSM) Testing & Analysis

PA introduction

NB-Iot shares some similar features with GSM. For example, they all occupy 200KHz carrier. In addition, the GSM also have a maximum power beyond 30dBm, which is similar to what the HPUE is designed for.

So, it is interesting to see how the legacy GSM PA could behave if used it as a high power NB-Iot PA. One more measurement is done to check GSM PA performance for NB-Iot and see what kind of power level could be achieved.

Figure 1. Block diagram of a GSM PA for verification

One commercial Integrated LB/MB GSM PA with antenna switch was selected. The test output port was as shown, and the antenna switching is already incorporated into this module, to used for other 4G/5G signals. There is an approximate 1~2dB IL between the GSM PA and the test output port, making the verification a bit more stringent actually. In another word, the extra IL needed after the test output port can be controlled to around 1dB.

Testing configurations and results

In order to simplify the test, only #10 (full RE allocation) from previous Table 6.2B.2.4.1-1 in 36.521-4 was selected, which is the worst full RE allocation.

Both n255 and n256 were covered, and a number of Pout were verified. It should be noted that 31.x is the maximum possible output power, since the PA is saturated and cannot achieve 32dBm output power.

Figure 2. GSM_ACLR results for a GSM PA (#10)

Figure 3. UTRA_ACLR results for a GSM PA (#10)

Analysis

It can be seen that all the ACLRs are within specification with reasonable margin for PC3 requirements (at least 5dB). From these results, although it seems that this GSM PA still cannot reach PC1 1Tx for NB-Iot signal, it actually get quite close:

- Since the antenna switch is already integrated, it needs only 1~2dB more IL to reach the antenna connector, this means that it can almost reach 29~30dBm in the antenna connector.

- In addition, this is full RE allocation, means that a 2dB MPR is allowed (for PC3). If similar value is allowed for PC1 1Tx, then a 29dBm output power can also satisfy PC1.

So, based on these results, the following observation can be made:

Observation 5: Based on the measurement for a GSM PA, the PC1 1Tx can almost be achieved, although margin may not be sufficient.

So based on these results and observations, it can be concluded that:

Observation 6: The implementation difficulty of PA to satisfy PC1 1Tx for NB-Iot based Iot-NTN, should be comparable to GSM with limited improvements. For PAs specifically designed for satellite communications, margins are considerable.

Feasibility Summary

In all, with current verification, it seems that for handheld UE, the feasibility of PC1 1Tx is as follows:

- NB-Iot Based: PA is feasible for PC1 1Tx; no duplexer needed, thus PC1 should be feasible.

- Half duplex eMTC based: PA may be feasible for PC1 1Tx, but margin may not sufficient. This is based on NR NTN verification data; no duplexer needed; thus PC1 may also be feasible. However, more data and analysis can also be pursed before a conclusion, and this can also be FFS;

- Full duplex eMTC based: PC1 1Tx may not be achieved by the PA, and the duplexer is needed but can not achieved. This would means that PC1 is not feasible.

A brief table is as following:

Observation 7a: A summary of current restriction for handheld UE.

Table 5. Current PC1 feasibility summary for handheld UE for Iot NTN

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| NB-Iot Based | Feasible with sufficient margin | N/A | Feasible |
| half duplex eMTC based | May be feasible, margin may not sufficient (Power Comparable to NR NTN, IL smaller) | N/A | May be feasible, can also be FFS |
| full duplex eMTC based | May not be feasible (Comparable to NR NTN) | Not feasible (Needed as NR NTN) | Not feasible |

For non-handheld UE, theoretically any PC is possible since we all have precedence currently in the spec. However, there may still need discussion on the requirements that needed.

Observation 7b: For non-handheld UE, all the current power classes could be feasible, based on similar assumption with previous discussion in TN LTE.

Based on previous analysis, the following proposals were provided.

Proposal 4: For NB-Iot based Iot NTN, confirm PC1 1Tx is feasible for handheld and non-handheld UE.

Proposal 5a: For half duplex eMTC based Iot NTN, handheld UE, FFS the feasibility of PC1 1Tx.

Proposal 5b: For full duplex eMTC based Iot NTN, handheld UE, confirm PC1 1Tx is NOT feasible.

Proposal 5c: For both full and half duplex eMTC based Iot NTN, non-handheld UE, confirm PC1 1Tx is feasible.

HPUE Impact for Tx Requirements

Duty cycle Related

Note: This section is basically the same to [7] which is for NR NTN.

In TN network, duty cycle related solution is widely used in TDD system, in which some types of power class fall back would be enforced if certain duty cycle would be exceeded. However, for FDD HPUE, no duty cycle solution have been defined, partly because this concept does not quite adaptive with FDD, and partly may also because duty cycle has some its internal problems.

However, for NTN system, since keeping the UE power might be more important compared to TN because of the tight link budget, the duty cycle solution is raised again by several companies. And the WF is as following:

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

In [4], originally only P-MPR is considered for simplicity. After some study, it is felt that the solution similar to TN duty cycle may also be useful.

Although the duty cycle concept is not quite aligned with FDD, the duty cycle method currently defined is actually quite adaptive. The key part actually did not use the wording “duty cycle”, but “the percentage of uplink symbols transmitted in a certain evaluation period” which actually can be adaptable to both FDD/TDD.

Observation 8: The wording of current duty cycle-based SAR solution is quite adaptive and can also be applied to FDD. This may also be useful to keep the power.

Proposal 6: Consider applying the current duty cycle-based SAR solution to Iot based NTN.

Others

For other requirements, most of them would be impacted by ACLR, which would be dependent on UE co-existence study, so the discussion was postponed in this paper.

Conclusion

In this paper, further feasibility analysis based on some measurements were collected, and a number of proposals were made based on last meeting’s progress.

Constraints

Observation 1a: Power amplifier is the main constraint for all the half duplex cases, including NB-Iot based Iot-NTN and half duplex eMTC based Iot-NTN;

Observation 1b: Power amplifier and duplexer are two main constraints for full duplex case, which is eMTC based Iot-NTN.

Proposal 1: Confirm that only full duplex eMTC based Iot-NTN need duplexer in Iot NTN.

Handheld and non-handheld for Iot NTN

Observation 2a: In LTE, NB-Iot and eMTC are not differentiate between handheld and non-handheld.

Observation 2b: Iot NTN may face different scenarios compared to legacy Iot in LTE.

Proposal 2: Iot NTN also can be differentiated between handheld and non-handheld devices.

1Tx vs 2Tx

Observation 3a: 1Tx is one of the most basic assumptions for NB-Iot and eMTC.

Observation 3b: Further extend to 2Tx may bring undue complexities and cost for Iot.

Observation 3c: Iot can achieve higher power even with 1Tx, which makes the 2Tx even less necessary.

Proposal 3: Only 1Tx is considered for Iot NTN, and 2Tx implementation not considered.

PA-1 (Satellite) testing & Analysis

Observation 4: Based on the measurement of a PA designed for handheld UE in satellite communication, PC1 1Tx have quite sufficient margin for NB-Iot based Iot-NTN.

PA-2 (GSM) testing & Analysis

Observation 5: Based on the measurement for a GSM PA, the PC1 1Tx can almost be achieved, although margin may not be sufficient.

Observation 6: The implementation difficulty of PA to satisfy PC1 1Tx for NB-Iot based Iot-NTN, should be comparable to GSM with limited improvements. For PAs specifically designed for satellite communications, margins are considerable.

Feasibility summary

Observation 7a: A summary of current restriction for handheld UE.

Table 5. Current PC1 feasibility summary for handheld UE for Iot NTN

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| NB-Iot Based | Feasible with sufficient margin | N/A | Feasible |
| half duplex eMTC based | May be feasible, margin may not sufficient (Power Comparable to NR NTN, IL smaller) | N/A | May be feasible, can also be FFS |
| full duplex eMTC based | May not be feasible (Comparable to NR NTN) | Not feasible (Needed as NR NTN) | Not feasible |

Observation 7b: For non-handheld UE, all the current power classes could be feasible, based on similar assumption with previous discussion in TN LTE.

Proposal 4: For NB-Iot based Iot NTN, confirm PC1 1Tx is feasible for handheld and non-handheld UE.

Proposal 5a: For half duplex eMTC based Iot NTN, handheld UE, FFS the feasibility of PC1 1Tx.

Proposal 5b: For full duplex eMTC based Iot NTN, handheld UE, confirm PC1 1Tx is NOT feasible.

Proposal 5c: For both full and half duplex eMTC based Iot NTN, non-handheld UE, confirm PC1 1Tx is feasible.

Duty cycle

Note: This section is basically the same to [7] which is for NR NTN.

Observation 8: The wording of current duty cycle-based SAR solution is quite adaptive and can also be applied to FDD. This may also be useful to keep the power.

Proposal 6: Consider applying the current duty cycle-based SAR solution to Iot based NTN.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] https://www.murata.com/products/productdata/8825373098014/DS-SAYRZ2G00BA0E0A.pdf

[6] https://www.murata.com/en-eu/products/productdata/8820841873438/DS-SAYRZ1G54BA0C0A.pdf

[7] R4-2415788, Feasibility study and analysis for NR NTN HPUE Tx Requirements, vivo, RAN4#112bis