3GPP TSG-RAN WG4 Meeting #112bis	R4-2416192

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On the NGSO Satellite Motion Trajectory used in testing

Agenda item:	6.8.4

Document for:	Discussion

Introduction

In RAN #101, RAN plenary has approved a simplification of the test framework for NGSO, amidst concerns of the time to complete the work item in RAN5 by the end of the Release 17. As a consequence, the specification work in RAN4 has concluded that the test cases for NGSO scenarios would be tested against a fixed (non-varying) propagation delay and fixed doppler delay.

As the speed of satellite in NGSO scenarios is a very relevant aspect of the NTN scenario that has not been covered by similar testing before and considering that NTN is a growing and relevant market, RAN has approved the WID RP-241985 [1] (NR_IoT_NTN_req_test_enh) to develop more meaningful and relevant test cases for this feature considering the motion trajectory of the satellite.

The following objectives were established for this WI:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

The topic was initially discussed in RAN4 #112, and the WF was captured in [2].

Discussion

Channel Model and Satellite Motion Trajectory

The enhancement of NGSO testing proposed in this work item aims at improving the testability of NTN UEs, specially those that support NGSO systems, by creating a more meaningful test environment compared to what the UEs will face when deployed in a Network.

This was explicitly conveyed in the work item description as highlighted below:

- Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios

This is an important aspect to be covered by this work item, because in NTN the channel model for varying timing and doppler are not only estimated at the UE side based on received signals. They are actually pre-compensated by the UE based on satellite assistance information conveyed by the network.

This means that for the transmit timing of the UE to match the UL reception time by the test equipment in the test setup, the pre-compensation applied by the UE has to match the time varying component of the channel model.

The pre-compensation performed by the UE has to match the time and doppler variation observed in the TE channel model.

As the UE can only apply pre-compensation based on satellite assistance information, as described in the specifications, then, the TE channel model has to match the satellite motion trajectory emulated by the test equipment.

The UE can only pre compensate as indicated in the satellite assistance information acquired via SIB19.

Hence, it must be a principle that TE channel model and the satellite motion trajectory in the test setup are matching. This was agreed in the previous WF:

Issue 1-2-3 Test procedure/configuration - Agreement During test, ephemeris information shall be updated periodically and aligned with TE-emulated channel model

One important aspect of the time and doppler variation caused by the satellite motion is that two different UEs on the ground, both within coverage of the same satellite footprint are likely to experience different doppler and propagation time conditions, as can be seen in Figure 1 below.

Figure 1 Delay distribution within a 100km x 100km beam directly below a satellite (i = 0° and ψ = 0° in the middle). Delays for points within the beam (top left), as seen by the User Equipment in the middle with ΔtSPREAD = 0.174ms (top right), as seen by the UE shifted by half of the radius with ΔtSPREAD = 0.255ms (bottom left), and as seen by the UE on the border of the beam ΔtSPREAD = 0.333ms (bottom right). Altitude = 600km

Therefore, in order to perform an accurate and fair test, the TE channel conditions might be generated considering the UE emulated position (regardless of whether GNSS emulation is used or AT commands):

The generation of the TE channel model must also match the conditions for the UE emulated location in the test setup.

Choosing the channel model

RAN4 has previously discussed different options for the TE channel model, and this was captured in the previous WF:

Issue 1-2-1 Methodology for Time varying Doppler and Delay shifts modelling - Agreement - RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options: - Option 1: Eckstein-Hechler model - Option 2: Keplerian model - Option 3: Existing model from TR 38.811 5.3.4.3 - Option 4: Proposal 2 from R4-2412552 dht=modvNGSO⋅t, 2dh,max-dh,max Tt=dtc=dh(t)2+hNGSO2c Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2 where - Tt: Propagation delay of the signal from the satellite at time t, - F(t): Doppler (frequency) shift of the signal from the satellite at time t. - dh(t): the horizontal distance between the satellite and UE at time t, - d(t): the distance between the satellite and UE at time t, - dh,max: the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E), - c: the speed of light, - fd: Maximum Doppler shift (Hz). - Option 5: Proposal 2 from R4-2412783 - The instant satellite speed v is given as: v=μR+h - The initial angle β of vector geocentric to UE relative to the x axis is given as: β=π-α-arcsin⁡R∙cosαR+h - The satellite position s is given as: s=j∙R+h - The UE position u is given as: u=R∙ej∙β+v∙tR+h - The Doppler fd is given as: fd=vc∙fc∙realu-su-s - The delay T is given as: T=¦u-s¦c where μ=398600.4418×109m3s2 is standard gravitational parameter, R=6371008.7714m is earth average radius, h=600000m is satellite altitude for LEO-600, α is initial angle of vector UE to satellite relative to vector UE to clockwise side horizon, t is the time, c=299792458ms is the speed of light, fc=2×109Hz is the carrier frequency.. - Other options not precluded Note: Companies are encouraged to bring more information for option 1 and option2. - Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model - FFS on the initial ephemeris information and parameters

In this contribution we will assess the proposed options and indicate which one we believe is the most suitable to reach the objectives of this work item.

Options 1 and 2 do not describe exactly a channel model as they are not intended to provide descriptors for the time and doppler variation observed in the channel. Rather they are models for the motion trajectories that could be adopted by the TE to emulate an orbital trajectory of the satellite (and generate the information to be conveyed in SIB 19).

Options 1 and 2 in the WF are motion trajectory model that could be used to generate the channel model and the satellite assistance information in SIB19 during the tests.

Among the two options above, Eckstein-Hechler model tends to produce the best approximation for an orbital model (See annex).

Adopt the Eckstein-Hechler model as the reference model for obtaining satellite motion trajectory in the test setup.

Option 3 is limited to the doppler calculation for a mobile terminal placed along the orbital plan (where the doppler shift is maximum). The formulation does not describe the satellite movement, but rather relies on knowing the satellite velocity and elevation angle to the UE to derive the doppler variation.  Option 5 is also an alternative option,very similar to that of option 3.

Option 3 might be used in combination with one satellite motion trajectory model to derive the channel model for the doppler variation, assuming the UE to be in the orbital plane. The same is valid for option 5.

For simplicity, and because it is already provided in one TR (38.821), we would prefer the doppler model to be adopted from Option 3.

Once the satellite trajectory model is chosen, the doppler shift of the channel model at any point in time is calculated using the formulation of TR 38.821 (5.3.4.3).

Option 4 cannot be utilized, as its approximations implicitly consider a flat Earth model, as the spherical coordinates are not considered when computing the distance of the UE to the satellite at lower angles. This can be observed by the graphs presented in R4-2412552 show a maximum propagation delay of 4 ms at an elevation angle of 30 degrees, compared to the actual value of 3.6 ms that is actually observed when the orbital motion and the spherical earth model are considered.

Option 4 is not applicable, as the channel model estimator would not correspond to the values produced by UE pre compensation, if the UE applies values to be observed on the field.

Frequency Error

One important part of this WI regards the RF frequency test, as presented in the WID:

Inform RAN5 to assist specifying RF frequency error tests, if needed Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement

For assessing this it is important to evaluate the RF frequency error core requirement and whether it can be met under the current TE model.

In TS 38.101-5 7 the current frequency error core requirements are described as follows:

6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

The highlighted part indicates that the UE is already expected to pre-compenate the UL modulated carrier frequency by the estimated Doppler shift, and that the estimation of the doppler shift is based on the satellite assistance information.

Therefore, as long as the TE channel model matches the time and doppler shift corresponding to the satellite assistance information broadcast in SIB 19 during the tests and the UE emulated position, the core requirements do not need to be altered as they are foresee the UE pre-compensating the doppler shift as expected.

Hence, we propose the following:

Confirm that the existing frequency error requirement can be met under the TE-emulated channel model without intention of modifying the existing core requirement.

Parameters

There are infinite valid orbits around Earth that could be used for testing purposes. If different TEs implement different orbital parameters, it might be difficult to guarantee that all UEs are tested under the same conditions and in certain cases it might get more difficult to specify the test metrics. Therefore, it is necessary to introduce some guidance for the performance configuration.

This was initially discussed in the previous WF as captured below:

Issue 1-2-2 Parameters for TE-emulated channel model - Agreement - RAN4 further discuss the necessary parameters with associated channel model, following parameters list for information purpose: - Satellite altitude: - Option 1: LEO -600km only - Option 2: Both LEO-600km and LEO-1200km Elevation angle range - UE position - Whether to consider a non-zero time-varying feeder link delay - Other parameters not precluded Issue 1-2-3 Test procedure/configuration - Agreement During test, ephemeris information shall be updated periodically and aligned with TE-emulated channel model

For the configuration of the satellite assistance information, it is important to define meaningful configuration.

it is not meaningful for the transmit timing cases that the elevation angle between the UE and the serving cell is set close to 90 degrees for NGSO scenarios, as there will be almost no variation on the UE transmit timing.

For mobility cases, it is not meaningful that the serving cell is configured such that the elevation angle between the satellite and the UE is close to 90 degrees (best propagation case scenario). In special, for the distance-based triggers.

We then have the following set of proposals:

The test setup must include a orbital motion trajectory emulated such that the UE is in the orbital plane.

The satellite heights to be considered in this work item are the same as currently existent in the RAN5 scope of test cases for NTN (600 and 1200 km).

In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE.

  - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator.

  - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test.

Inclusion in Specification

Following the discussion regarding how to define the channel model, one important aspect which should not be overlooked is how this channel model should be incorporated into the current 3GPP NTN specifications. Currently the NTN-TDL channels used within the performance specifications are contained within TR 38.811

Channel Models for NTN are currently contained within TR 38.811

However it is typical that a TR is stable once a release has been completed, and further we note that TR 38.811 is not listed in the effected documentation in the WID for Rel-19.

Currently it is not expected (within the WID) for TR 38.811 to be impacted.

Therefore, it could be considered whether the outcome of this work may have an impact to TR 38.811. Alternatively, the channel model itself could be included in the respective specification where the performance requirements are defined, as is the case for example with High Speed Train channels wit time varying doppler.

The varying doppler profiles could be captured in the appropriate performance specifications.

Thus, this leads to the following proposal.

RAN4 shall discuss how to incorporate the channel model into the specifications once agreed.

Conclusion

The paper has discussed the TE channel model for varying doppler and delay conditions under NGSO scenarios and the following set of observations and proposals were made:

Observation 1: The pre-compensation performed by the UE has to match the time and doppler variation observed in the TE channel model.

Observation 2: The UE can only pre compensate as indicated in the satellite assistance information acquired via SIB19.

Proposal 1: The generation of the TE channel model must also match the conditions for the UE emulated location in the test setup.

Observation 3: Options 1 and 2 in the WF are motion trajectory model that could be used to generate the channel model and the satellite assistance information in SIB19 during the tests.

Proposal 2: Adopt the Eckstein-Hechler model as the reference model for obtaining satellite motion trajectory in the test setup.

Observation 4: Option 3 might be used in combination with one satellite motion trajectory model to derive the channel model for the doppler variation, assuming the UE to be in the orbital plane. The same is valid for option 5.

Proposal 3: Once the satellite trajectory model is chosen, the doppler shift of the channel model at any point in time is calculated using the formulation of TR 38.821 (5.3.4.3).

Observation 5: Option 4 is not applicable, as the channel model estimator would not correspond to the values produced by UE pre compensation, if the UE applies values to be observed on the field.

Proposal 4: Confirm that the existing frequency error requirement can be met under the TE-emulated channel model without intention of modifying the existing core requirement.

Observation 6: it is not meaningful for the transmit timing cases that the elevation angle between the UE and the serving cell is set close to 90 degrees for NGSO scenarios, as there will be almost no variation on the UE transmit timing.

Observation 7: For mobility cases, it is not meaningful that the serving cell is configured such that the elevation angle between the satellite and the UE is close to 90 degrees (best propagation case scenario). In special, for the distance-based triggers.

Proposal 5: The test setup must include a orbital motion trajectory emulated such that the UE is in the orbital plane.

Proposal 6: The satellite heights to be considered in this work item are the same as currently existent in the RAN5 scope of test cases for NTN (600 and 1200 km).

Proposal 7: In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE.

a.	This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator.

b.	The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test.

Observation 8: Channel Models for NTN are currently contained within TR 38.811

Observation 9: Currently it is not expected (within the WID) for TR 38.811 to be impacted.

Observation 10: The varying doppler profiles could be captured in the appropriate performance specifications.

Proposal 8: RAN4 shall discuss how to incorporate the channel model into the specifications once agreed.

- References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105, Melbourne, Australia, September 9-12, 2024+

- R4-2413516, " Way Forward for [112][329] NTN_testing_NGSO_channel_model ", Samsung, RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- TS 38.1015-5 “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”, Release 18.7.0, 3GPP

- R1-2102360, “Considerations on UL timing and frequency synchronization in NTN”, Thales, RAN1 #104, e-Meeting, April 12th – April 20th, 2021

- Annex

The reference propagator must be such that the DP does not cause the reference TA to deviate significantly from the UL timing expected by the gNB during the reception of UL signals. It is fair to assume that the best way to measure that is if the “reference measurement” is as close as the gNB timing as possible. The dilution of precision was investigated during the SI phase and investigated in [4]  from which the table below is extracted:

|  |  |  | Maximum errors |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Without fitting | Model | Duration of prediction (minute) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
|  | Kepler | 3 | 261 | 1.74 | 2.64 | 17.60 |
|  | Eckstein Hechler (J6) | 21 | 303 | 2.02 | 0.48 | 3.20 |
|  | 2x2 num | 22,5 | 301.4 | 2.01 | 0.42 | 2.80 |
|  | 6x6 num | 27 | 302.5 | 2.02 | 0.44 | 2.93 |
|  |  |  | Maximum errors |  |  |  |
|  | Model | Duration of prediction (second) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
| Without fitting | Kepler | 45 | 43.4 | 0.29 | 0.689 | 45.93 |
|  | Eckstein Hechler (J6) | 300 | 43.4 | 0.29 | 0.093 | 6.20 |
|  | 2x2 num | 300 | 44.3 | 0.30 | 0.1 | 6.67 |
|  | 6x6 num | 350 | 44.2 | 0.29 | 0.084 | 5.60 |
| With fitting | TLE | 370 | 44,1 | 0.29 | 0.1 | 6.67 |
|  | Eckstein Hechler (J6) | 390 | 44,2 | 0.29 | 0.083 | 5.53 |
|  | 2x2 num | 375 | 44 | 0.29 | 0.092 | 6.13 |
|  | 6x6 num | 395 | 44,2 | 0.29 | 0.079 | 5.27 |

Based on this reference, we prefer to adopt a precise model, such as the Eckstein-Hechler as the reference model. Presuming the epoch time represents the most up-to-date information broadcasted by the network, at epoch time the reference satellite assistance information is that broadcasted in SIB19. The interval between epoch times is at most 10.24 s, according to the current definition provided in 3GPP TS38.331.