TSG-RAN Working Group 4 (Radio) meeting #112-bis	R4-2416267

Hefei, China, October 14-18, 2024

Source:	Ericsson

Title:	Conformance test aspects for the requirement on OTA spatial emission above the horizon for BS operating in band n104

Agenda item:	6.11.4

Document for:	Approval

# Introduction

In this contribution we present information relevant for BS RF conformance test specification TS 38.141-2. Companion contributions are also provided, a discussion paper on core aspects in [1], draft CR with specification text for TS 38.104 in [2] and draft CR for TS 38.141-2 in [3].

Note: This is a continuation of the discussion in [4].

In the following clauses we provide our position on the open issues outlined in the way forward approved during RAN4 #112 in [5]:

Issue 2-2: CAR related issue

Issue 4-3: Measurement grid over elevation angle and azimuth angle

Issue 4-4: RF channels

Issue 4-6: Test procedures

Issue 4-7: Measurement uncertainty and Confidence Interval

# Discussion

  - Conformance test aspects

    - BS RF test approach

For AAS BS conformance testing the concept including a set of test beams was proposed by ITU-R, where the test beams are supposed to be spread out within the intended coverage range.

Observation 1: When the test requirement and test procedure are defined in RAN4, usage of the term “horizon” should be avoided. For a BS. the relation to horizon depends on deployment parameters such as mechanical tilt and other site-specific details not part of the BS RF conformance testing specification.

During the test the AAS base station is configured with a test baseband functionality that will use the test models and apply beam forming weight vector, as shown in Figure 2.2.1-1. With the beamforming weight vector, test beams will be generated according to requirement definitions in TS 38.141-2.

The test is conducted in a Line-of-Sight (LOS) environment without feedback from the measurement receiver (UE). Unlike the UE conformance testing, the BS Down-link and Up-link are tested separately with no dependencies.

# Figure 2.2.1-1: Down-link test object configuration

The base station manufacturer shall declare the range of angles intended for directing the narrow beam under normal operation (also referred as CAR – Coverage Angular Range). This declaration will depend on intended deployment scenario. This would mean that if a specific base station implementation is intended for multiple deployment scenarios, (e.g., rural, sub-urban and urban), multiple declarations are needed.

The concept of static test beams allows for conformance testing in conventional antenna test ranges using standard test equipment (network analyser, signal generator, spectrum analyser, etc.).

Currently, the specification does not describe how beam-forming weights associated to test beams are applied to the test signal (test model) during test. Based on declared beam-peak direction, beam-forming weights are derived and applied to the test signal.

Observation 2: With the introduction of OTA spatial emission requirement RAN4 may need to consider how beamforming weights should be applied to the test model to guarantee a relevant test condition.

    - Test Concept

The conformance test required to show compliance to the EEIRP limit needs to be developed by RAN4. To show conformance to the EEIRP limits a large number of measured 3-D EIRP patterns for different array excitations will be needed. Since the RAN4 conformance testing is built around pre-defined test signals (test models and fixed refence channels) normal operation in fading environment capturing true layer-1 operation cannot be captured by the conformance test in RAN4 for AAS base stations.

The conformance test for EEIRP limits can be created around the concept of using test beams within a declared Coverage Angular Range (CAR). The CAR is the angular range along vertical and horizontal axis for which the base station is intended to provide coverage within.  Additionally, it has also been agreed in WF [5] any supported mechanical tilt for CAR/beam steering direction sets for BS conformance testing to be declared.

In the terminology section (clause 3 of TS 38.141-2), a definition of CAR is needed. For the purpose of EEIRP requirement the CAR can be defined as:

Coverage Angular Range: The coverage angular range is a set of directions describing the intended angular operation range for the test beams.

For each test beam, the 3-D EIRP pattern in the upper hemisphere is measured. Based on measured EIRP patterns the EEIRP profile can be calculated as the output from a multistage averaging process.

Multiple test beams with different directions are required to establish the average EIRP pattern corresponding to intended operation. For this purpose, K different test beams with different beam directions, distributed within the declared CAR for which the base station is intended to operate within, are required.

The intention with the test beams is to excite the array antenna in such a way that the array side-lobe suppression characteristics are captured.

For an array antenna with sparse element separation in the vertical domain using sub-arrays, side lobe response and eventual grating lobe response will be captured.

Multiple CARs can be declared for a base station supporting multiple deployment scenarios to map towards different scenarios with different UE distributions. Eventually, for each declared CAR, different beam power profiles are utilized to optimize coverage while meeting the EEIRP profile above the horizon.

For each test beam, the BS manufacturer declares the peak EIRP and direction as q and j angle pair.

For a solid test requirement, the details related to placement of the beam directions and weighting need to be captured in the specification. The location and distribution of beam directions will affect the averaging process significantly. RAN4 needs to define how many beam directions will be needed and how to distribute them.

Observation 3: Since the conformance test concept adopted in RAN4 currently does not capture operation in fading environment, manufacturer declarations will be needed to derive the angular coverage range. This means that the declaration and deployment scenarios are tied together.

The CAR is declared by the base station manufacturer using the angles listed in Table 2.2.2-1.

Table 2.2.2-1: CAR declarations

| Parameter | Description |
| --- | --- |
| qmin | Minimum angle along q axis |
| qmax | Maximum angle along q axis |
| jmin | Minimum angle along j axis |
| jmax | Maximum angle along j axis |

The CAR can be visualized as a rectangular area in the q/j plane, as shown in Figure 2.2.2-1 as a green rectangle.

For a typical wide area base station intended for 3-sector rural deployments values listed in Table 2.2.2-2 is reasonable.

Table 2.2.2-2: Typical values

| Parameter | Values (degrees) |
| --- | --- |
| qmin | 0 |
| qmax | 12 |
| jmin | -60 |
| jmax | 60 |

For other deployment scenarios (e.g., for wide area base station operating in an urban deployment scenario) other values may be declared.

Figure 2.2.2-1: Coverage angular range

The test beams will be located within the declared CAR. The number of test beams K and the location within the q/j plane will impact the measured EEIRP profile. In the background information relevant for the condition ITU-R stipulates that the beam directions should be distributed uniformly within the CAR. To adjust for beams located at the edge of the CAR a beam weight has been introduced.

The average EIRP pattern is calculated from K EIRP patterns as:

EIRPθ,φ=k=1KwkEIRPkθ,φ

, where wk is a beam weight and EIRPkθ,φ is the EIRP pattern per measured test beam.

Beam weight: fraction of the coverage angular range represented by the test beam.

For example, wk=1/K   represents the case where K uniformly and equally spaced test beams are assumed in azimuth and elevation, respectively, and where each beam covers an equal range of angles (e.g., equal area in q/j plane). The weight is required to account for test beam coverage for test beams located on the edge of the CAR, non-uniform distribution or allowing for fewer test beams to be considered as shown in the following examples.

Eventually, the maximum peak EIRP difference between declared beam needs to be limited to a certain value (e.g., 10 dB). Without a cap on the maximum difference, fake beams can be declared during testing that will produce incorrect measured EEIRP levels.

Observation 4: RAN4 need to define test beam set(s) that guarantees acceptable measurement uncertainty.

Proposal 1: RAN4 should define a concept where the test beam directions are related to the declared Coverage Angular Range (CAR) and thereby the number of test beam directions are specified.  The BS manufacturer declares CAR for different deployment situations (e.g. sectorization (3 sector or 6 sector or other), vertical coverage and different mechanical tilt settings). This means that the BS manufacturer can declare one or several CARs for which the OTA spatial emission requirement should be meet.

Based on the declaration of CAR parameters (qmin, qmax, jmin, jmax) RAN4 can establish a concept on where the test beams are located within the CAR. With this approach the number of used test beams and the distribution can be captured in the specification.

    - Conformance test beam set

The beams selected for conformance testing will have direct impact on the robustness of measuring the emission generated above the horizon. To be able to measure the EEIRP, the test object needs to be stimulated to generate beams within the intended coverage angular range. For this purpose, RAN4 need to establish a set of test beams to be used during conformance testing.

In this section we evaluate the EEIRP profiles for different test beam sets for a given array antenna geometry representative of a product in the band and described in Table 2.2.3-1. It should be noticed that these parameters are not selected with the intention to meet the EEIRP requirement profile. As the result will show, the specified EEIRP levels will not meet the ITU-R condition, which would indicate that additional mitigation techniques are needed. The intention with the parameters below is to evaluate the EEIRP characteristics due to different test beam sets.

Table 2.2.3-1: Antenna array assumptions

| Parameter | Value |
| --- | --- |
| Frequency | 6775 MHz |
| Element model | Vertical HPBW: 90° Horizontal HPBW: 90° Sidelobe suppression: 30dB Element directivity: 6.4 dBi |
| Array configuration | Number of rows: 8 Number of columns: 16 Sub-array: 3x1 Sub-array tilt: 6° Horizontal element separation: 0.5λ Vertical element separation: 0.7λ |
| TRP | 27 dBm/MHz |
| Peak EIRP target | 79 dBm/100 MHz |

To evaluate the impact of the number of beams, distribution and beam weights, several beam sets have been defined (Reference, A, B, C, D, E), considering different sizes of the test beam sets, distribution of test beams in the declared CAR, and weights. The CAR is set to -60 to 60 degrees in the horizontal plane and to 0 to 12 degrees (down-tilt) in the vertical plane.

For reference purposes a reference beam set of 325 uniformly distributed beams (25 x 13 equally spaced beams in horizontal and vertical plane, respectively) with uniform beam weight has been defined. This beam set is referred to as “Reference” in figures.

Uniform distribution

We first consider a uniform distribution of beams in the declared CAR. Two cases have been considered, Case A with different beam weights considering beams at the edge of the CAR and Case B, where non weighted average is considered.

Specifically, Beam set A and B are defined by 25 test beams distributed in a uniform lattice with 5 beams along the horizonal axis and 5 beams along the vertical axis, according to Figure 2.2.3-1.

Figure 2.2.3-1: Beam distribution for Beam set cases A and B

For beam set A, the beam pointing direction and corresponding beam weight for all beams are listed in Table 2.2.3-2. The beam directions are weighted to reflect the area that each beam covers within the CAR box.

Table 2.2.3-2: Beam weights for Beam set A

| Test Beam [k] | θ° | φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/32 |
| 3 | 6 | 60 | 1/32 |
| 4 | 6 | -30 | 1/16 |
| 5 | 6 | 30 | 1/16 |
| 6 | 3 | 0 | 1/16 |
| 7 | 9 | 0 | 1/16 |
| 8 | 0 | 0 | 1/32 |
| 9 | 12 | 0 | 1/32 |
| 10 | 0 | -60 | 1/64 |
| 11 | 0 | -30 | 1/32 |
| 12 | 3 | -60 | 1/32 |
| 13 | 3 | -30 | 1/16 |
| 14 | 0 | 30 | 1/32 |
| 15 | 0 | 60 | 1/64 |
| 16 | 3 | 30 | 1/16 |
| 17 | 3 | 60 | 1/32 |
| 18 | 9 | -60 | 1/32 |
| 19 | 9 | -30 | 1/16 |
| 20 | 12 | -60 | 1/64 |
| 21 | 12 | -30 | 1/32 |
| 22 | 9 | 30 | 1/16 |
| 23 | 9 | 60 | 1/32 |
| 24 | 12 | 30 | 1/32 |
| 25 | 12 | 60 | 1/64 |

For Beam set B, uniform beam weighting is assumed. The reason for this is to compare the impact of applying additional beam weighting to the resulted EEIRP profile. The beam directions and beam weight vector are listed in Table 2.2.3-3.

Table 2.2.3-3: Beam weights for Beam set I.B

| Test Beam [k] | θ° | φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/25 |
| 2 | 6 | -60 | 1/25 |
| 3 | 6 | 60 | 1/25 |
| 4 | 6 | -30 | 1/25 |
| 5 | 6 | 30 | 1/25 |
| 6 | 3 | 0 | 1/25 |
| 7 | 9 | 0 | 1/25 |
| 8 | 0 | 0 | 1/25 |
| 9 | 12 | 0 | 1/25 |
| 10 | 0 | -60 | 1/25 |
| 11 | 0 | -30 | 1/25 |
| 12 | 3 | -60 | 1/25 |
| 13 | 3 | -30 | 1/25 |
| 14 | 0 | 30 | 1/25 |
| 15 | 0 | 60 | 1/25 |
| 16 | 3 | 30 | 1/25 |
| 17 | 3 | 60 | 1/25 |
| 18 | 9 | -60 | 1/25 |
| 19 | 9 | -30 | 1/25 |
| 20 | 12 | -60 | 1/25 |
| 21 | 12 | -30 | 1/25 |
| 22 | 9 | 30 | 1/25 |
| 23 | 9 | 60 | 1/25 |
| 24 | 12 | 30 | 1/25 |
| 25 | 12 | 60 | 1/25 |

Reduced number of test beams

The impact on the EEIRP profile for the case where test beam directions are reduced have been evaluated by defining test beam directions according to Figure 2.2.3-2. Now the test beam directions are aligned with the q-axis and j-axis.

Figure 2.2.3-2: Test beam set II, to reduce number of test beam directions

For this beam set, three different variants of beam weight vectors can be evaluated. Therefore, three different beam sets have been defined C, D and E. The beam directions and beam weight vectors are listed in Table 2.2.3-4, Table 2.2.3-5 and Table 2.2.3-6.

Table 2.2.3-4: Beam weights for beam set C

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/32 |
| 3 | 6 | 60 | 1/32 |
| 4 | 6 | -30 | 1/16 |
| 5 | 6 | 30 | 1/16 |
| 6 | 3 | 0 | 1/4 |
| 7 | 9 | 0 | 1/4 |
| 8 | 0 | 0 | 1/8 |
| 9 | 12 | 0 | 1/8 |

Table 2.2.3-5: Beam weights for beam set D (uniform)

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/9 |
| 2 | 6 | -60 | 1/9 |
| 3 | 6 | 60 | 1/9 |
| 4 | 6 | -30 | 1/9 |
| 5 | 6 | 30 | 1/9 |
| 6 | 3 | 0 | 1/9 |
| 7 | 9 | 0 | 1/9 |
| 8 | 0 | 0 | 1/9 |
| 9 | 12 | 0 | 1/9 |

Table 2.2.3-6: Beam weights for beam set E

| Test Beam [k] | Direction θ° | Direction φ° | Beam weights [wk] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 2 | 6 | -60 | 1/8 |
| 3 | 6 | 60 | 1/8 |
| 4 | 6 | -30 | 1/4 |
| 5 | 6 | 30 | 1/4 |
| 6 | 3 | 0 | 1/16 |
| 7 | 9 | 0 | 1/16 |
| 8 | 0 | 0 | 1/32 |
| 9 | 12 | 0 | 1/32 |

Another alternative considers that the beam weighting is set to adjust for beam directions located on the edge of the CAR and to adjust for vertical angles having only one beam for all horizontal angles. In theory, this beam configuration may allow for an alternative, but incorrect weight values (Case E) as shown in Table 2.2.3-6. With the alternative beam weight , beams with horizontal steering are applied with a significant suppression. Hence the impact of using sub-arrays in the vertical domain will be suppressed, since beam causing grating lobes will be suppressed in the averaging process.

Test beam distribution evaluation

The number of test beams, the distribution within the declared CAR and the weighting, will impact the measured EEIRP level. Therefore, the EEIRP produced by a base station assuming a baseline antenna configuration, already introduced in Table 2.2.3-1, have been evaluated.

In the evaluation 5 different test beam configurations have been evaluated, as listed in Table 2.2.3-7, and compared to the reference beam set and the WRC-23 requirement.

Table 2.2.3-7: Test beam configurations

| Case | Test beam distribution | Beam weight | Detailed description |
| --- | --- | --- | --- |
| A | Uniform | Non-uniform | Table 2.2.3-1 |
| B | Uniform | Uniform | Table 2.2.3-2 |
| C | Non-uniform | Non-uniform | Table 2.2.3-3 |
| D | Non-uniform | Uniform | Table 2.2.3-4 |
| E | Non-uniform | Incorrect Non-uniform | Table 2.2.3-5 |

The EEIRP profiles corresponding to Beam set A, B, C, D and E are plotted in Figure 2.2.3-3.

Figure 2.2.3-3: EEIRP profile for Cases A, B, C, D and E (sub-array size 3x1)

It can be observed that the EEIRP profiles align well with the reference case for beam sets A and B, and quickly converge to it. On the other hand, for the reduced set of beams, beam sets cases C, D and E, deviate from the reference case and from beam sets A and B. As a result, we consider that a test beam-set of 25 beams can represent a good trad-off between accuracy and testing complexity. It can also be noticed that the WRC-23 level is violated for some angular ranges.

It can be noticed that case E significantly deviates from the reference case, which indicates that case E should not be used for conformance testing.

These results, which were already discussed in previous meeting, demonstrate that the test beams should be carefully selected and should be distributed within the whole coverage angular range, not only along the axis. To guarantee repeatability and acceptable measurement uncertainty, RAN4 need to define one common test beam set, taking into account the trade-off between accuracy and testability.

Another aspect to consider in relation to test beam sets is the impact on the EEIRP profile depending on considered array antenna structure, more specifically the size of the sub-array. In what follows we repeat the experiments considering a bigger sub-array size. Specifically, we consider sub-array size of 6x1 and compare it to the previously studied case of sub-array size 3x1.

From the reference beam set, with 325 (25 x 13 equally spaced beams in horizontal and vertical plane, respectively) uniformly distributed beams with uniform beam weights, we already observe a slightly different behaviour as a function of the sub-array size in Figure 2.2.3-4. Specifically, with larger sub-array size the EEIRP exceeds slightly more the requirement, compared to sub-array size 3x1, in certain directions.

Figure 2.2.3-4: EEIRP profile for the reference beam set with 3x1 and 6x1 sub-array size

Similar trend is observed also for other beam sets as described in Figure 2.2.3-5, Figure 2.2.3-6 and Figure 2.2.3-7. It can also be noticed that the same observations done for the sub-array size 3x1 are valid for sub-array size 6x1, in terms of better accuracy and convergence to the reference when considering beam set A and beam set B, with respect to the other cases studied for beam-set C, D and E.

Summary of EEIRP profiles for cases A, B, C, D and E, for sub-array size 6x1 is shown in Figure 2.2.3-8.

Figure 2.2.3-5: EEIRP profile for the beam sets A and B, with 3x1 and 6x1 sub-array size

Figure 2.2.3-6: EEIRP profile for the beam sets C and D, with 3x1 and 6x1 sub-array size

Figure 2.2.3-7: EEIRP profile for the beam set E, with 3x1 and 6x1 sub-array size

Figure 2.2.3-8: EEIRP profile for Cases A, B, C, D and E (sub-array size 6x1)

Observation 5: It can be concluded that the test beams should be carefully selected and should be distributed within the whole coverage angular range, not only along the axis. To guarantee repeatability and acceptable measurement uncertainty, RAN4 need to define one common test beam set.

Observation 6: The impact of the beam weight has significant impact on the measured EEIRP profile.

Observation 7: The trend observed in simulation results, in terms of better accuracy provided by beam sets with sufficient number of sample beams and good distribution, are confirmed also when considering different sub-array sizes.

Proposal 2: RAN4 need to develop a concept where the test beams are distributed within the whole coverage angular, range with a specified beam weight vector.

Observation 8: Comparing the EEIRP result from Beam set A and Beam set B it can be noticed that they produce similar EEIRP profiles, so that case B might be preferred to case A since a simple uniform beamforming weight is used.

Is seems that beam set B provides enough samples for the average to converge and an easy approach to describe how test beams are located within the CAR. From a specification drafting principle it would be good to define beam set B in the conformance test specification including description on how the beam are located within the CAR. For that purpose, equations are required.

Proposal 3: Based on the analysis in this contribution we propose to specify the minimum number if beams used within the CAR to 25 uniformly distributed with wk equal to 1/K and location on test beams in the BS RF conformance specification (referred in this contribution to as beam set B).

Proposal 4: RAN4 to specify 25 test beam directions, uniformly distributed directions within CAR. This can be done by capturing equations for each test beam with input parameters from declared CAR (qmin, qmax, jmin, jmax). The values listed in Table 2.2.3-3 can be expressed in equations for a more general approach.

    - Description of test method in specification

The information in this section is captured in TS 38.141-2 for each requirement. It gives detailed information about test object configuration and test scope.

      - Test scope

The number of EIRP measurement points required for one frequency, carrier bandwidth configuration, carrier configuration can be expressed as KNM samples. For K=25 (test beams), N=90 (Vertical samples, 1 deg. resolution) and M=360 (Horizontal samples, 1 deg. resolution), the total number of EIRP samples required will be 810000. Therefore, careful considerations of the test scope (RF channels to be tested, carrier bandwidths, carrier configurations, etc.) are required for a reasonable over all test time and complexity.

The impact on EEIRP profile due measured RF channel can be evaluated by studying Bottom (B), Middle (M) and Top (T) RF channels within band n104 considering different array configurations (28 cases considered with sub-array sizes of 3 up to 6 elements vertically spanning from 1.35l to 4.5l).

The simulation results are listed in Table 2.1.4.1-1:

Table 2.1.4.1-1: Simulation results for Bottom, Middle, Top RF channel

| RF channel | EEIRP limit violations (%) |
| --- | --- |
| Bottom | 53 |
| Middle | 19 |
| Top | 28 |

According to the analysis based on antenna models it is observed that it is the Bottom RF channel within the n104 band fails the limits of the OTA spatial emission requirement before other RF channels. This is most likely due to the wider coverage that is provided by an antenna unit-port/sub-array both with the main-lobe but also with the sidelobes above the horizon.

Results indicate that it is most likely that the OTA spatial emission requirement limits will be first met by the Bottom RF channel emissions. Also, with the highest deviation compared to other frequency points, meaning that there is no higher risk measuring the bottom frequency compared to the rest.

What matters for the requirement is the assigned Power Spectral Density (PSD) level. Hence it is important that the measurements are done considering the highest supported PSD in a product (This information could be extracted from the declarations of the supported carrier bandwidths and maximum supported power for a product).

The results indicate the number of RF channels to be considered for conformance testing can be reduced to two RF channels, if Bottom RF channel and Top RF channel are considered.

For the OTA spatial emission above horizon requirement the carrier bandwidth configuration does not really affect the emission upwards. Instead, what matter is the BS capability of generating power spectral density. Therefore, the number of tested carrier bandwidth configurations can be reduced to the widest supported carrier bandwidth with maximum PSD.

Proposal 5: It is sufficient to test OTA spatial emission above horizon requirement  for single carrier configuration.

Proposal 6: It is sufficient to test two RF channels (Bottom and Top RF channel).

Proposal 7: It is sufficient to test widest carrier bandwidth that generate maximum supported PSD.

      - Draft initial conditions and procedure

In this section information related to initial conditions, test object configuration and test object placement is captured. To prepare for specification drafting of “initial condition” and “procedure” sections in TR 38.141-2 for OTA spatial emission above horizon requirements, the structure and format from TS 38.141-2 is used as baseline.

Draft specification text for “initial conditions” and “procedure”.

x.y.4.1	Initial conditions

The test environment: Normal condition, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: B and T; see TS 38.141-2, subclause 4.9.1 using widest supported carrier bandwidth capable of maximum supported Power Spectral Density (PSD) level.

Beams to be tested: For a declared CAR test specified test beams, see TS 38.141-2, Annex X.

The test object is located on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is capturing power transmitted towards the sky, it is essential that the BS is mounted on the positioner such that blocking effects due to the positioner are minimized.

NOTE: 	For regular antenna pattern measurements typically, the focus is on the horizontal angle and then we often see blocking effects in along vertical angle.

For this requirement the same test model as for radiated transmit power in TS 38.141-2, subclause 6.2 should be used.

x.y.4.2	Procedure

The following high level test procedure describes essential steps required to measure the EEIRP profile.

- Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3.

- Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

- Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1.. N.

- Enable test beam  k for k=1..K.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 and 7 for all K test beams.

- Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

- Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TR 38.141-2, Annex Y.

Observation 9: Since the OTA spatial emission above horizon requirement is based on an absolute power level EEIRP mask. The calibration of the test range will be essential to guarantee an acceptable measurement uncertainty.

Observation 10: A test method for EEIRP profile would require EIRP samples to be measured for many test beam directions. Therefore, it is essential to capture a description in the conformance test specification of how EIRP samples are collected for different test beam directions.

Proposal 8: Use draft specification text for “initial conditions” and “procedure” as baseline for TS 38.141-2 draft CR. RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission.

    - EIRP pattern measurement

Traditionally, the measurement can be divided into two different stages: Test range calibration and measurement. For an expected EIRP measurement a third stage is introduced.

- Test range calibration

- EIRP pattern measurement

- Post processing

The post-processing stage is required for some test methods, e.g., TRP test methods which are based on spatial EIRP samples. For EEIRP, the post processing stage will be even more complex which would lead to that a new Annex would be required in the conformance test specification for the technical background information.

      - Test range calibration

In the calibration stage the antenna test range transmission loss is characterized with the intention to minimize error sources related to test setup. A common approach used is the substitution method, where a reference antenna with known antenna gain is used to determine the transmission loss in the test range. The test range calibration is described in detail in TR 37.941, subclause 8.3.

In addition, the measurement uncertainty can be improved by calibrating the absolute power measurement uncertainty of the measurement receiver (spectrum analyser with fairly high absolute power measurement uncertainty) using a power meter (with very low measurement uncertainty). A detailed description of suitable test ranges and corresponding calibration procedures can be found in TR 37.941, subclause 8.3.1.

Proposal 9: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation.

      - EIRP pattern measurement

For each test beam, the EIRP pattern above the horizon (in the upper hemi-sphere) is measured for horizontal angles within the range -π≤φ≤π and elevation angles within the range 0≤θ≤π2 .

The discrete EIRP pattern EIRPkθn,φm measured per test beam using a specific sampling grid with a specific sampling grid resolution. For the reference test method described in this document a uniform sampling grid is considered. Considering practical measurement aspects, such as test complexity and test time the EIRP pattern per test beam is measured for a set of discrete spatial angles θn,φm, where m=1..M and n=1..N.

The EIRP pattern measurement stage and corresponding details on maximum allowed sampling resolution needs to be documented in an Annex in TS 38.141-2.

Observation 11: For each test beam within the CAR a spatially discrete EIRP pattern is measured in the upper hemi-sphere.

Proposal 10: RAN4 need to describe the EIRP sampling grid in the upper hemi-sphere and define criteria for maximum allowed sampling resolution.

      - Post processing

In technical background information from ITU-R [2] the EEIRP profile is calculated from an integration over continues average EIRP patten. From a testing perspective, the averaging process should be described as an average over discrete spatial samples.

The EEIRP profile can be extracted from measured EIRP spectral density radiation patterns described in section 2.2.5.2 by applying multiple stage of averaging processes.

In the first stage the average EIRP radiation pattern over K test beams are calculated as:

EIRPθn,φm=k=1KwkEIRPkθn,φm		Eq. 2.1.5.3-1

, where EIRPkθn,θm  is the EIRP spectral density radiation pattern measured for the k-th test beam and wk refers to the weight for the k-th beamforming direction, i.e., the fraction of the steering range represented by the k-th beamforming direction (see detailed description in section 2.3.2).

In the second average stage the EEIRP level for the i-th elevation range is calculated as:

EEIRPi=θHi-θLisinθHi-sinθLiMN∙n=nLinHim=1MEIRPθn,φmcosθn		Eq. 2.1.5.3-2

where N  is the number of sampling points within the θLi and θHibounding range over upper hemisphere in the elevation range, nLiand nHi are the lowest and highest elevation sampling angles within the  (θLi, θHi) bounding range, with  N= nHi-nLi ; M is the number of sampling points in the azimuth range.

The post-processing stage with details description of the averaging calculations should be captured in a dedicated Annex in TS 38.141-2.

In TS 38.141-2 clause 4.14 there is a definition of reference coordinate system used for the present requirements. For this requirement, RAN4 may need to define the elevation angle according to conventions used in ITU-R.

Proposal 11: RAN4 need to describe the post-processing calculation in discrete form in the conformance test specification.

    - Measurement uncertainty evaluation

Unlike previous OTA requirement the new requirement for protecting FSS UL is constructed as a vertical profile with different absolute Power Spectral Density (PSD) maximum levels for a set of vertical ranges (often referred to as vertical bins). This means that the requirement unlike all previous requirements relies on information over a defined vertical range with different pass/fail absolute levels. This aspect requires careful consideration in the Measurement Uncertainty (MU) evaluation part of the general description of BS RF OTA test method.

At the first glance the requirement has some similarities with OTA test methods described in TS 38.141-2, Annex I and MU evaluation captured in TR 37.941. However, the details and relevant uncertainties to be considered differ. Therefore, it is not appropriate to adopt uncertainties from TRP test methods in an Anechoic Chamber (AC) without careful analysis.

For the OTA spatial emission above horizon requirement RAN4 needs to evaluate the Measurement Uncertainty (MU) for at least one test method. It would be reasonable to start the work after the test method is completely described in a draft CR to TS 38.141-2.

The conformance testing framework adopted by RAN4 for BS RF OTA requirement is described in TR 37.941, subclause 5.1. The framework is summarized considering EEIRP as:

- Multiple test methods may exist for each requirement. For EEIRP it would be appropriate to follow guidance on test method indicated by condition defined by WRC-23. Later, other test methods can also be considered.

- Each test method will require its own test procedure.

- A single conformance requirement applies for each core requirement, regardless of test procedure.

- Common maximum accepted test system uncertainty applies for all test methods addressing the same test requirement. Test methods producing significantly worse uncertainty than others at comparable cost should not impact the common maximum accepted test system uncertainty assessment.

- Common Test Tolerances (TT) apply for all test methods addressing the same test requirement. For EEIRP the requirement is defined by regulation, hence the TT is set to zero.

- A common way of establishing the uncertainty result from all test methods' individual budgets is established.

- A common method of making an uncertainty budget (not a common uncertainty budget) is established.

- Establish budget format examples for each addressed test method in the form of lists of uncertainty contributions. Contributions that may be negligible with some BS and substantial with others should be in this list. For each combination of measurement method and test parameter, develop a list with measurement uncertainties.

- Describe potential OTA test methods relevant for testing radiated requirements (e.g. directional, TRP, or co-location requirements). The description requires information about the applicable test range architectures and test procedures. Addressing each item in each uncertainty budget with respect to the expected distribution of the errors, the mechanism creating the error and how it interacts with properties of the BS. The EEIRP requirement is a new type of requirement based on a spatial requirement pass/fails mask.

- Providing example uncertainty budgets will be useful in order to demonstrate the way a budget should be defined and how calculating its resulting measurement uncertainty is done, but the figures used in the examples will clearly be only examples and not applicable in general.

- Each test instance may require an individual uncertainty budget applicable for the combination of the test facility, the BS and the test procedure and property tested. Here, the tester demonstrates that the uncertainty requirement is fulfilled during the conformance testing.

Further details on how the uncertainty calculation is done can be found in TR 37.941, subclause 5.2.

Observation 12: For EEIRP test method development it is essential to continue the direction set out in the outcome from WRC-23. Based on WRC-23 guidance define a test method and corresponding MU evaluation. At a later stage, if needed document additional test methods.

Observation 13: Since the EEIRP requirement cannot be categorized as Directional requirement, TRP requirement or Co-location requirement. It would be appropriate to define a new separate category for EEIRP based requirements.

Observation 14: The OTA spatial emission requirement is a regulatory requirement, the TT=0 which would result in that eventual MU needs to be accounted for in the design as power back-off or other mitigation.

For the OTA spatial emission requirement above horizon based on EEIRP following technical test challenges should be considered in the MU evaluation:

- The MU impact due to limited measurement receiver dynamic range. Since the EEIRP requirement profile is based on multiple vertical bins with different absolute power level the impact on OTA test range dynamic range should be studied.

- The MU impact due to limited number of used test beam directions. It has been concluded that the expected MU will depend on the number of considered test beams and where they are located in the CAR.

- The MU impact due to selected distribution of test beam directions and usage of the beam weight. If a concept with declarable beam weights is adopted the final MU will be declarable. This approach will not be accepted by regulation.

- The MU impact due to the selected EIRP patten spatial sampling resolution. The average calculation requires a certain number of samples to guarantee small variations. RAN4 need to define required maximum allowed sampling resolution.

- The MU impact due to number of power samples measured by the measurement receiver per test beam.

- The MU impact due to the absolute power measurement uncertainty determined by the calibration process.

- The MU impact due to positioner blocking effects in the vertical domain.

Proposal 12: Before starting work with MU evaluation, first focus on BS RF core draft specification text and BS RF conformance specification text. When draft CRs for TS 38.104 and TS 38.141-2 are complete then focus on measurement method MU evaluation in TR 37.941.

    - EIRP pattern grid resolution

Considering the requirement for OTA spatial emission above the horizon there are two aspects to consider in relation on how to define the EIRP pattern spatial resolution required to guarantee accurate measurement of EEIRP:

- The sampling resolution required to capture the radiation above the horizon. For this sufficient number of spatial samplers are needed to sufficiently capture emission generated in the upper hemisphere including side lobes and grating lobes.

- The size of the vertical bins varies over the elevation range from 5 degrees at low elevations to 30 degrees at high elevation angles. This means that the number of vertical samples with is limited by the bin size.

The first aspect can be resolved by taking enough spatial samples required the spatial sampling Nyquist criteria formulated as:

Δθ<λD180π

, where D is the diameter of a sphere enclosing the radiating source in m (in other words the antenna aperture of the base station) and l is the wavelength in m. The diameter D is calculated as:

D=w2+h2+d2

, where w is the radiating aperture width equal to Ndh and h is the radiating aperture height equal to MsubMdv and d is the depth of the BS enclosure (here assumed to be 0.1 m).

To investigate the required sampling resolution, different antenna geometries have been considered. In Table 2.1.7-1, the required sampling resolution is listed for band n104 centre frequency (6775 MHz) for different array antenna geometries assuming the base station enclosure depth of 0.1 m.

Table 2.1.7-1: Required maximum sampling resolution

| Array antenna configuration (MsubxNsub, MxN) | Diameter D (m) | Element separation (m) dv, dh | Required sampling resolution (degrees) Δθ |
| --- | --- | --- | --- |
| 3x1, 4x8 | 0.42 | 0.7l, 0.5l | 6.0 |
| 3x1, 8x16 | 0.83 | 0.7l, 0.5l | 3.0 |
| 3x1, 16x32 | 1.65 | 0.7l, 0.5l | 1.5 |

Considering the integration method (described by Eq. 2.1.5.3-2) from WRC-23 outcome and different types of array antenna sizes it would be safe to define maximum allowed resolution to 1 degree.

With 1 degree resolution the average process within respective bin is also secured. Even for the narrowest bin with only 5 samples and average value can be calculated.

Proposal 13: Based on technical analysis considering array sizes suitable for operation within band n104 the sampling resolution can be set to 1.0 degree both along vertical axis and horizontal axis.

# Conclusion

In this contribution we present an overview of the new requirement required to protect FSS UL within band n104 and aspects to consider when the requirement is included in RAN4 BS specifications.

Based on the information in this contribution we have observed following:

Observation 1: When the test requirement and test procedure are defined in RAN4, usage of the term “horizon” should be avoided. For a BS, the relation to horizon depends on deployment parameters such as mechanical tilt and other site-specific details not part of the BS RF conformance testing specification.

Observation 2: With the introduction of OTA spatial emission requirement RAN4 may need to consider how beamforming weights should be applied to the test model to guarantee a relevant test condition.

Observation 3: Since the conformance test concept adopted in RAN4 currently does not capture operation in fading environment, manufacturer declarations will be needed to derive the angular coverage range. This means that the declaration and deployment scenarios are tied together.

Observation 4: RAN4 need to define test beam set(s) that guarantees acceptable measurement uncertainty.

Observation 5: It can be concluded that the test beams should be carefully selected and should be distributed within the whole coverage angular range, not only along the axis. To guarantee repeatability and acceptable measurement uncertainty, RAN4 need to define one common test beam set.

Observation 6: The impact of the beam weight has significant impact on the measured EEIRP profile.

Observation 7: The trend observed in simulation results, in terms of better accuracy provided by beam sets with sufficient number of sample beams and good distribution, are confirmed also when considering different sub-array sizes.

Observation 9: Since the OTA spatial emission above horizon requirement is based on an absolute power level EEIRP mask. The calibration of the test range will be essential to guarantee an acceptable measurement uncertainty.

Observation 10: A test method for EEIRP profile would require EIRP samples to be measured for many test beam directions. Therefore, it is essential to capture a description in the conformance test specification of how EIRP samples are collected for different test beam directions.

Observation 11: For each test beam within the CAR a spatially discrete EIRP pattern is measured in the upper hemi-sphere.

Observation 12: For EEIRP test method development it is essential to continue the direction set out in the outcome from WRC-23. Based on WRC-23 guidance define a test method and corresponding MU evaluation. At a later stage, if needed document additional test methods.

Observation 13: Since the EEIRP requirement cannot be categorized as Directional requirement, TRP requirement or Co-location requirement. It would be appropriate to define a new separate category for EEIRP based requirements.

Observation 14: The OTA spatial emission requirement is a regulatory requirement, the TT=0 which would result in that eventual MU needs to be accounted for in the design as power back-off or other mitigation.

To further progress the work, we propose following:

Proposal 1: RAN4 should define a concept where the test beam directions are related to the declared Coverage Angular Range (CAR) and thereby the number of test beam directions are specified.  The BS manufacturer declares CAR for different deployment situations (e.g. sectorization (3 sector or 6 sector or other), vertical coverage and different mechanical tilt settings). This means that the BS manufacturer can declare one or several CARs for which the OTA spatial emission requirement should be meet.

Proposal 2: RAN4 need to develop a concept where the test beams are distributed within the whole coverage angular, range with a specified beam weight vector.

Proposal 3: Based on the analysis in this contribution we propose to specify the minimum number if beams used within the CAR to 25 uniformly distributed with wk equal to 1/K and location on test beams in the BS RF conformance specification (referred in this contribution to as beam set B).

Proposal 4: RAN4 to specify 25 test beam directions, uniformly distributed directions within CAR. This can be done by capturing equations for each test beam with input parameters from declared CAR (qmin, qmax, jmin, jmax). The values listed in Table 2.2.3-3 can be expressed in equations for a more general approach.

Proposal 5: It is sufficient to test OTA spatial emission above horizon requirement for single carrier configuration.

Proposal 6: It is sufficient to test on two RF channels (Bottom and Top RF channel).

Proposal 7: It is sufficient to test widest carrier bandwidth that generate maximum supported PSD.

Proposal 8: Use draft specification text for “initial conditions” and “procedure” as baseline for TS 38.141-2 draft CR. RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission.

Proposal 9: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation.

Proposal 10: RAN4 need to describe the EIRP sampling grid in the upper hemi-sphere and define criteria for maximum allowed sampling resolution.

Proposal 11: RAN4 need to describe the post-processing calculation in discrete form in the conformance test specification.

Proposal 12: Before starting work with MU evaluation, first focus on BS RF core draft specification text and BS RF conformance specification text. When draft CRs for TS 38.104 and TS 38.141-2 are complete then focus on measurement method MU evaluation in TR 37.941.

Proposal 13: Based on technical analysis considering array sizes suitable for operation within band n104 the sampling resolution can be set to 1.0 degree both along vertical axis and horizontal axis.

# References

[1]	R4-2416266, “	Core aspects for OTA spatial emission above the horizon requirement for BS operating in band n104”, Ericsson

[2]	R4-2416268, “Draft CR to TS 38.104: Addition of spatial emission requirement to protect FSS UL within band n104”, Ericsson

[3]	R4-2416269, “Draft CR to TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104”, Ericsson

[4]	R4-2413220, “Conformance test aspects for the requirement on OTA spatial emission above the horizon for BS operating in band n104”, RAN4 #112, Ericsson

[5]	R4-2413509, “WF for [112][305] NR_BS_RF”, RAN4#112, ZTE

# Annex

The is defined with respect to the elevation angle, where θ equal to 0 degrees refers to the angle towards the horizon and 90 degrees refers to an angle straight up into the sky.

The EEIRP for each interval of elevation angles is defined as two dimensional average EIRP:

EEIRPi=12πsinθHi-sinθLiθLiθHi-ππEIRPθ,φcosθdθdφ

, where EIRPθ,φ denotes the average 3-D EIRP spectral density pattern, averaged over K EIRP patterns, and θLi≤θ<θHi  is the elevation angular range, i is the index of elevation range, θ is the elevation angle and φ is the horizontal angle.

The average EIRP pattern is calculated from K EIRP patterns as:

EIRPθ,φ=k=1KwkEIRPkθ,φ

, where wk is a beam weighting and EIRPkθ,φ is the EIRP pattern per measured test beam.

To get the EEIRP discrete form, we assume M points in azimuth and N in elevation, equally spaced, so that:

dφ=2πM

dθ=θHi-θLiN

EEIRPiθn,φm=2πθHi-θLi2πsinθHi-sinθLiMNnLinHim=1MEIRPθn,φmcosθn

EEIRPiθn,φm=θHi-θLisinθHi-sinθLiMNnLinHim=1MEIRPθn,φmcosθn