3GPP TSG-RAN WG4 Meeting #112bis	R4-2416444

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Nokia

Title:	RRM Core Requirements for RedCap Positioning

Agenda item:	4.2.1.4

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning includes the enhancements for RedCap positioning. The WID depicts the RAN4 work related to this accuracy improvement technology. The RAN4#112 meeting [2] has made the following agreements on RRM performance requirements for RedCap positioning.

### Issue 2-1: Requirement on N_(hops,effect)for RedCap positioning core requirements Proposals: - Option 1: the parameter NrepPRS shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling window within each hop - Option 2: Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times. - Option 3: Other options are not precluded.

This contribution further investigates impacts to RRM performance requirements for NR positioning enhancements.

Discussion

The impacts to RRM performance requirements for some of the above enhancements are discussed in this section.

Requirement on Nhops,effect for RedCap positioning core requirements

The number of hops within a single MG occasion is given by Nhop=minNhops,effect,Nhop,max where Nhops,effect is the effective number of Rx hops within a MG instance. When defining Nhops,effect, the open issue is how to define NrepPRS. In the current definition, NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times. Based on the current definition, RF retuning times can be excluded from measuring the effective number of Rx hops within a measurement gap instance. Therefore, Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times.

RAN4 to define NrepPRS by using Option 2 (‘Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times’).

Conclusion

The following proposal is made in this contribution.

- RAN4 to define Nrep PRS by using Option 2 (‘Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times’).

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning,” Intel Corporation, RAN#99.

- R4-2414032, “WF on RedCap positioning and PRS/SRS bandwidth aggregation,” Ericsson, RAN4#112.