3GPP TSG-RAN WG4 Meeting #112bis	R4-2416462

# Hefei, China, 14 – 18 October 2024

Agenda item:		6.26.2.1

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for NR NTN RedCap

Document for:		Approval

## 1	Introduction

Work for third phase of NR NTN enhancements is on-going, with one of the objectives being specifying RedCap for NR NTN. In this contribution (e)RedCap UE RF requirements are discussed.

## 2	Discussion

The main remaining open issues for NR NTN eRedCap UE RF are HD-FDD sensitivity, maximum output power and simultaneous operation with GNSS. This contribution focuses on these open items.

Simultaneous NR NTN and GNSS operation

Since release 17, 3GPP has assumed that NR NTN is able to simultaneously operate with GNSS. This is needed for example for accurate Tx frequency pre-compensation. However, no studies were never done on this. The current work item explicitly mentions that “GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.”

Simultaneous GNSS and NR-NTN operation is especially challenging for operating bands n255 and n254 due to the narrow frequency separation between the NR-NTN UL and GNSS. This results in both Tx emissions leaking on top of GNSS Rx, as well as the NR-NTN UL acting as jammer to GNSS. Both of these effects can result in UE not being able to get a location fix, potentially leading to field issues.

There are multiple option on improving the likelihood of successful GNSS reception. For example, UE could take advantage of a more complex dual-frequency GNSS receiver. This would increase UE complexity receiving signals at both lower and upper L-band would need more complicated RF front-end and antennas, as well as more advanced digital receiver. All UEs do not have these capabilities.

Alternatively, UE could use more advanced RF filters which would reject Tx leakage to GNSS. At the same time, GNSS band would also need to use similarly advanced filter to avoid NR NTN UL from jamming the receiver. However, filtering solutions are not always viable. For example, n254 operates immediately adjacent to GLONASS frequencies, so one would need a perfect “brickwall filter” to fully avoid interference scenarios. This kind of perfect filter does not exists. In some cases filters can work, but they do add to cost and size of the RF solution.

UE could also try to rely on natural gaps in UL transmissions. The satellite beam covers a large area and could serve a large number of users, and all of these users cannot transmit at the same time. However, there can also be situations when the network load is low, and a single user has a long continuous data transmission. Even half-duplex operation cannot guarantee long transmission gaps for UL heavy traffic.

As such, there is no fool-proof solution which in all scenarios for all UEs could guarantee simultaneous GNSS and NR-NTN operation, and standards solution is needed.

Observation 1: Simultaneous GNSS and NR-NTN operation needs a standard-based solution.

Generally, GNSS needs a sufficiently long and sufficiently interference free time period to get a reliable location fix. As different UE implementations have different RF filtering, antenna performance, emission performance and jammer tolerance, the definition on “sufficient” will vary between implementations. To facilitate the co-existence between GNSS and NR-NTN, a standardized TDM-solution which can guarantee gaps in NR UL transmissions would be needed. Wide configurability would be needed to address different UE implementations, while it is recognized some advanced UEs may not need this solution at all.

Based on our studies, as an example of one implementation, a TDM pattern which can guarantee maximum 50% duty cycle for few seconds could work Within these few seconds, the UL transmissions cannot be continuously for a long time. This kind of gap would be needed regularly, multiple times in a minute.

It is suggested to ask RAN1 and RAN2 to ensure an TDM-based in-device co-existence solution can be relied on by UEs and emphasize that it needs to be supported in real deployments. The solution should be applicable to both RedCap NR NTN as well as non-RedCap NR NTN UEs, and that it would be beneficial if it can be made available in earlier release. Details of the solution can be left to RAN1 and RAN2.

RAN2 has already worked on in-device co-existence solutions in release-18, and it could be possible that existing solution can work. However, our understanding is that rel-18 IDC was never truly designed for NR NTN and GNSS concurrency, and it rather just copied old LTE solution without considered specific NTN aspects. Furthermore, the solution is not helpful if it is not supported by the network. Therefore, an LS to RAN1 and RAN2 is needed.

Proposal 1: Send an LS to RAN1 and RAN2 asking them to ensure UE can rely on in-device co-existence solution which guarantees sufficiently long gaps in NR UL transmission to guarantee successful simultaneous operation between GNSS and NR NTN. Draft LS is provided in appendix.

Maximum output power for PC3 HD-FDD RedCap UE

In previous meeting it was proposed that PC3 output power, or at least upper tolerance for maximum output power, could be increased for HD-FDD RedCap UEs due to lower front-end loss. We see some downsides in this proposal. Firstly, higher output power tolerance means higher uncertainty of the transmitted output power, while it does not guarantee any higher output power. Therefore, it would be a pure relaxation to UE requirements with no real gain.

Secondly, it is not obvious that front-end loss is in all cases reduced. As discussed for GNSS co-existence, in some cases complex filters could be used for co-existence with GNSS, and this would apply also for half-duplex operation.

Thirdly, applying this kind of relaxation to NR NTN RedCap UEs would diverge the requirements between TN RedCap and NTN RedCap UEs. Perhaps more importantly, there would be divergence between NTN IoT and NR RedCap output power requirements. While it is natural that not all requirements are the same, the motivation used by the proponent company applies also for TN RedCap and NTN IoT half duplex operation. It is possible that same UE supports both NTN IoT and NR NTN RedCap, and it would not be logical to apply relaxation only to NR NTN RedCap.

Proposal 2: Specify PC3 max output power for HD-FDD RedCap as 23 dBm +/- 2 dB

HD-FDD 2Rx sensitivity

HD-FDD 2Rx sensitivity is still open. Sensitivity for HD-FDD can in some cases be better than FD-FDD, given that there is no Tx interference for impacts from Rx IIP2. It is proposed to specify HD-FDD 2Rx sensitivity align with TDD bands and TN HD-FDD RedCap, resulting in values below in Table 1. It should be noted that values for n256 are already agreed.

Proposal 3: Specify HD-FDD 2Rx sensitivity as in Table 1

Table 1: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

## 3	Conclusion

In this contribution UE RF requirements for NR NTN RedCap were discussed. Following observations and proposals were made

Observation 1: Simultaneous GNSS and NR-NTN operation needs a standard-based solution.

Proposal 1: Send an LS to RAN1 and RAN2 asking them to ensure UE can rely on in-device co-existence solution which guarantees sufficiently long gaps in NR UL transmission to guarantee successful simultaneous operation between GNSS and NR NTN. Draft LS is provided in appendix.

Proposal 2: Specify PC3 max output power as 23 dBm +/- 2 dB

Proposal 3: Specify HD-FDD 2Rx sensitivity as in Table 1

Table 1: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

## Appendix

Title:	draft LS on NR NTN co-existence with GNSS

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

Cc:	-

Contact person:	Toni Lähteensuo

tlaehtee@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 has studied the emissions and blocking effects NR NTN transmission cause to GNSS frequencies and observed that successful GNSS reception cannot always be guaranteed, the problem being worst in nearby frequency band n254 and n255. To avoid any field performance issues both for NR NTN RedCap as well as NR NTN non-RedCap UEs, a reliable in-device co-existence solution needs to be supported in real deployments.

RAN4 therefore respectfully asks RAN2 and RAN1 to ensure that NR NTN UEs can rely on an in-device co-existence solution for GNSS co-existence. As one example, a TDM pattern which guarantees maximum 50% Tx duty cycle for a few seconds was reported in RAN4. This kind of pattern could be needed by the UE regularly, multiple times within a minute. The required parameters can have large differences between different UE implementations, with some UEs being able to perform completely without such solution.

# Actions

ACTIONS:

To RAN WG2 and WG1: RAN4 respectfully asks RAN2 and RAN1 to ensure that NR NTN UEs can rely on an in-device co-existence solution for GNSS co-existence with sufficient gaps in NR UL transmissions, as described above.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #113	Nov 18 - 22, 2024	Orlando, USA

TSG-RAN4 Meeting #114	Feb 17 - 21, 2025	Athens, Greece