3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2416482

# Hefei, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Discussion on Ambient IoT Device RF requirement impact

Agenda item:	6.21.3.2

Document for:	Discussion

Introduction

In 3GPP RAN #112, WF[1] was approved. In this paper, we provide our views on Ambient IoT Device RF requirement impact Discussion

- Issue 1-1: System parameter Agreement in RAN4#111: - RAN4 will define the D2R and/or R2D channel bandwidth and operating bands for A-IoT - Wait for the conclusions from other WGs to discuss the detailed parameters. - FFS on whether to have the same or different channel bandwidths for devices, BS and intermediate node Agreement in RAN4#112: - Requirements potentially needed: - channel bandwidth - FFS whether requirements are needed or not: - Transmission bandwidth configuration; - Channel spacing - Channel raster Note: The system parameters are defined from the network perspective. Some of them might not be needed for devices. - Requirements not needed: - Synchronization raster - Guard band/Guard RB Note: Guard band/RB can be considered as part of RF requirements and/or test cases. Agreement in RAN4#112: - BS class: Use Micro-BS as baseline in SI stage (reference to SID RP-240826). - BS type: Priority A-IoT BS type 1-C, FFS for 1-H. - Topic #3: A-IoT device - Issue 3-1: General Agreement in RAN4#112: - Different RF requirement for Ambient IoT Device 1, Device 2a and Device 2b can be specified - Issue 3-3: TX(D2R) Agreement in RAN4#112: - Requirements potentially needed: - Transmit OFF power for device 2b - Power control requirement for device 2b - ON/OFF time mask for device 2b - Frequency error for device 2b - In band emissions (IBE) for device 2b - Carrier leakage for device 2b - [SEM] - [ACLR] - [Spurious emissions] - FFS whether requirements are needed or not: - Maximum output power - EVM - Transmit OFF power for device 1/2a - Transmit time mask - Minimum output power - Frequency error for device 2a - In band emissions (IBE) for device 2a - Carrier leakage for device 2a - Occupied bandwidth - Transmit intermodulation - Requirements not needed: - Power control requirement for device 1/2a - ON/OFF time mask for device 1/2a - Frequency error for device 1 - In band emissions (IBE) for device 1 - Carrier leakage for device 1 - Issue 3-4: RX(R2D) Agreement in RAN4#112: - Requirements potentially needed: - Reference sensitivity - FFS whether requirements are needed or not: - Maximum input power - Receiver intermodulation - Rx spurious emission - ACS - ACSC - In-band blocking - Out-of-band blocking - Requirements not needed: - Spurious response - Issue 3-5: testability Agreement: in RAN4#111: - FFS on whether conducted conformance test is feasible for AIOT devices. - FFS on OTA test method, performance metric, etc. Agreement in RAN4#112: - Take OTA test as the baseline for at least Devices 1 and 2a

- Discussion

Transmit output power

Assuming there is a virtual power source called received CW, it need to define at the minimum distance and maximum distance. However, if the CW node is seen as external node, it is necessary to simply define as output power.

Proposal 1: For Device 1 and Device 2a, it need to simply define as output power or to define both minimum output power and maximum output power.

Maximum input power

It is important to determine whether there are elements in the device that are saturated due to high power.Considering the structure currently being discussed, device1 has no active elements. However, device2a has an LNA and reflection amplifier. At this aspect,

Proposal 2: Maximum input level is not needed for device 1 but Device 2a potentially need to define maximum input level.

Rx spurious emission

it is depend on co-existence analysis result and need to meet regulatory requirement

Proposal 3: For Device 1 and Device 2a, it is need to define Rx spurious emission and depend on co-existence result and need to meet regulatory requirement.

ACS and ACSC

It is depend on co-existence study

Proposal 4: ACS and ACSC is depend on co-existence study.

- Conclusion

In this contribution, we want to share some views on Ambient IoT Device RF requirement impact and proposals are made as following:

Proposal 1: For Device 1 and Device 2a, it need to simply define as output power or to define both minimum output power and maximum output power.

Proposal 2: Maximum input level is not needed for device 1 but Device 2a potentially need to define maximum input level.

Proposal 3: For Device 1 and Device 2a, it is need to define Rx spurious emission and depend on co-existence result and need to meet regulatory requirement.

Proposal 4: ACS and ACSC is depend on co-existence study.

- 3. Reference

[1] R4-2414305, “WF on impacts of A-IoT on RF requirements”