3GPP TSG-RAN WG4 Meeting #112bis										          R4-2415651

Hefei, Anhui, China, 14th – 18th Oct 2024

Agenda item:			6.1.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [112bis][204] NR_ENDC_RF_Ph4

Document for:	Information

Introduction

This email thread discusses the RRM requirements of WI on R19 NR_ENDC_RF_Ph4.

Online handling

(Online) Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE

(Online) Issue 1-2: SNR level for RLM and BFD testing for 6Rx capable UE

(Online) Issue 1-3: Antenna configurations for 6Rx capable UE

Topic #1:	RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415247 | CMCC | Proposal 1: it is proposed that the legacy interruption requirements at SRS antenna switching are applicable to 6RX capable UE. |
| R4-2415570 | Nokia, Nokia Shanghai Bell | Observation #1: In RAN1, 3T6R and 4T6R SRS antenna switching has not been specified. Observation #2: In R17, the interruption requirements for SRS antenna switching were specified considering up to 4Rx and SRS resource(s) is assumed being configured within the last 6 symbols of a slot. Observation #3: In R19, it is not clear SRS resources are always configured within the last 6 symbols of a slot pending on RAN1 discussion. Proposal 1: To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4. |
| R4-2415843 | Huawei, HiSilicon | Proposal 1: The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed. Proposal 2: No new/enhanced interruption requirements due SRS antenna switching are to be specified in this work item. Proposal 3: RLM and BFD testing for 6Rx capable UEs reuse SNR levels specified for 4Rx capable UE. Proposal 4: Regarding antenna configurations for 6Rx capable UE, a new clause is to be introduced and the requirements of “A.3.6.1.2 Antenna connection for 8 Rx capable UEs” shall apply with “8RX” being replaced with “6RX”. |
| R4-2416361 | Ericsson | Proposal 1: The legacy interruption requirements for SRS antenna port switching are reused for 4 Rx capable UEs. Proposal 2: Impact on SNR levels for RLM and BFD tests is discussed in the performance part of the WI. |

Open issues summary

Sub-topic 1: RRM requirements of WI on R19 NR_ENDC_RF_Ph4

(Online) Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE

- Proposals

  - Option 1(CMCC, Huawei, Ericsson): The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

  - Option 2(Nokia): To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4.

- Recommended WF

Further discussion.

(Online) Issue 1-2: SNR level for RLM and BFD testing for 6Rx capable UE

- Proposals

  - Option 1(Huawei): Reuse SNR levels specified for 4Rx capable UE.

  - Option 2 (Ericsson): Impact on SNR levels for RLM and BFD tests is discussed in the performance part of the WI.

- Recommended WF

Further discussion.

(Online) Issue 1-3: Antenna configurations for 6Rx capable UE

- Proposals

  - Option 1(Huawei): a new clause is to be introduced and the requirements of “A.3.6.1.2 Antenna connection for 8 Rx capable UEs” shall apply with “8RX” being replaced with “6RX”.

- Recommended WF

Further discussion.