3GPP TSG-RAN WG4 Meeting #112bis	R4-2415658

Hefei, Anhui, China, 14th – 18th October 2024

Agenda item:			6.18.4

Source:	Moderator (Nokia)

Title:	Topic summary for [212bis][211] FS_NR_AIML_Mob

Document for:	Information

# Introduction

This summary thread addresses the issues related to AI/ML mobility SI in RAN4. This is the first meeting of this SI in RAN4. The document is split into 3 high-level topics which are further split into several sub-topics and issues.

Moderator’s recommendations are to first agree on the Priorities of different use-cases, scenarios and sub use-cases. Therefore, following issues to be discussed first:

- Issue 1-1-1 Priorities of different use cases

- Issue 1-1-2 Scenario/sub use-case priorities

We will try to have a tentative agreement on above listed issues during the offline session (probably on Monday) in order to have a quick agreement in online session.

Next, following issues from General Aspects and Work Plan topic to be discussed:

- Issue 1-1-3: Evaluation in RAN4

- Issue 1-1-4: Work Plan

From RAN4 RRM requirements, moderator’s recommendation is to prioritize discussion on following issue in the online session:

- Issue 2-1-1 Performance Metrics/KPIs for Measurement Prediction Use case

- Issue 2-1-2 Impacts of measurement errors on the prediction accuracy performance

- Issue 2-1-3 Impacts on Core requirements

From Topic #3, Testability and interoperability, moderator’s recommendation is to prioritize discussion on following issue in the online session:

- Issue 3-1-1 New testability aspects

- Issue 3-1-2 Testing goals

- Issue 3-1-1 Testing in FR1

- Issue 3-3-1 Principle of FR2 testing

- Issue 3-3-2 New FR2 testing aspects

# Topic #1: General Aspects and Work Plan.

This section addresses general aspects and priorities of different use cases, scenarios and sub use-cases identified in the draft TR 38.744.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415504 | Apple | General Aspects and Work Plan Observation 1: It is too early for RAN4 to evaluate the testability, interoperability, and RRM impacts related to measurement event prediction and RLF/HO prediction Proposal 1: RAN4 should begin with use cases focused on RRM measurement prediction, then integrate RLF, and potentially include mobility event prediction based on advancements achieved in RAN2. Proposal 2: The RAN4 study can commence by concentrating on the accuracy of RRM measurement prediction before addressing RLF/HO and measurement event prediction. Proposal 3: RAN4 can start by examining the impact of RRM core requirements to RRM measurement prediction accuracy in simulations. Proposal 4: RAN4 should discuss ground truth and assess the feasibility of RAN4 metrics. Proposal 5: RAN4 should review the assumptions made in RAN2 simulations to determine whether RAN4 aspects have been considered. Proposal 6: RAN4 needs to investigate the impact on testability for measurement prediction (Sub-Case 1) involving neighbor cells Proposal 7: RAN4 should leverage agreements from these discussions for Case 1 in AI/ML concerning mobility in the NR Study Item (beam temporal prediction). Proposal 8: The discussion on applying or modifying the identified RAN4 requirements can be postponed to the WI phase. |
| R4-2415968 | Nokia, Oppo, MediaTek | - Work Plan for RAN4 Discussions on AI/ML for Mobility in NR Observation 1: RAN2 has started the study with the discussion of simulation assumptions and RRM measurement prediction use-case. RLF use-case was already considered but simulation result evaluation will continue only in 2025. HOF use-case is deprioritized. Evaluation of measurement event prediction use-case will be treated at the upcoming RAN2 meeting. LCM and generalization aspects and other scenarios for mobility are expected to be discussed at a later stage. Observation 2: The work plan for the FS_NR_AIML_Mob does not consider RAN4 work yet. Proposal 1: RAN4 to agree the RAN4 Work plan from the Tabel below. Table: RAN4 work plan. Time slot; Meeting#; RAN4 work plan. October; RAN4#112bis; - Discussion/agreement of the work plan, checking RAN4 TR 38.744 section. - Start RAN4 study of RRM measurement prediction use-case: - Agree on the scenarios to be considered in RAN4 - Discuss relevant core/performance requirements - Discus feasible requirement metrics - Start discussing Testability and Interoperability aspects for RRM measurement prediction use-case.. November; RAN4#113; - Continue discussion of relevant RAN4 requirements for RRM measurement prediction use-case. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability and Interoperability aspects.. Feb. 2025; RAN4#114; - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-cases - Initiate the discussion of Measurement event prediction use-case depending on the progress in RAN2. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability, considering Rel-19 WI progress.. April; RAN4#114bis; - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-case and corresponding metrics - Initiate the discussion of RLF use-case depending on the progress in RAN2. - Continue discussion of Testability aspects - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Initiate the discussion of monitoring, generalization, scalability, etc. – related requirements based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air - If possible, start the discussion of LCM-related issues based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air. May; RAN4#115; - Continue the discussion and evaluation of potential requirements per use-case. - Continue the discussion of testability issues - Initial draft of the TR/Draft CR(s) to the TR 38.744 - Continue or initiate the discussion of LCM-related issues and requirements. June; RAN#108; Initial TR for information. August; RAN4#116; - Conclude evaluation of testability, interoperability, and specifications impacts on RRM requirements and performance - Conclude identified LCM issues and their spec impact - Submit the final CR(s) to the TR with RAN4 conclusions. September; RAN#109; - TR for approval - Close the SI. |
| R4-2415931 | NTT DOCOMO, INC. | Discussions on general aspects of AI/ML for mobility Proposal 1: The study cases and prioritization in RAN4 should be same as RAN2 assumptions summarized in the table below. Case number; Prioritization; Evaluation scenario combination; target study goal. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal. |
| R4-2415844 | Huawei, HiSilicon | Discussion on general aspects in AIML mobility Proposal 1: RAN4 can discuss the following two options: - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement. - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters. - FFS: The value of MRRT. - FFS: Other parameter and the values, e.g., the prediction window. Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction. Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap. Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about the Work plan and related priorities for AI/ML Mobility use cases are summarized

#### Issue 1-1-1: Priorities of different use cases

- Background

  - Prioritization of different use cases considered in RAN2. Use-cases are listed below:

    - Measurement Prediction

    - Handover and Radio Link Failure Prediction, and

    - Measurement Event Prediction

- Proposals and Observations:

  - Proposal 1 (Nokia, Oppo, MediaTek, Qualcomm, CATT, Xiaomi):

    - From RANR#112bis, Start discussing Testability and Interoperability aspects for RRM measurement prediction use-case

  - Proposal 2 (Apple):

    - RAN4 should begin with use cases focused on RRM measurement prediction, then integrate RLF, and potentially include mobility event prediction based on advancements achieved in RAN2.

    - RAN4 should begin with use cases focused on RRM measurement prediction, then integrate RLF, and potentially include mobility event prediction based on advancements achieved in RAN2.

- Candidate options:

  - RAN4 to start the SI with discussions on RRM requirements and testability and interoperability aspects for RRM measurement prediction use-case.

  - For Other use-cases:

    - Option 1 [Nokia, Oppo, MediaTek]: From RAN4#114 meeting, RAN4 to initiate the discussion of Measurement event prediction use-case depending on the progress in RAN2. From RAN4#114bis meeting, RAN4 to initiate the discussion of RLF use-case depending on the progress in RAN2.

    - Option 2: RAN4 to start the discussion of RRM measurement prediction, then the discussion of RLF use-case and then the discussion of Measurement event prediction use-case.

- Recommended WF

  - RAN4 to initiate the discussion with RRM measurement prediction use-case.

    - From RAN4#114 meeting, RAN4 to initiate the discussion of Measurement event prediction use-case depending on the progress in RAN2.

    - From RAN4#114bis meeting, RAN4 to initiate the discussion of RLF use-case depending on the progress in RAN2

#### Issue 1-1-2: Scenario/sub-use-case priorities

- Background

In this issue, we have included different proposals from different companies on the issue of prioritization of scenarios and sub-use cases, irrespective of the agenda items in which the tdocs were submitted. Different companies submitted their tdocs, addressing this issue, in different agenda items.

- Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model) At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Proposals and Observations:

  - Proposal 1 (Xiaomi): For RRM measurement prediction, RAN4 to study the testability and interoperability for AI/ML mobility starting with the three prioritized scenarios:

    - FR1 to FR1 inter-frequency intra-cell temporal domain case B

    - FR1 to FR1 inter-frequency inter-cell frequency domain

    - FR2 to FR2 intra-frequency intra-cell temporal domain case A

  - Proposal 2 (Xiaomi): For Measurement event prediction, wait for RAN2 input to further study testability and interoperability issue.

  - Proposal 3 (Xiaomi): For RLF prediction, wait for RAN2 input to further study testability and interoperability issue.

  - Proposal 4 (CATT): Cell-level RSRP is assumed to be used as input/output in tests and further discuss the necessity of using beam-level RSRP.

  - Proposal 5 (QC): RAN4 to consider the use case of RRM measurement prediction for the testability and interoperability of AI/ML-aided mobility as a starting point

  - Proposal 6 (Samsung): In AI/ML mobility, regarding the test scope, suggest to prioritize discussions on AI/ML model inference in each use case and sub-use case.

    - Take Rel-18 AI/ML test principles/procedures/frameworks as much as possible for testing the AI/ML model inference functionality.

  - Proposal 7 (Huawei): RAN4 can focus on the high priority scenarios in RAN2 at first.

  - Proposal 8 (Ericsson): RAN4 to prioritize discussions on testability and interoperability issues for case numbers 2,3, and 4 in Table 1.

  - Proposal 9 (Ericsson): RAN4 to start discussing testability of AI/ML based mobility for scenarios 2 and 4 after the details related to the time window for prediction is further progressed or settled in RAN2.

  - Proposal 10 (Nokia): Study the requirements and testing framework for AI/ML based Measurement prediction use case considering the existing AI/ML based BM test framework as the starting point for FR2 scenarios.

  - Proposal 11 (Nokia): RAN4 first should start discussions on performance metrics/KPIs for different sub-use cases/scenarios of measurement prediction use case.

  - Proposal 12 (MediaTek):  Focus on scenarios 2,3,4 for RRM measurement prediction use case at first.

  - Proposal 13 (Oppo): For RRM measurement prediction, prioritize following scenarios (align with the prioritized scenarios of RRM measurement prediction in RAN2)

| Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- |
| High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

  - Proposal 15 (Nokia): RAN4 initially focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3, #4, and #6 (i.e. FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction, FR2 to FR2 intra-frequency temporal domain case A, and FR2 to FR2 intra-frequency spatial domain).

  - Proposal 16 (Samsung):

    - For RRM measurement prediction, it is suggested to consider all the three high prioritization sub-use cases for RAN4 discussion at current stage

    - For RRM measurement prediction, RAN4 RRM requirement’s study would be better to focus on the following with high priority:

      - L3 measurement enhancement

        - Temporal domain FR1 intra frequency

          - UE measurement reduction

        - Frequency domain FR1 inter frequency

          - UE measurement reduction

        - Temporal domain FR2 intra frequency

          - Handover performance enhancement

        - Other scenarios should wait for more progress from RAN2

- Recommended WF:

  - Moderator’s recommendation is to discuss the Use-case and Scenario priorities in a general topic #1.

  - Moderator’s recommendation is to discuss all sub-use cases for high and middle priority scenarios (2, 3, 4 and 6) agreed in RAN2

  - Here are considered sub use-cases in RAN2 (as mentioned in Clause 4.2 of draft TR 38.744)

    - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

    - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

    - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

  - For the sake of simplicity, we will refer them L1-L1, L3-L3 and L1-L3 in the following.

  - Discuss which of the following combinations of scenarios and sub use-cases to be considered in RAN4:

  - Scenario 2 FR1 to FR1 intra-frequency temporal domain case B

    - Opt 2-1: L1-L1

    - Opt 2-2: L3-L3

    - Opt 2-3: L1-L3

  - Scenario 3 FR1 to FR1 inter-frequency frequency domain

    - Opt 3-1: L1-L1

    - Opt 3-2: L3-L3

    - Opt 3-3: L1-L3

  - Scenario 4 FR2 to FR2 intra-frequency temporal domain case A

    - Opt 4-1: L1-L1

    - Opt 4-2: L3-L3

    - Opt 4-3: L1-L3

  - Scenario 6 FR2 to FR2 intra-frequency spatial domain

    - Opt 6-1: L1-L1

    - Opt 6-2: L3-L3

    - Opt 6-3: L1-L3

#### Issue 1-1-3: Evaluation in RAN4

- Proposals and Observations:

  - Proposal 1 (Apple): RAN4 can start by examining the impact of RRM core requirements to RRM measurement prediction accuracy in simulations

  - Proposal 2 (Apple): RAN4 should review the assumptions made in RAN2 simulations to determine whether RAN4 aspects have been considered.

- Recommended WF

  - Companies to discuss Proposal 1 and Proposal 2 as Options.

#### Issue 1-1-4: Work Plan

- Proposals and Observations:

  - Proposal 1 (Nokia, Oppo, MediaTek): RAN4 to agree the RAN4 Work plan from the Tabel below:

Table: RAN4 work plan.

| Time slot | Meeting# | RAN4 work plan |
| --- | --- | --- |
| October | RAN4#112bis | - Discussion/agreement of the work plan, checking RAN4 TR 38.744 section. - Start RAN4 study of RRM measurement prediction use-case: - Agree on the scenarios to be considered in RAN4 - Discuss relevant core/performance requirements - Discus feasible requirement metrics - Start discussing Testability and Interoperability aspects for RRM measurement prediction use-case. |
| November | RAN4#113 | - Continue discussion of relevant RAN4 requirements for RRM measurement prediction use-case. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability and Interoperability aspects. |
| Feb. 2025 | RAN4#114 | - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-cases - Initiate the discussion of Measurement event prediction use-case depending on the progress in RAN2. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability, considering Rel-19 WI progress. |
| April | RAN4#114bis | - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-case and corresponding metrics - Initiate the discussion of RLF use-case depending on the progress in RAN2. - Continue discussion of Testability aspects - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Initiate the discussion of monitoring, generalization, scalability, etc. – related requirements based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air - If possible, start the discussion of LCM-related issues based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air |
| May | RAN4#115 | - Continue the discussion and evaluation of potential requirements per use-case. - Continue the discussion of testability issues - Initial draft of the TR/Draft CR(s) to the TR 38.744 - Continue or initiate the discussion of LCM-related issues and requirements |
| June | RAN#108 | Initial TR for information |
| August | RAN4#116 | - Conclude evaluation of testability, interoperability, and specifications impacts on RRM requirements and performance - Conclude identified LCM issues and their spec impact - Submit the final CR(s) to the TR with RAN4 conclusions |
| September | RAN#109 | - TR for approval - Close the SI |

- Recommended WF

Discuss and agree the work plan and approve the work plan tdoc R4-2415968.

# Topic #2: RAN4 requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415194 | vivo | - Discussion on impacts on RAN4 requirements for AI mobility Observation 1: There would be no RAN4 impact on cell-level result based measurements and predictions. Observation 2: It is still pending in RAN2 discussion on whether the measurement prediction is based on reported L3 results, or derived L3 results by combining measurement samples. Proposal 1: More clarifications for the model input of beam-level results are needed and RAN4 could discuss the potential RRM impacts at least including the following aspects at the initial study stage: - The impact of measurement delay requirements due to the reduction of measurement samples; - The RRM measurement prediction accuracy requirements. Proposal 2: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error. |
| R4-2415073 | Xiaomi | Discussion on impacts on RAN4 requirement for AI mobility Proposal 1: RRM measurement prediction and RLF failure prediction will have RRM impact. RAN4 to study the impact of RRM measurement prediction first. Observation 1: In RAN2, RSRP prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell. Proposal 2: RAN4 needs to define L3 filtered cell-level RSRP prediction accuracy requirement. Proposal 3: If RAN4 would like to analyze the RSRP prediction accuracy degradation due to measurement error, suggest RAN4 to align the assumptions of input data and labels for training and test first. Observation 2: For Intra-frequency intra-cell temporal domain prediction, RAN2 will define two temporal domain cases in TR, e.g. case A and case B, based on observation window and prediction window. Observation 3: Observation window and prediction window will have RRM impact: - Observation window will have impact on measurement duration - Prediction window will have impact on scheduling restriction Observation 4: Observation window and prediction window length are dependent on specific sub-cases. Proposal 4: For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2. Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period. - Proposal 5: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period. |
| R4-2415033 | OPPO | Study of impacts on RAN4 requirements for AI mobility Proposal 1: In RAN4, prioritize the study on RRM measurement prediction. Proposal 2: For RRM measurement prediction, prioritize following scenarios (align with the prioritized scenarios of RRM measurement prediction in RAN2) Priority; Evaluation scenario; Target study goal; Methodology. High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Proposal 3: For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window. - above aspects can be referenced based on the progress of RAN2's research, particularly the evaluation assumptions Proposal 4: For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4: - the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5} - the window length should be multiple times of sampling period or measurement period of corresponding frequency range. Proposal 5: For FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion: - MRRT: {50%~80%} - OW: {40ms ~ 2000ms} - PW: {40ms~800ms} Proposal 6: In RAN4, reuse the intermediate KPIs that defined in RAN2 as candidate performance metrics for further study, - Metrics for measurement prediction accuracy Measurement prediction accuracy for cell-level RRM measurement prediction is defined as: Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged. - Assumptions on measurement reduction rate MRRT and MRRS that defined in RAN2 can be reused: MRRT = skipped measurement time instances / total measurement time instances MRRS = skipped beams to be measured/ total beams to be measured For cases that need to be evaluated for the 1st study goal(reduce measurement efforts) for both FR1 and FR2 scenario. MRRT(s) should be aligned among companies. Proposal 7: Postpone the discussion on Measurement event prediction to Feb. 2025, details(e.g. sub use cases, KPIs, etc.,) depending on the progress in RAN2 Proposal 8: Postpone the discussion on RLF event prediction to April 2025, details(e.g. sub use cases, KPIs, etc.,)depending on the progress in RAN2. Proposal 9: Down prioritized the discussion on HOF in RAN4 until new progress in RAN2. |
| R4-2415256 | CMCC | Discussion on impacts on RAN4 requirements for AI/ML for mobility Proposal 1: for RRM measurement prediction, it is proposed to introduce cell level RSRP prediction accuracy. Proposal 2: the cell level RSRP prediction accuracy is proposed as following: - Absolute cell level RSRP accuracy = predicted L3-RSRP of cell i – ideal measured L3-RSRP of cell i - For FR2, the ideal measured L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP) - For FR1, the ideal measured L3-RSRP is the transmitted power Proposal 3: for RRM measurement prediction, the measurement delay requirements need to be defined - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy - For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements |
| R4-2414931 | MediaTek Inc. | Discussion on RRM requirements of AI mobility Proposal 1: Focus on RRM measurement prediction use case this meeting and further discuss other use cases based on RAN2 progress. Proposal 2: Focus on scenarios 2,3,4 for RRM measurement prediction use case at first. Proposal 3: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated L3 cell level RSRP. Proposal 4: For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated L3 cell level RSRP. Proposal 5: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility. Proposal 6: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility. Proposal 7: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2. Proposal 8: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for inter-frequency layers which require normal measurement and inter-frequency layers which require model monitoring. Proposal 9: Scenario 4 has no impact on measurement delay requirements. |
| R4-2416449 | Nokia | On AIML Mobility Scenarios Evaluation and RRM Requirements Impacts Observation 1: RAN2 has initiated its work by focusing on measurement prediction, leading to more advanced progress in this area compared to other use cases. Proposal 1: RAN4 initially focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3, #4, and #6 (i.e. FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction, FR2 to FR2 intra-frequency temporal domain case A, and FR2 to FR2 intra-frequency spatial domain). Observation 2: For measurement prediction, different accuracy metrics are considered in RAN2 SI for AI/ML mobility and in RAN4 for AI/ML Air Interface WI for beam prediction. Proposal 2: RAN4 will need to study accuracy requirements for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered measurements for measurement reduction purpose. Observation 3: To ensure compatibility, RAN2 simulation assumptions must align with the RRM constraints outlined by RAN4. Proposal 3: RAN4 to check and compare RAN2 simulation assumptions to usual RAN4 RRM requirements/test configurations to conclude about their compatibility Observation 4: Frequency domain measurement prediction may need different accuracy and measurement gap requirements compared to temporal measurement prediction. Proposal 4: RAN4 to study the required RSRP accuracy of inter-frequency (frequency domain) measurement prediction, and the impact of frequency domain prediction on measurement gap. Observation 5: RAN4 AI/ML for Air Interface focus on FR2 use cases. RAN4 will need to study and evaluate RRM requirements for RAN2 prioritized FR1 AI/ML mobility scenarios. Observation 6: Performance of AI/ML-based measurement prediction is highly dependent on the accuracy of the input data. Proposal 5: RAN4 to study the impact of measurement error used as the input data on the accuracy of the predicted measurements. Observation 7: Depending on the use cases considered by RAN2, RAN4 may study requirements for reported predicted future measurements or for other predictions derived from those measurements with the goal to optimize mobility performance/HO without reduction of measurements. Proposal 6: RAN4 will need to study LCM required actions and the impact on RRM. Observation 9: RAN4 will need to study the required accuracy metrics and core requirements for measurement event prediction depending on the progress of this use-case in RAN2. |
| R4-2415176 | CATT | Views on impacts of AIML mobility on RAN4 requirements Proposal 1: LCM procedures have impacts on delay requirements and RAN4 waits for agreements in AI/ML for NR air WI. Proposal 2: RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility. Proposal 3: RAN4 discusses to reuse or tighten the existing accuracy requirements for RSRP measurements for RRM measurement prediction case. |
| R4-2400505 | Apple | Study of impacts on RAN4 requirement Observation: RAN2 has various KPIs tailored for mobility optimization. We need to understand the interactions between different RAN4 KPIs, such as RRM measurement accuracy, and RAN2 KPIs Observation: Accuracy requirements for predicted measurements might align with those established for baseline measurements. However, RAN4 should explore whether new or updated requirements are necessary when AI/ML-based predictions are reported to NW for the purpose of measurement reductions Proposal 1: RAN4 should examine whether AI/ML-based predicted beam-level L1/L3-filtered or cell-level L3-filtered measurements introduce any new or modified requirements when reported to NW for measurement reduction Observation: When defining requirements for measurement prediction accuracy, it is important to also investigate the availability and accuracy of ground truth reference values Observation: It is essential that RAN4 validates the simulation assumptions under which the mobility performance conclusions from the RAN2 study are based.(form case 1) This will help prevent scenarios where inaccuracies in measurements would affect predicted values, potentially impacting the conclusions drawn by the RAN2 study Proposal 2: If RAN2 study fails to achieve the desired mobility performance (case 3) under realistic measurement accuracies (as defined by RAN4 requirements), a second round of simulations may be needed. In this phase, RAN4’s involvement would focus on investigating the potential impact on RRM measurement requirements to the mobility performance across various RRM measurement prediction cases and evaluation scenarios. Proposal 3: If the RAN2 study fails to achieve the desired measurement prediction accuracy (Case 1) under realistic measurement accuracies, as defined by RAN4 requirements, a second round of simulations may be necessary. In this phase, RAN4’s involvement would focus on investigating the potential impact of RRM measurement requirements on measurement prediction accuracy across various RRM measurement prediction cases and evaluation scenarios. Proposal 4: Investigate translating the requirements of RAN2 KPIs into RAN4 RRM measurement accuracy requirements to establish the feasibility of the RAN2 study. This translation is crucial for aligning the performance metrics and ensuring that the mobility optimization goals are effectively met. |
| R4-2415577 | Samsung | General discussion on RRM requirement impact analyses on AI mobility Observation 1: RAN2 give first priority to L3 cell level measurement predictions, and put L3 beam level measurement predictions as low priority. For no matter FR1 or FR2, how to produce filtered L1 and L3 RSRP is still suspending Observation 2: The following scenarios are considered in RAN2 for AI/ML RRM measurement prediction - FR1-to-FR1 - Intra-frequency intra-cell in temporal domain prediction for the purpose of measurement overhead reduction - Inter-frequency inter-cell in frequency domain prediction for the purpose of measurement overhead reduction - Other domain including: Intra-frequency temporal domain with 2nd goal is low prioritization - Cluster approach for intra-frequency prediction need to be further clarified - FR2-to-FR2 - Intra-frequency intra-cell in temporal domain prediction for the purpose of HO performance enhancement - Other domains including: Intra-frequency temporal domain with 1st goal and intra-frequency intra-cell spatial domain with 1st goal are low and middle prioritization, separately - Cluster approach for intra-frequency prediction need to be further clarified Observation 3: Regarding measurement event predictions - Both direct measurement event prediction and indirect prediction are allowed - At least measurement event evaluation based on RRM measurement prediction result will be studied - Start with A3 as a baseline Observation 4: There are at least three main differences between AI/ML beam management and AI/ML mobility from high-level perspective - Motivation and KPIs - Multiple domains prediction - Cell/beam level measurement prediction - L3 filtered beam level prediction cases are lower priority in AI mobility Proposal 1: The general aspects part of RAN4 RRM work on AI/ML based beam prediction in Rel-18 could be reused for AI/ML based RRM measurement prediction as much as possible - RAN4 to discuss what general aspects could be reused to avoid the duplicate study - RAN4 to discuss what general aspects are Rel-19 AI mobility specific, and need to be defined accordingly Proposal 2: For RRM measurement prediction, it is suggested to consider all the three high prioritization sub-use cases for RAN4 discussion at current stage Proposal 3: For RRM measurement prediction sub-use case 1, consider AI-PHY beam management prediction framework/methodologies/requirements as starting point to study Proposal 4: For RRM measurement prediction sub-use case 2, the core and performance requirements, and test to be defined for such sub-use case could be deferred until RAN2’s conclusion Proposal 5: For RRM measurement prediction, RAN4 RRM requirement’s study would be better to focus on the following with high priority: - L3 measurement enhancement - Temporal domain FR1 intra frequency - UE measurement reduction - Frequency domain FR1 inter frequency - UE measurement reduction - Temporal domain FR2 intra frequency - Handover performance enhancement - Other scenarios should wait for more progress from RAN2 RRM requirement impacts are to be studied for core requirements, performance requirements as well as testability and interoperability requirements Proposal 6: For cell level measurement prediction, at least the following issues related to sub-use cases should subject to RAN2’s process and agreements - Non-AI benchmark - The evaluation methodologies for different scenarios - KPIs for measurement prediction (e.g., prediction accuracy) - Dataset generation for AI model training/test - Input/output of the direct/indirect AI model Proposal 7: The simulation parameters for verifying the feasibility of requirement/testing in RAN4 should follow RAN2’s progress Proposal 8: Based on RAN2’s progress and agreements to determine what kind of AI model (i.e., UE side model or NW side model) could be considered to define the AI/ML based requirement for RRM measurement prediction in RAN4 Proposal 9: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility Proposal 10: It is suggested for RAN4 to delay the Measurement event prediction related requirements discussion until more stable conclusions are achieved in RAN2 |
| R4-2415845 | Huawei, HiSilicon | Discussion on impacts of RAN4 requirements in AI/ML mobility Proposal 1: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed. Proposal 2: For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified. Proposal 3: For intra-frequency temporal domain prediction case, the RSRP prediction accuracy would be specified. Proposal 4: For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT may need to be discussed and specified. Certain limitation on the prediction length may also need to be considered. Proposal 5: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy would be specified. Proposal 6: The processing time of inference function may need to be considered in measurement reporting delay. Proposal 7: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified. Proposal 8: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements. Proposal 9: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface. Proposal 10: Measurement event prediction study can wait for more progress on measurement prediction. Proposal 11: The RRM impacts identified for measurement prediction in temporal domain case A can be used as a starting point for indirect RLF prediction, where RSRP is replaced by SINR. Observation 1: Measurement impairment may impact the prediction accuracy. Observation 2: The RSRP with measurement impairment can be obtained in reality, rather than the ideal RSRP. Observation 3: In RAN2 assumption, the ideal RSRP is assumed both for model input and the label of model output. Proposal 12: RAN4 to discuss the measurement impairment considered for verifying the actual prediction accuracy enabled by AI. Proposal 13: RAN4 can perform link level simulation to determine the RSRP measurement impairment in different SNR values if the measured RSRP is considered as model output. Observation 4: In RAN4 legacy, there is no requirement of prediction accuracy in point C, point A1 instead. In AI mobility, RAN2 uses the RSRP difference at point C as the prediction accuracy. Proposal 14: RAN4 will discuss whether to define prediction accuracy in point C. Proposal 15: RAN4 will study the following three potential options for defining prediction accuracy, training and testing dataset. Pros and Cons need to be analyzed. - Option 1: Use the prediction accuracy aligned in RAN2 to define the requirement in RAN4. The testing dataset is generated by link level channel model. - Option 2: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4. - Option 3: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4. Proposal 16: RAN4 will study the following two potential options for defining the testing and training dataset. - Option 1: Use the channel model and the sampling rules. Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner. |
| R4-2415932 | NTT DOCOMO, INC. | Discussions on RAN4 requirement impacts of AIML for mobility Observation 1: Although many points are under discussions, the KPI is based on how many numbers of measurement can be replaced by predicted value for cell quality evaluation. Proposal 1: In temporal domain case A, impacts on RAN4 requirements are at least measurement delay, interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption. Proposal 2: In temporal domain case B, the total measurement delay may be same as non-AI/ML measurement. Thus, impacts on RAN4 requirements can be interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption. Proposal 3: In frequency domain, all the requirements related to the inter-frequency measurements may be impacted because whole measurement may be skipped. Proposal 4: In spatial domain, firstly the investigation how to use the outcome of Rel-18 AI/ML for air interface is needed. After this investigation, same perspective for other temporal domain studies can be started. |
| R4-2416332 | Ericsson | On requirements for AI/ML based mobility Proposal 1: RAN4 to study requirements for temporal prediction, spatial prediction, and frequency prediction based on the outcome of RAN2 evaluations. Proposal 2: RAN4 to study requirements for event prediction based on the outcome of RAN2 evaluations. Proposal 3: RAN4 to use definition of AI/ML model inference from TR38.843 for spatial and frequency predictions while discussing the impact of spatial and frequency predictions on RAN4 requirements. Proposal 4: RAN4 to use the following definition of temporal prediction while discussing the impact of temporal prediction on RAN4 requirements. “Temporal prediction is a process of using a trained AI/ML model to produce a set of output(s) based on a set of input(s), where the output of the model should remain valid at a future reference time.” Proposal 5: Send an LS to RAN2 requesting to capture definition of temporal, spatial, and frequency prediction in TR38.744. Proposal 6: RAN4 to provide input to RAN2 on parameters and assumptions relevant to generalization study. |
| R4-2416381 | ZTE Corporation | Discussion impacts on requirements of AI/ML mobility Observation 1: Both cell level measurement prediction and beam level measurement prediction for intra-frequency intra and inter-cell spatial domain are within the scope of RRM measurement prediction. And there are some agreements on cell level measurement prediction. Proposal 1: RAN4 should analyse and define requirements for both cell level measurement prediction and beam level measurement prediction for intra-frequency intra and inter-cell spatial domain. Observation 2: AI-PHY beam management Case A and Case B from the RAN1 is reused for temporal domain measurement prediction both beam level and cell level in AI mob. Relative requirements, such as beam prediction accuracy related KPIs, can be used as baseline. Observation 3: as to the KPI of cell-level measurement prediction, focus on measurement prediction accuracy and RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell should be considered. Proposal 2: RAN4 should guarantee the preformance of cell-level measurement prediction and define measurement prediction accuracy. For beam-level measurement prediction, wait for RAN2’s further progress. Proposal 3：For measurement prediction accuracy of cell-level measurement prediction, metrics for beam management requirements/tests on AL/ML PHY can be used as start point, i.e. RSRP accuracy, beam prediction accuracy. Proposal 4: RAN4 should discuss the value of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances. Observation 4: Measurement event prediction is based on RRM measurement prediction and it should be discussed in RAN2 first. Observation 5: HOF prediction is downprioritized in RAN2’s study and simulation results on RLF are not expected before February. |
| R4-2415347 | Qualcomm Incorporated | Impact of AI based mobility on RRM requirements Observation 1: According to the objective of the study item, evaluation the potential benefits and gains of AI/ML-aided mobility should be studied by RAN2. As the item is still in a study phase, and given the limited TU allocated to RAN4 for the study item, RAN4 should focus on discussing potential impacts on core/performance requirements and testability/interoperability issues. Additionally, RAN4 should try to leverage the outcomes of the ongoing parallel work item (AI/ML air interface, particularly AI/ML-based beam management, LCM, and performance monitoring). Observation 2: According to the endorsed draft TR 38.744 for the study item R2-2406309 and the RAN2 progress, HOF (Handover Failure) prediction is down prioritized compared to RLF prediction. For RLM prediction, RLF due to physical layer problems (e.g., T310 timer expiry) is the major cause under the study. Among the use cases, RRM measurement prediction has made the most progress in RAN2 compared to measurement event prediction and RLF prediction. Observation 3: There may potentially be multiple aspects that are unique to the AI/ML-aided mobility study item compared to AI/ML-aided beam management work item due to the following differences between the two items: - BM is only for FR2, whereas Mobility is for both FR1 and FR2 - BM is only for L1 measurement/prediction, whereas Mobility is for both L1 and L3 measurement/prediction - BM is only for FR2 serving cell, whereas Mobility is serving and neighbor cells - BM is only for temporal and spatial domain, whereas Mobility is temporal, spatial, and frequency domain - BM and Mobility may need different side conditions in terms of RSRP/SINR and BW of reference signal for the input measurements - BM and Mobility may need different NW assistant information for multiple target cells/sites in the same or different frequency layers Observation 4: The following three scenarios are prioritized for RRM measurement prediction: - Case B intra-frequency intra-cell measurement prediction in the temporal domain in FR1 - Case A intra-frequency intra-cell measurement prediction in the temporal domain in FR2 - Iner-frequency measurement prediction in the frequency domain in FR1 Observation 5: The following three sub-cases are under consideration for RRM measurement prediction: - Sub-case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated - Sub-case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) - Sub-case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Observation 6: The following two approached are under consideration for RRM measurement prediction: - Cluster-based approach: Measurement results of multiple cells are used to predict the measurement of one or more cells. - Cell-based approach: AI/ML model is trained per cell/site and used for inferring the measurement results for the same cell. Observation 7: RAN2 evaluation assumptions are still under discussion, e.g. spatial consistency, correlation of large-scale fading characteristics across frequency layers, and the measurement and sampling period for the input of AI/ML models. Observation 8: There has not been meaningful progress on AI/ML-aided measurement event prediction Observation 9: There are still many open issues under the RAN2 study on AI/ML-aided RLF prediction. Proposal 1: RAN4 to wait for further progress from RAN1 to identify the detailed potential impact on the RRM core and performance requirements. The potential impact on RRM requirements can only be outlined at a high level, including aspects such as handover, RRC re-establishment, RLM, [BFD/CBD], and intra-/inter-frequency operations with and without measurement gaps. Proposal 2: Once the group gets more clarity on the overall framework of each AI/ML-aided use case, RAN4 can study the side conditions for requirement applicability and the overall procedures/sequences when identifying and defining potential impacts on RRM requirements. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about the RRM requirements for AI/ML Mobility use cases are summarized.

#### Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case

- Proposals and Observations:

  - Proposal 1 (vivo):

    - Prediction accuracy requirements should be considered

  - Proposal 2 (Xiaomi):

    - RAN4 needs to define L3 filtered cell-level RSRP prediction accuracy requirement.

  - Proposal 3 (Oppo): Reuse KPIs defined in RAN2:

    - Metrics for measurement prediction accuracy: Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged.

    - Metrics for measurement reduction rate:

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

  - Proposal 4 (CMCC):

    - Absolute cell level RSRP accuracy =  predicted L3-RSRP of cell i – ideal measured L3-RSRP of cell i

- For FR2, the ideal measured L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP)

- For FR1, the ideal measured L3-RSRP is obtained as the legacy way of conducted test

  - Proposal 5 (MediaTek):

    - For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated L3 cell level RSRP.

    - For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated L3 cell level RSRP.

  - Proposal 6 (Nokia):

    - RAN4 will need to study accuracy requirements for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered measurements for measurement reduction purpose.

    - RAN4 to check and compare RAN2 simulation assumptions to usual RAN4 RRM requirements/test configurations to conclude about their compatibility

    - RAN4 to study the required RSRP accuracy of inter-frequency (frequency domain) measurement prediction, and the impact of frequency domain prediction on measurement gap.

  - Proposal 7 (Apple):

    - RAN4 should examine whether AI/ML-based predicted beam-level L1/L3-filtered or cell-level L3-filtered measurements introduce any new or modified requirements when reported to NW for measurement reduction

    - Investigate translating the requirements of RAN2 KPIs into RAN4 RRM measurement accuracy requirements to establish the feasibility of the RAN2 study. This translation is crucial for aligning the performance metrics and ensuring that the mobility optimization goals are effectively met.

  - Proposal 8 (Huawei):

    - For intra-frequency temporal domain prediction case, the RSRP prediction accuracy would be specified.

    - For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT (Measurement reduction rate in temporal domain) may need to be discussed and specified. Certain limitation on the prediction length may also need to be considered.

    - For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy would be specified.

    - RAN4 will discuss whether to define prediction accuracy in point C as shown in the figure below. This figure is from TS 38.300 (Figure 9.2.4-1).

  - Proposal 9 (ZTE):

    - RAN4 should analyse and define requirements for both cell level measurement prediction and beam level measurement prediction for intra-frequency intra and inter-cell spatial domain.

    - RAN4 should guarantee the preformance of cell-level measurement prediction and define measurement prediction accuracy. For beam-level measurement prediction, wait for RAN2’s further progress.

    - For measurement prediction accuracy of cell-level measurement prediction, metrics for beam management requirements/tests on AL/ML PHY can be used as start point, i.e. RSRP accuracy, beam prediction accuracy.

  - Proposal 10 (Qualcomm):

    - RAN4 to wait for further progress from RAN1 to identify the detailed potential impact on the RRM core and performance requirements. The potential impact on RRM requirements can only be outlined at a high level, including aspects such as handover, RRC re-establishment, RLM, [BFD/CBD], and intra-/inter-frequency operations with and without measurement gaps.

    - Once the group gets more clarity on the overall framework of each AI/ML-aided use case, RAN4 can study the side conditions for requirement applicability and the overall procedures/sequences when identifying and defining potential impacts on RRM requirements.

  - Proposal 11 (Samsung):

    - For cell level measurement prediction, at least the following issues related to sub-use cases should subject to RAN2’s process and agreements

      - KPIs for measurement prediction (e.g., prediction accuracy)

  - Proposal 12 (Ericsson)

    - RAN4 to study requirements for temporal prediction, spatial prediction, and frequency prediction based on the outcome of RAN2 evaluations.

- Candidate options:

  - Option 1: RAN4 needs to define only L3 cell level RSRP (Point C) prediction accuracy requirements.

  - Option 2: RAN4 needs to define L1 (Point A1)/L3 beam-level (Point E) and L3 cell-level (Point C) prediction accuracy requirements

  - Option 3: Depending upon the scenario, RAN4 needs to define absolute and/or relative prediction accuracy requirements.

  - Option 4: RAN4 to wait for further progress from RAN1 to identify the detailed potential impact on the RRM core and performance requirements

  - Option 5: The performance Metrics/KPIs for Measurement Prediction use case considered in RAN4 shall subject to RAN2’s process and agreements

- Recommended WF

  - Discuss different options listed above.

#### Issue 2-1-2: Impacts of measurement errors on the prediction accuracy performance

- Proposals and Observations:

  - Proposal 1 (Vivo):

    - RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

  - Proposal 2 (Nokia):

    - RAN4 to study the impact of measurement error used as the input data on the accuracy of the predicted measurements.

  - Proposal 3 (CATT):

    - RAN4 discusses to reuse or tighten the existing accuracy requirements for RSRP measurements for RRM measurement prediction case.

  - Proposal 4 (Huawei):

    - RAN4 to discuss the measurement impairment considered for verifying the actual prediction accuracy enabled by AI.

    - RAN4 can perform link level simulation to determine the RSRP measurement impairment in different SNR values if the measured RSRP is considered as model output.

    - If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

  - Proposal 5 (Apple):

    - If RAN2 study fails to achieve the desired mobility performance (case 3) under realistic measurement accuracies (as defined by RAN4 requirements), a second round of simulations may be needed. In this phase, RAN4’s involvement would focus on investigating the potential impact on RRM measurement requirements to the mobility performance across various RRM measurement prediction cases and evaluation scenarios.

  - Proposal 6 (Xiaomi):

    - If RAN4 would like to analyze the RSRP prediction accuracy degradation due to measurement error, suggest RAN4 to align the assumptions of input data and labels for training and test first.

  - Proposal 7 (Samsung):

    - The simulation parameters for verifying the feasibility of requirement/testing in RAN4 should follow RAN2’s progress

  - Proposal 8 (MediaTek):

    - Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility.

- Candidate options:

  - Option 1: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

  - Option 2: suggest RAN2 to take into account the measurement errors in RAN2 evaluation.

  - Option 3: Wait for RAN2 evaluation results and if RAN2 doesn’t consider measurement errors then RAN4 should do it after RAN2 evaluations.

  - Option 4: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility

- Recommended WF

  - Discuss different options listed above. Companies should be encouraged to review RAN2 simulation assumptions as well.

#### Issue 2-1-3: Impacts on Core requirements

- Proposals and Observations:

  - Proposal 1 (Xiaomi):

    - For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2.

    - For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 2 (Oppo):

    - For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window.

    - For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4:

      - the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

      - the window length should be multiple times of sampling period or measurement period of corresponding frequency range.

    - For FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion:

      - MRRT: {50%~80%}

      - OW: {40ms ~ 2000ms}

      - PW: {40ms~800ms}

  - Proposal 3 (CMCC):

    - For RRM measurement prediction, the measurement delay requirements need to be defined/updated

      - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC and/or MGRP cannot be directly used  in the measurement delay requirements as legacy

      - For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements

  - Proposal 4 (MediaTek):

    - Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

    - Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

    - For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for inter-frequency layers which require normal measurement and inter-frequency layers which require model monitoring.

    - Scenario 4 has no impact on measurement delay requirements.

  - Proposal 5 (CATT):

    - RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility.

  - Proposal 6 (Samsung):

    - RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

  - Proposal 7 (Huawei):

    - For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

    - For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy would be specified.

    - The processing time of inference function may need to be considered in measurement reporting delay.

  - Proposal 8 (ZTE):

    - RAN4 should discuss the value of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances.

  - Proposal 9 (Vivo):

    - RAN4 could discuss the impact of measurement delay requirements due to the reduction of measurement samples;

  - Proposal 10 (NTT Docomo):

    - In temporal domain case B, the total measurement delay may be same as non-AI/ML measurement. Thus, impacts on RAN4 requirements can be interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption.

- Candidate options:

  - Option 1: For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2

  - Option 2: For inter-frequency inter-cell frequency domain prediction, RAN4 to study the side condition of frequency prediction (e.g., EPRE difference).

  - Option 3: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility

  - Option 4: RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility.

  - Option 5: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

- Recommended WF

  - Discuss above listed options.

#### Issue 2-1-4: Prediction definition

- Proposals and Observations:

  - Proposal 1 (Ericsson):

    - RAN4 to use definition of AI/ML model inference from TR38.843 for spatial and frequency predictions while discussing the impact of spatial and frequency predictions on RAN4 requirements.

    - RAN4 to use the following definition of temporal prediction while discussing the impact of temporal prediction on RAN4 requirements. “Temporal prediction is a process of using a trained AI/ML model to produce a set of output(s) based on a set of input(s), where the output of the model should remain valid at a future reference time.”

    - Send an LS to RAN2 requesting to capture definition of temporal, spatial, and frequency prediction in TR38.744.

- Recommended WF

  - Discuss different options listed above.

#### Issue 2-1-5: LCM related requirements

- Proposals and Observations:

  - Proposal 1 (Nokia):

    - RAN4 will need to study LCM required actions and the impact on RRM.

  - Proposal 2 (CATT):

    - LCM procedures have impacts on delay requirements and RAN4 waits for agreements in AI/ML for NR air WI.

  - Proposal 3 (Huawei):

    - If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

    - If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

- Recommended WF Candidate options / tentative agreements:

  - Option 1: RAN4 will need to study LCM required actions and the impact on RRM

  - Option 2: LCM procedures have impacts on delay requirements However RAN4 should wait for agreements in AI/ML for NR air WI.

- Recommended WF

  - LCM related discussions should be held in RAN4 potentially at a later stage. RAN2 needs to progress on this topic.

#### Issue 2-1-6: Training and testing dataset

- Proposals and Observations:

  - Proposal 1 (Huawei):

    - RAN4 will study the following three potential options for defining prediction accuracy, training and testing dataset.  Pros and Cons need to be analyzed.

      - Option 1: Use the prediction accuracy aligned in RAN2 to define the requirement in RAN4. The testing dataset is generated by link level channel model.

      - Option 2: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

      - Option 3: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

    - RAN4 will study the following two potential options for defining the testing and training dataset.

      - Option 1: Use the channel model and the sampling rules.

      - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

  - Proposal 2 (Qualcomm): In order to ensure data sanity in AI/ML model training and inference and consistency between data set and measurement configuration, RAN4 should carefully review the performance assessment assumptions and configurations when discussing the testability aspect.

- Recommended WF

  - Dataset related discussions should be held in RAN4 potentially at a later stage in the context of testing setup. Companies should share their views on the options listed above.

# Topic #3: Testability and interoperability

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2414932 | MediaTek Inc. | - Discussion on testability and interoperability of AI mobility Proposal 1: Focus on the testability issues in FR1. Check if there is any more testability issue for FR2 AI/ML mobility after RAN4 concluding the testability issues in “AI/ML for NR air interface”. Proposal 2: Use conductive test in FR1 for AI/ML mobility. Observation 1: Ground truth is available at TE using conductive test in FR1 for AI/ML mobility. |
| R4-2415034 | OPPO | - Study of testability and interoperability for AI mobility Proposal 1: Regarding the testability issues for RRM measurement prediction, at least following aspects need to be considered: - Testing in FR1 - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction - Testing L3-RSRP measurements instead of L1-RSRP Proposal 2: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it. Proposal 3: For FR1 prediction, it is necessary to consider whether the testing assumption for FR1 rises any additional testing issues. - RAN4 needs to determine whether the FR1 test should be performed through OTA testing or conductive testing. Proposal 4: For FR1 prediction, using conductive testing as the basic assumption for the test environment is more appropriate Proposal 5: For inter-carrier scenario (inter-frequency inter-cell prediction), how to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site should be considered. Proposal 6: For inter-carrier scenario (inter-frequency inter-cell prediction), how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship should be considered. - Stable time-domain RSRP signal for the tests might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time. Proposal 7: For inter-carrier scenario (inter-frequency inter-cell prediction), to ensure the testability of measurements for inter-cell prediction, channel modelling for inter-cell prediction should be considered. - Channel models proposed and analyzed in R19 BM discussions could be considered as the starting point. Proposal 8: Regarding L3 RSRP, compare with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions. |
| R4-2415064 | Xiaomi | Initial discussion on study of testability and interoperability for AIML mobility Proposal 1: For RRM measurement prediction, RAN4 to study the testability and interoperability for AI/ML mobility starting with the three prioritized scenarios: - FR1 to FR1 inter-frequency intra-cell temporal domain case B - FR1 to FR1 inter-frequency inter-cell frequency domain - FR2 to FR2 intra-frequency intra-cell temporal domain case A Proposal 2: For RRM measurement prediction, RAN4 to study the maximum numbers of Tx beam that has to be configured in the test and emulated by the test equipment for FR1 and FR2 separately. Proposal 3: For RRM measurement prediction, RAN4 to study which fading channel should be used in the test, as this may differ between FR1 and FR2. Proposal 4: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied. Proposal 5: For the scenario of FR2 to FR2 intra-frequency intra-cell temporal domain case A, multi-AoA setup in the test need to be further studied. Proposal 6: For Measurement event prediction, wait for RAN2 input to further study testability and interoperability issue. Proposal 7: For RLF prediction, wait for RAN2 input to further study testability and interoperability issue. |
| R4-2415177 | CATT | Discussion on testability and interoperability issues for AIML mobility Proposal 1: RAN4 can focus on the testability and interoperability study of case 1 with UE-sided models as the starting point. Proposal 2: Number of neighbour cells should be larger than 1 in test. The specific value is FFS. Observation 1: Up to 3 AoAs are supported for OTA tests in current spec. And study on the support of more than 3 AoAs is under discussion in AI/ML for NR air WI. Proposal 3: RAN4 to discuss the required number of AoAs in AI/ML based mobility tests. RAN4 shall wait for progress in AI/ML for NR air WI if it is identified that more than 3 AoAs are needed. Proposal 4: RAN4 to discuss the mapping relationship between SSB signals and AoAs when a single AoA is multiplexed by multiple cells. Proposal 5: Cell-level RSRP is assumed to be used as input/output in tests and further discuss the necessity of using beam-level RSRP. Proposal 6: For delay test metrics of RRM measurement prediction tests, RAN4 discussions can start from the following options: - Handover interruption. - Measurement reduction . - HOF/RLF. - Others. |
| R4-2415195 | vivo | Discussion on testability and interoperability for AI mobility Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion. Observation 2: There are no testability issues for cell-level result based measurements and predictions for AI mobility. Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system. Proposal 1: RAN4 to consider use one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell. Observation 4: A simplified RAN4 test system would skip Tx beams and the imput dimension mismatch issue may happened in the lab. Proposal 2: RAN4 to consider evaluate the impact of model input dimensions (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance. |
| R4-2415257 | CMCC | Discussion on testability and interoperability for AI/ML for mobility Proposal 1: for scenario number 6 (FR2 to FR2 intra-frequency spatial domain), it is proposed that the Test setup for beam management for AI/ML for NR air interface can be reused. Priposal 2: for temporal domain prediction, it is proposed to further discuss whether legacy test setup can be used as baseline. Proposal 3: it is proposed to study the impact of measurement error for the input to inference and dataset for training on prediction accuracy. |
| R4-2415348 | Qualcomm Incorporated | Testability and interoperability for AI based mobility Reference Use Case for Testability and Interoperability Discussions Proposal 1: RAN4 to consider the use case of RRM measurement prediction for the testability and interoperability of AI/ML-aided mobility as a starting point. Relation between AI/ML Aire Interface WI and AI/ML Mobility SI Proposal 2: RAN4 to consider the following differences between AI/ML-aided beam management and AI/ML-aided mobility in the testability and interoperability discussions: - BM is only for FR2, whereas Mobility is for both FR1 and FR2 - BM is only for L1 measurement/prediction, whereas Mobility is for both L1 and L3 measurement/prediction - BM is only for FR2 serving cell, whereas Mobility is serving and neighbor cells - BM is only for temporal and spatial domain, whereas Mobility is temporal, spatial, and frequency domain - BM and Mobility may need different side conditions in terms of RSRP/SINR and BW of reference signal for the input measurements - BM and Mobility may need different NW assistant information for multiple target cells/sites in the same or different frequency layers Proposal 3: RAN4 to consider FR1 for testability and interoperability issues in the AI/ML-aided mobility study item. And the discussion on testability and interoperability for FR2 can wait for further progress from AI/ML-aided Air interface work item, particularly beam management use case. Proposal 4: For the following aspects, RAN4 to wait for further progress from AI/ML-aided Air interface work item. - LCM and performance monitoring - Generalization - Interoperability Proposal 5: Testability issues need to be studied, taking the following into consideration when RAN2 settles on the detailed simulation assumptions: - Time-varying reception power and SNR due to UE trajectory - Whether/how to model the different time-varying characteristics per-cell - Whether/how to model the different variation per-test iteration Data Set Generation and Consistency Proposal 6: In order to ensure data sanity in AI/ML model training and inference and consistency between data set and measurement configuration, RAN4 should carefully review the performance assessment assumptions and configurations when discussing the testability aspect. |
| R4-2415503 | Apple | Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR: Study of testability and interoperability Proposal 1: RAN4 should consider a simplified probe layout as the baseline for AI/ML-based mobility similar to AI/ML beam management testing and discuss the testability requirements for the following aspects: - Set of CDL models to be considered (eg. minimum number of clusters for spatial prediction) - Which large scale parameters of the channel model should be modeled as frequency-dependent - to capture the effects of varying frequencies for frequency prediction - Investigate availability of ground truth/reference values for temporal prediction - Investigate feasibility and testability for Tx beam spatial prediction across multiple cells Proposal 2: To effectively test RLF detection, it is essential to define a testing dataset that mirrors the characteristics of the training dataset. This testing dataset will evaluate the time prediction capabilities of the AI/ML model. It should consist of various patterns of SINR waveforms, ensuring a comprehensive assessment of the model's performance across different scenarios Proposal 3: specify time intervals for measurements (M) and predictions (P) within the waveforms. This specification will help to structure the analysis and ensure clarity in assessing the model's performance at distinct intervals in time (e.g AI/ML model predicts the probability of RLF detection inside interval P) |
| R4-2415576 | Samsung | General discussion on AI mobility regarding testability and interoperability Proposal 1: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements - Follow the conclusions on generalization achieved in AI/ML for NR air interface as much as possible - Study depends on sufficient research progress in RAN2 Proposal 2: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases - The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures - RAN4 study should be aligned with the agreements in RAN1 and RAN2 Proposal 3: In AI/ML mobility, regarding the test scope, suggest to prioritize discussions on AI/ML model inference in each use case and sub-use case. - Take Rel-18 AI/ML test principles/procedures/frameworks as much as possible for testing the AI/ML model inference functionality. Proposal 4: RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows. - RRM measurement prediction - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained - HO failure/RLF prediction - Directly RLF prediction by AI/ML models - To verify whether the HO performance gain of AI/ML model can be obtained - To verify whether the successful/failure HO is classified correctly in datasets with class imbalance cases - Indirectly RLF prediction based on the temporal domain serving cell measurement predictions - To verify whether the prediction accuracy performance of AI/ML model is reliable - To verify whether the HO performance gain of AI/ML model can be obtained - FFS how to design the AI mobility specific test cases including test setup - FFS the definition of ground truth - Metrics/KPIs subject to RAN2’s progress and agreements |
| R4-2415846 | Huawei, HiSilicon | Discussion on testability and interoperability issues in AIML mobility Proposal 1: Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results) and Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM. Proposal 2: RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results). |
| R4-2416333 | Ericsson | On testability issues related to AI/ML for mobility Proposal 1: RAN4 to prioritize discussions on testability and interoperability issues for case numbers 2,3, and 4 in Table 1. Proposal 2: RAN4 to start discussing testability of AI/ML based mobility for scenarios 2 and 4 after the details related to the time window for prediction is further progressed or settled in RAN2. |
| R4-2416382 | ZTE Corporation, Sanechips | Discussion on the Interoperability and testability aspects Observation 1: One side model is considered in AI mobility. For one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happened between UE and NW mainly depended on signalling. Proposal 1：RAN4 to consider interoperability for one side model, including - UE sided and NW sided model for cell-level and beam-level measurement prediction - UE sided model for HO failure/RLF prediction - UE sided model for Measurement events prediction And detail should be further discussed based on RAN2’s input. Observation 2: Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition. Proposal 2: it’s necessary to ensure the performance compared to legacy measurement and the testing goal is to verify whether the minimum performance of AI/ML Mobility can be achieved. Observation 3: the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility, which can easily lead to measurement results used for inference not being applicable to the inference stage, resulting performance would degrade. Proposal 3: RAN4 should further discussion whether the performance in practical network can be guaranteed based on prediction measurement results. Observation 4: Generalization performance is important for prediction, RAN2 thinks that AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found, and RAN4 should discuss whether and how to define and test the generalization performance of AI/ML model. Proposal 4: From the perspective of test, core part and performance part requirements should be considered, i.e. measurement delay and measurement accuracy. Proposal 5: RAN4 to discuss the number of cell to measurement for AI/ML mob, and the number of cells and SSBs used for legacy intra-f/inter-f measurement can serve as a starting point for discussion. |
| R4-2416441 | Nokia | On Testability and Interoperability Issues for AI/ML Mobility Proposal 1: Study the requirements and testing framework for AI/ML based Measurement prediction use case considering the existing AI/ML based BM test framework as the starting point for FR2 scenarios. Observation 1: Different sub use cases and scenarios for measurement prediction use case may have different performance metrics/KPIs. Observation 2: Discussion on performance metric/KPI will impact the study of aspects related to testability and interoperability of AI/ML mobility use cases. Proposal 2: RAN4 first should start discussions on performance metrics/KPIs for different sub-use cases/scenarios of measurement prediction use case. Proposal 3: RAN4 should study any additional impact on the life cycle management related issues coming from AI/ML mobility use cases. The framework developed for rel. 19 WI (AI/ML for Air Interface) should be used as the baseline. Observation 3: In AI/ML mobility SI, RAN4 also need to consider inter-cell aspects in the testability aspects and testing setup. Observation 4: In FR2, existing principles of testing UE-reported RSRP accuracy based on setup parameters and measurement uncertainties allow for very wide range of valid RSRP values. This devaluates such tests for verification of AI/ML-enabled RSRP/best beam prediction. Observation 5: Ground truth extraction should be handled differently for FR1 and FR2 as it involves different testing frameworks (conducted, OTA). Proposal 4: For scenarios involving FR2 (Scenarios 4, 5 and 6), reported RSRP measurements are used as an approximate of the ground truth, as agreed in AI/ML Beam Management use case Proposal 5: For scenarios involving FR1 (Scenario 1, 2 and 3), actual ground truth measurements are considered since it can be realized in conducted mode similar to legacy approach. Proposal 6: For OTA testing in FR2, AI/ML mobility SI should follow the channel model selected for AI/ML BM use case in rel. 19 WI (RP-234039, AI/ML for NR Air Interface). Proposal 7: RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about the main testability aspects and priority of scenarios are summarized.

#### Issue 3-1-1: New testability aspects

- Proposals and Observations:

  - Proposal 1 (OPPO): Regarding the testability issues for RRM measurement prediction, at least following aspects need to be considered:

    - Testing in FR1

    - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction

    - Testing L3-RSRP measurements instead of L1-RSRP

  - Proposal 2 (Qualcomm): RAN4 to consider the following differences between AI/ML-aided beam management and AI/ML-aided mobility in the testability and interoperability discussions:

    - BM is only for FR2, whereas Mobility is for both FR1 and FR2

    - BM is only for L1 measurement/prediction, whereas Mobility is for both L1 and L3 measurement/prediction

    - BM is only for FR2 serving cell, whereas Mobility is serving and neighbor cells

    - BM is only for temporal and spatial domain, whereas Mobility is temporal, spatial, and frequency domain

    - BM and Mobility may need different side conditions in terms of RSRP/SINR and BW of reference signal for the input measurements

    - BM and Mobility may need different NW assistant information for multiple target cells/sites in the same or different frequency layers

  - Proposal 3 (Qualcomm): Testability issues need to be studied, taking the following into consideration when RAN2 settles on the detailed simulation assumptions:

    - Time-varying reception power and SNR due to UE trajectory

    - Whether/how to model the different time-varying characteristics per-cell

    - Whether/how to model the different variation per-test iteration

  - Proposal 5 (CATT): Number of neighbour cells should be larger than 1 in test. The specific value is FFS.

  - Proposal 6 (OPPO): Regarding L3 RSRP, compare with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions.

  - Proposal 7 (Xiaomi): For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied.

  - Proposal 8 (Huawei): Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results) and Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM.

  - Proposal 9 (Huawei): RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results).

- Recommended WF:

  - RAN4 will need to consider at least the following testability aspects for AI/ML-enabled mobility:

    - Testing in FR1

    - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction

    - Testing cell-level L3-RSRP measurements, e.g., L3 filtering

    - Consideration of serving and neighbor cells

    - BM and Mobility may need different side conditions in terms of RSRP/SINR and BW of reference signal for the input measurements

    - BM and Mobility may need different NW assistant information for multiple target cells/sites in the same or different frequency layers

    - Time-varying reception power and SNR due to UE trajectory

      - Whether/how to model the different time-varying characteristics per-cell

      - Whether/how to model the different variation per-test iteration

    - Test setup for different sub-use cases.

      - FFS: reuse the test setup in legacy for Sub use Case 2;

      - The testability discussion of Case 1 and case 3 wait the progress of the testability study in AI-BM.

    - The analysis of other aspects is not precluded

#### Issue 3-1-2: Testing goals

- Background

  - In NR_AIML_Air the following agreements was achieved at RAN4#112 [R4-2414323]:

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

- Proposals and Observations:

  - Proposal 1 (Samsung): RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows.

    - RRM measurement prediction

      - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

      - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained

  - Observation 2 (ZTE): Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition.

  - Proposal 3 (ZTE): it’s necessary to ensure the performance compared to legacy measurement and the testing goal is to verify whether the minimum performance of AI/ML Mobility can be achieved.

  - Proposal 4 (Huawei): RAN4 can discuss the following two options:

      - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

      - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

- Candidate options:

  - Option 1 [Moderator]: Reuse testing goal definition from NR_AIML_Air WI:

    - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

  - Option 1a [ZTE]: Testing goal is to verify whether the minimum performance of AI/ML Mobility can be achieved

  - Option 2 [Samsung]: Consider additional testing aims:

    - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

    - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained

  - Option 3 [Huawei]: RAN4 can discuss the following two options:

    - Option 3-1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

    - Option 3-2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - Other options are not precluded

- Recommended WF

  - Further discuss testing goals during the meeting.

#### Issue 3-1-3: Interoperability

- Background

  - RAN4 objective is to [RP-242393]

    - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.)

- Proposals and Observations:

  - Observation 1 (ZTE): One side model is considered in AI mobility. For one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happened between UE and NW mainly depended on signalling.

  - Proposal 1 (ZTE): RAN4 to consider interoperability for one side model, including

    - UE sided and NW sided model for cell-level and beam-level measurement prediction

    - UE sided model for HO failure/RLF prediction

    - UE sided model for Measurement events prediction

And detail should be further discussed based on RAN2’s input.

- Recommended WF

  - RAN4 to discuss further which interoperability issues to discuss.

#### Issue 3-1-4: Testing setup restrictions

- Background

  - TBA.

- Proposals and Observations:

  - Observation 1 (Vivo): A simplified RAN4 test system would skip Tx beams and the imput dimension mismatch issue may happened in the lab.

  - Proposal 2 (Vivo): RAN4 to consider evaluate the impact of model input dimensions (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance.

  - Observation 2 (ZTE): the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility, which can easily lead to measurement results used for inference not being applicable to the inference stage, resulting performance would degrade.

  - Proposal 3 (ZTE): RAN4 should further discussion whether the performance in practical network can be guaranteed based on prediction mesurement results.

- Recommended WF:

  - RAN4 will need to discuss further how to treat the difference in the testing setup and real deployments, such as

    - Mismatch in between training dataset and testing dataset/setup configuration,

      - E.g., mismatch in the number of beams

    - Whether the performance in practical network can be guaranteed based on prediction measurement results

### Sub-topic 3-2: Testing in FR1

Sub-topic description:

In this sub-topic the company proposals about the FR1 testing aspects are summarized

#### Issue 3-2-1: Testing in FR1

- Proposals and Observations:

  - Proposal 1 (MediaTek): Focus on the testability issues in FR1. Check if there is any more testability issue for FR2 AI/ML mobility after RAN4 concluding the testability issues in “AI/ML for NR air interface”.

  - Proposal 2 (MediaTek): Use conductive test in FR1 for AI/ML mobility.

  - Observation 1 (MediaTek): Ground truth is available at TE using conductive test in FR1 for AI/ML mobility.

  - Proposal 3 (OPPO): For FR1 prediction, it is necessary to consider whether the testing assumption for FR1 rises any additional testing issues.

    - RAN4 needs to determine whether the FR1 test should be performed through OTA testing or conductive testing.

  - Proposal 4 (OPPO): For FR1 prediction, using conductive testing as the basic assumption for the test environment is more appropriate

  - Proposal 3 (Qualcomm): RAN4 to consider FR1 for testability and interoperability issues in the AI/ML-aided mobility study item. And the discussion on testability and interoperability for FR2 can wait for further progress from AI/ML-aided Air interface work item, particularly beam management use case.

  - Proposal 4 (Xiaomi): For RRM measurement prediction, RAN4 to study the maximum numbers of Tx beam that has to be configured in the test and emulated by the test equipment for FR1 and FR2 separately.

  - Proposal 5 (ZTE): RAN4 to discuss the number of cell to measurement for AI/ML mob, and the number of cells and SSBs used for legacy intra-f/inter-f measurement can serve as a starting point for discussion.

- Recommended WF

  - What way of testing to consider for FR1:

    - Option 1: OTA testing

    - Option 2: Conducted testing

    - Option 3: Both OTA and Conducted

  - RAN4 can study further the following aspects of testing in FR1:

    - FFS whether it is possible to assume that ground truth is available at the TE for the conducted tests

    - Maximum numbers of Tx beam/SSBs to be configured in the test

    - Channel model

    - Number of cells to measure in the test

    - Other aspects are not precluded

### Sub-topic 3-3: Testing in FR2

Sub-topic description:

In this sub-topic the company proposals about the FR2 testing aspects are summarized

#### Issue 3-3-1: Principle of FR2 testing

- Proposals and Observations:

  - Proposal 1 (OPPO): For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

  - Observation 1 (CATT): Up to 3 AoAs are supported for OTA tests in current spec. And study on the support of more than 3 AoAs is under discussion in AI/ML for NR air WI.

  - Observation 2 (Vivo): Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion.

  - Observation 4 (Vivo): There are no testability issues for cell-level result based measurements and predictions for AI mobility.

  - Observation 4 (Vivo): In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system.

  - Proposal 4 (Vivo): RAN4 to consider use one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

  - Proposal 5 (CMCC): for scenario number 6 (FR2 to FR2 intra-frequency spatial domain), it is proposed that the Test setup for beam management for AI/ML for NR air interface can be reused.

  - Proposal 6 (CMCC): for temporal domain prediction, it is proposed to further discuss whether legacy test setup can be used as baseline.

  - Proposal 7 (Apple): RAN4 should consider a simplified probe layout as the baseline for AI/ML-based mobility similar to AI/ML beam management testing and discuss the testability requirements for the following aspects:

    - Set of CDL models to be considered (eg. minimum number of clusters for spatial prediction)

    - Which large scale parameters of the channel model should be modeled as frequency-dependent

    - to capture the effects of varying frequencies for frequency prediction

    - Investigate availability of ground truth/reference values for temporal prediction

    - Investigate feasibility and testability for Tx beam spatial prediction across multiple cells

  - Proposal 8 (Xiaomi): For the scenario of FR2 to FR2 intra-frequency intra-cell temporal domain case A, multi-AoA setup in the test need to be further studied.

  - Proposal 9 (Nokia): For OTA testing in FR2, AI/ML mobility SI should follow the channel model selected for AI/ML BM use case in rel. 19 WI (RP-234039, AI/ML for NR Air Interface).

  - Proposal 10 (MediaTek): Focus on the testability issues in FR1. Check if there is any more testability issue for FR2 AI/ML mobility after RAN4 concluding the testability issues in “AI/ML for NR air interface”.

- Recommended WF:

  - Option 1: For FR2 measurement prediction, proposals, conclusions and testing setup that RAN4 made for AI/ML BM can be reused as a baseline

  - Option 2: Reuse BM test setup for Scenario 6 (FR2 to FR2 intra-frequency spatial domain) and FFS for temporal domain

  - Option 3: Need to further study/reconsider applicability of BM testing setup in FR2 for AIML mobility

  - Other options are not precluded

#### Issue 3-3-2: New FR2 testing aspects

- Proposals and Observations:

  - Proposal 2 (CATT): RAN4 to discuss the required number of AoAs in AI/ML based mobility tests. RAN4 shall wait for progress in AI/ML for NR air WI if it is identified that more than 3 AoAs are needed.

  - Proposal 3 (CATT): RAN4 to discuss the mapping relationship between SSB signals and AoAs when a single AoA is multiplexed by multiple cells.

  - Proposal 1 (Xiaomi): For RRM measurement prediction, RAN4 to study the maximum numbers of Tx beam that has to be configured in the test and emulated by the test equipment for FR1 and FR2 separately.

  - Proposal 2 (Xiaomi): For RRM measurement prediction, RAN4 to study which fading channel should be used in the test, as this may differ between FR1 and FR2.

  - Proposal 3 (ZTE): RAN4 to discuss the number of cell to measurement for AI/ML mob, and the number of cells and SSBs used for legacy intra-f/inter-f measurement can serve as a starting point for discussion.

  - Proposal 4 (Vivo): RAN4 to consider use one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

- Recommended WF:

  - The following new aspects for testing in FR2 can be considered by interested companies:

    - Number of AoAs in AI/ML based mobility tests, i.e., if more than 3 AoA are needed

    - Relationship between SSB signals and AoAs when a single AoA is multiplexed by multiple cells

    - Maximum numbers of Tx beam that has to be configured

    - Which fading channel should be used

    - Number of cells to measurement

    - Whether to consider one probe to emulate different Tx beams by applying different power

    - Other aspects are not precluded

### Sub-topic 3-4: Testing of inter-carrier scenario

Sub-topic description:

In this sub-topic the company proposals about the testing of inter-carrier scenarios are summarized

#### Issue 3-4-1: Testability of inter-frequency scenario

- Proposals and Observations:

  - Proposal 1 (OPPO): For inter-carrier scenario (inter-frequency inter-cell prediction), how to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site should be considered.

  - Proposal 2 (OPPO): For inter-carrier scenario (inter-frequency inter-cell prediction), how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship should be considered.

    - Stable time-domain RSRP signal for the tests might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time.

  - Proposal 3 (OPPO): For inter-carrier scenario (inter-frequency inter-cell prediction), to ensure the testability of measurements for inter-cell prediction, channel modelling for inter-cell prediction should be considered.

    - Channel models proposed and analyzed in R19 BM discussions could be considered as the starting point.

  - Proposal 4 (ZTE): RAN4 to discuss the number of cell to measurement for AI/ML mob, and the number of cells and SSBs used for legacy intra-f/inter-f measurement can serve as a starting point for discussion.

  - Proposal 5 (Nokia): RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

  - Proposal 6 (Xiaomi): For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied.

- Recommended WF:

  - The following aspects of inter-carrier scenario testing can be considered further:

    - Relation in between different frequencies:

      - How to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site

      - Channel model and its consistency between different carrier components

      - How to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship

    - Multi-cell setup and number of cells to measure

    - Other aspects are not precluded

### Sub-topic 3-5: Other aspects

Sub-topic description:

In this sub-topic the company proposals about remaining aspects of testability and interoperability are summarized

#### Issue 3-5-1: RLF/HOF scenario

- Proposals and Observations:

  - Proposal 1 (Samsung): HO failure/RLF prediction

    - Directly RLF prediction by AI/ML models

    - To verify whether the HO performance gain of AI/ML model can be obtained

    - To verify whether the successful/failure HO is classified correctly in datasets with class imbalance cases

    - Indirectly RLF prediction based on the temporal domain serving cell measurement predictions

    - To verify whether the prediction accuracy performance of AI/ML model is reliable

    - To verify whether the HO performance gain of AI/ML model can be obtained

  - Proposal 2 (Apple): To effectively test RLF detection, it is essential to define a testing dataset that mirrors the characteristics of the training dataset. This testing dataset will evaluate the time prediction capabilities of the AI/ML model. It should consist of various patterns of SINR waveforms, ensuring a comprehensive assessment of the model's performance across different scenarios

  - Proposal 3 (Apple): specify time intervals for measurements (M) and predictions (P) within the waveforms. This specification will help to structure the analysis and ensure clarity in assessing the model's performance at distinct intervals in time (e.g AI/ML model predicts the probability of RLF detection inside interval P)

- Recommended WF:

  - RLF/HOF discussion can be postponed for a few meetings to wait for more agreements in RAN2.

#### Issue 3-5-2: LCM aspects

- Proposals and Observations:

  - Proposal 1 (Samsung): LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

    - The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

    - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

    - RAN4 study should be aligned with the agreements in RAN1 and RAN2

  - Proposal 2 (Nokia): RAN4 should study any additional impact on the life cycle management related issues coming from AI/ML mobility use cases. The framework developed for rel. 19 WI (AI/ML for Air Interface) should be used as the baseline.

- Recommended WF:

  - LCM discussion can be postponed for now to get more details from the NR_AIML_Air WI and fro mRAN2.

#### Issue 3-5-3: Generalization aspects

- Proposals and Observations:

  - Proposal 1 (Samsung): For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

    - Follow the conclusions on generalization achieved in AI/ML for NR air interface as much as possible

    - Study depends on sufficient research progress in RAN2

  - Observation 1 (ZTE): Genralization performance is important for prediction, RAN2 thinks that AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found, and RAN4 should discuss whether and how to define and test the generalization performance of AI/ML model.

  - Proposal 2 (Ericsson): RAN4 to provide input to RAN2 on parameters and assumptions relevant to generalization study.

- Recommended WF:

  - Generalization discussion can be postponed for now to get more details from the NR_AIML_Air WI and fro mRAN2.