3GPP TSG-RAN WG4 Meeting # 112bis											R4-2415667

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.25.3

Source:	Moderator (Nokia)

Title:	Topic summary for [112bis][220] NR_XR_Ph3

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for extended reality for NR Phase 3, under thread [112bis][225] NR_XR_Ph3. [112bis][220] NR_XR_Ph3

This summary is based on the contributions submitted to the agenda items:

- 6.25	XR for NR Phase 3

  - 6.25.1	General aspects and work plan

  - 6.25.2	RRM core requirements

  - 6.25.3	Moderator summary and conclusions

The topics for discussion are organized as follows:

Topic #1: Scenarios for XR enhancements

- Sub-topic 1-1 Scenarios for measurement skipping

- Sub-topic 1-2 Measurements with gaps

- Sub-topic 1-3 XR skipping requirements impact

- Sub-topic 1-4 Timeline for measurement skipping processing

- Sub-topic 1-5 Need/feasibility of UE assistance information

- Sub-topic 1-6 Additional skipping schemes

- Sub-topic 1-7 Reply LS to RAN1

The order of the issues that are proposed to be discussed is as follows:

- Sub-topic 1-3 XR skipping requirements impact

  - Issue 1-3-1: How to capture impact of measurement skipping

  - Issue 1-3-2: Principles for measurement delay calculation

  - Issue 1-3-8: Conditions for measurement skipping

- Sub-topic 1-7 Reply LS to RAN1

  - Issue 1-7-1: Response on RAN1 working assumption

- Sub-topic 1-4 Timeline for measurement skipping processing

  - Issue 1-4-1: Timeline requirement for measurement gap skipping

- Sub-topic 1-2 Measurements with gaps

  - Issue 1-2-1: General approach for defining requirements for measurements with gaps

  - Issue 1-2-2: Types of measurement gaps

- Sub-topic 1-5 Need/feasibility of UE assistance information

  - Issue 1-5-1: General on UAI

  - Issue 1-5-2: Expected gNB behaviour

  - Issue 1-5-3: Information related to measurement occasions

- Sub-topic 1-1 Scenarios for measurement skipping

  - Issue 1-1-1: Deployment scenarios

  - Issue 1-1-2: Types of measurements to be considered

  - Issue 1-1-3: Measurements without gaps

  - Issue 1-1-4: L1 measurements

  - Issue 1-1-5: L1/L2 triggered mobility measurements

- Sub-topic 1-6 Additional skipping schemes

  - Issue 1-6-1: Semi-static schemes

The latest WID can be found at:

- RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia

# Topic #1: RRM requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2416204 | Ericsson | Draft LS response on UE assistance information |
| R4-2416205 | Ericsson | Draft LS response on UE gaps and restrictions that are caused by RRM measurements |
| R4-2414919 | InterDigital Canada | Discussion on gap skipping occasions mechanism - NR XR_Ph3 Observation 1: Type 1 MGs are supported by all UEs. Proposal 1: RAN4 to consider in Rel-19 normative work in order of priorities Type 1 MG configured via GapConfig without suffix and if time allows Type 2 configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r1. Observation 2: For UL PUSCH, DCI format 0_2 can be covered by format 0_1 and 0_3, while format 1_1 and 1_3 can fulfill 1_2 as well for DL PDSCH, and thus 'Minimum applicable scheduling offset indicator’ from subclause 5.3.1 of TS38.214 can be used as a baseline and would cover the legacy behaviour as well. Proposal 2: RAN4 to assume subclause 5.3.1 of TS38.214 as a baseline for X offset scheduling determination and MG skipping scope expansion. |
| R4-2415198 | vivo | Further discussion on RRM impacts for XR and reply LS Proposal 1: Type 1 MG and Type 2 MG, including per-UE gap and per-FR gap, and pre-configured MG is considered in XR WI in Rel-19, where Type 2 MG needs further clarification. Proposal 2: Skipping SMTC occasions for L3 measurements outside gaps are deprioritized. Proposal 3: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction. Proposal 4: RAN4 to discuss and decide whether enabling Rx/Tx in gaps/restrictions can be supported by any UE. Proposal 5: RAN4 to study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception after solution for enabling Rx/Tx in gaps is clear. Proposal 6: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements. Proposal 7: RAN4 to study possible restrictions on dynamic indication by DCI, if it is confirmed, to achieve good trade-off between data deliver delay and mobility performance. Proposal 8: If supported, RAN4 to study whether and how to define new core requirements when some of SMTC occasions that need to be used for measurements outside gaps are enabled for data transmission/reception when scheduling restrictions exists. Proposal 9: UE should provide information about gaps not used for measurement for enabling transmission/reception in these gaps. Proposal 10: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information. Proposal 11: NW may indicate the percentage of measurement gaps not used for measurement for UE to provide information about gaps not used for measurement. Proposal 12: If supported, UE should provide information about SMTC occasions not used for L3 measurement for enabling transmission/reception on the SMTC occasions. Proposal 13: If supported, RAN4 to discuss the information about SMTC occasions not used for measurement are reported as UE capability or UE assistance information. Proposal 14: If supported, NW may indicate the percentage about SMTC occasions not used for L3 measurement outside gaps for UE to provide information about SMTC occasions not used for measurement. Proposal 15: If supported, the UE assistance information about SMTC occasions not used for measurement should be provided on each CC/MO. Proposal 16: The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as UE assistance information. Proposal 17: UE assistance information/UE capability regarding gaps/SMTC occasions (if supported) not used for measurement is reported in response to RRCReconfiguration. Proposal 18: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured. Proposal 19: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC. Proposal 20: It needs to be clarified that when per-FR gaps are configured, the indication by DCI of the particular gap to be skipped is for which gap pattern(s). Proposal 21: Minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms. Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements. Observation 2: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied. Observation 3: It is up to UE implementation which gaps are not used for measurements under current measurement configuration. Observation 4: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance. Observation 5: Depending on measurement configuration, e.g., SMTC, MO etc., some of the SMTC occasions may not be used for measurements while measurement requirements being satisfied. Observation 6: It is up to UE implementation which SMTC occasions are not used for measurements under current measurement configuration. Observation 7: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Moreover, additional information about gaps/SMTCs not used for measurement is still needed. Observation 8: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified. |
| R4-2415242 | CMCC | Discussion on XR (eXtended Reality) for NR Phase 3 Observation 1: the delay related requirments, e.g. delay for PSS/SSS detection, acquisition the index of the SSB, and measurement, are minimum requirements. Not all the measurment occasions are used for measurement for better UE implementation or in good radio conditions. Observation 2: Since how UE perform measurement is up to UE implementation, network has no information about whether measurement is performed in a cetain measurement occasion. Observation 3: if the skipped measurement occasions are limited to the unused measurement occasions reported by UAI, the legacy measurement requirements can be applied, and no need to relax measurement requirements which may result in connection failure. Proposal 1: it is proposed to introduce UE assistance information related to measurement occasions, e.g. the number of needed measurement occasions for measurement or the number of measurement occasions which can be skipped. Proposal 2: it is proposed that - Apply legacy measurement delay and accuracy requirements for the following conditions: - the number of skipped measurement occasions indicated by network is within the number of unused measurement occasion reported by UAI, or - UE in bad radio conditions (e.g. at cell edge) - Extend measurement delay while keep legacy accuracy requirements unchanged for the following conditions: - there is no UAI, or - The number of skipped measurement occasions indicated by network is larger than the number of unused measurement occasion reported by UAI, or - UE in good radio conditions Proposal 3: for RAN1 LS on solutions to enable transmission/reception in gaps/restrictions, it is proposed to confirm RAN1 working assumption. Observation 4: In Rel-17 positioning, N2 is used as the time offset between UE determination of the presence of DL channels of higher priority than the DL PRS in the DL PRS processing window and the first symbol of the DL PRS processing window, to determine whether UE is required to receive DL channels and drop all PRS within the DL PRS processing window (N2 is about UE PUSCH preparation procedure time). Proposal 4: the minimum time offset(s) X between indication to skip and skipped measurement occasion is proposed as N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214). Proposal 5: it is proposed to consider Tx/Rx in occasions of scheduling restriction due to L1 operation. |
| R4-2415278 | Meta | Discussion on RRM Impacts for XR Observation 1: Incorporating UE assistance information related to skipping measurement occasions can be valuable in reducing RRM performance degradation. Proposal 1: UE assistance information related to measurement occasions should be considered. |
| R4-2415493 | Apple | Discussion of RRM impact of XR for NR Phase 3 Observation 1: Pre-MG deactivated delay is 6~8ms per RAN4 requirements. Proposal 1: as baseline, 7ms minimum time offset is needed between indication to skip and skipped measurement occasion. Observation 2: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap. Observation 3: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement. Proposal 2: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap. Proposal 3: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. Proposal 4: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. Proposal 5: max number of occasions cancelled during a measurement period shall be defined. Proposal 6: RAN4 can continue discussion on other solution to address negative impact on measurement due to cancellation for XR traffic. Proposal 7: at least the following types of measurement gaps shall be included in this work item: - Type-1 gap - Type-2 gap - NCSG - concurrent gaps - pre-MG Proposal 8: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation. |
| R4-2415494 | Apple | Reply LS on gaps/restrictions that are caused by RRM measurements |
| R4-2415704 | ZTE Corporation, Sanechips | Discussion on RRM aspects of R19 XR Proposal 1: For L1 operation skipping, the two items are preferred: - Not to consider the L1 measurement skipping when overlapping with XR Tx/Rx; - Allow the L1 measurement for mobility skipping when overlapping with XR Tx/Rx. Proposal 2: For the L3 measurement and L1 measurement for mobility, both the measurement within gap and without gap should be considered to skip when overlapping with XR Tx/Rx. Proposal 3: Regarding which type of MG to be consider to skip, we have the following suggestion: - First priority: Type 1 MG - Second priority: Pre-configured MG - Not consider: NCSG, Type 2 MG, MUSIM, Positioning, R18 concurrent MG Observation 1: After receiving the dynamic indication of MG skipping through DCI, instead of retuning the RF chain from data Tx/Rx to performing measurement, the UE just keep the RF chain targeting the data Tx/Rx. Proposal 4: For the time offset between the dynamic indication and the first applicable gap instance, besides the DCI decoding delay, no additional processing delay is needed. Observation 2: In legacy, before and after the scheduling restriction caused by the measurement, not any RF retuning operation involved in, UE just needs some BB adaptation. Proposal 5: For the time offset between the dynamic indication and the first applicable scheduling restriction, besides the DCI decoding delay, no additional processing delay is needed. Proposal 6: For the valid duration of the dynamic indication, since the gap/restriction indication field is included in the scheduling DCI, one-shot validity is preferred, i.e. only valid for the gap/restriction(s) overlapping with the XR Tx/Rx scheduled by this DCI. Proposal 7: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. Proposal 8: Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered. Proposal 9: To guarantee the measurement performance when the MG/restriction skipping happens, one straightforward solution is to introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be considered when determining the exact scaling factor. Proposal 10: The UAI is beneficial in the XR scenario, and the regular UAI mechanism could be applied, i.e. UE reports UAI when necessary, and UE makes decision. Proposal 11: Within all candidates of UAI in RAN1 LS, we believe the following types are useful and feasible: - UE assistance information related to channel conditions: e.g. RSRP is below/above search threshold (s-MeasureConfig); - UE assistance information related to traffic: e.g. PSI (PDU set importance); - UE assistance information related to UE mobility: e.g. L3 parameters related to mobility, e.g., static or not. |
| R4-2415836 | Huawei, HiSilicon | Discussion on RRM requirements for Rel-19 XR Observation 1: No significant on RRM impact based on the WID updating in RAN#105. Proposal 1: RAN4 to only consider Tx/Rx in occasions of L3 measurement. Proposal 2: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. Observation 2: In some scenarios, the RRM measurement performance (e.g. latency, accuracy) may have higher priority than XR service to guarantee the mobility performance. Observation 3: NW shall have clear expectation on RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception. Proposal 3: RAN4 shall define corresponding RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception. Observation 4: Regarding the RRM behavior/performance when the measurement occasion is skipped/canceled, following options can be taken as starting point: - Alt1: Same requirement still applies - Alt2: The measurement delay to be extended Observation 5: From NW point views, it is safe to support this feature when it can provide additional scheduling opportunity but will not impacting the mobility performance. Observation 6: UAI for measurement occasion may be helpful for enabling transmission/reception in MGs/SMTC with restriction. And the flexibility of scheduling shall be guaranteed as NW can properly scheduling the data transmission/reception. Proposal 4: RAN4 to introduce UAI at least for following case: - UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when there is no impact on L3 measurement performance Proposal 5: RAN4 to discuss whether to introduce UAI for following case: - UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when the measurement delay could be extended. FFS on the impact on delay extension. Observation 7: The dynamic indication (Alt.1) can give NW more flexibility on scheduling. Proposal 6: RAN4 reply LS that: The dynamic approach (Alt.1) should be supported from RAN4 perspective, which can give more flexibility on NW scheduling. RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion. |
| R4-2415935 | NTT DOCOMO, INC. | Discussions on RRM core requirements for XR Proposal 1: RAN4 to work based on the following table Cap types; First prio; Second prio; Don’t include. Type 1 (Gap(s) configured via GapConfig without suffix); Should be first priority because most basic MG and will be the baseline for XR as well. Type 2 (Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17); Can be excluded. NCSG; Should be second priority because it can be a solution for XR. MUSIM; Can be excluded. Positioning; Can be excluded. Rel 18 concurrent gap; Can be excluded. PreConfigured; Should be second priority because it can be a solution for XR. Proposal 2: No UE assistance information related to measurement occasion is needed. |
| R4-2416125 | OPPO | Discussion on RRM requirements for XR Observation 1: Alt 1-1 in the LS R1-2407561 is similar as Pre-MG deactivation upon DCI-based BWP switch and no technical issue is identified in RAN4. Proposal 1: Deprioritize Tx/Rx on the scheduling restricted symbols due to L1 measurement. Proposal 2: Deprioritize Tx/Rx on the scheduling restricted symbols due to L3 measurement without gaps. Proposal 3: To reflect the RRM impacts due to measurement cancellation/skipping, extend the delay and keep the accuracy requirements could be considered as the baseline. Proposal 4: In Rel-19 XR WI, - Consider type-1 gap, type-2 gap (Rel-17 concurrent gap), NCSG and pre-MG with first priority - Postpone Rel-18 concurrent gaps - Exclude MUSIM gap and positioning gap Proposal 5: If type-2 gap (Rel-17 concurrent gap) is considered, the execution order between Rel-17 priority-based gap collision rule and Rel-19 gap skipping rule should be discussed. Proposal 6: Alt 1-1 in the LS R1-2407561 could be confirmed from RAN4 perspective. Proposal 7: The minimum time offset X between DCI indication to skip and the skipped measurement occasion should be DCI decoding time defined in TS 38.214 + [5]ms time margin. |
| R4-2416203 | Ericsson | On RRM core requirements for XR - Proposal 1: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR. - Proposal 2: For UEs not equipped with an additional rx chain for XR, RAN4 to discuss possible enhancements to the legacy RRM requirements to account for the UE XR operation, e.g., discuss the options: - pre-defined fixed extended RRM measurement period for all UEs under XR operation, - dynamically extendable RRM measurement period for all UEs under XR operation, - pre-defined fixed extended RRM measurement period only for UEs under XR operation in poor radio conditions, - dynamically extendable RRM measurement period only for UEs under XR operation in poor radio conditions. NOTE: 1) and 2) are based on Approach 2, while 3) and 4) are based on Approach 3. - Proposal 3: RAN4 to discuss and agree on the applicable scenarios: - FR2-1, - FFS: FR1; - NR SA, - FFS: MR-DC, including NR-DC, EN-DC, and NE-DC; - FFS: Carrier aggregation. - Proposal 4.1: RAN4 considers Type 1 MGs for MG cancelling in Rel-19 XR enhancements, as a baseline. - Proposal 4.2: MGs associated with PRS (positioning) measurements shall not be cancelled. - Proposal 4.3: MUSIM MGs shall not be cancelled. - Proposal 4.4: FFS: Network controlled pre-configured MGs, at least until the core requirements impacts are settled for Type 1 MGs. - Proposal 4.5: FFS: Concurrent MGs, at least until the core requirements impacts are settled for Type 1 MGs. - Proposal 5: RAN4 to agree on the RRM measurements which may be impacted, e.g., intra/inter-frequency, inter-RAT, specific-purpose measurements, e.g., positioning, etc. - Proposal 6: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g., - minimum MGL, - maximum MGL, - maximum MGRP, etc. - Proposal 7: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements. - Proposal 8: RAN4 to discuss the impact of DRX on MG cancellation. - Proposal 9 (UAI): RAN4 will not specify any explicit or implicit requirement on gNB behaviour in relation to the UE assistance data. - Proposal 10 (UAI): RAN4 to discuss whether the information related to the maximum number of measurement occasions allowed for reallocated to XR over a time can be defined in RAN4 specification. - Proposal 11 (UAI): UE assistance containing patterns still cannot guarantee that gNB configuration will follow the UE pattern, since the configuration is ultimately the network decision. - Proposal 12 (UAI): RAN4 considers the impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side. - Proposal 13 (time offset): RAN4 to discuss the range of acceptable time offset values between DCI reception and the intended MG, considered for XR operation, including: - the minimum time offset value, and - the maximum time offset value. |
| R4-2416454 | MediaTek inc. | Discussion on RRM requirements for XR Proposal 1: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact. Proposal 2: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic. Proposal 3: RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window. Proposal 4: In order to reduce the measurement delay, RAN4 shall specify that the UE operating with XR is not required to measure 2G/3G frequency layers. Proposal 5: RAN4 shall consider Type-1 MG with high priority, while all other measurement gaps/interruption can be considered at later stage. Proposal 6: RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic. Proposal 7: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution. Proposal 8: In order to maintain short time offset for gap cancellation, RAN4 RRM shall not consider 3G/2G for UE operating with XR data traffic. |
| R4-2416468 | Nokia | Discussion on XR ph3 RRM requirements Observation 1: FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS. Proposal 1: Measurement skipping apply for FR1 and FR2-1 measurements Proposal 2: RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Application of the requirements to other types of gap should be dealt with in a second stage. Proposal 3: Once the impact on requirements for measurement skipping for Type 1 MG has been agreed, Type 2 MG, NCSG and MG for positioning should be considered as 2nd priority. Proposal 4: MUSIM, Preconfigured, and Rel-18 concurrent gaps are not considered for Rel 19 XR ph3 enhancements. Proposal 5: Measurement skipping is not applicable for MUSIM, Preconfigured, and Rel-18 Concurrent gaps measurement gap based on the Rel 19 XR DCI skipping indication. Observation 2: RAN1 is defining enhancements for skipping measurements that cause scheduling restrictions which are agnostic to the types of measurement gaps or scheduling restrictions. Observation 3: Intra-frequency measurements always cause scheduling restrictions in FR2, around SSB symbols to measure. Observation 4: Interruptions will cause scheduling problems for latency sensitive traffic. Observation 5: Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time. Observation 6: Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions. Proposal 6: Define requirements for measurement skipping only for measurement with gaps. Observation 7: In Rel-16, RAN4 defined requirements for NR_unlic, where RRM measurement delay is extended due to CCA/LBT failures without impact to measurement accuracy. Proposal 7: Measurement skipping is reflected on RRM measurements via extension of measurement delay. Proposal 8: Reuse existing measurement accuracy requirements. Proposal 9: RAN4 to define requirements such that the network can configure which frequency layers are measured with higher priority Observation 8: Once the UE has realized that it has received a RRM measurement skipping indication command, the UE only needs to continue normal operation without executing complex RF retuning and gap activation procedures. Observation 9: Shorter time between DCI skipping command to measurement gap enables better XR capacity for the same skipping ratio. Proposal 10: RAN4 shall define processing requirements for a UE to act on a DCI with RRM measurement skipping indication as a fraction of a slot. Observation 10: UAI only makes sense if UAI information is used to provide skipping implies on no extension of measurement delay and no impact on measurement accuracy. Observation 11: Discuss the feasibility and accuracy of UE reporting UAI in relation to XR capacity; Observation 12: Discuss the feasibility and accuracy of UE reporting UAI in relation to measurement performance. Observation 13: Discuss the benefits of UE reporting UAI in relation to XR capacity; Observation 14: Discuss the benefits of UE reporting UAI in relation to measurement performance. Proposal 11: When considering the feasibility of UAI related to measurement occasions, it should be ensured that measurement delay and measurement accuracy are not impacted when measurement skipping is influenced by UAI. Proposal 12: Any proposals for UAI related to measurement occasions should be justified by clear performance benefit either in terms of measurement accuracy and delay or XR capacity. Proposal 13: For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network when the condition is met (i.e. defined RSRP threshold is exceeded). Proposal 14: For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network that it is stopping doing RRM measurements because the s-MeasureConfig condition is no longer fulfilled. Observation 15: RAN4 already defined requirements for features where dynamic adaptation of measurements were necessary, e.g. NR-U and Pre-configured gaps. Proposal 15: Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline. |
| R4-2416470 | Qualcomm Incorporated | RRM requirements for XR enhancements Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. Proposal 2: Following DCI-based pre-configured MG activation/deactivation delay, specify the minimum offset X between the indication to skip and the measurement occasion to be skipped as 5 ms. Proposal 3: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L1 measurements. Proposal 4: RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network. - For DCI based dynamic indication, follow the NR-U approach to extend the L3 delays by the number of MGs deactivated during the period. Proposal 5: When overlapping between gaps/restrictions that are caused by RRM measurements occurs, the dynamic indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied after the overlapping is resolved. Proposal 6: RAN4 to consider introducing UAI to indicate one or more of the following aspects for MG deactivation - Maximum number of consecutive MGs that can be skipped - Maximum number of MGs that can be skipped within a duration - MG type that can be skipped |

## Open issues summary

### Sub-topic 1-1 Scenarios for measurement skipping

#### Issue 1-1-1: Deployment scenarios

- Proposals

  - Proposal 1 (Ericsson): RAN4 to discuss and agree on the applicable scenarios:

    - FR2-1,

      - FFS: FR1;

    - NR SA,

      - FFS: MR-DC, including NR-DC, EN-DC, and NE-DC;

      - FFS: Carrier aggregation."

  - Proposal 2 (Mediatek): In order to maintain short time offset for gap cancellation, RAN4 RRM shall not consider 3G/2G for UE operating with XR data traffic.

  - Proposal 3 (Nokia): Measurement skipping apply for FR1 and FR2-1 measurements

  - Proposal 4 (Ericsson): To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

- Recommended WF

  - Please discuss which options are agreeable.

#### Issue 1-1-2: Types of measurements to be considered

- Proposals

  - Proposal 1: Intra-frequency

  - Proposal 2: Inter- frequency

  - Proposal 3: Inter-RAT

    - Proposal 3a: LTE inter-RAT

    - Proposal 3b: 2G/3G inter-RAT

  - Proposal 4: specific-purpose measurements, e.g., positioning, etc.

- Recommended WF

  - Please discuss which measurement types RAN4 should discuss.

#### Issue 1-1-3: Measurements without gaps

- Proposals

  - Option 1 (Apple, ZTE): support measurements without gaps

  - Option 2 (Mediatek, OPPO, vivo): de-prioritize measurements without gaps

  - Option 3 (Nokia): measurement skipping do not apply for measurements without gaps

- Recommended WF

  - Please discuss the options regarding measurements without gaps.

#### Issue 1-1-4: L1 measurements

- Proposals

  - Option 1 (CMCC)– support skipping of L1 measurements

  - Option 2 (Apple)– support skipping of RLM and BFD measurements

  - Option 3 (OPPO)– de-prioritize L1 measurements

  - Option 4 (Huawei, Qualcomm, vivo, ZTE, MTK)- measurement skipping do not apply for L1 measurements

- Recommended WF

  - Please discuss the options regarding L1 measurements.

#### Issue 1-1-5: L1/L2 triggered mobility measurements

- Proposals

  - Option 1 (ZTE) – support skipping of L1 measurements for mobility within and outside gap

  - Option 2 – de-prioritize L1 measurements for mobility

  - Option 3 - measurement skipping do not apply for L1 measurements for mobility

- Recommended WF

  - Please discuss the options regarding L1 measurements for mobility.

### Sub-topic 1-2 Measurements with gaps

#### Issue 1-2-1: General approach for defining requirements for measurements with gaps

- Proposals

  - Proposal 1 (Huawei): RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

  - Proposal 2 (Nokia): RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Application of the requirements to other types of gap should be dealt with in a second stage.

- Recommended WF

  - The selection of the measurement gap types is a topic that has been taking long discussion. Please consider if a combination of proposals 1 and 2 can be considered as a baseline.

#### Issue 1-2-2: Types of measurement gaps

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | Apple, NTT DOCOMO, Ericsson, Interdigital, Mediatek, Nokia, OPPO, vivo, ZTE |  |  |
| Type 2 | Apple, OPPO, vivo | Interdigital, Nokia | NTT DOCOMO, ZTE |
| NCSG | Apple, OPPO | NTT DOCOMO, Nokia | ZTE |
| MUSIM |  |  | NTT DOCOMO, Ericsson, Nokia, OPPO, ZTE |
| Positioning |  | Nokia | NTT DOCOMO, Ericsson, OPPO, ZTE |
| Rel 18 concurrent gap |  | OPPO | NTT DOCOMO, Nokia, ZTE |
| PreConfigured | Apple, OPPO, vivo | NTT DOCOMO, ZTE | Nokia |

- Recommended WF

  - The selection of the measurement gap types is a topic that has been taking long discussion. Please consider if agreement on Issue 1-2-1 is possible.

#### Issue 1-2-3: How to consider gap types that are not included for RRM requirements

- Proposal

  - Proposal 1 (Nokia): Measurement skipping based on the Rel 19 XR DCI skipping indication is not applicable for gap types labelled as “Don’t include” in Issue 1-2-2.

- Recommended WF

  - Please discuss the meaning of “Don’t include" in issue 1-2-2.

### Sub-topic 1-3 XR skipping requirements impact

#### Issue 1-3-1: How to capture impact of measurement skipping

- Proposals related to measurement delay

  - Proposal 1 (Huawei): RAN4 shall define corresponding RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception.

  - Proposal 2 (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

  - Proposal 3 (Mediatek): RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

  - Proposal 4 (Nokia, MTK): Measurement skipping is reflected on RRM measurements via extension of measurement delay.

  - Proposal 5 (OPPO): To reflect the RRM impacts due to measurement cancellation/skipping, extend the delay and keep the accuracy requirements could be considered as the baseline.

  - Proposal 6 (Qualcomm): RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

    - For DCI based dynamic indication, follow the NR-U approach to extend the L3 delays by the number of MGs deactivated during the period."

  - Proposal 7 (vivo): RAN4 to study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception after solution for enabling Rx/Tx in gaps is clear.

    - When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

- Proposals related to measurement accuracy

  - Proposal 7 (Nokia): Reuse existing measurement accuracy requirements.

- Recommended WF

  - Considering that the proposals have a general line, please consider if Proposal 2 and Proposal 7 can be agreed as a baseline.

#### Issue 1-3-2: Principles for measurement delay calculation

- Proposals

  - Proposal 1 (Mediatek): RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

  - Proposal 2 (ZTE): To guarantee the measurement performance when the MG/restriction skipping happens, one straightforward solution is to introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be considered when determining the exact scaling factor.

- Recommended WF

  - Discuss the proposals

#### Issue 1-3-3: Pre-defined vrs dynamic delay calculation

- Proposals

  - Proposal 1: measurement delay extension due to measurement skipping is calculated as a pre-defined fixed extended RRM measurement period,

    - Proposal 1a: pre-defined fixed extended RRM measurement period only for UEs under XR operation in poor radio conditions,

  - Proposal 2: measurement delay extension due to measurement skipping is calculated as a dynamically extendeable RRM measurement period,

    - Proposal 2a: dynamically extendable RRM measurement period only for UEs under XR operation in poor radio conditions.

- Recommended WF

  - Discuss the proposals

#### Issue 1-3-4: RRM delay calculation based on radio condition

- Proposals

  - Option 1: measurement delay extension due to measurement skipping is different for good or poor radio condition

  - Option 2: measurement delay extension due to measurement skipping is independent of radio condition

- Recommended WF

  - Discuss the options

#### Issue 1-3-5: Impact on Per-FR gaps

- Proposals

  - Proposal 1 (vivo): It needs to be clarified that when per-FR gaps are configured, the indication by DCI of the particular gap to be skipped is for which gap pattern(s).

  - Proposal 2 (ZTE): For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

- Recommended WF

  - Discuss the proposals

#### Issue 1-3-6: Impact on different frequency layers

- Proposals

  - Proposal 1 (Mediatek): In order to reduce the measurement delay, RAN4 shall specify that the UE operating with XR is not required to measure 2G/3G frequency layers.

  - Proposal 2 (Nokia): RAN4 to define requirements such that the network can configure which frequency layers are measured with higher priority

- Recommended WF

  - Discuss the proposals

#### Issue 1-3-8: Conditions for measurement skipping

- Proposals

  - Proposal 1 (Apple): max number of occasions cancelled during a measurement period shall be defined.

  - Proposal 2 (Ericsson): RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g.,

    - minimum MGL,

    - maximum MGL,

    - maximum MGRP, etc.

  - Proposal 3 (Ericsson): RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

- Recommended WF

  - Discuss the proposals

#### Issue 1-3-9: DRX

- Proposal

  - Proposal 1 (Ericsson): RAN4 to discuss the impact of DRX on MG cancellation.

- Recommended WF

  - Discuss the proposal

#### Issue 1-3-10: Concurrent gaps

- Proposal

  - Proposal 1 (OPPO): If type-2 gap (Rel-17 concurrent gap) is considered, the execution order between Rel-17 priority-based gap collision rule and Rel-19 gap skipping rule should be discussed.

  - Proposal 2 (Qualcomm): When overlapping between gaps/restrictions that are caused by RRM measurements occurs, the dynamic indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied after the overlapping is resolved.

- Recommended WF

  - Discussion depends on the outcome of which gaps are considered.

#### Issue 1-3-11: Other proposals

- Proposals

  - Proposal 1 (Apple): RAN4 can continue discussion on other solution to address negative impact on measurement due to cancellation for XR traffic.

  - Proposal 2 (vivo): If supported, RAN4 to study whether and how to define new core requirements when some of SMTC occasions that need to be used for measurements outside gaps are enabled for data transmission/reception when scheduling restrictions exists.

  - Proposal 3 (vivo): RAN4 to study possible restrictions on dynamic indication by DCI, if it is confirmed, to achieve good trade-off between data deliver delay and mobility performance.

- Recommended WF

  - Discuss the proposals.

### Sub-topic 1-4 Timeline for measurement skipping processing

#### Issue 1-4-1: Timeline requirement for measurement gap skipping

- Proposals degrading minimum time offset between indication to skip and skipped measurement gap

  - Proposal 1 (Apple): 7 ms

  - Proposal 2 (CMCC): N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window  (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

  - Proposal 3 (Interdigital): assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

  - Proposal 4 (Nokia): Fraction of a slot

  - Proposal 5 (OPPO): DCI decoding time defined in TS 38.214 + [5]ms

  - Proposal 6 (Qualcomm, vivo): 5 ms

  - Proposal 7 (ZTE): besides the DCI decoding delay, no additional processing delay is needed.

  - Proposal 8 (MTK): Proposal 8: In order to maintain short time offset for gap cancellation, RAN4 RRM shall not consider 3G/2G for UE operating with XR data traffic.

- Recommended WF

  - Discussion needed

#### Issue 1-4-2: Timeline requirement for skipping measurements without gaps

- Proposals

  - Proposal 1 (ZTE): For the time offset between the dynamic indication and the first applicable scheduling restriction, besides the DCI decoding delay, no additional processing delay is needed.

- Recommended WF

  - Discussion should take place after decision of whether measurements without gaps are considered for XR enhancements

### Sub-topic 1-5 Need/feasibility of UE assistance information

Sub-topic description

RAN1 has sent an LS (R1-2405736) to RAN4, requesting RAN4 to decide whether to introduce any UE assistance information related to measurement occasions. This sub-topic includes issues relating to the need and feasibility of such UE assistance information.

Open issues and candidate options before meeting:

#### Issue 1-5-1: General on UAI

- Proposals

  - Option 1 (Mediatek): RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution.

  - Option 2 (ZTE): The UAI is beneficial in the XR scenario, and the regular UAI mechanism could be applied, i.e. UE reports UAI when necessary, and NW makes decision.

- Recommended WF

  - Please discuss which option can be agreed.

#### Issue 1-5-2: Expected gNB behaviour

- Proposals

  - Proposal 1 (Ericsson): RAN4 will not specify any explicit or implicit requirement on gNB behaviour in relation to the UE assistance data.

  - Proposal 2 (Ericsson): UE assistance containing patterns still cannot guarantee that gNB configuration will follow the UE pattern, since the configuration is ultimately the network decision.

- Recommended WF

  - Please discuss if the proposal is agreed. Please have in mind that the LS from RAN1 already includes an agreement stating that there is no mandated gNB behaviour expected in response to any UE assistance information that might be specified in this WID.

#### Issue 1-5-3: Information related to measurement occasions

- Proposals

  - Option 1 (Nokia):

    - When considering the feasibility of UAI related to measurement occasions, it should be ensured that measurement delay and measurement accuracy are not impacted when measurement skipping is influenced by UAI.

    - Any proposals for UAI related to measurement occasions should be justified by clear performance benefit either in terms of measurement accuracy and delay or XR capacity.

  - Option 2 (Apple): UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

  - Option 3 (NTT DOCOMO): No UE assistance information related to measurement occasion is needed.

  - Option 4 (Meta): UE assistance information related to measurement occasions should be considered.

  - Option 5 (vivo):

    - UE should provide information about gaps not used for measurement for enabling transmission/reception in these gaps.

    - If supported, UE should provide information about SMTC occasions not used for L3 measurement for enabling transmission/reception on the SMTC occasions.

- Recommended WF

  - Please discuss the options above

#### Issue 1-5-4: If Information related to measurement occasions is introduced, which aspects should be considered

In the following, the proposals regarding what should be considered for UAI related to measurement occasions are listed. Please consider that this issue depends on Issue 1-5-3.

- Proposals

  - Proposal 1 (CMCC): it is proposed to introduce UE assistance information related to measurement occasions, e.g. the number of needed measurement occasions for measurement or the number of measurement occasions which can be skipped.

  - Proposal 2 (Ericsson): RAN4 to discuss whether the information related to the maximum number of measurement occasions allowed for reallocated to XR over a time can be defined in RAN4 specification.

  - Proposal 3 (Huawei): RAN4 to introduce UAI at least for following case:

    - UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when there is no impact on L3 measurement performance"

RAN4 to discuss whether to introduce UAI for following case:

    - UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when the measurement delay could be extended. FFS on the impact on delay extension."

  - Proposal 5 (Qualcomm): RAN4 to consider introducing UAI to indicate one or more of the following aspects for MG deactivation

    - Maximum number of consecutive MGs that can be skipped

    - Maximum number of MGs that can be skipped within a duration

    - MG type that can be skipped

  - Proposal 6 (vivo):

    - RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information.

    - If supported, RAN4 to discuss the information about SMTC occasions not used for measurement are reported as UE capability or UE assistance information.

    - The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as UE assistance information.

- Recommended WF

  - Please discuss the proposals

#### Issue 1-5-5: Information related to channel conditions

- Proposals

  - Proposal 1 (Nokia):

    - For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network when the condition is met (i.e. defined RSRP threshold is exceeded).

    - For UEs configured with search threshold (s-MeasureConfig), the UE shall inform the network that it is stopping doing RRM measurements because the s-MeasureConfig condition is no longer fulfilled.

  - Proposal 2 (ZTE): Within all candidates of UAI in RAN1 LS, we believe the following types are useful and feasible:

    - UE assistance information related to channel conditions:

    - e.g. RSRP is below/above search threshold (s-MeasureConfig);

- Recommended WF

  - Discus the proposals.

#### Issue 1-5-6: Information related to mobility

- Proposals

  - Proposal 1 (ZTE): Within all candidates of UAI in RAN1 LS, we believe the following types are useful and feasible:

    - UE assistance information related to traffic:

    - e.g. PSI (PDU set importance);.

- Recommended WF

  - Discus the proposal.

#### Issue 1-5-7: Information related to traffic

- Proposals

  - Proposal 1 (ZTE): Within all candidates of UAI in RAN1 LS, we believe the following types are useful and feasible:

    - UE assistance information related to UE mobility:

    - e.g. L3 parameters related to mobility, e.g., static or not.

- Recommended WF

  - Discus the proposal.

#### Issue 1-5-8: Requirements impact of Information related to measurement occasions

If RAN4 decides that UAI information related to measurement occasions, how does that reflect RRm requirements?

- Proposals

  - Proposal 1 (CMCC): it is proposed that

    - Apply legacy measurement delay and accuracy requirements for the following conditions:

      - the number of skipped measurement occasions indicated by network is within the number of unused measurement occasion reported by UAI, or

      - UE in bad radio conditions (e.g. at cell edge)

    - Extend measurement delay while keep legacy accuracy requirements unchanged for the following conditions:

      - there is no UAI, or

      - The number of skipped measurement occasions indicated by network is larger than the number of unused measurement occasion reported by UAI, or

      - UE in good radio conditions"

  - Proposal 2 (vivo): When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

- Recommended WF

  - Discus the proposals.

#### Issue 1-5-9: Other details on UAI Information related to measurement occasions

- Proposals

  - Proposal 1 (vivo):

    - If supported, the UE assistance information about SMTC occasions not used for measurement should be provided on each CC/MO.

    - UE assistance information/UE capability regarding gaps/SMTC occasions (if supported) not used for measurement is reported in response to RRCReconfiguration.

- Recommended WF

  - Discus the proposals.

#### Issue 1-5-10: DRX

- Proposals

  - Proposal 1 (Ericsson): RAN4 considers the impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side.

  - Proposal 2 (vivo): RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

- Recommended WF

  - Discus the proposals.

#### Issue 1-5-10: LS response

- Proposals

  - Proposal 1 (Ericsson) draft LS response in R4-2416204.

### Sub-topic 1-6 Additional skipping schemes

#### Issue 1-6-1: Semi-static schemes

- Proposals

  - Proposal 1 (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

    - Proposal 1a: (Huawei): RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

  - Proposal 2 (vivo): To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC

- Recommended WF

  - Please discuss the proposals, considering the existing agreements from RAN1.

#### Issue 1-6-2: Partial measurement skipping

- Proposals

  - Proposal 1 (ZTE): Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered.

- Recommended WF

  - Please discuss the proposal.

#### Issue 1-6-3: Network indication

- Proposals

  - Proposal 1 (vivo):

    - NW may indicate the percentage of measurement gaps not used for measurement for UE to provide information about gaps not used for measurement.

    - If supported, NW may indicate the percentage about SMTC occasions not used for L3 measurement outside gaps for UE to provide information about SMTC occasions not used for measurement.

- Recommended WF

  - Please discuss the proposal.

### Sub-topic 1-7 Reply LS to RAN1

#### Issue 1-7-1: Response on RAN1 working assumption and time offset

- Proposals

  - Option 1:

    - Option 1a (CMCC): for RAN1 LS on solutions to enable transmission/reception in gaps/restrictions, it is proposed to confirm RAN1 working assumption.

    - Option 1b (Nokia): Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline

    - Option 1c (OPPO): Alt 1-1 in the LS R1-2407561 could be confirmed from RAN4 perspective.

  - Option 2 (Huawei) RAN4 reply LS that:

    - The dynamic approach (Alt.1) should be supported from RAN4 perspective, which can give more flexibility on NW scheduling.

    - RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

    - Whether UE supports both approaches can be up to UE capability discussion."

  - Option 3 (Ericsson, draft LS in R4-2416205) RAN4 replies that:

    - RAN4 to discuss the range of acceptable time offset values between DCI reception and the intended MG and will agree on:

      - the minimum time offset value, and

      - the maximum time offset value.

    - It is RAN4 understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received.

- Recommended WF

  - Please discuss the options.