| 3GPP TR 38.719-03-01 V0.2.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-19 NR Inter-band CA/DC configurations including inter band CA for 3 different bands DL with x different bands UL (x=1,2) (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	13

1	Scope	15

2	References	15

3	Definitions of terms, symbols and abbreviations	15

3.1	Terms	15

3.2	Symbols	15

3.3	Abbreviations	16

4	Background	16

4.1	Introduction	16

4.2	TR Maintenance	16

5	Three bands within FR1 Carrier Aggregation: Specific Band Combination Part	16

5.0	CA_nX-nY-nZ	16

5.0.1	Common for 1 band UL and 2 bands UL CA	16

5.0.1.1	Operating bands for CA	16

5.0.1.2	Channel bandwidths per operating band for CA	17

5.0.1.3	∆TIB,c and ∆RIB,c values	17

5.0.2	Specific for 2 bands UL CA	18

5.0.2.1	UE co-existence studies	18

5.0.2.1.1	Co-existence studies for 2UL band with 1CC per band	18

5.0.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	18

5.0.2.2	REFSENS requirements	19

5.1	CA_n28-n41-n79	19

5.1.1	Common for 1 band UL and 2 bands UL CA	19

5.1.1.1	Operating bands for CA	19

5.1.1.2	Channel bandwidths per operating band for CA	20

5.1.1.3	∆TIB,c and ∆RIB,c values	20

5.1.2	Specific for 2 bands UL CA	20

5.1.2.1	UE co-existence studies	20

5.1.2.1.1	Co-existence studies for 2UL band with 1CC per band	20

5.1.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	20

5.1.2.2	REFSENS requirements	21

5.2	CA_n8-n20-n75	21

5.2.1	Common for 1 band UL and 2 bands UL CA	21

5.2.1.1	Operating bands for CA	21

5.2.1.2	Channel bandwidths per operating band for CA	22

5.2.1.3	∆TIB,c and ∆RIB,c values	22

5.2.2	Specific for 2 bands UL CA	22

5.2.2.1	UE co-existence studies	22

5.2.2.1.1	Co-existence studies for 2UL band with 1CC per band	22

5.2.2.2	REFSENS requirements	23

5.3	CA_n8-n28-n75	23

5.3.1	Common for 1 band UL and 2 bands UL CA	23

5.3.1.1	Operating bands for CA	23

5.3.1.2	Channel bandwidths per operating band for CA	24

5.3.1.3	∆TIB,c and ∆RIB,c values	24

5.3.2	Specific for 2 bands UL CA	24

5.3.2.1	UE co-existence studies	24

5.3.2.1.1	Co-existence studies for 2UL band with 1CC per band	24

5.3.2.2	REFSENS requirements	25

5.4	CA_n20-n28-n75	25

5.4.1	Common for 1 band UL and 2 bands UL CA	25

5.4.1.1	Operating bands for CA	25

5.4.1.2	Channel bandwidths per operating band for CA	26

5.4.1.3	∆TIB,c and ∆RIB,c values	26

5.4.2	Specific for 2 bands UL CA	26

5.4.2.1	UE co-existence studies	26

5.4.2.1.1	Co-existence studies for 2UL band with 1CC per band	26

5.4.2.2	REFSENS requirements	27

5.5	CA_n3-n7-n75	27

5.5.1	Common for 1 band UL and 2 bands UL CA	27

5.5.1.1	Operating bands for CA	27

5.5.1.2	Channel bandwidths per operating band for CA	28

5.5.1.3	∆TIB,c and ∆RIB,c values	28

5.5.2	Specific for 2 bands UL CA	28

5.5.2.1	UE co-existence studies	28

5.5.2.1.1	Co-existence studies for 2UL band with 1CC per band	28

5.5.2.2	REFSENS requirements	29

5.6	CA_n3-n75-n78	30

5.6.1	Common for 1 band UL and 2 bands UL CA	30

5.6.1.1	Operating bands for CA	30

5.6.1.2	Channel bandwidths per operating band for CA	30

5.6.1.3	∆TIB,c and ∆RIB,c values	30

5.6.2	Specific for 2 bands UL CA	30

5.6.2.1	UE co-existence studies	30

5.6.2.1.1	Co-existence studies for 2UL band with 1CC per band	31

5.6.2.2	REFSENS requirements	31

5.7	CA_n7-n75-n78	32

5.7.1	Common for 1 band UL and 2 bands UL CA	32

5.7.1.1	Operating bands for CA	32

5.7.1.2	Channel bandwidths per operating band for CA	32

5.7.1.3	∆TIB,c and ∆RIB,c values	33

5.7.2	Specific for 2 bands UL CA	33

5.7.2.1	UE co-existence studies	33

5.7.2.1.1	Co-existence studies for 2UL band with 1CC per band	33

5.7.2.2	REFSENS requirements	33

5.8	CA_n1-n7-n75	34

5.8.1	Common for 1 band UL and 2 bands UL CA	34

5.81.1	Operating bands for CA	34

5.8.1.2	Channel bandwidths per operating band for CA	35

5.8.1.3	∆TIB,c and ∆RIB,c values	35

5.8.2	Specific for 2 bands UL CA	35

5.8.2.1	UE co-existence studies	35

5.8.2.1.1	Co-existence studies for 2UL band with 1CC per band	35

5.8.2.2	REFSENS requirements	36

5.9	CA_n1-n3-n7	37

5.9.1	Common for 1 band UL and 2 bands UL CA	37

5.9.1.1	Operating bands for CA	37

5.9.1.2	Channel bandwidths per operating band for CA	37

5.9.1.3	∆TIB,c and ∆RIB,c values	37

5.9.2	Specific for 2 bands UL CA	37

5.9.2.1	UE co-existence studies	37

5.9.2.1.1	Co-existence studies for 2UL band with 1CC per band	38

5.9.2.2	REFSENS requirements	40

5.10	CA_n41-n66-n71	40

5.10.1	Common for 1 band UL and 2 bands UL CA	40

5.10.1.1	Operating bands for CA.	40

5.10.1.2	Channel bandwidths per operating band for CA	40

5.10.2	Specific for 2 bands UL CA	40

5.10.2.1	UE co-existence studies	40

5.10.2.1.1	Co-existence studies for 2UL band with 1CC per band	40

5.10.2.2	REFSENS requirements	41

5.11	CA_n1-n41-n78	41

5.11.1	Common for 1 band UL and 2 bands UL CA	41

5.11.1.1	Operating bands for CA	41

5.11.1.2	Channel bandwidths per operating band for CA	42

5.11.1.3	∆TIB,c and ∆RIB,c values	42

5.11.2	Specific for 2 bands UL CA	42

5.11.2.1	UE co-existence studies	42

5.11.2.2	REFSENS requirements	44

5.12	CA_n1-n5-n8	45

5.12.1	Common for 1 band UL and 2 bands UL CA	45

5.12.1.1	Operating bands for CA	45

5.12.1.2	Channel bandwidths per operating band for CA	45

5.12.1.3	∆TIB,c and ∆RIB,c values	45

5.13	CA_n3-n5-n8	46

5.13.1	Common for 1 band UL and 2 bands UL CA	46

5.13.1.1	Operating bands for CA	46

5.13.1.2	Channel bandwidths per operating band for CA	46

5.13.1.3	∆TIB,c and ∆RIB,c values	46

5.14	CA_n25-n29-n77	47

5.14.1	Common for 1 band UL and 2 bands UL CA	47

5.14.1.1	Operating bands for CA	47

5.14.1.2	Channel bandwidths per operating band for CA	47

5.14.1.3	∆TIB,c and ∆RIB,c values	47

5.14.2	Specific for 2 bands UL CA	48

5.14.2.1	UE co-existence studies	48

5.14.2.1.1	Co-existence studies for 2UL band with 1CC per band	48

5.14.2.2	REFSENS requirements	49

5.15	CA_n1-n75-n78	49

5.15.1	Common for 1 band UL and 2 bands UL CA	49

5.15.1.1	Operating bands for CA	49

5.15.1.2	Channel bandwidths per operating band for CA	49

5.15.1.3	∆TIB,c and ∆RIB,c values	49

5.15.2	Specific for 2 bands UL CA	49

5.15.2.1	UE co-existence studies	49

5.15.2.1.1	Co-existence studies for 2UL band with 1CC per band	49

5.15.2.2	REFSENS requirements	50

5.16	CA_n1-n28-n40	50

5.16.1	Common for 1 band UL and 2 bands UL CA	50

5.16.1.1	Operating bands for CA	50

5.16.1.2	Channel bandwidths per operating band for CA	51

5.16.1.3	∆TIB,c and ∆RIB,c values	51

5.16.2	Specific for 2 bands UL CA	51

5.16.2.1	UE co-existence studies	51

5.16.2.1.1	Co-existence studies for 2UL band with 1CC per band	51

5.16.2.2	REFSENS requirements	54

5.17	CA_n1-n20-n71	54

5.17.1	Common for 1 band UL and 2 bands UL CA	54

5.17.1.1	Operating bands for CA	54

5.17.1.2	Channel bandwidths per operating band for CA	55

5.17.1.3	∆TIB,c and ∆RIB,c values	55

5.17.2	Specific for 2 bands UL CA	55

5.17.2.1	UE co-existence studies	55

5.17.2.1.1	Co-existence studies for 2UL band with 1CC per band	55

5.17.2.2	REFSENS requirements	58

5.18	CA_n1-n20-n78	58

5.18.1	Common for 1 band UL and 2 bands UL CA	58

5.18.1.1	Operating bands for CA	58

5.18.1.2	Channel bandwidths per operating band for CA	59

5.18.1.3	∆TIB,c and ∆RIB,c values	59

5.18.2	Specific for 2 bands UL CA	59

5.18.2.1	UE co-existence studies	59

5.18.2.1.1	Co-existence studies for 2UL band with 1CC per band	59

5.18.2.2	REFSENS requirements	62

5.19	CA_n20-n41-n71	62

5.19.1	Common for 1 band UL and 2 bands UL CA	62

5.19.1.1	Operating bands for CA	62

5.19.1.2	Channel bandwidths per operating band for CA	63

5.19.1.3	∆TIB,c and ∆RIB,c values	63

5.19.2	Specific for 2 bands UL CA	63

5.19.2.1	UE co-existence studies	63

5.19.2.1.1	Co-existence studies for 2UL band with 1CC per band	63

5.19.2.2	REFSENS requirements	66

5.20	CA_n20-n41-n77	66

5.20.1	Common for 1 band UL and 2 bands UL CA	66

5.20.1.1	Operating bands for CA	66

5.20.1.2	Channel bandwidths per operating band for CA	67

5.20.1.3	∆TIB,c and ∆RIB,c values	67

5.20.2	Specific for 2 bands UL CA	68

5.20.2.1	UE co-existence studies	68

5.20.2.1.1	Co-existence studies for 2UL band with 1CC per band	68

5.20.2.2	REFSENS requirements	70

5.21	CA_n20-n41-n78	71

5.21.1	Common for 1 band UL and 2 bands UL CA	71

5.21.1.1	Operating bands for CA	71

5.21.1.2	Channel bandwidths per operating band for CA	72

5.21.1.3	∆TIB,c and ∆RIB,c values	72

5.21.2	Specific for 2 bands UL CA	72

5.21.2.1	UE co-existence studies	72

5.21.2.1.1	Co-existence studies for 2UL band with 1CC per band	72

5.21.2.2	REFSENS requirements	75

5.22	CA_n20-n71-n78	76

5.22.1	Common for 1 band UL and 2 bands UL CA	76

5.22.1.1	Operating bands for CA	76

5.22.1.2	Channel bandwidths per operating band for CA	76

5.22.1.3	∆TIB,c and ∆RIB,c values	76

5.22.2	Specific for 2 bands UL CA	77

5.22.2.1	UE co-existence studies	77

5.22.2.1.1	Co-existence studies for 2UL band with 1CC per band	77

5.22.2.2	REFSENS requirements	79

5.23	CA_n3-n20-n71	80

5.23.1	Common for 1 band UL and 2 bands UL CA	80

5.23.1.1	Operating bands for CA	80

5.23.1.2	Channel bandwidths per operating band for CA	80

5.23.1.3	∆TIB,c and ∆RIB,c values	80

5.23.2	Specific for 2 bands UL CA	81

5.23.2.1	UE co-existence studies	81

5.23.2.1.1	Co-existence studies for 2UL band with 1CC per band	81

5.23.2.2	REFSENS requirements	83

5.24	CA_n3-n20-n77	84

5.24.1	Common for 1 band UL and 2 bands UL CA	84

5.24.1.1	Operating bands for CA	84

5.24.1.2	Channel bandwidths per operating band for CA	84

5.24.1.3	∆TIB,c and ∆RIB,c values	84

5.24.2	Specific for 2 bands UL CA	85

5.24.2.1	UE co-existence studies	85

5.24.2.1.1	Co-existence studies for 2UL band with 1CC per band	85

5.24.2.2	REFSENS requirements	87

5.25	CA_n1-n3-n71	88

5.25.1	Common for 1 band UL and 2 bands UL CA	88

5.25.1.1	Operating bands for CA	88

5.25.1.2	Channel bandwidths per operating band for CA	89

5.25.1.3	∆TIB,c and ∆RIB,c values	89

5.25.2	Specific for 2 bands UL CA	89

5.25.2.1	UE co-existence studies	89

5.25.2.1.1	Co-existence studies for 2UL band with 1CC per band	89

5.25.2.2	REFSENS requirements	92

5.26	CA_n1-n3-n8	93

5.26.1	Common for 1 band UL and 2 bands UL CA	93

5.26.1.1	Operating bands for CA	93

5.26.1.2	Channel bandwidths per operating band for CA	94

5.26.1.3	∆TIB,c and ∆RIB,c values	94

5.26.2	Specific for 2 bands UL CA	94

5.26.2.1	UE co-existence studies	94

5.26.2.1.1	Co-existence studies for 2UL band with 1CC per band	94

5.26.2.2	REFSENS requirements	97

5.27	CA_n1-n41-n71	97

5.27.1	Common for 1 band UL and 2 bands UL CA	97

5.27.1.1	Operating bands for CA	97

5.27.1.2	Channel bandwidths per operating band for CA	97

5.27.1.3	∆TIB,c and ∆RIB,c values	97

5.27.2	Specific for 2 bands UL CA	98

5.27.2.1	UE co-existence studies	98

5.27.2.1.1	Co-existence studies for 2UL band with 1CC per band	98

5.27.2.2	REFSENS requirements	101

5.28	CA_n1-n8-n41	101

5.28.1	Common for 1 band UL and 2 bands UL CA	101

5.28.1.1	Operating bands for CA	101

5.28.1.2	Channel bandwidths per operating band for CA	102

5.28.1.3	∆TIB,c and ∆RIB,c values	102

5.28.2	Specific for 2 bands UL CA	102

5.28.2.1	UE co-existence studies	102

5.28.2.1.1	Co-existence studies for 2UL band with 1CC per band	102

5.28.2.2	REFSENS requirements	105

5.29	CA_n3-n41-n71	106

5.29.1	Common for 1 band UL and 2 bands UL CA	106

5.29.1.1	Operating bands for CA	106

5.29.1.2	Channel bandwidths per operating band for CA	106

5.29.1.3	∆TIB,c and ∆RIB,c values	106

5.29.2	Specific for 2 bands UL CA	107

5.29.2.1	UE co-existence studies	107

5.29.2.1.1	Co-existence studies for 2UL band with 1CC per band	107

5.29.2.2	REFSENS requirements	110

5.30	CA_n1-n7-n20	110

5.30.1	Common for 1 band UL and 2 bands UL CA	110

5.30.1.1	Operating bands for CA	110

5.30.1.2	Channel bandwidths per operating band for CA	110

5.30.1.3	∆TIB,c and ∆RIB,c values	111

5.30.2	Specific for 2 bands UL CA	111

5.30.2.1	UE co-existence studies	111

5.30.2.1.1	Co-existence studies for 2UL band with 1CC per band	111

5.30.2.2	REFSENS requirements	114

5.31	CA_n1-n3-n75	115

5.31.1	Common for 1 band UL and 2 bands UL CA	115

5.31.1.1	Operating bands for CA	115

5.31.1.2	Channel bandwidths per operating band for CA	115

5.31.1.3	∆TIB,c and ∆RIB,c values	115

5.31.2	Specific for 2 bands UL CA	115

5.31.2.1	UE co-existence studies	115

5.31.2.1.1	Co-existence studies for 2UL band with 1CC per band	115

5.31.2.2	REFSENS requirements	116

5.32	CA_n1-n7-n78	117

5.32.1	Common for 1 band UL and 2 bands UL CA	117

5.32.1.1	Operating bands for CA	117

5.32.1.2	Channel bandwidths per operating band for CA	117

5.32.1.3	∆TIB,c and ∆RIB,c values	117

5.32.2	Specific for 2 bands UL CA	117

5.32.2.1	UE co-existence studies	117

5.32.2.1.1	Co-existence studies for 2UL band with 1CC per band	117

5.32.2.2	REFSENS requirements	119

5.33	CA_n7-n25-n29	120

5.33.1	Common for 1 band UL and 2 bands UL CA	120

5.33.1.1	Operating bands for CA	120

5.33.1.2	Channel bandwidths per operating band for CA	120

5.33.1.3	∆TIB,c and ∆RIB,c values	120

5.33.2	Specific for 2 bands UL CA	121

5.33.2.1	UE co-existence studies	121

5.33.2.1.1	Co-existence studies for 2UL band with 1CC per band	121

5.33.2.2	REFSENS requirements	121

5.34	CA_n7-n29-n66	122

5.34.1	Common for 1 band UL and 2 bands UL CA	122

5.34.1.1	Operating bands for CA	122

5.34.1.2	Channel bandwidths per operating band for CA	122

5.34.1.3	∆TIB,c and ∆RIB,c values	122

5.34.2	Specific for 2 bands UL CA	123

5.34.2.1	UE co-existence studies	123

5.34.2.1.1	Co-existence studies for 2UL band with 1CC per band	123

5.34.2.2	REFSENS requirements	123

5.35	CA_n7-n29-n77	124

5.35.1.1	Operating bands for CA	124

5.35.1.2	Channel bandwidths per operating band for CA	124

5.35.1.3	∆TIB,c and ∆RIB,c values	124

5.35.2.1	UE co-existence studies	125

5.35.2.1.1	Co-existence studies for 2UL band with 1CC per band	125

5.35.2.2	REFSENS requirements	125

5.36	  n3A-n40A-n79A	126

5.36.1	Common for 1 band UL and 2 bands UL CA	126

5.36.1.1	 Operating bands for CA	126

5.36.1.2	Channel bandwidths per operating band for CA	126

5.36.1.3	∆TIB,c and ∆RIB,c values	126

5.36.2	Specific for 2 bands UL CA	126

5.36.2.1	UE co-existence studies	126

5.36.2.2	REFSENS requirements	127

5.37	CA_n3-n7-n77	127

5.37.1	Common for 1 band UL and 2 bands UL CA	127

5.37.1.1	Operating bands for CA	127

5.37.1.2	Channel bandwidths per operating band for CA	128

5.37.1.3	∆TIB,c and ∆RIB,c values	128

5.37.2	Specific for 2 bands UL CA	128

5.37.2.1	UE co-existence studies	128

5.37.2.1.1	Co-existence studies for 2UL band with 1CC per band	128

5.37.2.2	REFSENS requirements	131

5.38	CA_n1-n20-n41	131

5.38.1	Common for 1 band UL and 2 bands UL CA	131

5.38.1.1	Operating bands for CA	131

5.38.1.2	Channel bandwidths per operating band for CA	132

5.38.1.3	∆TIB,c and ∆RIB,c values	132

5.38.2	Specific for 2 bands UL CA	132

5.38.2.1	UE co-existence studies	132

5.38.2.1.1	Co-existence studies for 2UL band with 1CC per band	132

5.38.2.2	REFSENS requirements	134

5.39	CA_n1-n20-n77	135

5.39.1	Common for 1 band UL and 2 bands UL CA	135

5.39.1.1	Operating bands for CA	135

5.39.1.2	Channel bandwidths per operating band for CA	135

5.39.1.3	∆TIB,c and ∆RIB,c values	135

5.39.2	Specific for 2 bands UL CA	136

5.39.2.1	UE co-existence studies	136

5.39.2.1.1	Co-existence studies for 2UL band with 1CC per band	136

5.39.2.2	REFSENS requirements	138

5.40	CA_n1-n71-n77	139

5.40.1	Common for 1 band UL and 2 bands UL CA	139

5.40.1.1	Operating bands for CA	139

5.40.1.2	Channel bandwidths per operating band for CA	139

5.40.1.3	∆TIB,c and ∆RIB,c values	139

5.40.2	Specific for 2 bands UL CA	140

5.40.2.1	UE co-existence studies	140

5.40.2.1.1	Co-existence studies for 2UL band with 1CC per band	140

5.40.2.2	REFSENS requirements	143

5.41	CA_n3-n20-n41	143

5.41.1	Common for 1 band UL and 2 bands UL CA	143

5.41.1.1	Operating bands for CA	143

5.41.1.2	Channel bandwidths per operating band for CA	144

5.41.1.3	∆TIB,c and ∆RIB,c values	144

5.41.2	Specific for 2 bands UL CA	144

5.41.2.1	UE co-existence studies	144

5.41.2.1.1	Co-existence studies for 2UL band with 1CC per band	144

5.41.2.2	REFSENS requirements	147

5.42	CA_n3-n71-n77	147

5.42.1	Common for 1 band UL and 2 bands UL CA	147

5.42.1.1	Operating bands for CA	147

5.42.1.2	Channel bandwidths per operating band for CA	148

5.42.1.3	∆TIB,c and ∆RIB,c values	148

5.42.2	Specific for 2 bands UL CA	148

5.42.2.1	UE co-existence studies	148

5.42.2.1.1	Co-existence studies for 2UL band with 1CC per band	148

5.42.2.2	REFSENS requirements	151

5.43	CA_n1-n41-n78	152

5.43.1	Common for 1 band UL and 2 bands UL CA	152

5.43.1.1	Operating bands for CA	152

5.43.1.2	Channel bandwidths per operating band for CA	152

5.43.1.3	∆TIB,c and ∆RIB,c values	152

5.43.2	Specific for 2 bands UL CA	153

5.43.2.1	UE co-existence studies	153

5.43.2.1.1	Co-existence studies for 2UL band with 1CC per band	153

5.43.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	153

5.43.2.2	REFSENS requirements	154

5.44	CA_n1-n71-n78	154

5.44.1	Common for 1 band UL and 2 bands UL CA	154

5.44.1.1	Operating bands for CA	154

5.44.1.2	Channel bandwidths per operating band for CA	155

5.44.1.3	∆TIB,c and ∆RIB,c values	155

5.44.2	Specific for 2 bands UL CA	155

5.44.2.1	UE co-existence studies	155

5.44.2.1.1	Co-existence studies for 2UL band with 1CC per band	155

5.44.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	158

5.44.2.2	REFSENS requirements	159

5.45	CA_n1-n8-n78	160

5.45.1	Common for 1 band UL and 2 bands UL CA	160

5.45.1.1	Operating bands for CA	160

5.45.1.2	Channel bandwidths per operating band for CA	160

5.45.2	Specific for 2 bands UL CA	160

5.45.2.1	UE co-existence studies	160

5.45.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	160

5.45.2.2	REFSENS requirements	161

5.46	CA_n3-n41-n78	162

5.46.1	Common for 1 band UL and 2 bands UL CA	162

5.46.1.1	Operating bands for CA	162

5.46.1.2	Channel bandwidths per operating band for CA	162

5.46.1.3	∆TIB,c and ∆RIB,c values	162

5.46.2	Specific for 2 bands UL CA	162

5.46.2.1	UE co-existence studies	162

5.46.2.1.1	Co-existence studies for 2UL band with 1CC per band	162

5.46.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	162

5.46.2.2	REFSENS requirements	163

5.47	CA_n3-n71-n78	163

5.47.1	Common for 1 band UL and 2 bands UL CA	163

5.47.1.1	Operating bands for CA	163

5.47.1.2	Channel bandwidths per operating band for CA	164

5.47.1.3	∆TIB,c and ∆RIB,c values	164

5.47.2	Specific for 2 bands UL CA	165

5.47.2.1	UE co-existence studies	165

5.47.2.1.1	Co-existence studies for 2UL band with 1CC per band	165

5.47.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	167

5.47.2.2	REFSENS requirements	168

5.48	CA_n3-n8-n78	168

5.48.1	Common for 1 band UL and 2 bands UL CA	168

5.48.1.1	Operating bands for CA	168

5.48.1.2	Channel bandwidths per operating band for CA	169

5.48.2	Specific for 2 bands UL CA	169

5.48.2.1	UE co-existence studies	169

5.48.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	169

5.48.2.2	REFSENS requirements	170

5.49	CA_n41-n71-n78	170

5.49.1	Common for 1 band UL and 2 bands UL CA	170

5.49.1.1	Operating bands for CA	170

5.49.1.2	Channel bandwidths per operating band for CA	170

5.49.1.3	∆TIB,c and ∆RIB,c values	170

5.49.2	Specific for 2 bands UL CA	171

5.49.2.1	UE co-existence studies	171

5.49.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	171

5.49.2.2	REFSENS requirements	171

5.50	CA_n8-n41-n78	172

5.50.1	Common for 1 band UL and 2 bands UL CA	172

5.50.1.1	Operating bands for CA	172

5.50.1.2	Channel bandwidths per operating band for CA	172

5.50.1.3	∆TIB,c and ∆RIB,c values	172

5.50.2	Specific for 2 bands UL CA	173

5.50.2.1	UE co-existence studies	173

5.50.2.1.1	Co-existence studies for 2UL band with 1CC per band	173

5.50.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	175

5.50.2.2	REFSENS requirements	176

5.51	CA_n1-n3-n78	177

5.51.1	Common for 1 band UL and 2 bands UL CA	177

5.51.1.1	Operating bands for CA	177

5.51.1.2	Channel bandwidths per operating band for CA	177

5.51.1.3	∆TIB,c and ∆RIB,c values	177

5.51.2	Specific for 2 bands UL CA	177

5.51.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	177

5.51.2.2	REFSENS requirements	178

6	Dual Connectivity: Specific Band Combination Part	179

6.0	DC_nX-nY-nZ	179

6.0.1	Configurations for DC_nX-nY-nZ	179

Annex <A> (normative): Band group definition	180

Annex <B> (normative): Valid UL configurations	181

Annex <C> (informative): Change history	182

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC configurations including inter-band CA for 3 different bands DL with x different bands UL (x=1,2) under Rel-19 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 3 bands DL with up to 2 bands UL for the Rel-19 band combinations.

This TR contains the RF requirements of band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

4	Background

4.1	Introduction

The present document is a technical report for NR inter-band CA and DC configurations including inter-band CA for 3 different bands DL with x different bands UL (x=1,2) under Rel-19 time frame. The document covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination) including:

-	Common issues for both 1 band UL and 2 bands UL NR CA, including ∆TIB and ∆RIB requirements.

-	2 bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

It shall be noted that no new issue for inter-band NR DC combination, and the 2 bands UL NR CA specific issues shall be re-used.

4.2	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

Editor's note: It is not recommended to bring TP to TR for the following cases:

-	NR CA configurations with additional BCS other than BCS0 (such as BCS1) if there is no additional technical issue.

-	High order DL NR CA configurations, such as DL NR CA configuration CA_nXA-nYA-nZC

5	Three bands within FR1 Carrier Aggregation: Specific Band Combination Part

5.0	CA_nX-nY-nZ

5.0.1	Common for 1 band UL and 2 bands UL CA

5.0.1.1	Operating bands for CA

Editor's note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.x.1.1-1 below.

Table 5.0.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nZ | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |

5.0.1.2	Channel bandwidths per operating band for CA

Table 5.0.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_nXA-nYA-nZA | CA_nXA-nYA | nX | Channel BWs | X |
|  | CA_nXA-nZA | nY | Channel BWs |  |
|  | CA_nYA-nZA | nZ | Channel BWs |  |
| CA_nXA/B/C/(2A)-nYA/B/C/(2A)- nZA/B/C/(2A) | CA_nXA/B/C-nYA CA_nXA/B/C-nZA CA_nYA/B/C-nZA | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |
|  |  | nZ | Channel BW or CA BCS |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.0.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_nX-nY-nZ, the TIB,c and RIB,c values are given in the tables below.

Table 5.0.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_nx-ny-nz |  |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.0.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_nx-ny-nz |  |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.0.2	Specific for 2 bands UL CA

5.0.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.0.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.0.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_nXA-nYA-nZA with UL CA_nXA-nYA.

Table 5.0.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

5.0.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Editor’s Note: Since the triple beat tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.0.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band nZ interference analysis for CA_nXA/B/C-nYA/B/C with nX/YB/C transmitting with a XXX MHz maximum instantaneous bandwidth.

Table 5.0.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (nX, nY) | – |  | – |  |
| fDL (nZ) | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the two UL band with intra-band UL CA triple beat shall be included here, even if no issues are identified. |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in table A.1, of band one (nX) or two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.0.2.2	REFSENS requirements

Editor's note 1: Text can be added if IMD due to 2 bands UL with 2 UL carriers or triple beat due to 2 bands UL with 3 UL carriers issues are identified in Table 5.0.2.1.1-1 and Table 5.0.2.1.2-1, respectively.

Editor's note 2: Proponent are encouraged to provide the detailed technical analysis of the MSD requirements. For example: RF architecture, RF components parameter, and etc. In the case where the proponent is missing some elements to calculate the REFSENS exception cases, the TP can be submitted to the “not for block approval” AI for expert support.

5.1	CA_n28-n41-n79

5.1.1	Common for 1 band UL and 2 bands UL CA

5.1.1.1	Operating bands for CA

Table 5.1.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 703 MHz – 748MHz | FDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.1.1.2	Channel bandwidths per operating band for CA

Table 5.1.1.2-1: Supported bandwidths per CA band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- |
| CA_n28A-n41A-n79C | CA_n79C CA_n28A-n41A CA_n28A-n79A CA_n28A-n79C CA_n41A-n79A CA_n41A-n79C | n28 | n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |  |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |  |  |
| CA_n28A-n41C-n79A | CA_n41C CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |  |  |
|  |  | n79 | n41 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n28A-n41C-n79C | CA_n41C CA_n79C CA_n28A-n41A CA_n28A-n79A CA_n28A-n79C CA_n41A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |  |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |  |  |

5.1.1.3	∆TIB,c and ∆RIB,c values

∆TIB,c and ∆RIB,c values have already been defined in the current spec.

5.1.2	Specific for 2 bands UL CA

5.1.2.1	UE co-existence studies

The co-existence studies of fallback configurations of 2UL band with one CC per band can be applied.

5.1.2.1.1	Co-existence studies for 2UL band with 1CC per band

For CA_n28A-n41A/C-n79A/C, the two UL bands with one CC per band IMD interference analysis have already been analysed in the fallback.

5.1.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.1.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n28A-n79C with n79C transmitting with a 200MHz maximum instantaneous bandwidth.

Table 5.1.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n28 |  | CA_n79C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 703 -- 748 |  | 4400 -- 5000 |  |
| fDL | 758 -- 803 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -- 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+ max2CCBW |
| IMD3 (MHz) | 503 -- 703 |  | 748 -- 948 |  |
| Analysis | There are no triple beat products caused by UL CA_n28A-n79C falls into band n41. |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.1.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n28 interference analysis for CA_n41A-n79C with n79C transmitting with a 200MHz maximum instantaneous bandwidth.

Table 5.1.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n41 |  | CA_n79C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 2496 -- 2690 |  | 4400 -- 5000 |  |
| fDL | 2496 -- 2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -- 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+ max2CCBW |
| IMD3 (MHz) | 2296 -- 2496 |  | 2690 -- 2890 |  |
| Analysis | There are no triple beat products caused by UL CA_n41A-n79C falls into band n28. |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.1.2.2	REFSENS requirements

There is no need to define additional REFSEN requirements (i.e. MSD) for CA_n28A-n41A/C-n79C due to there are no additional triple beat co-existence issue.

5.2	CA_n8-n20-n75

5.2.1	Common for 1 band UL and 2 bands UL CA

5.2.1.1	Operating bands for CA

Table 5.2.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.2.1.2	Channel bandwidths per operating band for CA

Table 5.2.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n20A-n75A | CA_n8A-n20A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.2.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n20-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.2.2	Specific for 2 bands UL CA

5.2.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.2.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.2.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n20A-n75A with UL CA_n8A-n20A.

Table 5.2.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18–83 |  | 1712–1777 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898–998 |  | 749–844 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592–2692 |  | 2544–2639 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778–1913 |  | 1581–1706 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 36–166 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472–3607 |  | 3376–3501 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3424–3554 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2413–2568 |  | 2658–2828 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 666–826 |  | 916–1081 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208–4363 |  | 4352–4522 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256–4416 |  | 4304–4469 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD produced by UL CA_n8A-n20A will fall into the n75 Rx band.

5.2.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

5.3	CA_n8-n28-n75

5.3.1	Common for 1 band UL and 2 bands UL CA

5.3.1.1	Operating bands for CA

Table 5.3.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.3.1.2	Channel bandwidths per operating band for CA

Table 5.3.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n28A-n75A | CA_n8A-n28A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.3.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.3.2	Specific for 2 bands UL CA

5.3.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.3.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.3.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n28A-n75A with UL CA_n8A-n28A.

Table 5.3.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 132–212 |  | 1583–1663 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012–1127 |  | 491–616 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463–2578 |  | 2286–2411 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892–2042 |  | 1194–1364 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 264–424 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343–3493 |  | 2989–3159 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3166–3326 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1897–2112 |  | 2772–2957 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 279–484 |  | 1144–1339 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692–3907 |  | 4008–4223 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869–4074 |  | 4046–4241 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, there is no IMD produced by UL CA_n8A-n28A will fall into the n75 DL spectrum.

5.3.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

5.4	CA_n20-n28-n75

5.4.1	Common for 1 band UL and 2 bands UL CA

5.4.1.1	Operating bands for CA

Table 5.4.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.4.1.2	Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n28A-n75A | CA_n20A-n28A | n20 | n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.4.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.4.2	Specific for 2 bands UL CA

5.4.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.4.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.4.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n28A-n75A with UL CA_n20A-n28A.

Table 5.4.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 84–159 |  | 1535–1610 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 916–1021 |  | 544–664 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367–2472 |  | 2238–2358 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1748–1883 |  | 1247–1412 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 168–318 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199–3334 |  | 2941–3106 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3070–3220 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950–2160 |  | 2580–2745 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 385–580 |  | 1000–1180 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644–3854 |  | 4031–4196 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773–3968 |  | 3902–4082 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, there is no IMD produced by UL CA_n20A-n28A will fall inte the n75 DL spectrum.

5.4.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

5.5	CA_n3-n7-n75

5.5.1	Common for 1 band UL and 2 bands UL CA

5.5.1.1	Operating bands for CA

Table 5.5.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.5.1.2	Channel bandwidths per operating band for CA

Table 5.5.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| CA_n3A-n7A-n75A | CA_n3A-n7A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.5.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.5.2	Specific for 2 bands UL CA

5.5.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.5.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.5.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n75A with UL CA_n3A-n7A.

Table 5.5.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715–860 |  | 4210–4355 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850–1070 |  | 3215–3430 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920–6140 |  | 6710–6925 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2560–2855 |  | 5715–6000 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1430–1720 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7630–7925 |  | 9210–9495 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8420–8710 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8215–8570 |  | 4270–4640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3930–4290 |  | 10–355 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710–12065 |  | 9340–9710 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920–11280 |  | 10130–10495 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 4th order IMD generated by UL CA_n3A-n7A may fall into own Rx of Band n75

5.5.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.5.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.5.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.5.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n75 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 10.4 | SDL | IMD4 |

5.6	CA_n3-n75-n78

5.6.1	Common for 1 band UL and 2 bands UL CA

5.6.1.1	Operating bands for CA

Table 5.6.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.6.1.2	Channel bandwidths per operating band for CA

Table 5.6.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n75A-n78A | CA_n3A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.6.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.6.2	Specific for 2 bands UL CA

5.6.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.6.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.6.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n75A-n78A with UL CA_n3A-n78A.

Table 5.6.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515–2090 |  | 5010–5585 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 270–380 |  | 4815–5890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720–7370 |  | 8310–9385 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330–2055 |  | 8115–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3030–4180 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430–9155 |  | 11610–13185 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020–11170 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415–13490 |  | 3040–3840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330–7980 |  | 1245–2470 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910–16985 |  | 10140–10940 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320–14970 |  | 11730–12955 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 4th and 5th order IMD generated by UL CA_n3A-n78A may fall into own Rx of Band n75

5.6.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.6.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.6.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.6.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n75-n78 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 10.8 | SDL | IMD4 |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1490 | 1.0 | SDL | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |

5.7	CA_n7-n75-n78

5.7.1	Common for 1 band UL and 2 bands UL CA

5.7.1.1	Operating bands for CA

Table 5.7.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.7.1.2	Channel bandwidths per operating band for CA

Table 5.7.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n75A-n78A | CA_n7A-n78A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.7.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n7-n75-n78, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.7.2	Specific for 2 bands UL CA

5.7.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.7.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.7.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n75A-n78A with UL CA_n7A-n78A.

Table 5.7.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1300 |  | 5800–6370 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200–1840 |  | 4030–5100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–8940 |  | 9100–10170 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700–4410 |  | 7330–8900 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460–2600 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11510 |  | 12400–13970 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–12740 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–12700 |  | 6200–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–6400 |  | 100–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–17770 |  | 13300–14080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–16540 |  | 14100–15310 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 3rd and 4th order IMD generated by UL CA_n7A-n78A may fall into own Rx of Band n75

5.7.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.7.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.7.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.7.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n75-n78 | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1470 | 16 | SDL | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1500 | 10.5 | SDL | IMD4 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |

5.8	CA_n1-n7-n75

5.8.1	Common for 1 band UL and 2 bands UL CA

5.81.1	Operating bands for CA

Table 5.8.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.8.1.2	Channel bandwidths per operating band for CA

Table 5.8.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n7A-n75A | CA_n1A-n7A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.8.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n1-n7-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.8.2	Specific for 2 bands UL CA

5.8.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.8.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.8.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n75A with UL CA_n1A-n7A.

Table 5.8.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520–650 |  | 4420–4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270–1460 |  | 3020–3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340–6530 |  | 6920–7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190–3440 |  | 5520–5790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1040–1300 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260–8510 |  | 9420–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8840–9100 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8020–8360 |  | 5110–5420 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3540–3870 |  | 620–940 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920–12260 |  | 10180–10490 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340–11670 |  | 10760–11080 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 3rd order IMD generated by UL CA_n1A-n7A may fall into own Rx of Band n75

5.8.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.8.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.8.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.8.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n7-n75 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 13.2 | SDL | IMD3 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.9	CA_n1-n3-n7

5.9.1	Common for 1 band UL and 2 bands UL CA

5.9.1.1	Operating bands for CA

Table 5.9.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |

5.9.1.2	Channel bandwidths per operating band for CA

Table 5.9.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n7A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.9.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n1A-n3A-n7A, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.9.2	Specific for 2 bands UL CA

5.9.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.9.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.9.2.1.1-1, 5.9.2.1.1-2 and 5.9.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n7A with UL CA_n1A-n3A, CA_n1A-n7A and CA_n3A-n7A

Table 5.9.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1300 |  | 5800–6370 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200–1840 |  | 4030–5100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–8940 |  | 9100–10170 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700–4410 |  | 7330–8900 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460–2600 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11510 |  | 12400–13970 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–12740 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–12700 |  | 6200–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–6400 |  | 100–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–17770 |  | 13300–14080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–16540 |  | 14100–15310 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD produced by UL CA_n1A-n3A will fall into the n7 Rx band.

Table 5.9.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520–650 |  | 4420–4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270–1460 |  | 3020–3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340–6530 |  | 6920–7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190–3440 |  | 5520–5790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1040–1300 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260–8510 |  | 9420–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8840–9100 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8020–8360 |  | 5110–5420 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3540–3870 |  | 620–940 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920–12260 |  | 10490–10180 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340–11670 |  | 10760–11080 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD produced by UL CA_n1A-n7A will fall into the n3 Rx band.

Table 5.9.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320–1880 |  | 5220–5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40–660 |  | 4620–5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140–7760 |  | 8520–9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960–2640 |  | 7920–9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2640–3760 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060–9740 |  | 11820–13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440–11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220–13280 |  | 3880–4620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940–7560 |  | 660–1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120–17180 |  | 10980–11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740–15360 |  | 12360–13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above tables, no IMD produced by UL CA_n3A-n7A will fall into the n1 Rx band.

5.9.2.2	REFSENS requirements

Based on the co-existence studies, there is no need to define MSD values.

5.10	CA_n41-n66-n71

5.10.1	Common for 1 band UL and 2 bands UL CA

5.10.1.1	Operating bands for CA.

Table 5.10.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz –2690 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2100 MHz – 2200 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |

5.10.1.2	Channel bandwidths per operating band for CA

| Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n25+n41+n77NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n41C-n66A-n71A | n417,9 CA_n41A-n66A7 CA_n41C-n66A CA_n41A-n71A7 CA_n41C-n71A CA_n41C7 CA_n66A-n71A |  | n41 | CA_n41C_BCS0 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  |  | n71 | 5, 10, 15, 20 |  |
|  |  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n71 | 5, 10, 15, 20 |  |
|  |  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

5.10.2	Specific for 2 bands UL CA

5.10.2.1	UE co-existence studies

5.10.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.10.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)Table 5.10.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71.

Table 5.10.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n66 |  | CA_n41C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1710 MHz – 1780 MHz |  | 2496 MHz– 2690 MHz |  |
| fDL | 2110 MHz – 2200 MHz |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 190 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1520 MHz – 1710 MHz |  | 1780 MHz – 1970 MHz |  |
| Analysis | No issue |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.10.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n66.

Table 5.10.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n71 |  | CA_n41C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 663 MHz – 698 MHz |  | 2496 MHz– 2690 MHz |  |
| fDL | 617 MHz – 652 MHz |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 190 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 473 MHz – 663 MHz |  | 698 MHz – 888 MHz |  |
| Analysis | No issue |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.10.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA, 1st order triple beat IMD is not falling into bands n66 and n71, so there is no need to define MSD values.

5.11	CA_n1-n41-n78

5.11.1	Common for 1 band UL and 2 bands UL CA

5.11.1.1	Operating bands for CA

When adding CA_n1-n41-n78 to Table 5.2A.2.2-1 in 38.101-1, it must be marked with Note 3 as CA_n1-n41-n783.

Table 5.11.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

This combination supports inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability.

5.11.1.2	Channel bandwidths per operating band for CA

Table 5.11.1.2-1: Supported bandwidths per CA band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n41A-n78A | CA_n1A-n41A CA_n1A-n78A CA_n41A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

5.11.1.3	∆TIB,c and ∆RIB,c values

Requirements for CA_n1-n41-n78, for the TIB,c and RIB,c values are given in the tables below following CA_n1-n41-n77.

Table 5.11.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n78 | 0.5 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.11.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n78 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.11.2	Specific for 2 bands UL CA

5.11.2.1	UE co-existence studies

Table 5.11.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n78A with UL CA_n1A-n4A.

Table 5.11.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9220 - 8952 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.11.2.1.1-1, 3rd, 4th and 5th order IMD from band n1 and Band n41 may also fall into Rx frequencies of band n78.

Table 5.11.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 1880 |  | 5220 - 5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 - 660 |  | 4620 - 5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 7760 |  | 8520 - 9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 2640 |  | 7920 - 9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3760 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 9740 |  | 11820 - 13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11440 - 10560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.11.2.1.1-2, 4th order IMD from band n1 and Band n78 may also fall into Rx frequencies of band n41.

Table 5.11.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1304 |  | 5796 - 6490 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 - 2080 |  | 3910 - 5104 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9180 |  | 9096 - 10290 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 - 4770 |  | 7210 - 8904 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2608 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 11870 |  | 12396 - 14090 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12592 - 11980 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 - 10510 |  | 7460 - 6184 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 - 4520 |  | 1470 - 112 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 17890 |  | 13284 - 14560 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 16780 |  | 14088 - 15670 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.11.2.1.1-3, 4th order IMD from band n41 and Band n78 may also fall into Rx frequencies of band n1.

From the tables it is found that:

-  Band n78 may be subject to IMD3, IMD4 and IMD5.

-  Band n41 may be subject to IMD4.

-  Band n1 may be subject to IMD4.

5.11.2.2	REFSENS requirements

MSD values have been taken from CA_n1A-n41A-n77A.

Table 5.11.2.2-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n41-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3330 | 19.6 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2140 | 9.3 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.12	CA_n1-n5-n8

5.12.1	Common for 1 band UL and 2 bands UL CA

5.12.1.1	Operating bands for CA

Table 5.12.1.1-1: CA band combination constituent bands definition

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n5-n8 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

5.12.1.2	Channel bandwidths per operating band for CA

Table 5.12.1.2-1: Supported bandwidths per CA band combination of band n1-n5-n8

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n5A-n8A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n8 | 5, 10, 15, 20 |  |

5.12.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n5-n8, since there is only single carrier UL the following TIB,c  values are proposed:

Table 5.12.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n5-n8 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The RIB,c values are proposed to be defined following CA_n5-n8 as in the following table

Table 5.12.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n5-n8 | - | 0.4 | 0.4 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.13	CA_n3-n5-n8

5.13.1	Common for 1 band UL and 2 bands UL CA

5.13.1.1	Operating bands for CA

Table 5.13.1.1-1: CA band combination constituent bands definition

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n5-n8 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

5.13.1.2	Channel bandwidths per operating band for CA

Table 5.13.1.2-1: Supported bandwidths per CA band combination of band n3-n5-n8

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n5A-n8A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n8 | 5, 10, 15, 20 |  |

5.13.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n5-n8, since there is only single carrier UL the following TIB,c  values are proposed:

Table 5.13.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n5-n8 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The RIB,c values are proposed to be defined following CA_n5-n8 as in the following table.

Table 5.13.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n5-n8 | - | 0.4 | 0.4 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

## 5.14	CA_n25-n29-n77

### 5.14.1	Common for 1 band UL and 2 bands UL CA

#### 5.14.1.1	Operating bands for CA

Table 5.14.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

#### 5.14.1.2	Channel bandwidths per operating band for CA

Table 5.14.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n25A-n29A-n77A | CA_n25A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n29A-n77(2A) | CA_n25A-n77A CA_n77(2A) | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n25A-n29A-n77(3A) | CA_n25A-n77A CA_n77(2A) | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |

#### 5.14.1.3	∆TIB,c and ∆RIB,c values

For CA_n25-n29-n77 the TIB,c and RIB,c values are given in the tables below.

Table 5.14.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n25-n29-n77 | 0.6 | N/A | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.14.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n25-n29-n77 | 0.2 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.14.2	Specific for 2 bands UL CA

#### 5.14.2.1	UE co-existence studies

##### 5.14.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.14.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n25A-n29A-n77A with UL CA_n25A-n77A.

Table 5.14.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385–2350 |  | 5150–6115 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 500–530 |  | 4685–6550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000–8030 |  | 8450–10315 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1350–2445 |  | 7985–10750 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2770–4700 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850–9945 |  | 11750–14515 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10300–12230 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11285–14950 |  | 3200–4360 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6070–8900 |  | 855–2850 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050–18715 |  | 10700–11860 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600–16430 |  | 12150–14145 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, there is no IMD generated by UL CA_n25A-n77A fall into own Rx of Band n29.

In addition, there is no additional MSD due to UL CA_n77(2A) needs to specified for three bands DL configuration.

#### 5.14.2.2	REFSENS requirements

Based on the co-existence studies, there is no need to define MSD values.

5.15	CA_n1-n75-n78

5.15.1	Common for 1 band UL and 2 bands UL CA

5.15.1.1	Operating bands for CA

Table 5.15.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz –1980 MHz | 2110 MHz –2170 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.15.1.2	Channel bandwidths per operating band for CA

Table 5.15.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n75A-n78A | CA_n1A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

5.15.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n75-n78, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.15.2	Specific for 2 bands UL CA

5.15.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.15.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.15.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n75A-n78A with UL CA_n1A-n78A.

Table 5.15.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320–1880 |  | 5220–5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40–660 |  | 4620–5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140–7760 |  | 8520–9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960–2640 |  | 7920–9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2640–3760 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060–9740 |  | 11820–13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440–11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220–13280 |  | 3880–4620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940–7560 |  | 660–1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120–17180 |  | 10980–11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740–15360 |  | 12360–13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd and 5th order IMD generated by UL CA_n1A-n78A may fall into own Rx of Band n75

5.15.2.2	REFSENS requirements

IMD5 generated by UL CA_n1A-n78A fall into own Rx of Band n75.However, since the MSD value is 0 dB, there is no need to define MSD of IMD5. Referring to DC_1A_n75A-n78A, the MSD requirements can be specified as below:

Table 5.15.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n75-n78 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1470 | 30.4 | SDL | IMD2 |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | TDD | N/A |

5.16	CA_n1-n28-n40

5.16.1	Common for 1 band UL and 2 bands UL CA

5.16.1.1	Operating bands for CA

Table 5.16.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n28 | 703-748 | 758-803 | FDD |
| n40 | 2300-2400 | 2300-2400 | TDD |

5.16.1.2	Channel bandwidths per operating band for CA

Table 5.16.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n28A-n40A | CA_n1A-n28A CA_n1A-n40A CA_n28A-n40A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.16.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n28-n40, the TIB,c and RIB,c values are already captured in the specification.

5.16.2	Specific for 2 bands UL CA

5.16.2.1	UE co-existence studies

5.16.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.16.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n28A-n40A with UL CA_n1A-n28A.

Table 5.16.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 - 1172 |  | 2623 - 2728 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 - 3257 |  | 574 - 424 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 - 4708 |  | 3326 - 3476 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 - 5237 |  | 129 - 324 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2344 - 2554 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 - 6688 |  | 4029 - 4224 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5246 - 5456 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 - 832 |  | 7217 - 6932 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 - 1851 |  | 4534 - 4264 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 - 4972 |  | 8383 - 8668 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 - 6204 |  | 7166 - 7436 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.16.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n28A-n40A with UL CA_n1A-n40A.

Table 5.16.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 - 480 |  | 4220 - 4380 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 - 1660 |  | 2620 - 2880 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 - 6360 |  | 6520 - 6780 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 - 3640 |  | 4920 - 5280 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 960 - 640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 - 8340 |  | 8820 - 9180 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8440 - 8760 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 - 7220 |  | 5620 - 5280 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 - 2940 |  | 1340 - 960 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 - 11580 |  | 9980 - 10320 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 - 11160 |  | 10360 - 10740 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.16.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n28A-n40A with UL CA_n28A-n40A.

Table 5.16.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1552 - 1697 |  | 3003 - 3148 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 994 - 804 |  | 3852 - 4097 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3706 - 3896 |  | 5303 - 5548 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 291 - 56 |  | 6152 - 6497 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3394 - 3104 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4409 - 4644 |  | 7603 - 7948 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6006 - 6296 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8897 - 8452 |  | 692 - 412 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5794 - 5404 |  | 2356 - 2691 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9903 - 10348 |  | 5112 - 5392 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8306 - 8696 |  | 6709 - 7044 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.16.2.1.1-1 there is IMD4 from n1 and n28 into RX band n40.

Based on the analysis in Table 5.16.2.1.1-2 there is IMD4 from n1 and n40 into RX band n28.

5.16.2.2	REFSENS requirements

The MSD value has been taken from CA_n1-n5-n40.

Table 5.16.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n40 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 8 | FDD | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2390 | 9.0 | TDD | IMD4 |

5.17	CA_n1-n20-n71

5.17.1	Common for 1 band UL and 2 bands UL CA

5.17.1.1	Operating bands for CA

Table 5.17.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n71 | 663-698 | 617-652 | FDD |

5.17.1.2	Channel bandwidths per operating band for CA

Table 5.17.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n71A | CA_n1A-n20A CA_n1A-n71A CA_n20A-n71A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n71 | 5, 10, 15, 20 |  |

5.17.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n71, the TIB,c and RIB,c values are re-used from CA_n1-n5-n105 as given in the tables below.

Table 5.17.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n71 | - | 0.2 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.17.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n71 | 0.3 | 0.6 | 0.6 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.17.2	Specific for 2 bands UL CA

5.17.2.1	UE co-existence studies

5.17.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.17.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n71A with UL CA_n1A-n20A.

Table 5.17.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.17.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n71A with UL CA_n1A-n71A.

Table 5.17.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.17.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n71A with UL CA_n20A-n71A.

Table 5.17.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.17.2.1.1-1 there is IMD4 from n1 and n20 into RX band n71.

Based on the analysis in Table 5.17.2.1.1-2 there is IMD5 from n1 and n71 into RX band n20.

Based on the analysis in Table 5.17.2.1.1-3 there is IMD3 from n20 and n71 into RX band n1.

5.17.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n5-n105.

Table 5.17.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n71 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 859 | 5 | 25 | 818 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 627 | 9.4 | FDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 4.6 | FDD | IMD5 |
|  | n71 | 689 | 5 | 25 | 643 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167 | 16 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n71 | 666 | 5 | 25 | 620 | N/A | FDD | N/A |

5.18	CA_n1-n20-n78

5.18.1	Common for 1 band UL and 2 bands UL CA

5.18.1.1	Operating bands for CA

Table 5.18.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.18.1.2	Channel bandwidths per operating band for CA

Table 5.18.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n78A | CA_n1A-n20A CA_n1A-n78A CA_n20A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.18.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n78, the TIB,c and RIB,c values are already captured in the specification.

5.18.2	Specific for 2 bands UL CA

5.18.2.1	UE co-existence studies

5.18.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.18.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n78A with UL CA_n1A-n20A.

Table 5.18.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.18.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n78A with UL CA_n1A-n78A.

Table 5.18.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 1880 |  | 5220 - 5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 - 660 |  | 4620 - 5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 7760 |  | 8520 - 9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 2640 |  | 7920 - 9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3760 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 9740 |  | 11820 - 13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 17180 |  | 10980 - 11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.18.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n78A with UL CA_n20A-n78A.

Table 5.18.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 2968 |  | 4132 - 4662 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 - 1576 |  | 5738 - 6768 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5524 |  | 7432 - 8462 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 - 714 |  | 9038 - 10568 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5936 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6386 |  | 10732 - 12262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 9324 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 - 12338 |  | 148 - 472 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 - 8176 |  | 4014 - 5104 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 16062 |  | 6628 - 7248 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 13124 |  | 9096 - 10186 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.18.2.1.1-1 there is IMD3 from n1 and n20 into RX band n78.

Based on the analysis in Table 5.18.2.1.1-2 there is IMD5 from n1 and n78 into RX band n20.

Based on the analysis in Table 5.18.2.1.1-3 there is IMD3 from n20 and n78 into RX band n1.

5.18.2.2	REFSENS requirements

The MSD value has been taken from CA_n1-n26-n78.

Table 5.18.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3644 | 15.7 | TDD | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.1 | FDD | IMD5 |
|  | n78 | 3328 | 10 | 50 | 3328 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2120 | 18.1 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

5.19	CA_n20-n41-n71

5.19.1	Common for 1 band UL and 2 bands UL CA

5.19.1.1	Operating bands for CA

Table 5.19.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |

5.19.1.2	Channel bandwidths per operating band for CA

Table 5.19.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n41A-n71A | CA_n20A-n41A CA_n20A-n71A CA_n41A-n71A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |

5.19.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n41-n71, the TIB,c values are taken from CA_n5-n7-n105 as given in the tables below.

Table 5.19.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n71 | 0.5 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

RIB,c values are taken from DC_5-7_n71 as given in the tables below

Table 5.19.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n71 | - | - | 0.2 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.19.2	Specific for 2 bands UL CA

5.19.2.1	UE co-existence studies

5.19.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.19.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n71A with UL CA_n20A-n41A.

Table 5.19.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.19.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n71A with UL CA_n20A-n71A.

Table 5.19.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.19.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n71A with UL CA_n41A-n71A.

Table 5.19.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2027 - 1798 |  | 3159 - 3388 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4294 - 4717 |  | 1364 - 1100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5655 - 6078 |  | 3822 - 4086 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6790 - 7407 |  | 701 - 402 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3596 - 4054 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8151 - 8768 |  | 4485 - 4784 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6318 - 6776 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 296 - 38 |  | 10097 - 9286 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2898 - 3391 |  | 6744 - 6092 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5148 - 5482 |  | 10647 - 11458 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6981 - 7474 |  | 8814 - 9466 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.19.2.1.1-1 there is IMD5 from n20 and n41 into RX band n71.

Based on the analysis in Table 5.19.2.1.1-2 there is IMD5 from n20 and n71 into RX band n41.

5.19.2.2	REFSENS requirements

The MSD requirements are based on CA_n18-n28-n41.

Table 5.19.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n41-n71 | n20 | 834 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | 2686 | 10 | 25 | 2686 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 650 | 5.9 | FDD | IMD5 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2660 | 6.5 | TDD | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |

5.20	CA_n20-n41-n77

5.20.1	Common for 1 band UL and 2 bands UL CA

5.20.1.1	Operating bands for CA

Table 5.20.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.20.1.2	Channel bandwidths per operating band for CA

Table 5.20.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n41A-n77A | CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n20 | 5,10,15,20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n20A-n41A-n77(2A) | CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n20 | 5,10,15,20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.20.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n41-n77, the TIB,c and RIB,c values are re-used from CA_n5-n41-n77 as given in the tables below.

Table 5.20.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n77 | 0.6 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.20.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n77 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.20.2	Specific for 2 bands UL CA

5.20.2.1	UE co-existence studies

5.20.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.20.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n77A with UL CA_n20A-n41A.

Table 5.20.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.20.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n77A with UL CA_n20A-n77A.

Table 5.20.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.20.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n77A with UL CA_n41A-n77A.

Table 5.20.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1704 |  | 5796 - 6890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 792 - 2080 |  | 3910 - 5904 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9580 |  | 9096 - 11090 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3288 - 4770 |  | 7210 - 10104 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3408 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 12270 |  | 12396 - 15290 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11592 - 13780 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14304 - 10510 |  | 7460 - 5784 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7608 - 4520 |  | 1470 - 912 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 19490 |  | 13284 - 14960 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 17980 |  | 14088 - 16470 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.20.2.1.1-1 there are IMD2, IMD3 and IMD4 from n20 and n41 into RX band n77.

Based on the analysis in Table 5.20.2.1.1-2 there are IMD2 and IMD3 from n20 and n77 into RX band n41.

Based on the analysis in Table 5.20.2.1.1-3 there are IMD2 and IMD3 from n41 and n77 into RX band n20.

5.20.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n5-n41-n77.

Table 5.20.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n41-n77 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2593 | 5 | 25 | 2593 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 29.7 | TDD | IMD22 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2506 | 5 | 25 | 2506 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4165 | 16.1 | TDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 5 | N/A | 2593 | 30.1 | TDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n41 | N/A | 5 | N/A | 2516 | 13.2 | TDD | IMD3 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 30.2 | FDD | IMD2 |
|  | n41 | 2594 | 5 | 25 | 2594 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 817 | 15.2 | FDD | IMD3 |
|  | n41 | 2501 | 5 | 25 | 2501 | N/A | TDD | N/A |
|  | n77 | 4185 | 10 | 50 | 4185 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.21	CA_n20-n41-n78

5.21.1	Common for 1 band UL and 2 bands UL CA

5.21.1.1	Operating bands for CA

Table 5.21.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.21.1.2	Channel bandwidths per operating band for CA

Table 5.21.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n41A-n78A | CA_n20A-n41A CA_n20A-n78A CA_n41A-n78A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.21.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n41-n78, the TIB,c and RIB,c values are re-used from DC_20_n41-n78 as given in the tables below.

Table 5.21.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n78 | 0.5 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.21.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n78 | - | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.21.2	Specific for 2 bands UL CA

5.21.2.1	UE co-existence studies

5.21.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.21.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n78A with UL CA_n20A-n41A.

Table 5.21.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.21.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n78A with UL CA_n20A-n78A.

Table 5.21.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 2968 |  | 4132 - 4662 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 - 1576 |  | 5738 - 6768 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5524 |  | 7432 - 8462 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 - 714 |  | 9038 - 10568 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5936 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6386 |  | 10732 - 12262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 9324 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 - 12338 |  | 148 - 472 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 - 8176 |  | 4014 - 5104 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 16062 |  | 6628 - 7248 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 13124 |  | 9096 - 10186 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.21.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n78A with UL CA_n41A-n78A.

Table 5.21.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1304 |  | 5796 - 6490 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 - 2080 |  | 3910 - 5104 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9180 |  | 9096 - 10290 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 - 4770 |  | 7210 - 8904 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2608 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 11870 |  | 12396 - 14090 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11592 - 12980 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 - 10510 |  | 7460 - 6184 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 - 4520 |  | 1470 - 112 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 17890 |  | 13284 - 14560 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 16780 |  | 14088 - 15670 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.21.2.1.1-2 there are IMD2 and IMD4 from n20 and n41 into RX band n78.

Based on the analysis in Table 5.21.2.1.1-2 there is IMD2 from n20 and n78 into RX band n41.

Based on the analysis in Table 5.21.2.1.1-3 there are IMD2 and IMD5 from n41 and n78 into RX band n20.

5.21.2.2	REFSENS requirements

The MSD requirements for IMD2 components are re-used from DC_20A-n41A-n78A.

The MSD requirements for IMD4 on Rx n78 and IMD5 on Rx n20 are reuse from CA_n5-n7-n78.

Table 5.21.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n41-n78 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2593 | 10 | 50 | 2593 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3440 | 28.8 | TDD | IMD2 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2601 | 10 | 50 | 2601 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3508 | 9.7 | TDD | IMD4 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2593 | 29.8 | TDD | IMD2 |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 30.8 | FDD | IMD2 |
|  | n41 | 2593 | 10 | 50 | 2593 | N/A | TDD | N/A |
|  | n78 | 3399 | 10 | 50 | 3399 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.3 | FDD | IMD5 |
|  | n41 | 2635 | 10 | 50 | 2635 | N/A | TDD | N/A |
|  | n78 | 3549.5 | 10 | 50 | 3549.5 | N/A | TDD | N/A |

5.22	CA_n20-n71-n78

5.22.1	Common for 1 band UL and 2 bands UL CA

5.22.1.1	Operating bands for CA

Table 5.22.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.22.1.2	Channel bandwidths per operating band for CA

Table 5.22.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n71A-n78A | CA_n20A-n71A CA_n20A-n78A CA_n71A-n78A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.22.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n71-n78, the TIB,c and RIB,c values are re-used from CA_n5-n78-n105 as given in the tables below.

Table 5.22.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n71-n78 | 0.7 | 0.7 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.22.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n71-n78 | 0.2 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.22.2	Specific for 2 bands UL CA

5.22.2.1	UE co-existence studies

5.22.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.22.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n71A-n78A with UL CA_n20A-n71A.

Table 5.22.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.22.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n71A-n78A with UL CA_n20A-n78A.

Table 5.22.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 2968 |  | 4132 - 4662 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 - 1576 |  | 5738 - 6768 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5524 |  | 7432 - 8462 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 - 714 |  | 9038 - 10568 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5936 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6386 |  | 10732 - 12262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 9324 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 - 12338 |  | 148 - 472 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 - 8176 |  | 4014 - 5104 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 16062 |  | 6628 - 7248 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 13124 |  | 9096 - 10186 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.22.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n71A-n78A with UL CA_n71A-n78A.

Table 5.22.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3137 |  | 3963 - 4498 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2474 - 1904 |  | 5902 - 6937 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5196 |  | 7263 - 8298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1811 - 1206 |  | 9202 - 10737 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6274 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 5894 |  | 10563 - 12098 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 8996 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14537 - 12502 |  | 508 - 1148 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10074 - 8504 |  | 4506 - 5611 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 15898 |  | 5952 - 6592 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 12796 |  | 8589 - 9694 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.22.2.1.1-1 there is IMD5 from 20 and n71 into RX band n78.

Based on the analysis in Table 5.22.2.1.1-3 there is IMD5 from n71 and n78 into RX band n20.

5.22.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n5-n78-n105.

Table 5.22.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n71-n78 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3567 | 4 | TDD | IMD5 |
|  | n20 | N/A | 5 | N/A | 808 | 3.8 | FDD | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n78 | 3528 | 10 | 50 | 3528 | N/A | TDD | N/A |

5.23	CA_n3-n20-n71

5.23.1	Common for 1 band UL and 2 bands UL CA

5.23.1.1	Operating bands for CA

Table 5.23.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n71 | 663-698 | 617-652 | FDD |

5.23.1.2	Channel bandwidths per operating band for CA

Table 5.23.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n20A-n71A | CA_n3A-n20A CA_n3A-n71A CA_n20A-n71A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n71 | 5, 10, 15, 20 |  |

5.23.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n20-n71, the TIB,c and RIB,c values are re-used from DC_3_n5-n105 as given in the tables below.

Table 5.23.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n71 | 0.3 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.23.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n71 | - | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.23.2	Specific for 2 bands UL CA

5.23.2.1	UE co-existence studies

5.23.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.23.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n71A with UL CA_n3A-n20A.

Table 5.23.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 953 - 848 |  | 2542 - 2647 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 - 2738 |  | 121 - 14 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 - 4432 |  | 3374 - 3509 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 - 4523 |  | 711 - 876 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1696 - 1906 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 - 6217 |  | 4206 - 4371 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5084 - 5294 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 - 1543 |  | 6308 - 5978 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 834 - 1074 |  | 3691 - 3406 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 - 5233 |  | 7672 - 8002 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6156 |  | 6794 - 7079 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.23.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n71A with UL CA_n3A-n71A.

Table 5.23.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.23.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n71A with UL CA_n20A-n71A.

Table 5.23.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.23.2.1.1-3 there are IMD4 and IMD5 from n20 and n71 into RX band n3.

5.23.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n20-n28 for IMD4 component and DC_2A-5A_n12A for IMD5 component.

Table 5.23.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n20-n71 | n3 | N/A | 5 | N/A | 1843 | 9.4 | FDD | IMD4 |
|  | n20 | 841 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1841 | 5.9 | FDD | IMD5 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n71 | 672 | 5 | 25 | 626 | N/A | FDD | N/A |

5.24	CA_n3-n20-n77

5.24.1	Common for 1 band UL and 2 bands UL CA

5.24.1.1	Operating bands for CA

Table 5.24.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.24.1.2	Channel bandwidths per operating band for CA

Table 5.24.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n20A-n77A | CA_n3A-n20A CA_n3A-n77A CA_n20A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n20A-n77(2A) | CA_n3A-n20A CA_n3A-n77A CA_n20A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.24.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n20-n77, the TIB,c and RIB,c values are re-used from CA_n3-n20-n78 as given in the tables below.

Table 5.24.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n77 | 0.5 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.24.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n77 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.24.2	Specific for 2 bands UL CA

5.24.2.1	UE co-existence studies

5.24.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.24.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n77A with UL CA_n3A-n20A.

Table 5.24.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 953 - 848 |  | 2542 - 2647 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 - 2738 |  | 121 - 14 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 - 4432 |  | 3374 - 3509 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 - 4523 |  | 711 - 876 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1696 - 1906 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 - 6217 |  | 4206 - 4371 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5084 - 5294 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 - 1543 |  | 6308 - 5978 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 834 - 1074 |  | 3691 - 3406 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 - 5233 |  | 7672 - 8002 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6156 |  | 6794 - 7079 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.24.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n77A with UL CA_n3A-n77A.

Table 5.24.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2490 |  | 5010 - 5985 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 - 270 |  | 4815 - 6690 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7770 |  | 8310 - 10185 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 - 2055 |  | 8115 - 10890 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4980 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9555 |  | 11610 - 14385 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 - 11415 |  | 3840 - 2640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 - 6330 |  | 1245 - 3270 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 18585 |  | 10140 - 11340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 16170 |  | 11730 - 13755 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.24.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n77A with UL CA_n20A-n77A.

Table 5.24.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.24.2.1.1-1 there are IMD3 and IMD5 from n3 and n20 into RX band n77.

Based on the analysis in Table 5.24.2.1.1-3 there is IMD3 from n20 and n77 into RX band n3.

5.24.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n18-n77.

Table 5.24.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n20-n77 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3441 | 16.3 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1842 | 15.7 | FDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | 3536 | 10 | 50 | 3536 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.25	CA_n1-n3-n71

5.25.1	Common for 1 band UL and 2 bands UL CA

5.25.1.1	Operating bands for CA

Table 5.25.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n71 | 663-698 | 617-652 | FDD |

5.25.1.2	Channel bandwidths per operating band for CA

Table 5.25.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n71A | CA_n1A-n3A CA_n1A-n71A CA_n3A-n71A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | 5,10,15,20,25,30,35,40,45,50 |  |
|  |  | n71 | 5,10,15,20 |  |
| CA_n1A-n3(2A)-n71A | CA_n1A-n3A CA_n1A-n71A CA_n3A-n71A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS 4 and 5 |  |
|  |  | n71 | 5,10,15,20 |  |

5.25.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n71, the TIB,c and RIB,c values are re-used from CA_n1-n3-n105 and are given in the tables below.

Table 5.25.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n3-n71 | 0.3 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.25.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n3-n71 | 0.3 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.25.2	Specific for 2 bands UL CA

5.25.2.1	UE co-existence studies

5.25.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.25.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n71A with UL CA_n1A-n3A.

Table 5.25.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 - 135 |  | 3630 - 3765 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 - 2250 |  | 1440 - 1650 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 - 5745 |  | 5340 - 5550 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 - 4230 |  | 3150 - 3435 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 270 - 540 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 - 7725 |  | 7050 - 7335 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 - 7530 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 - 4860 |  | 6210 - 5895 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 - 1170 |  | 2520 - 2190 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 - 9120 |  | 9390 - 9705 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 - 9315 |  | 9180 - 9510 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.25.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n71A with UL CA_n1A-n71A.

Table 5.25.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.25.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n71A with UL CA_n3A-n71A.

Table 5.25.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.25.2.1.1-2 there is IMD5 from n1 and n71 into RX band n3.

Based on the analysis in Table 5.25.2.1.1-3 there is IMD4 from n3 and n71 into RX band n1.

5.25.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n3-n105.

Table 5.25.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n71 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |

5.26	CA_n1-n3-n8

5.26.1	Common for 1 band UL and 2 bands UL CA

5.26.1.1	Operating bands for CA

Table 5.26.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n8 | 880-915 | 925-960 | FDD |

5.26.1.2	Channel bandwidths per operating band for CA

Table 5.26.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n8A | CA_n1A-n3A CA_n1A-n8A CA_n3A-n8A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | 5,10,15,20,25,30,35,40,45,50 |  |
|  |  | n8 | 5,10,15,20 |  |
| CA_n1A-n3(2A)-n8A | CA_n1A-n3A CA_n1A-n8A CA_n3A-n8A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | CA_n3(2A) BCS 4 & 5 |  |
|  |  | n8 | 5,10,15,20 |  |

5.26.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n8, the TIB,c and RIB,c values have already been specified.

5.26.2	Specific for 2 bands UL CA

5.26.2.1	UE co-existence studies

5.26.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.26.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n8A with UL CA_n1A-n3A.

Table 5.26.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 - 135 |  | 3630 - 3765 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 - 2250 |  | 1440 - 1650 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 - 5745 |  | 5340 - 5550 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 - 4230 |  | 3150 - 3435 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 270 - 540 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 - 7725 |  | 7050 - 7335 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 - 7530 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 - 4860 |  | 6210 - 5895 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 - 1170 |  | 2520 - 2190 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 - 9120 |  | 9390 - 9705 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 - 9315 |  | 9180 - 9510 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.26.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n8A with UL CA_n1A-n8A.

Table 5.26.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 - 1005 |  | 2800 - 2895 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 - 3080 |  | 220 - 90 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 - 4875 |  | 3680 - 3810 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 - 5060 |  | 660 - 825 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2010 - 2200 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 - 6855 |  | 4560 - 4725 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5600 - 5790 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 - 1540 |  | 7040 - 6765 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 - 1320 |  | 4180 - 3930 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 - 5640 |  | 8560 - 8835 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 - 6705 |  | 7520 - 7770 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.26.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n8A with UL CA_n3A-n8A.

Table 5.26.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 - 795 |  | 2590 - 2700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 - 2690 |  | 25 - 120 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 - 4485 |  | 3470 - 3615 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4215 - 4475 |  | 855 - 1035 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1590 - 1810 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6010 - 6270 |  | 4350 - 4530 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5180 - 5400 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 - 1735 |  | 6260 - 5925 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 675 - 930 |  | 3595 - 3300 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 - 5445 |  | 7720 - 8055 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 - 6315 |  | 6890 - 7185 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The analysis of the intermodulation products show that the added uplink CA combinations create no intermodulation products inside the third receiver band.

5.26.2.2	REFSENS requirements

No new requirements needed

5.27	CA_n1-n41-n71

5.27.1	Common for 1 band UL and 2 bands UL CA

5.27.1.1	Operating bands for CA

Table 5.27.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |

5.27.1.2	Channel bandwidths per operating band for CA

Table 5.27.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n41A-n71A | CA_n1A-n41A CA_n1A-n71A CA_n41A-n71A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n71 | 5,10,15,20 |  |

5.27.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n41-n71, the TIB,c and RIB,c values are re-used from CA_n1-n40-n105 and are given in the tables below.

Table 5.27.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n71 | 0.5 | 0.5 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.27.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n71 | - | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.27.2	Specific for 2 bands UL CA

5.27.2.1	UE co-existence studies

5.27.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.27.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n71A with UL CA_n1A-n41A.

Table 5.27.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 - 9340 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.27.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n71A with UL CA_n1A-n71A.

Table 5.27.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.27.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n71A with UL CA_n41A-n71A.

Table 5.27.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2027 - 1798 |  | 3159 - 3388 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4294 - 4717 |  | 1364 - 1100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5655 - 6078 |  | 3822 - 4086 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6790 - 7407 |  | 701 - 402 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3596 - 4054 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8151 - 8768 |  | 4485 - 4784 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6318 - 6776 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 296 - 38 |  | 10097 - 9286 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2898 - 3391 |  | 6744 - 6092 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5148 - 5482 |  | 10647 - 11458 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6981 - 7474 |  | 8814 - 9466 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.27.2.1.1-1 there is IMD2 and IMD5 from n1 and n41 into RX band n71.

Based on the analysis in Table 5.27.2.1.1-2 there is IMD2 and IMD4 from n1 and n71 into RX band n41.

5.27.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n7-n105.

Table 5.27.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n41-n71 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n41 | 2565 | 10 | 50 | 2565 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 28.7 | FDD | IMD2 |
|  | n1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n41 | 2565 | 10 | 50 | 2565 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 645 | 1 | FDD | IMD5 |
|  | n1 | 1968.5 | 5 | 25 | 2158.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2634.5 | 27 | FDD | IMD22 |
|  | n71 | 666 | 5 | 25 | 620 | N/A | FDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.28	CA_n1-n8-n41

5.28.1	Common for 1 band UL and 2 bands UL CA

5.28.1.1	Operating bands for CA

Table 5.28.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n8 | 880-915 | 925-960 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |

5.28.1.2	Channel bandwidths per operating band for CA

Table 5.28.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n8A-n41A | CA_n1A-n8A CA_n1A-n41A CA_n8A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |

5.28.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n8-n41, the TIB,c and RIB,c values are given in the tables below following CA_n1-n8-n40.

Table 5.28.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n8-n41 | 0.3 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.28.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n8-n41 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.28.2	Specific for 2 bands UL CA

5.28.2.1	UE co-existence studies

5.28.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.28.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n8A-n41A with UL CA_n1A-n8A.

Table 5.28.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 - 1005 |  | 2800 - 2895 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 - 3080 |  | 220 - 90 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 - 4875 |  | 3680 - 3810 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 - 5060 |  | 660 - 825 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2010 - 2200 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 - 6855 |  | 4560 - 4725 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5600 - 5790 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 - 1540 |  | 7040 - 6765 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 - 1320 |  | 4180 - 3930 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 - 5640 |  | 8560 - 8835 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 - 6705 |  | 7520 - 7770 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.28.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n8A-n41A with UL CA_n1A-n41A.

Table 5.28.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 - 9340 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.28.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n8A-n41A with UL CA_n8A-n41A.

Table 5.28.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1581 - 1810 |  | 3376 - 3605 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 - 666 |  | 4077 - 4500 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4256 - 4520 |  | 5872 - 6295 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 50 - 249 |  | 6573 - 7190 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3620 - 3162 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5136 - 5435 |  | 8368 - 8985 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6752 - 7210 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9880 - 9069 |  | 1164 - 830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6310 - 5658 |  | 2247 - 2740 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10864 - 11675 |  | 6016 - 6350 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9248 - 9900 |  | 7632 - 8125 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

From the tables it is found that:

-  Band n8 may be subject to IMD5 from n1+n41.

5.28.2.2	REFSENS requirements

The MSD value has been taken from CA_n1A-n7A-n8A.

Table 5.28.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n8-n41 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 927.5 | 1.0 | FDD | IMD5 |
|  | n41 | 2502.5 | 10 | 50 | 2502.5 | N/A | TDD | N/A |

5.29	CA_n3-n41-n71

5.29.1	Common for 1 band UL and 2 bands UL CA

5.29.1.1	Operating bands for CA

Table 5.29.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |

5.29.1.2	Channel bandwidths per operating band for CA

Table 5.29.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n41A-n71A | CA_n3A-n41A CA_n3A-n71A CA_n41A-n71A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n71 | 5,10,15,20 |  |

5.29.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n41-n71, the TIB,c and RIB,c values are re-used from CA_n3-n7-n105 and aregiven in the tables below.

Table 5.29.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n41-n71 | 0.5 | 0.5 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.29.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n41-n71 | - | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.29.2	Specific for 2 bands UL CA

5.29.2.1	UE co-existence studies

5.29.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.29.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n41A-n71A with UL CA_n3A-n41A.

Table 5.29.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 - 980 |  | 4206 - 4475 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 - 1074 |  | 3207 - 3670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6260 |  | 6702 - 7165 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 - 2859 |  | 5703 - 6360 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1960 - 1422 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 - 8045 |  | 9198 - 9855 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8412 - 8950 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 - 8199 |  | 4644 - 4150 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 - 3918 |  | 363 - 250 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 - 12545 |  | 9336 - 9830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 - 11640 |  | 10122 - 10735 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.29.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n41A-n71A with UL CA_n3A-n71A.

Table 5.29.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.29.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n41A-n71A with UL CA_n41A-n71A.

Table 5.29.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2027 - 1798 |  | 3159 - 3388 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4294 - 4717 |  | 1364 - 1100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5655 - 6078 |  | 3822 - 4086 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6790 - 7407 |  | 701 - 402 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3596 - 4054 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8151 - 8768 |  | 4485 - 4784 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6318 - 6776 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 296 - 38 |  | 10097 - 9286 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2898 - 3391 |  | 6744 - 6092 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5148 - 5482 |  | 10647 - 11458 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6981 - 7474 |  | 8814 - 9466 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.29.2.1.1-1 there is IMD2 from n41 and n71 into RX band n3.

5.29.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n7-n105.

Table 5.29.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n71 | n3 | N/A | 5 | N/A | 1875 | 16.5 | FDD | IMD2 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | FDD | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |

5.30	CA_n1-n7-n20

5.30.1	Common for 1 band UL and 2 bands UL CA

5.30.1.1	Operating bands for CA

.

Table 5.30.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz –2170 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |

5.30.1.2	Channel bandwidths per operating band for CA

| Table 5.30.1.2-1: Supported bandwidths per CA band combination of band n25+n41+n77NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n7A-n20A | CA_n1A-n7A CA_n1A-n20A CA_n7A-n20A |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |

5.30.1.3	∆TIB,c and ∆RIB,c values

Requirements for CA_n1-n7-n20, for the TIB,c and RIB,c values are given in the tables below following CA_n1-n7, CA_n1-n20 and Ca_n7-n20  values.

Table 5.30.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n7-n20 | 0.5 | 0.6 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.30.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n7-n20 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.30.2	Specific for 2 bands UL CA

5.30.2.1	UE co-existence studies

5.30.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.30.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n20A with UL CA_n1A-n7A.

Table 5.30.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 650 – 520 |  | 4420 - 4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270- 1460 |  | 3020 - 3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 - 6530 |  | 6920 - 7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190- 3440 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1300 - 1040 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260 - 8510 |  | 9420 - 9100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7088- 6818 |  | 1528 - 1348 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 - 3540 |  | 940 - 620 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4276 - 4036 |  | 1254 - 1464 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 - 7664 |  | 6336 - 6546 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.30.2.1.1-1, 5th order IMD from Band n1 and Band n7 may fall into Rx frequencies of band n20.

Table 5.30.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n20A with UL CA_n1A-n20A.

Table 5.30.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 316 - 196 |  | 2978 - 3128 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3584 - 3704 |  | 4672 - 4822 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 516 - 666 |  | 4898 - 5108 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2296 - 2116 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4416 - 4566 |  | 6592 - 6802 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11440 - 10560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.30.2.1.1-2, there is no IMD impact from UL CA_n1A-n20A into Rx frequencies of band n7.

Table 5.30.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n20A with UL CA_n7A-n20A.

Table 5.30.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1738 - 1638 |  | 3332 - 3432 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 906 - 776 |  | 4138 - 4308 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4164 - 4294 |  | 5832 - 6002 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 - 86 |  | 6638 - 6878 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3476 - 3276 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 - 5156 |  | 8332 - 8572 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6664 - 6864 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9448 - 9138 |  | 948 - 758 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6046 - 5776 |  | 2414 - 2644 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 - 11142 |  | 5828 - 6018 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 - 9434 |  | 7496 - 7726 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.30.2.1.1-3, there is no IMD impact from UL CA_n7A-n20A into Rx frequencies of band n1.

From the tables above it is found that:

-  Band n20 may be subject to IMD5.

5.30.2.2	REFSENS requirements

MSD values have been taken from CA_n1A-n7A-n28A.

Table 5.30.2.2-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n7-n20 | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 800 | 4.5 | FDD | IMD5 |

5.31	CA_n1-n3-n75

5.31.1	Common for 1 band UL and 2 bands UL CA

5.31.1.1	Operating bands for CA

Table 5.31.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.31.1.2	Channel bandwidths per operating band for CA

Table 5.31.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n75A | CA_n1A-n3A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

5.31.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.31.2	Specific for 2 bands UL CA

5.31.2.1	UE co-existence studies

5.31.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.31.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n75A with UL CA_n1A-n3A.

Table 5.31.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135–270 |  | 3630–3765 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055–2250 |  | 1440–1650 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550–5745 |  | 5340–5550 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975–4230 |  | 3150–3435 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 270–540 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470–7725 |  | 7050–7335 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260–7530 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4860–5220 |  | 5895–6210 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1170–1515 |  | 2190–2520 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760–9120 |  | 9390–9705 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970–9315 |  | 9180–9510 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd and 5th order IMD generated by UL CA_n1A-n3A may fall into own Rx of Band n75.

5.31.2.2	REFSENS requirements

We analyze the IMD using the following component linearity assumptions:

Table 5.31.2.2-1 General linearity parameters

| Component | IP3 (dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Triplexer | 82 |
| PA forward mixing | 30 |
| PA reverse mixing | 28 |
| LNA | -6 |

Based on these parameters we get the following MSD.

Table 5.31.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n75 | n1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1480 | 24.2 | SDL | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.32	CA_n1-n7-n78

5.32.1	Common for 1 band UL and 2 bands UL CA

5.32.1.1	Operating bands for CA

Table 5.32.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.32.1.2	Channel bandwidths per operating band for CA

Table 5.32.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

5.32.1.3	∆TIB,c and ∆RIB,c values

For CA_n1A-n7A-n78A, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.32.2	Specific for 2 bands UL CA

5.32.2.1	UE co-existence studies

5.32.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.32.2.1.1-1, Table 5.32.2.1.1-2 and Table 5.32.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n78A with UL CA_n1A-n7A, UL CA_n1A-n78A and UL CA_n7A-n78A.

Table 5.32.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520–650 |  | 4420–4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270–1460 |  | 3020–3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340–5745 |  | 6920–7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190–3440 |  | 5520–5790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1040–1300 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260–8510 |  | 9420–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8840–9100 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8020–8360 |  | 5110–5420 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3540–3870 |  | 620–940 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920–12260 |  | 10180–10490 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340–11670 |  | 10760–11080 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th and 5th order IMD generated by UL CA_n1A-n7A may fall into own Rx of Band n78

Table 5.32.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320–1880 |  | 5520–5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40–660 |  | 4620–5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140–7760 |  | 8520–9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960–2640 |  | 7920–79480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2640–3760 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060–9740 |  | 11820–13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440–11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220–13280 |  | 3880–4620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940–7560 |  | 660–1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120–17180 |  | 10980–11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740–15360 |  | 12360–13540 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th order IMD generated by UL CA_n1A-n78A may fall into own Rx of Band n7.

Table 5.32.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1300 |  | 5800–6370 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200–1840 |  | 4030–5100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–8940 |  | 9100–10170 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700–4410 |  | 7330–8900 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460–2600 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11510 |  | 12400–13970 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–12740 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–12700 |  | 6200–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–6400 |  | 100–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–17770 |  | 13300–14080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–16540 |  | 14100–15310 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th order IMD generated by UL CA_n7A-n78A may fall into own Rx of Band n1.

5.32.2.2	REFSENS requirements

For CA_n1-n7-n78, the MSD values have already been specified in TS 38.101-1. However，the IMD5 from CA_n1A-n7A to Rx n78 is missing. Here IMD5 to n78 is added.

| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n7 | N/A | 5 | N/A | 2627.5 | 9.1 | FDD | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 8.7 | FDD | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 10.1 | TDD | IMD4 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3770 | 1.3 | TDD | IMD5 |

## 5.33	CA_n7-n25-n29

### 5.33.1	Common for 1 band UL and 2 bands UL CA

#### 5.33.1.1	Operating bands for CA

Table 5.33.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |

#### 5.33.1.2	Channel bandwidths per operating band for CA

Table 5.33.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n25A-n29A | CA_n7A-n25A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |

#### 5.33.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n25-n29, the TIB,c and RIB,c values are given in the tables below.

Table 5.33.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n25-n29 | 0.5 | 0.5 | N/A |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.33.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n25-n29 | - | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.33.2	Specific for 2 bands UL CA

#### 5.33.2.1	UE co-existence studies

##### 5.33.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.33.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n25A-n29A with UL CA_n7A-n25A.

Table 5.33.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 585–720 |  | 4350–4485 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3085–3290 |  | 1130–1330 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6850–7055 |  | 6200–6400 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5585–5860 |  | 2980–3245 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1170–1440 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9350–9625 |  | 8050–8315 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8700–8970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4830–5160 |  | 8085–8430 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 410–745 |  | 3670–4010 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9900–10230 |  | 11850–12195 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10550–10885 |  | 11200–11540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 2nd and 5th order IMD generated by UL CA_n7A-n25A may fall into own Rx of Band n29.

#### 5.33.2.2	REFSENS requirements

Based on the co-existence studies, there is a need to define MSD values. MSD values are specified as following.

Table 5.33.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n25-n29 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 719.5 | 26 | SDL | IMD2 |
|  | n7 | 2503 | 5 | 25 | 2623 | N/A | FDD | N/A |
|  | n25 | 1910 | 5 | 25 | 1990 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 724 | 4.5 | SDL | IMD5 |

## 5.34	CA_n7-n29-n66

### 5.34.1	Common for 1 band UL and 2 bands UL CA

#### 5.34.1.1	Operating bands for CA

Table 5.34.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |

#### 5.34.1.2	Channel bandwidths per operating band for CA

Table 5.34.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n29A-n66A | CA_n7A-n66A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |

#### 5.34.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n29-n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.34.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n66 | 0.5 | N/A | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.34.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n66 | 0.5 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.34.2	Specific for 2 bands UL CA

#### 5.34.2.1	UE co-existence studies

##### 5.34.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.34.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n29A-n66A with UL CA_n7A-n66A.

Table 5.34.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 720–860 |  | 4210–4350 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220–3430 |  | 850–1060 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6710–6920 |  | 5920–6130 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5720–6000 |  | 2560–2840 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1440–1720 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9210–9490 |  | 7630–7910 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8420–8700 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4270–4620 |  | 8220–8570 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10–340 |  | 3940–4290 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9340–9690 |  | 11710–12060 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10130–10480 |  | 10920–11270 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 2nd order IMD generated by UL CA_n7A-n66A may fall into own Rx of Band n29.

#### 5.34.2.2	REFSENS requirements

Based on the co-existence studies, there is a need to define MSD values. MSD values from DC_12-66_n7 are reused as below.

Table 5.34.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n29-n66 | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 725 | 31 | SDL | IMD2 |
|  | n66 | 1777.5 | 5 | 25 | 2197.5 | N/A | FDD | N/A |

## 5.35	CA_n7-n29-n77

### 5.35.1	Common for 1 band UL and 2 bands UL CA

#### 5.35.1.1	Operating bands for CA

Table 5.35.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

#### 5.35.1.2	Channel bandwidths per operating band for CA

Table 5.35.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n29A-n77A | CA_n7A-n77A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n29A-n77(2A) | CA_n7A-n77A CA_n77(2A) | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n7A-n29A-n77(3A) | CA_n7A-n77A CA_n77(2A) | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |

#### 5.35.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n29-n77 the TIB,c and RIB,c values are given in the tables below, reusing from CA_n7-n12-n77.

Table 5.35.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n77 | 0.5 | N/A | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.35.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n77 | 0.2 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.35.2	Specific for 2 bands UL CA

#### 5.35.2.1	UE co-existence studies

##### 5.35.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.35.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n29A-n77A with UL CA_n7A-n77A.

Table 5.35.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1700 |  | 5800–6770 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800–1840 |  | 4030–5900 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–9340 |  | 9100–10970 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3300–4410 |  | 7330–10100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460-3400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11910 |  | 12400–15170 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–13540 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–14300 |  | 5800–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–7600 |  | 690–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–19370 |  | 13300–14480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–17740 |  | 14100–16110 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, IMD5 generated by UL CA_n7A-n77A fall into own Rx of Band n29.

In addition, there is no additional MSD due to UL CA_n77(2A) needs to be specified for three bands DL configuration.

#### 5.35.2.2	REFSENS requirements

Based on the co-existence studies, MSD for IMD5 is introduced as following, the values are reused from DC_7A-29A_n78A.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n29-n77 | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 720 | 3.0 | SDL | IMD5 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |

  - 5.36			n3A-n40A-n79A

    - 5.36.1	Common for 1 band UL and 2 bands UL CA

      - 5.36.1.1		Operating bands for CA

Table 5.36.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

      - 5.36.1.2	Channel bandwidths per operating band for CA

Table 5.36.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n40A-n79A | CA_n3A-n40A CA_n3A-n79A CA_n40A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

      - 5.36.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n40-n79, the TIB,c and RIB,c values are proposed in the following tables:

Table 5.36.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n40-n79 | 0.5 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.36.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n40-n79 | - | - | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n3 the band order from left to right is n1, n3 and n3. |  |  |  |

    - 5.36.2	Specific for 2 bands UL CA

      - 5.36.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n40A, CA_n3A-n79A and CA_n40A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD5 issue caused by n3+n40, i.e.  fn40-4*fn3, fall into the its own band n79 Rx;

IMD5 issue caused by n3+n79, i.e.  fn79-4*fn3, fall into the its own band n40 Rx;

IMD5 issue caused by n40+n79, i.e.  2*fn79-3*fn40,  fall into the its own band n3 Rx;

      - 5.36.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined. The MSD values are proposed in table 5.36.2.2-1..

Table 5.36.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3-n40-n79 | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n79 | N/A | 40 | N/A | 4820 | 3.3 | IMD5 |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n40 | N/A | 5 | N/A | 2310 | 5.9 | IMD5 |
|  | n79 | 4820 | 40 | 216 | 4820 | N/A | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 4.7 | IMD5 |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |

5.37	CA_n3-n7-n77

5.37.1	Common for 1 band UL and 2 bands UL CA

5.37.1.1	Operating bands for CA

Table 5.37.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n7 | 2500-2570 | 2620-2690 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.37.1.2	Channel bandwidths per operating band for CA

Table 5.37.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n7A-n77A | CA_n3A-n7A CA_n3A-n77A CA_n7A-n77A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n77(2A) | CA_n77(2A) CA_n3A-n7A CA_n3A-n77A CA_n7A-n77A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.37.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n7-n77, the TIB,c and RIB,c values are given in the tables below as taken from CA_n3-n7-n78.

Table 5.37.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n7-n77 | 0.6 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.37.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n7-n77 | 0.2 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.37.2	Specific for 2 bands UL CA

5.37.2.1	UE co-existence studies

5.37.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.37.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n77A with UL CA_n3A-n7A.

Table 5.37.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 - 860 |  | 4210 - 4355 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 - 1070 |  | 3215 - 3430 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 - 6140 |  | 6710 - 6925 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2560 - 2855 |  | 5715 - 6000 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1720 - 1430 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7630 - 7925 |  | 9210 - 9495 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8420 - 8710 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 - 8215 |  | 4640 - 4270 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 - 3930 |  | 355 - 10 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 - 12065 |  | 9340 - 9710 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 - 11280 |  | 10130 - 10495 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.37.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n77A with UL CA_n3A-n77A.

Table 5.37.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2490 |  | 5010 - 5985 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 - 270 |  | 4815 - 6690 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7770 |  | 8310 - 10185 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 - 2055 |  | 8115 - 10890 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4980 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9555 |  | 11610 - 14385 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 - 11415 |  | 3840 - 2640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 - 6330 |  | 1245 - 3270 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 18585 |  | 10140 - 11340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 16170 |  | 11730 - 13755 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.37.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n77A with UL CA_n7A-n77A.

Table 5.37.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 - 1700 |  | 5800 - 6770 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 - 1840 |  | 4030 - 5900 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 - 9340 |  | 9100 - 10970 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3300 - 4410 |  | 7330 - 10100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3400 - 1460 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 - 11910 |  | 12400 - 15170 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600 - 13540 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14300 - 10630 |  | 6980 - 5800 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7600 - 4760 |  | 1110 - 900 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 - 19370 |  | 13300 - 14480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 - 17740 |  | 14100 - 16110 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.37.2.1.1-1 there is IMD3 and IMD5 from n3 and n7 into RX band n77.

Based on the analysis in Table 5.37.2.1.1-2 there is IMD5 from n3 and n77 into RX band n7.

Based on the analysis in Table 5.37.2.1.1-3 there is IMD3 and IMD4 from n7 and n77 into RX band n3.

5.37.2.2	REFSENS requirements

The MSD requirements are based on CA_n3-n7-n78 and DC_3-7_n77.

Table 5.37.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n77 | n3 | N/A | 5 | N/A | 1820 | 17.6 | FDD | IMD3 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 8.6 | FDD | IMD4 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2685 | 3.4 | FDD | IMD5 |
|  | n77 | 4175 | 10 | 50 | 4175 | N/A | TDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD31 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.38	CA_n1-n20-n41

5.38.1	Common for 1 band UL and 2 bands UL CA

5.38.1.1	Operating bands for CA

Table 5.38.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |

5.38.1.2	Channel bandwidths per operating band for CA

Table 5.38.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n41A | CA_n1A-n20A CA_n1A-n41A CA_n20A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |

5.38.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n41, the TIB,c values are reused from CA_n1-n18-n41 which are given in the tables below.

Table 5.38.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n41 | 0.5 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

RIB,c values are reused from CA_n1-n28-n41 which are given in the tables below.

Table 5.38.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n41 | - | 0.2 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.38.2	Specific for 2 bands UL CA

5.38.2.1	UE co-existence studies

5.38.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.38.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n41A with UL CA_n1A-n20A.

Table 5.38.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.38.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n41A with UL CA_n1A-n41A.

Table 5.38.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 - 9340 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.38.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n41A with UL CA_n20A-n41A.

Table 5.38.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.38.2.1.1-2 there is IMD5 from n1 and n41 into RX band n20.

5.38.2.2	REFSENS requirements

The MSD value has been taken from CA_n1-n18-n41.

Table 5.38.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n41 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 810 | 3.3 | FDD | IMD5 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |

5.39	CA_n1-n20-n77

5.39.1	Common for 1 band UL and 2 bands UL CA

5.39.1.1	Operating bands for CA

Table 5.39.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.39.1.2	Channel bandwidths per operating band for CA

Table 5.39.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n77A | CA_n1A-n20A CA_n1A-n77A CA_n20A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 4 and 5 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n20A-n77(2A) | CA_n1A-n20A CA_n1A-n77A CA_n20A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 4 and 5 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.39.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n77, the TIB,c and RIB,c values are re-used from CA_n1-n20-n78 as given in the tables below.

Table 5.39.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n77 | 0.3 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.39.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n77 | - | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.39.2	Specific for 2 bands UL CA

5.39.2.1	UE co-existence studies

5.39.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.39.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n77A with UL CA_n1A-n20A.

Table 5.39.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.39.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n77A with UL CA_n1A-n77A.

Table 5.39.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 2280 |  | 5220 - 6180 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 - 660 |  | 4620 - 6480 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 8160 |  | 8520 - 10380 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 - 2640 |  | 7920 - 10680 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4560 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 10140 |  | 11820 - 14580 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 12360 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14880 - 11220 |  | 4620 - 3480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8760 - 5940 |  | 660 - 2640 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 18780 |  | 10980 - 12120 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 16560 |  | 12360 - 14340 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.39.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n77A with UL CA_n20A-n77A.

Table 5.39.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.39.2.1.1-1 there are IMD3 and IMD5 from n1 and n20 into RX band n77.

Based on the analysis in Table 5.39.2.1.1-2 there is IMD5 from n1 and n77 into RX band n20.

Based on the analysis in Table 5.39.2.1.1-3 there is IMD3 from n20 and n77 into RX band n1.

5.39.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n18-n77.

Table 5.39.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3644 | 15.7 | TDD | IMD31 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.5 | FDD | IMD5 |
|  | n77 | 3328 | 10 | 50 | 3328 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 16.4 | FDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | 3834 | 10 | 50 | 3834 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.40	CA_n1-n71-n77

5.40.1	Common for 1 band UL and 2 bands UL CA

5.40.1.1	Operating bands for CA

Table 5.40.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.40.1.2	Channel bandwidths per operating band for CA

Table 5.40.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n71A-n77A | CA_n1A-n71A CA_n1A-n77A CA_n71A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n1A-n71A-n77(2A) | CA_n1A-n71A CA_n1A-n77A CA_n71A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |

5.40.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n71-n77, the TIB,c and RIB,c values are re-used from CA_n1-n78-n105 and are given in the tables below.

Table 5.40.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n77 | 0.3 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.40.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n77 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.40.2	Specific for 2 bands UL CA

5.40.2.1	UE co-existence studies

5.40.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.40.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n77A with UL CA_n1A-n71A.

Table 5.40.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.40.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n77A with UL CA_n1A-n77A.

Table 5.40.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 2280 |  | 5220 - 6180 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 - 660 |  | 4620 - 6480 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 8160 |  | 8520 - 10380 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 - 2640 |  | 7920 - 10680 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4560 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 10140 |  | 11820 - 14580 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 12360 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14880 - 11220 |  | 4620 - 3480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8760 - 5940 |  | 660 - 2640 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 18780 |  | 10980 - 12120 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 16560 |  | 12360 - 14340 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.40.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n77A with UL CA_n71A-n77A.

Table 5.40.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3537 |  | 3963 - 4898 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 - 1904 |  | 5902 - 7737 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5596 |  | 7263 - 9098 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 - 1206 |  | 9202 - 11937 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7074 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 6294 |  | 10563 - 13298 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 9796 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 - 12502 |  | 508 - 1548 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 - 8504 |  | 4506 - 6411 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 17498 |  | 5952 - 6992 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 13996 |  | 8589 - 10494 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.40.2.1.1-1 there is IMD3 and IMD4 from n1 and n71 into RX band n77.

Based on the analysis in Table 5.40.2.1.1-2 there is IMD3 from n1 and n77 into RX band n71.

Based on the analysis in Table 5.40.2.1.1-3 there is IMD3 and IMD4 from n71 and n77 into RX band n1.

5.40.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n78-n105.

Table 5.40.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n71-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3992 | 9.4 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n77 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2141 | 10.1 | FDD | IMD4 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 4181 | 10 | 50 | 4181 | N/A | TDD | N/A |

5.41	CA_n3-n20-n41

5.41.1	Common for 1 band UL and 2 bands UL CA

5.41.1.1	Operating bands for CA

Table 5.41.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |

5.41.1.2	Channel bandwidths per operating band for CA

Table 5.41.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n20A-n41A | CA_n3A-n20A CA_n3A-n41A CA_n20A-n41A | n3 | 5, 10, 15, 20, 25, 30, 45, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |

5.41.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n20-n41, the TIB,c and RIB,c values are re-used from CA_n3-n18-n41 as given in the tables below.

Table 5.41.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n41 | 0.5 | 0.3 | 0.31 / 0.82 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |  |

Table 5.41.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n41 | - | - | 01 / 0.52 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |  |

5.41.2	Specific for 2 bands UL CA

5.41.2.1	UE co-existence studies

5.41.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.41.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n41A with UL CA_n3A-n20A.

Table 5.41.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 953 - 848 |  | 2542 - 2647 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 - 2738 |  | 121 - 14 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 - 4432 |  | 3374 - 3509 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 - 4523 |  | 711 - 876 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1696 - 1906 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 - 6217 |  | 4206 - 4371 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5084 - 5294 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 - 1543 |  | 6308 - 5978 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 834 - 1074 |  | 3691 - 3406 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 - 5233 |  | 7672 - 8002 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6156 |  | 6794 - 7079 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.41.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n41A with UL CA_n3A-n41A.

Table 5.41.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 - 980 |  | 4206 - 4475 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 - 1074 |  | 3207 - 3670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6260 |  | 6702 - 7165 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 - 2859 |  | 5703 - 6360 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1960 - 1422 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 - 8045 |  | 9198 - 9855 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8412 - 8950 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 - 8199 |  | 4644 - 4150 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 - 3918 |  | 363 - 250 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 - 12545 |  | 9336 - 9830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 - 11640 |  | 10122 - 10735 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.41.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n41A with UL CA_n20A-n41A.

Table 5.41.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.41.2.1.1-2 there are IMD2 and IMD3 from n3 and n20 into RX band n41.

Based on the analysis in Table 5.41.2.1.1-2 there are IMD2 and IMD3 from n3 and n41 into RX band n20.

Based on the analysis in Table 5.41.2.1.1-3 there is IMD2 from n20 and n41 into RX band n3.

5.41.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n18-n41, excepts for IMD2 from n3 and n20 into RX band n41 where MSD requirement is re-used from CA_n5-n41-n66.

Table 5.41.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n20-n41 | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2623 | 29 | TDD | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2594 | 16 | TDD | IMD3 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 28.9 | FDD | IMD2 |
|  | n41 | 2553 | 10 | 50 | 2553 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 19 | FDD | IMD3 |
|  | n41 | 2624 | 10 | 50 | 2624 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1831 | 28.9 | FDD | IMD2 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2678 | 10 | 50 | 2678 | N/A | TDD | N/A |

5.42	CA_n3-n71-n77

5.42.1	Common for 1 band UL and 2 bands UL CA

5.42.1.1	Operating bands for CA

Table 5.42.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.42.1.2	Channel bandwidths per operating band for CA

Table 5.42.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n71A-n77A | CA_n3A-n71A CA_n3A-n77A CA_n71A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n3A-n71A-n77(2A) | CA_n3A-n71A CA_n3A-n77A CA_n71A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS_4 and 5 |  |

5.42.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n71-n77, the TIB,c and RIB,c values are given in the tables below(Re-use CA_n3-n78-n105).

Table 5.42.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n77 | 0.6 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.42.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n77 | 0.2 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.42.2	Specific for 2 bands UL CA

5.42.2.1	UE co-existence studies

5.42.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.42.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n77A with UL CA_n3A-n71A.

Table 5.42.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.42.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n77A with UL CA_n3A-n77A.

Table 5.42.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2490 |  | 5010 - 5985 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 - 270 |  | 4815 - 6690 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7770 |  | 8310 - 10185 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 - 2055 |  | 8115 - 10890 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4980 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9555 |  | 11610 - 14385 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 - 11415 |  | 3840 - 2640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 - 6330 |  | 1245 - 3270 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 18585 |  | 10140 - 11340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 16170 |  | 11730 - 13755 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.42.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n77A with UL CA_n71A-n77A.

Table 5.42.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3537 |  | 3963 - 4898 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 - 1904 |  | 5902 - 7737 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5596 |  | 7263 - 9098 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 - 1206 |  | 9202 - 11937 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7074 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 6294 |  | 10563 - 13298 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 9796 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 - 12502 |  | 508 - 1548 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 - 8504 |  | 4506 - 6411 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 17498 |  | 5952 - 6992 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 13996 |  | 8589 - 10494 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.42.2.1.1-1 there is IMD3, IMD4 and IMD5 from n3 and n71 into RX band n77.

Based on the analysis in Table 5.42.2.1.1-2 there is IMD3 from n3 and n77 into RX band n71.

Based on the analysis in Table 5.42.2.1.1-3 there is IMD4 from n71 and n77 into RX band n3.

5.42.2.2	REFSENS requirements

The MSD values are re-used from CA_n66-n71-n77 for IMD3 into n77 and n71 and DC_71A_n25A-n77A for the IMD4 into n3.

Table 5.42.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n71-n77 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4140 | 15.9 | TDD | IMD31 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3787 | 10.1 | TDD | IMD4 |
|  | n3 | 1748 | 5 | 25 | 1843 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 632 | 15.3 | FDD | IMD3 |
|  | n77 | 4128 | 10 | 50 | 4128 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1843 | 12.5 | FDD | IMD4 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 3883 | 10 | 50 | 3883 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.43	CA_n1-n41-n78

5.43.1	Common for 1 band UL and 2 bands UL CA

5.43.1.1	Operating bands for CA

Table 5.43.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.43.1.2	Channel bandwidths per operating band for CA

Table 5.43.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n41A-n78C | CA_n78C CA_n1A-n41A CA_n1A-n78A CA_n1A-n78C CA_n41A-n78A CA_n41A-n78C | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |

5.43.1.3	∆TIB,c and ∆RIB,c values

Already completed in R4-2414365 of RAN4#112.

5.43.2	Specific for 2 bands UL CA

5.43.2.1	UE co-existence studies

5.43.2.1.1	Co-existence studies for 2UL band with 1CC per band

Already completed in R4-2414365 of RAN4#112.

5.43.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.43.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n1A-n78C with n1A/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.43.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n1 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1920-1980 |  | 3300-3800 |  |
| n41 fDL | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.43.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n41A-n78C with n41A/78C transmitting with a 200 MHz maximum instantaneous bandwidth

Table 5.43.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n41 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 2496-2690 |  | 3300-3800 |  |
| n1 fDL | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.43.2.2	REFSENS requirements

No additional Refsens requirements needed.

5.44	CA_n1-n71-n78

5.44.1	Common for 1 band UL and 2 bands UL CA

5.44.1.1	Operating bands for CA

Table 5.44.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.44.1.2	Channel bandwidths per operating band for CA

Table 5.44.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n71A-n78A | CA_n1A-n71A CA_n1A-n78A CA_n71A-n78A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n1A-n71A-n78C | CA_n78C CA_n1A-n71A CA_n1A-n78A CA_n1A-n78C CA_n71A-n78A CA_n71A-n78C | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.44.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n71-n78, the TIB,c and RIB,c values are re-used from CA_n1-n78-n105 given in the tables below.

Table 5.44.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n78 | 0.3 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.44.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n78 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.44.2	Specific for 2 bands UL CA

5.44.2.1	UE co-existence studies

5.44.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.44.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n1A-n71A.

Table 5.44.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.44.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n1A-n78A.

Table 5.44.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 1880 |  | 5220 - 5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 - 660 |  | 4620 - 5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 7760 |  | 8520 - 9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 2640 |  | 7920 - 9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3760 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 9740 |  | 11820 - 13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 17180 |  | 10980 - 11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.44.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n71A-n78A.

Table 5.44.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3137 |  | 3963 - 4498 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2474 - 1904 |  | 5902 - 6937 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5196 |  | 7263 - 8298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1811 - 1206 |  | 9202 - 10737 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6274 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 5894 |  | 10563 - 12098 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 8996 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14537 - 12502 |  | 508 - 1148 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10074 - 8504 |  | 4506 - 5611 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 15898 |  | 5952 - 6592 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 12796 |  | 8589 - 9694 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.44.2.1.1-1 there is IMD3 from n1 and n71 into RX band n78.

Based on the analysis in Table 5.44.2.1.1-2 there is IMD3 from n1 and n78 into RX band n71.

Based on the analysis in Table 5.44.2.1.1-3 there is IMD3 from n71 and n78 into RX band n1.

5.44.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.44.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71 interference analysis for CA_n1A-n78C with n1/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.44.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n1 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n1 fUL | 1920-1980 |  | 3300-3800 |  |
| n71 fDL | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.44.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n71A -n78C with n71/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.44.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n71 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n71 fUL | 663-698 |  | 3300-3800 |  |
| n1 fDL | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.44.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n78-n105.

Table 5.44.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n71-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |

5.45	CA_n1-n8-n78

5.45.1	Common for 1 band UL and 2 bands UL CA

5.45.1.1	Operating bands for CA

Table 5.45.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n8 | 880-915 | 925-960 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.45.1.2	Channel bandwidths per operating band for CA

Table 5.45.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n8A-n78C | CA_n78C CA_n1A-n78C CA_n8A-n78C | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 0 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.45.2	Specific for 2 bands UL CA

5.45.2.1	UE co-existence studies

5.45.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.45.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n8 interference analysis for CA_n1A -n78C with n1/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.45.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n1 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n1 fUL | 1920-1980 |  | 3300-3800 |  |
| n8 fDL | 925-960 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.45.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n8A-n78C with n8/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.45.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n8 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n8 fUL | 880-915 |  | 3300-3800 |  |
| n1 fDL | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 680-880 |  | 915-1115 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.45.2.2	REFSENS requirements

No requirements since the bands are of different band groups [1].

5.46	CA_n3-n41-n78

5.46.1	Common for 1 band UL and 2 bands UL CA

5.46.1.1	Operating bands for CA

Table 5.46.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.46.1.2	Channel bandwidths per operating band for CA

Table 5.46.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n41A-n78C | CA_n78C CA_n3A-n41A CA_n3A-n78A CA_n3A-n78C CA_n41A-n78A CA_n41A-n78C | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |
| CA_n3(2A)-n41A-n78C | CA_n3A-n41A CA_n3A-n78A CA_n41A-n78A | n3 | CA_n3(2A)_BSC0 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |

5.46.1.3	∆TIB,c and ∆RIB,c values

Already captured in fallback.

5.46.2	Specific for 2 bands UL CA

5.46.2.1	UE co-existence studies

5.46.2.1.1	Co-existence studies for 2UL band with 1CC per band

Already captured in fallback.

5.46.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.46.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n3A-n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.46.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n3 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1710-1785 |  | 3300-3800 |  |
| n41 fDL | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.46.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n41A-n78C with n41/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.46.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n41 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 2496-2690 |  | 3300-3800 |  |
| n3 fDL | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.46.2.2	REFSENS requirements

No additional Refsens requirements needed.

5.47	CA_n3-n71-n78

5.47.1	Common for 1 band UL and 2 bands UL CA

5.47.1.1	Operating bands for CA

Table 5.47.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.47.1.2	Channel bandwidths per operating band for CA

Table 5.47.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n71A-n78A | C A_n3A-n71A CA_n3A-n78A CA_n71A-n78A | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n3(2A)-n71A-n78A | C A_n3A-n71A CA_n3A-n78A CA_n71A-n78A | n3 | CA_n3(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n3A-n71A-n78C | CA_n78C CA_n3A-n71A CA_n3A-n78A CA_n3A-n78C CA_n71A-n78A CA_n71A-n78C | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.47.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n71-n78, the TIB,c and RIB,c values are given in the tables below(Re-use CA_n3-n78-n105).

Table 5.47.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n78 | 0.6 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.47.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n78 | 0.2 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.47.2	Specific for 2 bands UL CA

5.47.2.1	UE co-existence studies

5.47.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.47.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n78A with UL CA_n3A-n71A.

Table 5.47.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.47.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n78A with UL CA_n3A-n78A.

Table 5.47.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2090 |  | 5010 - 5585 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 - 270 |  | 4815 - 5890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7370 |  | 8310 - 9385 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 - 2055 |  | 8115 - 9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4180 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9155 |  | 11610 - 13185 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11170 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 - 11415 |  | 3840 - 3040 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 - 6330 |  | 1245 - 2470 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 16985 |  | 10140 - 10940 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 14970 |  | 11730 - 12955 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.47.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n78A with UL CA_n71A-n78A.

Table 5.47.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3137 |  | 3963 - 4498 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2474 - 1904 |  | 5902 - 6937 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5196 |  | 7263 - 8298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1811 - 1206 |  | 9202 - 10737 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6274 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 5894 |  | 10563 - 12098 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 8996 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14537 - 12502 |  | 508 - 1148 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10074 - 8504 |  | 4506 - 5611 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 15898 |  | 5952 - 6592 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 12796 |  | 8589 - 9694 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.47.2.1.1-1 there is IMD4 and IMD5 from n3 and n71 into RX band n78.

Based on the analysis in Table 5.47.2.1.1-3 there is IMD4 from n71 and n78 into RX band n3.

5.47.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.47.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71 interference analysis for CA_n3A-n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.47.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n3 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n3 fUL | 1710-1785 |  | 3300-3800 |  |
| n71 fDL | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.47.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n71A-n78C with n71/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.47.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n71 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n71 fUL | 663-698 |  | 3300-3800 |  |
| n3 fDL | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.47.2.2	REFSENS requirements

The MSD values are re-used from CA_n3-n78-n105, where the IMD4 into n3 has been omitted, since the test frequencies cannot form an interferer inside the full band n3.

Table 5.47.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n71-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n71 | 670 | 5 | 25 | 624 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3740 | 17.3 | TDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.48	CA_n3-n8-n78

5.48.1	Common for 1 band UL and 2 bands UL CA

5.48.1.1	Operating bands for CA

Table 5.48.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n8 | 880-915 | 925-960 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.48.1.2	Channel bandwidths per operating band for CA

Table 5.48.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n8A-n78C | CA_n78C CA_n3A-n8A CA_n3A-n78A CA_n3A-n78C CA_n8A-n78A CA_n8A-n78C | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n8 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.48.2	Specific for 2 bands UL CA

5.48.2.1	UE co-existence studies

5.48.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.48.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n8 interference analysis for CA_n3A -n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.48.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n3 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n3 fUL | 1710-1785 |  | 3300-3800 |  |
| n8 fDL | 925-960 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.48.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n8A-n78C with n8/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.48.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n8 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n8 fUL | 880-915 |  | 3300-3800 |  |
| n3 fDL | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 680-880 |  | 915-1115 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.48.2.2	REFSENS requirements

There are no additional requirements.

5.49	CA_n41-n71-n78

5.49.1	Common for 1 band UL and 2 bands UL CA

5.49.1.1	Operating bands for CA

Table 5.49.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.49.1.2	Channel bandwidths per operating band for CA

Table 5.49.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41A-n71A-n78C | CA_n78C CA_n41A-n71A CA_n41A-n78A CA_n41A-n78C CA_n71A-n78A CA_n71A-n78C | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.49.1.3	∆TIB,c and ∆RIB,c values

Already provided in fallback.

5.49.2	Specific for 2 bands UL CA

5.49.2.1	UE co-existence studies

5.49.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.49.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71 interference analysis for CA_n41A -n78C with n41/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.49.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n41 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n41 fUL | 2496-2690 |  | 3300-3800 |  |
| n71 fDL | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.49.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n71A-n78C with n71/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.49.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n71 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n71 fUL | 663-698 |  | 3300-3800 |  |
| n41 fDL | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.49.2.2	REFSENS requirements

There are no additional Refsens requirements.

5.50	CA_n8-n41-n78

5.50.1	Common for 1 band UL and 2 bands UL CA

5.50.1.1	Operating bands for CA

Table 5.50.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880-915 | 925-960 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.50.1.2	Channel bandwidths per operating band for CA

Table 5.50.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n41A-n78A | CA_n8A-n41A CA_n8A-n78A CA_n41A-n78A | n8 | 5, 10, 15, 20 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n8A-n41A-n78C | CA_n78C CA_n8A-n41A CA_n8A-n78A CA_n8A-n78C CA_n41A-n78A CA_n41A-n78C | n8 | 5, 10, 15, 20 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.50.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n41-n78, the TIB,c and RIB,c values are given in the tables below, as they are not found in current fallback.

Table 5.50.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8-n41-n78 | 0.6 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.50.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8-n41-n78 | 0.2 | 0.4 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.50.2	Specific for 2 bands UL CA

5.50.2.1	UE co-existence studies

5.50.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.50.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n41A-n78A with UL CA_n8A-n41A.

Table 5.50.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1581 - 1810 |  | 3376 - 3605 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 - 666 |  | 4077 - 4500 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4256 - 4520 |  | 5872 - 6295 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 50 - 249 |  | 6573 - 7190 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3620 - 3162 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5136 - 5435 |  | 8368 - 8985 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6752 - 7210 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9880 - 9069 |  | 1164 - 830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6310 - 5658 |  | 2247 - 2740 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10864 - 11675 |  | 6016 - 6350 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9248 - 9900 |  | 7632 - 8125 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.50.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n41A-n78A with UL CA_n8A-n78A.

Table 5.50.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 - 2920 |  | 4180 - 4715 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 - 1470 |  | 5685 - 6720 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 - 5630 |  | 7480 - 8515 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1160 - 555 |  | 8985 - 10520 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5840 - 4770 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 - 6545 |  | 10780 - 12315 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360 - 9430 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 - 12285 |  | 360 - 280 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 - 8070 |  | 3855 - 4960 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 - 16115 |  | 6820 - 7460 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 - 13230 |  | 9240 - 10345 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.50.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n41A-n78A with UL CA_n41A-n78A.

Table 5.50.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1304 |  | 5796 - 6490 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 - 2080 |  | 3910 - 5104 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9180 |  | 9096 - 10290 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 - 4770 |  | 7210 - 8904 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2608 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 11870 |  | 12396 - 14090 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11592 - 12980 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 - 10510 |  | 7460 - 6184 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 - 4520 |  | 1470 - 112 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 17890 |  | 13284 - 14560 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 16780 |  | 14088 - 15670 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

For Table 5.50.2.1.1-1 there is IMD2 and IMD4 into band n78

For Table 5.50.2.1.1-2 there is IMD2 into band n41

For Table 5.50.2.1.1-3 there is IMD2 and IMD5 into band n8

5.50.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.50.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n8A -n78C with n8/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.50.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n8 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n8 fUL | 880-915 |  | 3300-3800 |  |
| n41 fDL | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 680-880 |  | 915-1115 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.50.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n8 interference analysis for CA_n41A-n78C with n41/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.50.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n41 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n41 fUL | 2496-2690 |  | 3300-3800 |  |
| n8 fDL | 925-960 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.50.2.2	REFSENS requirements

The MSD values are taken from DC_8-41_n78 and DC_7A_n8A-n78A.

Table 5.50.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n41-n78 | n8 | N/A | 10 | N/A | 950 | 29.1 | FDD | IMD2 |
|  | n41 | 2630 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | 3580 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | N/A | 10 | N/A | 940 | 4.0 | FDD | IMD5 |
|  | n41 | 2680 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | 3550 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 28.0 | TDD | IMD2 |
|  | n78 | 3545 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n41 | 2555 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | TDD | IMD22 |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.51	CA_n1-n3-n78

5.51.1	Common for 1 band UL and 2 bands UL CA

5.51.1.1	Operating bands for CA

Table 5.51.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.51.1.2	Channel bandwidths per operating band for CA

Table 5.51.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n78C | CA_n78C CA_n1A-n3A CA_n1A-n78A CA_n1A-n78C CA_n3A-n78A CA_n3A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 4 and 5 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78C_BCS4 & 5 |  |
| CA_n1A-n3(2A)-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 4 and 5 |
|  |  | n3 | CA_n3(2A)_BSC0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3(2A)-n78C | CA_n78C CA_n1A-n3A CA_n1A-n78A CA_n1A-n78C CA_n3A-n78A CA_n3A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 4 and 5 |
|  |  | n3 | CA_n3(2A)_BSC0 |  |
|  |  | n78 | CA_n78C_BCS4 & 5 |  |

5.51.1.3	∆TIB,c and ∆RIB,c values

Already in spec. Only triple beat analysis is required.

5.51.2	Specific for 2 bands UL CA

Only triple beat analysis is required.

5.51.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.51.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n1A-n78C with n1/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.51.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n1 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n1 fUL | 1920-1980 |  | 3300-3800 |  |
| n3 fDL | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is an issue of IMD3 |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.51.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n3A -n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.51.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n3 |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n3 fUL | 1710-1785 |  | 3300-3800 |  |
| n1 fDL | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.51.2.2	REFSENS requirements

Refsens requirements are provided in Table 5.51.2.2-1.

Table 5.51.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n78 | n1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | 25 | 1875 | N/A | 15.3 | IMD3 |
|  | n78 | 3300 | 100 | 1 (RBstart=203) | 3300 | N/A | TDD | N/A |
|  |  | 3400 | 100 | 1 (RBstart=67) | 3400 |  |  |  |

6	Dual Connectivity: Specific Band Combination Part

6.0	DC_nX-nY-nZ

Editor’s note: The texts for NR DC can only be added associated with the texts for the corresponding inter-band 2 bands UL CA above, which means pure TP to TR to included NR DC configuration is not allowed.

6.0.1	Configurations for DC_nX-nY-nZ

Table 6.0.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA-nZ | DC_nXA-nYA DC_nXA-nZA DC_nYA-nZA |
| DC_nXA/B/C/(2A)-nYA/B/C/(2A)- nZA/B/C/(2A) | DC_nXA/B/C-nYA DC_nXA/B/C-nZA DC_nYA/B/C-nZA |

Annex <A> (normative):
Band group definition

For three DL and 2UL/3CC triple beat MSD to be considered, the victim band should be part of the same or adjacent band group than one of the UL band as defined in Table A-1.

Table A-1: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Annex <B> (normative):
Valid UL configurations

For CA_nX-nY-nZ three band DL inter band combinations with two UL bands, the following UL configurations are applicable:

-	Two band UL with one CC per band: CA_nXA-nYA.

-	Two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC.

-	The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A).

-	Combinations with four UL CCs are not supported.

Annex <C> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2411825 |  |  |  | TR skeleton for TR 38.719-03-01 | 0.0.0 |
| 2024-08 | RAN4 #112 | R4-2411826 |  |  |  | (1) R4-2414340, TP for TR38.719-03-01_CA_n28A-n41A/C-n79A/C, ZTE Corporation, Sanechips. (2) R4-2414355, TP to TR 38.719-03-01 include CA_n8A-n20A-n75A, Huawei, HiSilicon, DT. (3) R4-2414356, TP to TR 38.719-03-01 include CA_n8A-n28A-n75A, Huawei, HiSilicon, DT. (4) R4-2414357, TP to TR 38.719-03-01 include CA_n20A-n28A-n75A, Huawei, HiSilicon, DT. (5) R4-2414358, TP to TR 38.719-03-01 include CA_n3A-n7A-n75A, Huawei, HiSilicon, DT. (6) R4-2414359, TP to TR 38.719-03-01 include CA_n3A-n75A-n78A, Huawei, HiSilicon, DT. (7) R4-2414360, TP to TR 38.719-03-01 include CA_n7A-n75A-n78A, Huawei, HiSilicon, DT. (8) R4-2414361, TP to TR 38.719-03-01 include CA_n1A-n7A-n75A, Huawei, HiSilicon, DT. (9) R4-2414362, TP to TR 38.719-03-01 include CA_n1A-n3A-n7A, Huawei, HiSilicon, DT. (10) R4-2414364, TP for 38.719-03-01 to add UL CA_n41C-n71A and CA_n41C-n66A for CA_n41C-n66-n71A, Ericsson, T-Mobile USA. (11) R4-2414365, TP to TR 38.719-03-01 Addition of CA_n1A-n41A-n78A, Nokia, Etisalat. (12) R4-2414366, TP to TR 38.719-03-01 Addition of CA_n1A-n5A-n8A, Nokia. (13) R4-2414367, TP to TR 38.719-03-01 Addition of CA_n3A-n5A-n8A, Nokia. | 0.1.0 |
| 2024-10 | RAN4 #112bis | R4-2416096 |  |  |  | (1) R4-2414961, TP for TR 38.719-03-01 CA_n25-n29-n77, Samsung, TELUS, Bell Mobility (2) R4-2414972, TP to TR 38.719-03-01 to add CA_n1A-n75A-n78A, Huawei, Hisilicon, Deutsche Telekom (3) R4-2415400 TP to TR 38.719-03-01 CA_n1-n28-n40, Nokia, STC (4) R4-2415404 TP to TR 38.719-03-01 CA_n1-n20-n71, Nokia, Mobily (5) R4-2415406, TP to TR 38.719-03-01 CA_n1-n20-n78, Nokia, Mobily (6) R4-2415408, TP to TR 38.719-03-01 CA_n20-n41-n71, Nokia, Mobily (7) R4-2415409, TP to TR 38.719-03-01 CA_n20-n41-n77, Nokia, Mobily (8) R4-2415410, TP to TR 38.719-03-01 CA_n20-n41-n78, Nokia, Mobily (9) R4-2415411, TP to TR 38.719-03-01 CA_n20-n71-n78, Nokia, Mobily (10) R4-2415413, TP to TR 38.719-03-01 CA_n3-n20-n71, Nokia, Mobily (11) R4-2415414, TP to TR 38.719-03-01 CA_n3-n20-n77, Nokia, Mobily (12) R4-2415419, TP to TR 38.719-03-01 CA_n1-n3-n71, Nokia, Etisalat UAE (13) R4-2415420, TP to TR 38.719-03-01 CA_n1-n3-n8, Nokia, Etisalat UAE (14) R4-2415421, TP to TR 38.719-03-01 CA_n1-n41-n71, Nokia, Etisalat UAE (15) R4-2415424, TP to TR 38.719-03-01 CA_n1-n8-n41, Nokia, Mobily (16) R4-2415426, TP to TR 38.719-03-01 CA_n3-n41-n71, Nokia, Etisalat UAE, Mobily (17) R4-2416219, TP for 38.719-03-01 to add CA_n1A-n7A-n20A, Ericsson, BT plc (18) R4-2417132, TP to TR 38.719-03-01 include CA_n1A-n3A-n75A, Huawei, Hisilicon, Deutsche Telekom (19) R4-2417133, TP to TR 38.719-03-01 include CA_n1A-n7A-n78A, Huawei, Hisilicon, Deutsche Telekom (20) R4-2417134, TP for TR 38.719-03-01 CA_n7-n25-n29, Samsung, TELUS, Bell Mobility, Qualcomm, Skyworks (21) R4-2417135, TP for TR 38.719-03-01 CA_n7-n29-n66, Samsung, TELUS, Bell Mobility (22) R4-2417136, TP for TR 38.719-03-01 CA_n7-n29-n77, Samsung, TELUS, Bell Mobility (23) R4-2417154, TP for TR38.719-03-01_3DL_2UL CA_n3A-n40A-n79A, ZTE Corporation, Sanechips (24) R4-2417137, Corrections of TR 38.719-03-01, Nokia (25) R4-2417138, TP to TR 38.719-03-01 CA_n3-n7-n77, Nokia, BT plc (26) R4-2417154, TP for TR38.719-03-01_3DL_2UL CA_n3A-n40A-n79A, ZTE Corporation, Sanechips (27) R4-2417140, TP to TR 38.719-03-01 CA_n1-n20-n41, Nokia, BT plc (28) R4-2417141, TP to TR 38.719-03-01 CA_n1-n20-n77, Nokia, Mobily (29) R4-2417142, TP to TR 38.719-03-01 CA_n1-n71-n77, Nokia, Mobily (30) R4-2417143, TP to TR 38.719-03-01 CA_n3-n20-n41, Nokia, Mobily (31) R4-2417144, TP to TR 38.719-03-01 CA_n3-n71-n77, Nokia, Mobily (32) R4-2417145 TP to TR 38.719-03-01 CA_n1-n41-n78 w ULCA, Nokia, Etisalat UAE (33) R4-2417146, TP to TR 38.719-03-01 CA_n1-n71-n78 w ULCA, Nokia, Etisalat UAE (34) R4-2417147, TP to TR 38.719-03-01 CA_n1-n8-n78 w ULCA, Nokia, Etisalat UAE (35) R4-2417148, TP to TR 38.719-03-01 CA_n3-n41-n78 w ULCA, Nokia, Etisalat UAE (36) R4-2417149, TP to TR 38.719-03-01 CA_n3-n71-n78 w ULCA, Nokia, Etisalat UAE (37) R4-2417150, TP to TR 38.719-03-01 CA_n3-n8-n78 w ULCA, Nokia, Etisalat UAE (38) R4-2417151, TP to TR 38.719-03-01 CA_n41-n71-n78 w ULCA, Nokia, Etisalat UAE (40) R4-2417152, TP to TR 38.719-03-01 CA_n8-n41-n78 w ULCA, Nokia, Etisalat UAE (41) R4-2417153, TP to TR 38.719-03-01 Uplink additions of CA_n1A-n3A-n78C, Nokia, Etisalat UAE | 0.2.0 |