| 3GPP TR 36.719-01-01 V0.0.1 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Rel-19 LTE Advanced CA for x bands DL (1<=x<=6) with y bands UL (y=1, 2) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

4.1	TR maintenance	7

5	Specific Band Combination Part	7

5.1	LTE-A intra-band CA	7

5.1.x	CA_a	7

5.1.x.1	Channel bandwidths per operating band	7

5.1.x.2	Co-existence studies	7

5.1.x.3	REFSENS requirements	8

5.1.x.3	AMPR UL CA bandwidth class X	8

5.2	LTE-A inter-band CA for x (x>1) bands DL with 1 band UL	8

5.2.x	CA_a-b	8

5.2.x.1	Channel bandwidths per operating band for CA	8

5.2.x.2	Co-existence studies	9

5.2.x.3	∆TIB and ∆RIB values	10

5.2.x.4	REFSENS requirements	10

5.3	LTE-A inter-band CA for 2 bands DL with 2 bands UL	10

5.3.x	CA_a-b	10

5.3.x.1	Channel bandwidths per operating band for CA	10

5.3.x.2	Co-existence studies	10

5.3.x.3	∆TIB and ∆RIB values	10

5.3.x.4	REFSENS requirements	10

5.4	LTE-A inter-band CA for x (x>2) bands DL with 2 bands UL	11

5.4.x	LTE-A inter-band CA: up to 6 bands DL(Band U and Band V and Band W and Band X and Band Y and Band Z)  with 2 bands UL	11

5.4.x.1	Channel bandwidths per operating band for CA	11

5.4.x.2	Co-existence studies	11

5.4.x.3	∆TIB and ∆RIB values	12

5.4.x.4	REFSENS requirements	13

Annex A: Change history	13

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report on intra-band CA and inter-band CA for x bands DL (1≤x≤6) with y bands UL (y=1,2) under Rel-19 timeframe.

The purpose is to gather the relevant background information and studies, in order to address Rel-19 band combinations that are related to x bands (1≤x≤6) DL with y bands UL (y=1,2) bands UL CA requirements. The band combinations are requested in the Excel file of the WID [2] or its revisions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-241676, “New WID Rel-19 LTE Advanced CA for x (x<=6) bands DL with y bands (y=1, 2) UL”, RAN#104.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for intra-band Carrier Aggregation and inter-band CA for x bands DL (1≤x≤6) with y bands UL (y=1,2) under Rel-19 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Specific Band Combination Part

Intra-band Carrier Aggregation, as well as, Inter-band Carrier Aggregation combination with x (1≤x≤6) and a single UL and double UL bands within Rel-19 timeframe are specified in this section.

## 5.1	LTE-A intra-band CA

### 5.1.x	CA_a

#### 5.1.x.1	Channel bandwidths per operating band

< Editor's note: Text will be added, the examples is given as follows>

Table 5.1.x.1-1: Intra-band CA operating band

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| xx | xx MHz | – | xxxx MHz | xx MHz | – | xx MHz | FDD/TDD |

Table 5.1.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_xxA | CA_xxA | xx |  |  | Yes | Yes | Yes | Yes | 20 | 0 |
| CA_xxC | CA_xxC | xx |  |  | Yes | Yes | Yes | Yes | 40 | 0 |

< Editor's note: If the UL CA is proposed for the CA configuration, the Uplink CA configurations column should be added in the table.>

#### 5.1.x.2	Co-existence studies

< Editor’s note: Text will be added, the examples is given as follows. The harmonics and harmonics mixing issues should be analysed based on this table. >

Table 5.1.2-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA _ xx.

Table 5.1.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| xx | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

< Editor’s note: Harmonic relation should be captured as far as there is harmonic interference, e.g. n = floor(xx_DL High Band Edge/xx_UL Low Band Edge). >

#### 5.1.x.3	REFSENS requirements

< Editor's note: Text will be added if harmonics and/or harmonic mixing, etc. issues are identified, and only REFSENS numbers for bands have these issues need to be provided in the table.>

Table 5.1.x.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_xxA CA_xxC | xx | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

#### 5.1.x.3	AMPR UL CA bandwidth class X

< Editor's note: Text will be added if AMPR studies and simulations are required..>

## 5.2	LTE-A inter-band CA for x (x>1) bands DL with 1 band UL

### 5.2.x	CA_a-b

#### 5.2.x.1	Channel bandwidths per operating band for CA

< Editor's note: Text will be added, the examples are given as follows for the case of x=2 >

Table 5.2.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| xx | xx MHz | – | xx MHz | xx MHz | – | xx MHz | FDD/TDD |
| yy | yy MHz | – | yy MHz | yy MHz | – | yy MHz | FDD/TDD |

Table 5.2.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_xxA-yyA | - | xx |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | yy |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_xxA-yyC | - | xx |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | yy | See the CA_yyC Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_xxC-yyA | - | xx | See the CA_xxC Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | yy |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_xxA-yyC | CA_yCC | xx |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | yy | See the CA_yyC Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

< Editor's note: If the UL CA is proposed for the CA configuration, the Uplink CA configurations column should be added in the table.>

#### 5.2.x.2	Co-existence studies

< Editor’s  note: Co-existence studies are needed till x=2 since these issue shall be treated on fallback combinations. Text will be added, the examples is given as follows. The harmonics and harmonics mixing issues shouldbe analysed based on this table. >

Table 5.2.2-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA _ xx-yy.

Table 5.2.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| xx | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| yy | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

< Editor’s  note: Harmonic relation should be captured as far as there is harmonic interference, e.g. n = floor(yy or xx_DL High Band Edge/xx or yy_UL Low Band Edge). >

Table 5.2.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| xx | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| yy | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

< Editor’s  note: Harmonic mixing relation should be captured as far as there is harmonic mixing interference, e.g. m= floor(yy or xx_UL High Band Edge/xx or yy_DL Low Band Edge)>

#### 5.2.x.3	∆TIB and ∆RIB values

< Editor's note: Text will be added, the examples is given as follows. IB,c and R IB,c can be added based on band>

Table 5.2.x.3-1: IB,c

| CA_xx-yy | xx | TBD |
| --- | --- | --- |
|  | yy | TBD |

Table 5.2.x.3-2: R IB,c

| CA_xx-yy | xx | TBD |
| --- | --- | --- |
|  | yy | TBD |

#### 5.2.x.4	REFSENS requirements

< Editor's note: Text will be added if harmonics and/or harmonic mixing, etc. issues are identified, and only REFSENS numbers for bands have these issues need to be provided in the table.>

Table 5.2.x.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_xxA-yyA CA_xxA-yyC CA_xxC-yyA | yy | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

## 5.3	LTE-A inter-band CA for 2 bands DL with 2 bands UL

### 5.3.x	CA_a-b

#### 5.3.x.1	Channel bandwidths per operating band for CA

<Text will be added.>

#### 5.3.x.2	Co-existence studies

<Text will be added. Spurious emission band UE co-existence for CA, i.e. the tables similar to Table 6.6.3.2A-0 in TS 36.101 should be reflected here. Noted that the protected bands are common set of the UL CA bands.>

#### 5.3.x.3	∆TIB and ∆RIB values

<Text will be added.>

#### 5.3.x.4	REFSENS requirements

<Text will be added if it’s necessary, only REFSENS numbers for bands with exception due to harmonics and/or harmonic mixing need to be provided in the table.>

## 5.3	LTE-A inter-band CA for 2 bands DL with 2 bands UL

### 5.3.1	CA_8-11

#### 5.3.1.1	Channel bandwidths per operating band for CA

Table 5.3.1.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |

Table 5.3.1.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_8A-11A | CA_8A-11A | 8 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |

#### 5.3.1.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.1.2-1.

Table 5.3.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1427.9 | 1447.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 2855.8 | 2895.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 4283.7 | 4343.7 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 567.9 | 512.9 | 2307.9 | 2362.9 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 312.1 | 402.1 | 1940.8 | 2015.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3187. | 3277.9 | 3735.8 | 3810.8 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1135.8 | 1025.8 | 4615.8 | 4725.8 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1192.1 | 1317.1 | 3368.7 | 3463.7 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4067.9 | 4192.9 | 5163.7 | 5258.7 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4911.6 | 4796.6 | 2232.1 | 2072.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6591.6 | 6706.6 | 4947.9 | 5107.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2583.7 | 2453.7 | 110.8 | 255.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6043.7 | 6173.7 | 5495.8 | 5640.8 |

Based on analysis on above table, there is no IMD issue for this combination.

Table 5.3.1.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_8-11 | Frequency range | 860 | - | 890 | -40 | 1 | 3, 11 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 11 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. |  |  |  |  |  |  |  |

#### 5.3.1.3	∆TIB and ∆RIB values

Relaxation values for CA_8-11 have already been specified in TS 36.101.

#### 5.3.1.4	REFSENS Requirements

Based on analysis of 5.3.1.2, there are no additional MSD requirements for this combination.

### 5.3.2	CA_1-68

#### 5.3.2.1	Channel bandwidths per operating band for CA

Table 5.3.2.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |

Table 5.3.6-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-68A | CA_1A-68A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 68 |  |  | Yes | Yes | Yes |  |  |  |

#### 5.3.2.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.2.2-1.

Table 5.3.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 728 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1456 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2184 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1192 | 2618 | 2708 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 584 | 464 | 3112 | 3262 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3316 | 3436 | 4538 | 4688 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2564 | 2384 | 5236 | 5416 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 114 | 264 | 5032 | 5242 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4014 | 4164 | 6458 | 6668 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7222 | 6952 | 992 | 812 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8378 | 8648 | 4712 | 4892 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4544 | 4304 | 1656 | 1866 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7156 | 7396 | 5934 | 6144 |

Based on Table 5.3.2.2-1, the 3rd harmonic of band 68 may fall into RX frequencies of band 1.

Table 5.3.2.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1-68 | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27, 44 |

#### 5.3.2.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_1-28 are reused.

Table 5.3.2.3-1: IB,c

| CA_1-68 | 1 | 0.3 |
| --- | --- | --- |
|  | 68 | 0.6 |

Table 5.3.2.3-2: R IB,c

| CA_1-68 | 1 | 0 |
| --- | --- | --- |
|  | 68 | 0.2 |

#### 5.3.2.4	REFSENS requirements

The following REFSENS requirement for CA has already been specified for CA_1A-28A and can be reused for CA_1A-68A:

Table 5.3.2.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-68A5,6 | 1 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

### 5.3.3	CA_3-68

#### 5.3.3.1	Channel bandwidths per operating band for CA

Table 5.3.3.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |

Table 5.3.6-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-68A | CA_3A-68A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 68 |  |  | Yes | Yes | Yes |  |  |  |

#### 5.3.3.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.3.2-1.

Table 5.3.3.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 728 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1456 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2184 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1087 | 982 | 2408 | 2513 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 389 | 254 | 2692 | 2872 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3106 | 3241 | 4118 | 4298 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2174 | 1964 | 4816 | 5026 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 309 | 474 | 4402 | 4657 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3804 | 3969 | 5828 | 6083 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6442 | 6112 | 1202 | 1007 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7538 | 7868 | 4502 | 4697 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3959 | 3674 | 1236 | 1476 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6526 | 6811 | 5514 | 5754 |

Based on Table 5.3.3.2-1, there are no IMD issues.

Table 5.3.3.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_3-68 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |

#### 5.3.3.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_3-28 are reused.

Table 5.3.3.3-1: IB,c

| CA_3-68 | 3 | 0.3 |
| --- | --- | --- |
|  | 68 | 0.3 |

Table 5.3.3.3-2: R IB,c

| CA_3-68 | 3 | 0 |
| --- | --- | --- |
|  | 68 | 0 |

#### 5.3.3.4	REFSENS requirements

#### Based on analysis of 5.3.3.2, there are no additional MSD requirements for this combination.

### 5.3.4	CA_7-68

#### 5.3.4.1	Channel bandwidths per operating band for CA

Table 5.3.4.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |

Table 5.3.6-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7A-68A | CA_7A-68A | 7 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 68 |  |  | Yes | Yes | Yes |  |  |  |

#### 5.3.4.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.4.2-1.

Table 5.3.4.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 728 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1456 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2184 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1872 | 1772 | 3198 | 3298 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1174 | 1044 | 4272 | 4442 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3896 | 4026 | 5698 | 5868 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3744 | 3544 | 6396 | 6596 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 476 | 316 | 6772 | 7012 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4594 | 4754 | 8198 | 8438 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9582 | 9272 | 412 | 222 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10698 | 11008 | 5292 | 5482 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6314 | 6044 | 2816 | 3046 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8896 | 9166 | 7094 | 7324 |

Based on Table 5.3.4.2-1, there are no IMD issues.

Table 5.3.4.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_7-68 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |

#### 5.3.4.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_7-28 are reused.

Table 5.3.4.3-1: IB,c

| CA_7-68 | 7 | 0.3 |
| --- | --- | --- |
|  | 68 | 0.3 |

Table 5.3.4.3-2: R IB,c

| CA_7-68 | 7 | 0 |
| --- | --- | --- |
|  | 68 | 0 |

#### 5.3.4.4	REFSENS requirements

#### Based on analysis of 5.3.4.2, there are no additional MSD requirements for this combination.

### 5.3.5	CA_8-68

#### 5.3.5.1	Channel bandwidths per operating band for CA

Table 5.3.5.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |

Table 5.3.6-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_8A-68A | CA_8A-68A | 8 |  |  | Yes | Yes | - | - | 25 | 0 |
|  |  | 68 |  |  | Yes | Yes | Yes |  |  |  |

#### 5.3.5.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.5.2-1.

Table 5.3.5.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 728 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1456 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2184 | 2640 | 2745 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 217 | 152 | 1578 | 1643 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 481 | 576 | 1032 | 1132 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2276 | 2371 | 2458 | 2558 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 434 | 304 | 3156 | 3286 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1179 | 1304 | 1912 | 2047 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2974 | 3099 | 3338 | 3473 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2962 | 2792 | 2032 | 1877 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4218 | 4388 | 3672 | 3827 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1349 | 1184 | 424 | 264 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4036 | 4201 | 3854 | 4014 |

Based on Table 5.3.5.2-1, there are no IMD issues.

Table 5.3.5.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_8-68 | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |

#### 5.3.5.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_8-28 are reused.

Table 5.3.5.3-1: IB,c

| CA_8-68 | 8 | 0.6 |
| --- | --- | --- |
|  | 68 | 0.5 |

Table 5.3.5.3-2: R IB,c

| CA_8-68 | 8 | 0.2 |
| --- | --- | --- |
|  | 68 | 0.1 |

#### 5.3.5.4	REFSENS requirements

### Based on analysis of 5.3.5.2, there are no additional MSD requirements for this combination.5.3.6	CA_20-68

#### 5.3.6.1	Channel bandwidths per operating band for CA

Table 5.3.6.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |

Table 5.3.6-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_20A-68A | CA_20A-68A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 68 |  |  | Yes | Yes | Yes |  |  |  |

#### 5.3.6.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.6.2-1.

Table 5.3.6.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 728 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1456 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2184 | 2496 | 2586 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 164 | 104 | 1530 | 1590 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 534 | 624 | 936 | 1026 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2228 | 2318 | 2362 | 2452 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 328 | 208 | 3060 | 3180 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1232 | 1352 | 1768 | 1888 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2926 | 3046 | 3194 | 3314 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2750 | 2600 | 2080 | 1930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4026 | 4176 | 3624 | 3774 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1190 | 1040 | 520 | 370 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3892 | 4042 | 3758 | 3908 |

Based on Table 5.3.6.2-1, there are no IMD issues.

Table 5.3.6.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_20-68 | Frequency range | 758 | - | 788 | -50 | 1 |  |

#### 5.3.6.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_20-28 are reused.

Table 5.3.6.3-1: IB,c

| CA_20-68 | 20 | 0.5 |
| --- | --- | --- |
|  | 68 | 0.5 |

Table 5.3.6.3-2: R IB,c

| CA_20-68 | 20 | 0 |
| --- | --- | --- |
|  | 68 | 0 |

#### 5.3.6.4	REFSENS requirements

Based on analysis of 5.3.6.2, there are no additional MSD requirements for this combination.

### 5.3.7	CA_40-68

#### 5.3.7.1	Channel bandwidths per operating band for CA

Table 5.3.7.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |

Table 5.3.6-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_40A-68A | CA_40A-68A | 40 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 68 |  |  | Yes | Yes | Yes |  |  |  |

#### 5.3.7.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.7.2-1.

Table 5.3.7.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 728 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1456 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2184 | 6900 | 7200 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ | ¦fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1702 | 1572 | 2998 | 3128 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1004 | 844 | 3872 | 4102 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3696 | 3856 | 5298 | 5528 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3404 | 3144 | 5996 | 6256 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 306 | 116 | 6172 | 6502 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4394 | 4584 | 7598 | 7928 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8902 | 8472 | 612 | 392 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9898 | 10328 | 5092 | 5312 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5804 | 5444 | 2416 | 2706 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8296 | 8656 | 6694 | 6984 |

Based on Table 5.3.7.2-1, there are no IMD issues.

Table 5.3.7.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_40-68 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |

#### 5.3.7.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_n28-n40 are reused.

Table 5.3.7.3-1: IB,c

| CA_40-68 | 40 | 0.3 |
| --- | --- | --- |
|  | 68 | 0.3 |

Table 5.3.7.3-2: R IB,c

| CA_40-68 | 40 | 0 |
| --- | --- | --- |
|  | 68 | 0 |

#### 5.3.7.4	REFSENS requirements

#### Based on analysis of 5.3.7.2, there are no additional MSD requirements for this combination.

## 5.4	LTE-A inter-band CA for x (x>2) bands DL with 2 bands UL

### 5.4.x	LTE-A inter-band CA: up to 6 bands DL(Band U and Band V and Band W and Band X and Band Y and Band Z)  with 2 bands UL

[Editor Note] It will be updated in future

#### 5.4.x.1	Channel bandwidths per operating band for CA

< Editor's note: Text will be added, the examples are given as follows for the case of x=2 >

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| xx | xx MHz | – | xx MHz | xx MHz | – | xx MHz | FDD/TDD |
| yy | yy MHz | – | yy MHz | yy MHz | – | yy MHz | FDD/TDD |
| zz | zz MHz | – | zz MHz | zz MHz | – | zz MHz | FDD/TDD |

< Editor's note: Table 5.4.x.1-1 can be extended for 3 to 6 E-UTRA bands >

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_xxA-yyA-zzA | CA_xxA-yyA | xx |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | yy |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | zz |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_xxA-yyC-zzA | CA_xxA-yyC | xx |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | yy | See the CA_yyC Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | zz |  |  | Yes | Yes | Yes | Yes |  |  |

< Editor's note: When similar E-UTRA CA Configurations with different channel bandwidths are proposed, their Bandwidth combination set number should be different.

Table 5.4.x.1-2 can be extended for the inter band CA configurations with 3 to 6 DL bands and 2 UL bands >

#### 5.4.x.2	Co-existence studies

< Editor’s  note: the text will be added here to include IMD analysis of the UL inter band CA combinations, if needed. >

Table 5.4.x.2-1 summarizes frequency ranges where IMD components due to  CA_xxA-yyA could fall.

Table 5.4.x.2-1: IMD analysis CA_xxA-yyA

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | xx | xx | yy | yy |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) |  |  |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) |  |  |  |  |

#### 5.4.x.3	∆TIB and ∆RIB values

< Editor's note: Text will be added, the examples is given as follows. IB,c and R IB,c can be added based on the band>

Table 5.4.x.3-1: IB,c

| CA_xx-yy-zz | xx | TBD |
| --- | --- | --- |
|  | yy | TBD |
|  | zz | TBD |

Table 5.4.x.3-2: R IB,c

| CA_xx-yy-zz | xx | TBD |
| --- | --- | --- |
|  | yy | TBD |
|  | zz | TBD |

#### 5.4.x.4	REFSENS requirements

< Editor's note: Text will be added if IMD related issues are identified, for CA configurations with 3 to 6 DL bands and 2 UL bands  >

Table 5.4.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_xxA-yyA-zzA | xx | X | X | X | X | X | FDD | IMDX |
|  | yy | X | X | X | X | N/A | FDD | N/A |

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | 3GPP RAN4#112 |  |  |  |  | Initial TR skeleton | 0.0.0 |
| 2024-10 | 3GPP RAN4#112bis |  |  |  |  | R4-2417168: CA_8-11 R4-2417170: CA_1-68 R4-2417171: CA_3-68 R4-2417172: CA_7-68 R4-2417173: CA_8-68 R4-2417174: CA_20-68A R4-2417175: CA_40-68A | 0.0.1 |