3GPP TSG-RAN WG4 Meeting #112-bis	R4-2414908

Hefei, Anhui, China, 14th – 18th October, 2024

3GPP TSG-RAN WG1 #118                                                                                          	          R1- 2407548

Maastricht, NL, August 19th – 23rd, 2024

Title:	Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Response to: 	R1-2405786(R2-2405769)

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	RAN1

To:	RAN2, RAN4

CC:

Contact Person:

#### Name:	Fangyu Cui

E-mail Address:	cui.fangyu@zte.com.cn

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response:

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2?

RAN1’s response:

From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases:

- NB-IoT with 3.75kHz SCS.

- eMTC CE mode A.

Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2

Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively.

Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the response into account.

To RAN4:

ACTION: 	RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #118bis                      14th – 18th October 2024                                            Hefei, China

TSG RAN WG1 Meeting #119                           18th – 22nd November 2024                                       Orlando, US