3GPP TSG-RAN WG4 Meeting #112bis				                                          R4-2415180

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Source:	CATT

Agenda Item:	6.15.2.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC and give our views.

2 Discussion

Regarding the applicability of CSSF optimization, it was discussed in last RAN4 meeting with no consensus reached. But in RAN#105 meeting, the issue was agreed and the WID was updated as below [3]:

- The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

Following the agreements above, the CSSF optimization can apply to CA/DC scenario independently of the UE support of Multi-Rx capabilities. But even if UE supports Multi-Rx, the requirements are based on the assumption that UE is not operating with Multi-Rx based L3 measurement, i.e., we do not consider Multi-Rx and CSSF optimization together.

Proposal 1: Follow RAN plenary conclusions about the applicability of CSSF optimization.

Regarding the scenarios/measurement procedure to apply CSSF outside gap optimization, the following agreement was achieved in last meeting:

Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

In our understanding, all the three aspects should be considered to apply CSSF optimization and no need to further discuss priority. Because we understand all the intra-frequency, inter-frequency and inter-RAT measurement requirements refer to the same section and use the same CSSF definition. Once the solutions are decided, the enhanced CSSF can be applied to all the three types of measurement. One concern in last meeting is that some of the solutions can only be used for certain type of measurement (e.g., intra-frequency), but we think it can be reflected in the corresponding measurement requirements rather than in the CSSF definition. For that part, we can discuss in detailed solution. And once it impacts the CSSF definition or calculation, it will also apply to all the measurements.

Proposal 2: The enhanced CSSF apply to all the following aspects and do not define the priority.

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement without gap;

- Inter-RAT measurement without gap.

In last meeting, the searcher assumption was also discussed. Although it is indicated in the WID that the baseline assumption is 2 searchers, we think other options are not precluded. And it is not reasonable to preclude some of the solutions just because of the searcher assumption although it does not change/increase the existing UE implementation complexity. It can be discussed together with solutions of CSSF optimization. If the solutions with additional searcher assumption are feasible under certain conditions, they should be also considered.

Proposal 3: Not to preclude the solutions based on 3 searchers assumption in current stage.

In previous meeting, different solutions were raised to enhance CSSF outside gap, including adjusting searcher allocation, reducing unnecessary measurement and allowing additional searcher assumption and others. After the discussion in last meeting, the solutions are summarized as the following:

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC - FFS: - Option 1 (Apple, CTC. Ericsson, Intel, Samsung, ZTE): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 2 (CATT, Apple, CTC, ZTE): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - Option 4 (CATT, HW): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers. (HW, CATT) - We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed. (HW)

We are open to discuss all the solutions above and multiple solutions can be defined, but some further clarifications are needed. For option 1, as discussed in last meeting, it can be implemented by network configuration, e.g., only one servingCellMO is configured on one SCC and no MOs are configured on the other SCCs, if multiple serving CCs are in the same band. Otherwise, if NW configures multiple CCs but UE only measures one of them, it needs to clarify how to interpret other CCs in this band. If it means the measurement on single CC can represent the channel quality of all the CCs in the same band, some additional conditions may be needed between these CCs.

For option 2, the searcher occupation ratio for PCC/PSCC and SCC needs to be discussed. In our understanding, it is better to define one or multiple fixed ratios in the specification rather than to dynamically indicate the ratio. And if multiple ratios are defined, the applied ratio can be indicated by network. Meanwhile we understand such solution would extend the PCC/PSCC measurement and may impact the communication in some of scenarios, so some additional conditions (e.g., the serving cell measurement quality is larger than a threshold) are needed.

For option 4, it is based on existing UE capability, and no additional UE implementation complexity is required. But according to the discussion in last meeting, it may not apply to all the cases and some conditions (e.g., both FR1 and FR2 measurement objects are configured) are needed. We are open to discuss the applicable scenarios.

Based on the discussions above, our preferred solutions are option 4 and option 2. Option 1 can be considered with further clarifications.

Proposal 4: Multiple solutions can be considered for CSSF optimization, and option 4 and option 2 are preferred.

- Option 4: Three searchers assumption for UE supporting per-FR gap

- Option 2: Using PCC/PSCC searcher to speed up SCC measurement

Proposal 5: For option 2, define one or multiple fixed occupation ratios in the specification. If multiple ratios are defined, UE shall follow the NW indicated ratio under certain conditions (e.g., the serving cell measurement is larger than a threshold).

Proposal 6: For option 4, define the applicable scenarios (e.g., both FR1 and FR2 measurement objects are configured).

For the feature capability, based on the solution discussions above, we think it depends on the specified solutions, e.g., new capability is not needed for solution #4 but is needed for other solutions.

Proposal 7: The feature capability depends on the solution:

- For option 1 and option 2, a new UE capability is needed to indicate the UE support of new solution.

- For option 4, the existing per-FR gap capability can be reused.

3 Summary

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Proposal 1: Follow RAN plenary conclusions about the applicability of CSSF optimization.

Proposal 2: The enhanced CSSF apply to all the following aspects and do not define the priority.

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement without gap;

- Inter-RAT measurement without gap.

Proposal 3: Not to preclude the solutions based on 3 searchers assumption in current stage.

Proposal 4: Multiple solutions can be considered for CSSF optimization, and option 4 and option 2 are preferred.

- Option 4: Three searchers assumption for UE supporting per-FR gap

- Option 2: Using PCC/PSCC searcher to speed up SCC measurement

Proposal 5: For option 2, define one or multiple fixed occupation ratios in the specification. If multiple ratios are defined, UE shall follow the NW indicated ratio under certain conditions (e.g., the serving cell measurement is larger than a threshold).

Proposal 6: For option 4, define the applicable scenarios (e.g., both FR1 and FR2 measurement objects are configured).

Proposal 7: The feature capability depends on the solution:

- For option 1 and option 2, a new UE capability is needed to indicate the UE support of new solution.

- For option 4, the existing per-FR gap capability can be reused.

4 References

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, RAN4#112

- R4-2411813, Topic summary for [112][218] NR_RRM_Ph5_Part1, Apple, RAN4#112

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN#105