3GPP RAN WG4 Meeting #112bis																R4-2415314

Hefei, China, 14th – 18th October 2024

Agenda item:	5.17.2

Source:	Apple

Title:	Initial view on the UE power back-off while operating in 2000-2020 MHz UL range

WI/SI:	NR_NTN_Sband-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2]. So-called extended L-band was also completed in Rel-18.

And after the RAN#104 meeting another spectrum WI was agreed for a satellite band operating in the S-band frequency range [3]. The UL block of a new NTN band covers the frequency range of 2000-2020MHz, which is close to the DL of another terrestrial band n25. And as discussed during the previous RAN4#112 meeting, there are some concerns on the potential impact from UL transmission on a new NTN band n252 to the DL receivers operating in the terrestrial band n25. Consequently, there were also concerns on the potential power back-off that band n252 UE may apply to meet existing requirements to protect band n25 receivers. To facilitate further technical discussions, we conducted a set of simulations and present them in this paper.

# 2	UE power back-off values

Here we present power back-off results for the NTN band n252 with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Additional requirements: -50dBm/MHz protection criterion for band n25

Figure 2-1 below presents the simulation results for the 5MHz channel, which is placed in the following locations within the band: left edge of the band (Fc=2002.5MHz) and two other centre frequencies shifted further away from the left edge (Fc=2007.5MHz and Fc=2012.5MHz).  For the sake of brevity, we present only DFT-s-OFDM results because as indicated in our previous studies CP-OFDM has even higher power back-off values. So, referring to Figure 2-1 the following general observations could be made for the 5MHz channel:

-	If the 5MHz channel is right at the left edge of the band, then large UL allocations may need large power back-off of up to 11dB. On the other hand, if we consider small UL allocations, which are likely to be the case for the NTN channels, then required power back-off is rather marginal. There is however a region constrained by RBstart=5 and LCRB=7 where even small UL allocations might have a noticeable power back-off.

-	If the 5MHz channel is shifted farther away from the band lower edge, then required power back-off reduces dramatically. Only large UL allocations may need power back-off, whereas small UL allocations either do not need any power back-off or the back-off stays within the limits of 0.5dB.

Figure 2-1: Power back-off values for the 5MHz channel with DFT-s-OFDM

Next, Figure 2-2 presents power back-off values for the 20MHz channel, which is the largest channel that can be configured in this band. So, there is only one scenario with the 20MHz channel placed within a new NTN band n252. Referring to the presented results, like for other NTN bands that have stringent requirements, large allocations and/or small(-er) allocations at the edge of the band will require noticeable power back-off.

Figure 2-2: Power back-off values for the 20MHz channel with DFT-s-OFDM

# 3	Conclusions

In this discussion paper we presented power back-off simulation results for a UE operating in the UL range of 2000-2020 MHz, i.e. NTN band n252 UL, in presence of the existing requirements to protect band n25 DL. These results can be used for further technical discussions to decide how to introduce the NTN band n252 to the 3GPP specifications.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241661, "New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)", Dish Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"