3GPP TSG-RAN WG4 # 112-bis	R4- 2415382

Hefei, China, Oct 14 – Oct 18, 2024

Agenda item:	6.14.2

Source:	MediaTek Inc.

Title:	Discussion on Spatial Channel Modelling

Document for:	Discussion

# Introduction

The Rel-19 study item [1] aims to investigate practical spatial channel modelling methodologies for both SU-MIMO and MU-MIMO demodulation and CSI reporting requirements. It has been identified that the current TDL-based models and spatial correlation matrices poorly match with realistic propagation conditions that have been used as the design assumption for the NR downlink precoder (PMI) codebooks. We also believe that the test scenarios and channel models should match with realistic transmit strategies and channel conditions in order to correctly support UE algorithm design. When RAN4 test cases are unrealistic, the UE algorithm designers may need to compromise between real-world behavior/performance and RAN4 tests.

In our view, the problem with the existing RAN4 spatial correlation models is that they amplify the signal only in the broadside direction (AOD/AOA) of the TX and RX antenna arrays. In order to support a higher number of layers, multiple separate spatial signal directions should exist. This can be achieved by introducing spatial clusters, where each cluster has its own separable mean AOD/AOA.

The current RAN4 MIMO tests employ TDL (Tapped Delay Line) channel models with specific spatial correlation matrices. For spatial channel model development, both CDL (Clustered Delay Line) and TDL models can be considered as baseline. In this contribution, we describe how to achieve desired spatial channel properties by employing simple “multi-cluster TX-RX beam steering” on top of TDL channels.

Proposal #1: We propose RAN4 to consider the option of multi-cluster TX-RX beam steering with TDL model for spatial channel modelling.

# Discussion

# 2.1 Background: TDL channel models with MIMO correlation

In many current RAN4 demodulation tests, spatial selectivity is introduced by applying MIMO correlation matrices on top of an uncorrelated fading TDL MIMO channel. Correlation models are defined for two different antenna array assumptions: Uniform Linear Array (ULA) and Cross-polarized Antennas (X-pol).

The spatial selectivity of a MIMO channel can be depicted as power angular distribution (PAD) that represents the average channel power response as function of TX or RX direction. Figure 2-1(a) illustrates the power angular distribution (PAD) measured from an 8TX-8RX X-pol medium correlated TDL channel model (α1=0.3, β=0.6). As can be seen, the channel is spatially selective, favoring the AODs and AOAs close to zero degrees. The PAD at the TX side is slightly wider than at the RX side because the TX-correlation is lower than the RX-correlation (α < β). Similarly, Figure 2-1(b) shows the PAD measured from a single-cluster 8TX-8RX CDL channel model with mean AOA = 0 degrees. Here, the angular spread is at the TX side CASD = 1.3710 and at the RX side CASA = 15.632 degrees, and the assumed antenna element radiation patterns is flat. As can be seen, the PAD curves for the correlated TDL channel and the single-cluster CDL channel are similar.

Observation #1: The spatial selectivity pattern of a correlated TDL channel and a single-cluster CDL channel are similar.

- (b)

Figure 2-1: Spatial selectivity of X-pol correlated TDL channel (a) and single-cluster CDL channel (b).

Figure 2-2 depicts the TX-antenna correlation measured from a single-cluster CDL channel model with 16 TX antennas and mean AOA = 0 degrees for different values of angular spread of departure (ASD). Here, only the first column of the correlation matrix is shown. As can be seen, higher angular spread results in lower channel correlation between antenna elements. It can be concluded that the effect of different angular spreads can be modelled by using corresponding spatial correlation models. This suggests that spatial selectivity can be modelled well via both TDL (correlation) and CDL (angular spread) approaches.

Observation #2: Higher angular spread results in lower MIMO antenna correlation and vice versa.

Observation #3: Spatial selectivity can be modelled via both TDL (correlation) and CDL (angular spread) approaches.

Figure 2-2: MIMO correlation at TX side of a single-cluster CDL channel.

# 2.2 Background: Dual-cluster beam steering

In current RAN4 specifications, slowly time-varying (dual-cluster) beam steering ([2] TS 38.101-4: B.2.3.2.3A) is employed in some PMI reporting tests. Note that beam steering is a part of the channel model, it is not an action taken by the transmitter or the receiver.

The target of beam steering (BS) is to cover a wide range of downlink transmit beam directions so that the UE’s ability to identify optimal PMIs is properly tested. BS is currently specified for cross-polarized (X-pol) antenna arrays and spatial correlations, but it can be defined for ULA-model as well if needed. The X-pol model matches with the assumption of a 2D antenna array with two polarizations (2×N1×N2 antenna elements) at gNB. On the other hand, at the UE side the model assumes a 1D linear antenna array with two polarizations so that each polarization has Nrx/2 antenna elements.

In RAN4 test cases, each cluster typically employs “high” correlation where the correlation parameters over each linear array dimension are: α1 = α2 = β = 0.9. As a result of the real-valued spatial correlations (before beam steering) the strongest beam direction is at 0⁰ AOD/AOA angle (broadside) of each linear array dimension. The transmit directions (AOD) are further shifted by cluster-specific diagonal BS matrices.

Note that in this form, adding clusters does not increase the spatial degrees of freedom in the MIMO channel, and the optimal number of layers remains low. This is because at the UE Rx all clusters have the same mean AOA of 0⁰. Therefore, the clusters are not separable, and the supported/optimal rank is mainly restricted by the underlying X-pol correlation model.

Observation #4: The existing dual-cluster beam steering feature does not increase the spatial degrees of freedom of the MIMO channel since the clusters have the same AOA in the RX side.

In the next section, we propose to amend the BS model with RX beam steering so that the clusters become separable, and the MIMO channel can support a higher number of layers.

# 2.3 Proposal: Multi-cluster TX-RX beam steering

In this section we present a spatial channel modelling concept that re-uses and extends the idea of beam steering (BS). We define a spatial MIMO channel model as a combination of N spatial clusters, where each cluster is associated with a statistically independent time-variant TDL channel employing the X-pol correlation model. For each cluster, we define TX and RX beam steering corresponding to desired AOD/AOA angles. An example is illustrated in Figure 2-3.

Let the MIMO channel per subcarrier and time instant be:

H=n=1Npn∙Dn(rx)HnDn(tx), where

- pn is the power of cluster n so that n=1Npn=1

- Dn(rx) is the diagonal RX-BS matrix of cluster n with dimensions Nrx× Nrx

- Dn(tx) is the diagonal TX-BS matrix of cluster n with dimensions Ntx× Ntx

- Hn is the X-pol TDL MIMO channel matrix of cluster n with dimensions Nrx× Ntx

Parameters:

- Number of clusters: N

- gNB TX-array dimensions: N1 and N2 (Ntx = 2N1N2)

- Number of UE RX antennas: Nrx

- TX-phases: ϴ1(n) and ϴ2(n) per cluster n

- RX-phases: ϴrx(n) per cluster n

Beam steering vectors and matrices:

- stx1(n)=1ejθ1(n)⋯ejθ1(n)N1-1T , length N1

- stx2(n)=1ejθ2(n)⋯ejθ2(n)N2-1T, length N2

- srx(n)=1ejθrx(n)⋯ejθrx(n)Nrx/2-1T , length Nrx/2

- Dn(tx)=1001⊗diag(stx1n)⊗diag(stx2n)

- Dn(rx)=1001⊗diag(srxn)

Note that the BS matrices can be either fixed or slowly time-variant. In general, the model parameters can be chosen to match the scope of each test case. For example, the number of clusters can be selected so that the desired number of layers (rank) is supported. The BS directions can be tailored, for example:

- Let TX directions rotate over time to test transmit precoders and PMI feedback performance.

- Let TX and RX directions rotate over time to test varying levels of cross-layer interference.

- Keep TX and RX BS directions constant to test fixed rank + PMI + MCS with different MCS per CW

Optionally, we could also apply cluster-specific Doppler shifts and additional path delays if needed.

Figure 2-3: TX-RX beam steering channel with two spatial clusters.

To demonstrate the effect of adding orthogonal equally powerful TX-RX BS clusters to correlated TDL-channels, we simulated fixed ranks 4 and 8 with adaptive wideband Type I PMI feedback and with adaptive MCS. The tput results are depicted in Figure 2-4. As can be seen, two spatial clusters can well support rank 4, while rank 8 struggles if there are fewer than 4 clusters in the channel.

Observation #5: Increasing the number of orthogonal TX-RX beam steering clusters increases the effective spatial degrees of freedom of the channel.

Figure 2-4: 8TX-8RX adaptive PMI/MCS simulations for ranks 4 and 8.

# 2.4 Example: Multi-cluster TX-RX beam steering for 2-CW fixed-MCS testing

As an example, testing of two-codeword (up to 8 layers) MIMO demodulation – so that consistently different channel quality per CW is maintained – has turned out to be challenging with current RAN4 correlation models. In this section, we present an example how to construct a PDSCH demodulation test with fixed rank, fixed Type I PMI, and different fixed MCS per CW.

For fixed PMI testing, the per-cluster phase shifts ϴ1(n) and ϴ2(n) can be chosen to be fixed and to match the beams of the transmit PMI. In Type I codebook, each CW often maps to a subset of the PMI-beams. This is illustrated below for the 5-layer precoder ([3] TS 38.214: 5.2.2.2.1). Consequently, per-CW channel quality levels can be adjusted by adjusting the per-cluster powers pn.

To minimize cross-layer/CW interference at the UE, the RX-phases ϴrx(n) can be chosen so that the corresponding steering vectors are orthogonal. Alternatively, by choosing non-orthogonal RX steering vectors, cross-layer/CW interference can be increased if so desired.

Finally, the selection of the X-pol spatial correlation model parameters (α1, α2, β) determines how strictly the mean spatial directions are maintained. Higher correlation is analogous to narrower angular spread per cluster.

To demonstrate the concept, we have simulated rank 5 PDSCH in a three-cluster (N = 3) channel setup as detailed in Table 1. The results are depicted in Figure 2-5. Here, CW0 employs 16QAM while CW1 has QPSK, but as clusters corresponding to CW0 are 6dB stronger, the BLER performances of the two CWs are almost on par.

Observation #6: CW-specific TX-RX BS clusters can be used to effectively scale CW-specific demodulation and decoding performance.

Table 1: Simulation setup

| Antenna setup: N1, N2, Nrx | 2, 2, 8 |
| --- | --- |
| TX PMI: i1,1, i1,2, i2 1st dim PMI beam indexes: l, l’, l” 2nd dim PMI beam indexes: m, m’, m” | 0, 0, 0 0, O1, O1 => phase 2πl/O1N1 = 0, π, π 0, 0, O2 => phase 2πm/O2N2 = 0, 0, π |
| Cluster powers: p1, p2, p3 | 4/6, 1/6, 1/6 (beam 1 is 6dB stronger than beams 2 and 3) |
| Cluster TX BS in 1st array dim: ϴ1(1), ϴ1(2), ϴ1(3) Cluster TX BS in 2nd array dim: ϴ2(1), ϴ2(2), ϴ2(3) | 0, -π, -π 0, 0, -π |
| Cluster RX BS: ϴrx(1), ϴrx(2), ϴrx(3) | 0, π/2, π |
| TDL channel model per cluster | TDLA-30 / 10Hz Doppler / X-pol high correlation |
| MCS / CW0 MCS / CW1 | MCS13, 16-QAM, code rate 0.48 MCS7, QPSK, code rate 0.51 |

Figure 2-5: 2-CW fixed MCS case in three-cluster TDL-A channel.

# 2.5 Comparison of TDL and CDL approaches

In this section, we compare the TDL and CDL approaches. There are clear similarities between the CDL model and the TDL model extended by multi-cluster TX-RX beam-steering, as both approaches employ a number of clusters that each have their own mean relative power and average AOA/AOD. However, many differences can be identified between the two. Summary of the comparison is given in Table 2.

The TDL approach builds on top of the existing RAN4 channel models. New models can be simply configured by setting/choosing a handful of new high-level parameters, and the parameters can be chosen to achieve desired channel properties and to match the scope of different performance tests. Since TDL-models are already well tested within RAN4 and by TE-vendors, the behavior/performance of new TDL-based models should be easily aligned between different companies. Moreover, the spatial properties of the clusters (in terms of AOA/AOD) could be defined to be time-varying to further improve the test coverage.

Observation #7: The parameters of the multi-cluster TX-RX beam-steering channel model can be selected to create desired channel properties, and to match the scope of any MIMO performance test.

The CDL models have not previously been employed in RAN4 conducted demodulation tests. Each CDL channel model consists of a large set of pre-defined low-level spatial parameters such as cluster-specific powers, mean AOA/AOD, and angular spreads. The performance in CDL channels has earlier turned out to be difficult to align between different companies. Even though the modelling principle is realistic, in terms of spatial properties, each CDL model is like a single random snapshot from the environment. This is because the mean AOAs/AODs are pre-defined constants with no support for a slowly time-varying component. Furthermore, the spatial richness (and therefore the optimal number of layers) of the model is not easily adjustable. To design a new RAN4 MIMO test case with some desired scope, a match between a suitable channel model and test setup can only be found via trial and error, and still there is no guarantee of finding a good match.

Observation #8: Spatial properties of CDL channel models are pre-defined and fixed, and there is no guarantee of finding suitable models for all MIMO performance tests.

Proposal #2: We propose spatial properties of channel model to be configurable to guarantee finding of suitable models for all MIMO performance tests.

It can be argued that the CDL models are more realistic and better resemble the phenomena of physical environments. In particular, in CDL models, each cluster has its own path delay and Doppler shift, resulting from the relation of AOA and UE direction of movement. However, if deemed necessary for the test purposes, such per-cluster delays and additional Doppler shifts can be introduced in the TDL approach as well.

Table 2: Comparison of advanced channel modelling approaches

|  | CDL | TDL with Multi-cluster TX-RX-BS |
| --- | --- | --- |
| RAN4 scope | New for conducted RAN4 demodulation tests | Combines and extends existing RAN4 models used in demodulation tests |
| Spatial properties | Low- and high-level properties pre-defined and fixed | High-level properties can be selected as desired |
| Test creation | Trial-and-error to match test with existing channel models, or define new channel models with desired properties | Choose a handful of high-level channel parameters to match tested feature |
| Performance alignment | Results difficult to align between companies | Should be easy to align between companies |
| Implementation | Complex to implement | Simple to implement on top of TDL |
| AOD/AOA time-domain variation | Time-domain variation not supported by current link-level models | Time-domain variation of cluster directions can be specified similarly as in existing BS functionality |

# 2.6 Discussion of CDL-C UMa FR1 channel model

In this section, we share some observations about implementing and simulating the CDL-C UMa FR1 channel model with fixed TX-beamforming as described in TR 38.827 Section 7.3, and with the agreed cross-polarized RX-antenna array geometry. We have noticed that implementing the CDL model is not trivial as the many details of the model are scattered between multiple TRs and their many sections. It will be interesting to see how well the results align between companies.

One of our observations is that due to the TX-beamforming definition included in the channel model, the average channel power response does not necessarily converge to one (while for other RAN4 channel models it does). Therefore, the definition of SNR must be agreed on to align the results. We have defined SNR in our results by measuring the average power of the 2x8, 4x8, 8x8 channels over the simulation, and by adding this result to SNR in dB.

Observation #9: Due to the integrated TX-beamforming feature, the average power response of the CDL-C UMa FR1 (as specified in TR 38.827) is not equal to one.

Another observation is that the CDL-C model includes subclusters for which the propagation delays and powers differ, but the TX-RX (departure/arrival) angles are identical as shown in Figure 2-6. These subclusters are highlighted in the TR-snippet below. Due to identical ray-specific incident angles, the ray-specific Doppler shifts for the subclusters become identical as well. Thus, there is a risk that the fading processes between the sub-clusters becomes correlated. This correlation depends on the ray-specific random initial complex phases. Correlation between subclusters can result in uneven average power response in frequency domain, which does not disappear even by averaging the fading over infinite time.

Observation #10: There is a risk that the fading processes between the sub-clusters become correlated, depending on their random initialization.

Proposal #3: RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it.

Furthermore, we see that focusing to this single CDL channel model offers limited opportunities to support a wider range of tests for demodulation and CSI feedback. Modifying this channel model for other test cases and purposes appears difficult. A more flexible way of defining new channel model variants would be desirable.

Finally, it is not clear whether this TX-beamforming model (with multiple parallel beamformers), potentially combined with outer (TypeI or eTypeII) precoding constitutes a realistic transmission strategy that could take place in live networks. It would be valuable if the network side companies could clarify or comment on this.

Proposal #4: Network vendors in RAN4 should clarify whether the TX-beamforming method (with multiple beamformers), potentially combined with outer (TypeI or eTypeII) precoding constitutes a realistic transmission strategy that could take place in live networks.

Figure 2-6: Channel model parameters for UMa CDL-C at 3.5 GHz.

# Conclusion

In this paper we provided the view on the spatial channel modelling. The following observations and proposals are made:

Proposal #1: We propose RAN4 to consider the option of multi-cluster TX-RX beam steering with TDL model for spatial channel modelling.

Observation #1: The spatial selectivity pattern of a correlated TDL channel and a single-cluster CDL channel are similar.

Observation #2: Higher angular spread results in lower MIMO antenna correlation and vice versa.

Observation #3: Spatial selectivity can be modelled via both TDL (correlation) and CDL (angular spread) approaches.

Observation #4: The existing dual-cluster beam steering feature does not increase the spatial degrees of freedom of the MIMO channel since the clusters have the same AOA in the RX side.

Observation #5: Increasing the number of orthogonal TX-RX beam steering clusters increases the effective spatial degrees of freedom of the channel.

Observation #6: CW-specific TX-RX BS clusters can be used to effectively scale CW-specific demodulation and decoding performance.

Observation #7: The parameters of the multi-cluster TX-RX beam-steering channel model can be selected to create desired channel properties, and to match the scope of any MIMO performance test.

Observation #8: Spatial properties of CDL channel models are pre-defined and fixed, and there is no guarantee of finding suitable models for all MIMO performance tests.

Proposal #2: We propose spatial properties of channel model to be configurable to guarantee finding of suitable models for all MIMO performance tests.

Observation #9: Due to the integrated TX-beamforming feature, the average power response of the CDL-C UMa FR1 (as specified in TR 38.827) is not equal to one.

Observation #10: There is a risk that the fading processes between the sub-clusters become correlated, depending on their random initialization.

Proposal #3: RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it.

Proposal #4: Network vendors in RAN4 should clarify whether the TX-beamforming method (with multiple beamformers), potentially combined with outer (TypeI or eTypeII) precoding constitutes a realistic transmission strategy that could take place in live networks.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- 3GPP TS 38.101-4 V18.5.0 (2024-09), “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)”

- 3GPP TS 38.214 V18.4.0 (2024-09), “Physical layer procedures for data (Release 18)”